3GPP TSG-RAN WG4 Meeting #113	R4-2419741

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.23.4.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM core requirements for LP-WUS

Document for:	Discussion

# Introduction

The RRM core requirements for Rel-19 LP-WUS/WUR was further discussed in the last RAN4 meeting, and the achieved agreements are captured in the WF [1]. In this paper, we continue the discussion on the open issues in the RRM core part and provide our view. The WID [2] is also provided as below for reference.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

# Discussion

## Cases/states of RRM relaxation for LP-WUS capable UE

In the last meeting, RAN4 further discussed the possible RRM offloading/relaxation cases for LP-WUS capable UE to be considered in Rel-19 LP-WUS. In RAN4#111 meeting, RAN4 agreed on Case#1 (RRM offloading case) first and then in RAN4#112 meeting, RAN4 agreed on Case#3 (RRM relaxation case) as captured below.

RAN4#111 agreement: Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. RAN4#112 agreement Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Regarding whether to discuss other cases for LP-WUS, in our view the current agreed cases in RAN4, Case#1 and Case#3, are sufficient to consider both scenarios of MR RRM offloading and MR RRM relaxation of serving and neighbour cell measurements, which align with the WID objective in [2]. Therefore, RAN4 should focus on defining the core requirements for the current agreed cases, and if some issues are detected later to consider other cases, it can be revisited.

Proposal 1: No need to discuss further cases for LP-WUS in RAN4.

## Entry/Exit conditions

Following the progress in RAN4, LP-WUS capable UE may operate under three scenarios (see Figure 1):

- RRM offloading (Case#1): when MR is deactivated and LR is activated.

- RRM relaxation (Case#3): when both MR and LR are activated.

- Legacy UE: when MR is activated and LR is deactivated.

Figure 1: Different cases of LP-WUR UE

In Case#1, MR is deactivated and LR is activated. LR should perform wake-up signal monitoring and serving cell measurements in RRC IDLE/INACTIVE mode. All other tasks performed by MR (paging, serving and neighbour cell RRM measurements) are deactivated for power saving. Since in this case serving cell measurement is exclusively performed by LR, UE needs to rely on the LR’s RRM measurements to determine whether to stay in the current state or switch to another state. In this case, UE can either exit to legacy UE behaviour or enter Case#3 as shown in Figure 2. For both scenarios, UE can evaluate the conditions (to exit from Case#1 or to enter Case#3) based on LR measurements only and with respect to the configured thresholds.

Figure 3: UE at Case#3.

Figure 2: UE at Case#1.

In Case#3, both MR and LR are activated. LR performs wake-up signal monitoring and serving cell measurements in RRC IDLE/INACTIVE mode. MR performs relaxed serving and neighbour cell measurements in IDLE/INACTIVE mode. Since, in this case, serving cell measurements are performed by both MR and LR, in the last meeting RAN4 agreed not to consider combining the measurements across the two radios, i.e., MR and LR as captured in the WF [1]. This means RAN4 need to discuss which measurements to consider when transiting between different states. When UE at Case#3, UE can either exit to legacy UE behaviour or enter Case#1 as shown in Figure 3. For both scenarios, LR or MR are capable to evaluate the conditions (to exit from Case#3 or to enter Case#1) based on the serving cell measurements and with respect to the configured thresholds. It is noted that, RAN2 agreed that exit condition is based on LR measurements only without MR measurements as captured in [4].

RAN2#127-bis agreement: - The LPWUS monitoring exit condition does not include MR measurements.

Therefore, for both cases (Case#1 and Case#3) RAN4 can define same evaluation requirements for exit conditions based on LR measurements only. On the other hand, the entry conditions when switching between Case#1 and Case#3 can be different. Entry from Case#1 to Case#3 can be based on LR only, but entry from Case#3 to Case#1 can be based on LR or LR+MR. Therefore, in our view there is no need to complicate the requirements when switching between Case#1 and Case#3 and RAN4 can define the same requirements for both based on LR measurements only (as it is the common between both cases).

Proposal 2: Based on LR measurements only, RAN4 to define same requirements to evaluate the exit condition for each: 1) exiting from Case#1 to legacy UE and 2) exiting from Case#3 to legacy UE.

Proposal 3: RAN4 to use the same evaluation requirements for entry condition when switching from Case#1 to Case#3 and from Case#3 to Case#1, where

- entry conditions from Case#1 to Case#3 can be based on LR only.

- entry conditions from Case#3 to Case#1 can be based on LR or LR+MR.

Proposal 4: RAN4 to consider LR measurements only to define same requirements to evaluate the entry condition for each: 1) switching from Case#1 to Case#3 and 2) switching from Case#3 to Case#1.

On the other hand, when UE operates as legacy (only MR), evaluating the entry condition to enter Case#1 or Case#3 should be based on MR measurements only and with respect to the configured thresholds.

Proposal 5: RAN4 to define requirements to evaluate the entry condition from legacy UE to Case#1 or Case#3 based on MR measurements only.

Another issue needs to be discussed is that how to avoid the ping pong behaviour when transitioning between different cases. Activating and deactivating MR can consume more power and takes longer time compared to LR. Therefore, it is important to ensure UE can confidently switch to any case without having ping pong impact.

Proposal 6: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Regarding the criteria (entry/exit conditions) for RRM fully offloading and MR RRM measurement relaxation. It is also our understanding that RAN2 is responsible for defining these criteria while RAN4 needs to define delay requirement to evaluate these criteria. In fact, RAN2 has already discussed the criteria and reached the following agreement [4]:

RAN2#127-bis agreement: - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Proposal 7: The criteria (entry/exit conditions) for RRM fully offloading and MR RRM measurement relaxation is up to RAN2, but the delay requirement to evaluate these criteria is RAN4 work.

## MR RRM relaxation

The MR RRM relaxation can be achieved by using a scaling factor for the RRM measurements performed in IDLE/INACTIVE mode. It is noted that different scaling factors can offer different power saving gain for the UE. RAN4 can discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

There is a trade-off between power saving gain and the delay. The higher the scaling factor, the greater the power savings that can be achieved, and the longer the delay will be (also vice versa). In the last meeting, some companies supported option 1 and others supported option 2. RAN4, however, can consider multiple stages of RRM relaxation with different scaling factors (i.e., not to limit the relaxation on a single value), which can depend on some conditions (e.g., channel condition, UE mobility, UE location). For example, MR RRM can be relaxed with a larger value when some conditions are met (e.g., UE closer to cell centre) otherwise a smaller value can be used. Note that this relaxation will only apply for Case#3 (when both MR and LR are on).

Proposal 8: RAN4 to discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

Proposal 9: RAN4 to consider multiple stages of RRM relaxation, with different scaling factors (e.g., 8 in 1st stage and 16 in 2nd stage) depending on some conditions (e.g., channel condition, UE mobility).

On the other hand, when UE operates in Case#3 (MR and LR are on), the relaxation factor applied for MR serving cell measurement can also be applied for MR neighbour cell measurements.

Proposal 10: Same relaxation factor applies to serving and neighbour cell measurements.

## Others

Some other issues were also discussed in RAN4 as captured in the WF [1] below:

Issue 1-2-1: Accuracy requirements Issue 1-2-4: On LP-WUR cell selection evaluation requirements for case 1 Issue 1-4-1: LP-WUR at CONNECTED mode

For Issue 1-2-1, we support that not to capture dedicated accuracy requirement in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 11: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

For Issue 1-2-4, since in Case#1 only LR is activated in idle/inactive mode, RAN4 need to discuss whether to define cell selection requirements (evaluate S criteria) based on LR measurements. The issue will depend on the threshold configuration for the LR exit condition. If LR exit threshold is higher S criteria threshold then LR will exit Case#1 anyway even when the measurement is higher than S criteria threshold. Then MR can meet cell selection requirement. This also apply for Case 3 as well.

Proposal 12: No need to define the LR based cell selection criterion evaluation in Case#1 and Case#3.

For issue 1-4-1, we also agree that no RRM objectives is expected for connected mode according to the WID.

Proposal 13: No RRM objectives is expected for connected mode in this WI.

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: No need to discuss further cases for LP-WUS in RAN4.

Proposal 2: Based on LR measurements only, RAN4 to define same requirements to evaluate the exit condition for each: 1) exiting from Case#1 to legacy UE and 2) exiting from Case#3 to legacy UE.

Proposal 3: RAN4 to use the same evaluation requirements for entry condition when switching from Case#1 to Case#3 and from Case#3 to Case#1, where

- entry conditions from Case#1 to Case#3 can be based on LR only.

- entry conditions from Case#3 to Case#1 can be based on LR or LR+MR.

Proposal 4: RAN4 to consider LR measurements only to define same requirements to evaluate the entry condition for each: 1) switching from Case#1 to Case#3 and 2) switching from Case#3 to Case#1.

Proposal 5: RAN4 to define requirements to evaluate the entry condition from legacy UE to Case#1 or Case#3 based on MR measurements only.

Proposal 6: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Proposal 7: The criteria (entry/exit conditions) for RRM fully offloading and MR RRM measurement relaxation is up to RAN2, but the delay requirement to evaluate these criteria is RAN4 work.

Proposal 8: RAN4 to discuss the following options for RRM relaxation:

- Option 1: MR RRM Relaxation from 8 to 16 times

- Option 2: MR RRM Relaxation >=16 times

Proposal 9: RAN4 to consider multiple stages of RRM relaxation, with different scaling factors (e.g., 8 in 1st stage and 16 in 2nd stage) depending on some conditions (e.g., channel condition, UE mobility).

Proposal 10: Same relaxation factor applies to serving and neighbour cell measurements.

Proposal 11: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 12: No need to define the LR based cell selection criterion evaluation in Case#1 and Case#3.

Proposal 13: No RRM objectives is expected for connected mode in this WI.

# References

- R4-2416861, WF on RRM requirements for NR_LPWUS, vivo.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO.

- 3GPP TR 38.869, “Study on low-power wake-up signal and receiver for NR (Release 18)”, V18.0.0.

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD, CATT