3GPP TSG-RAN WG4 Meeting #113	R4-2419742

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.15.2.1

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The objective for R19 NR RRM Phase-5 work item was updated in the WID as captured below [1]. In this paper, we continue the discussion on the first objective, particularly on optimizing Rx beam sweeping factor for L3 measurement delay reduction when UE support multiple-Rx simultaneous reception.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In R18, the feature of multiple-Rx simultaneous reception was introduced to enhance the RRM requirements (L1-RSRP measurement delay, RLM, BFD/CBD etc.) for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier in FR2-1. In R19, similar enhancement is brought to L3 measurements (SSB based) in FR2-1. For UE capable of multiple-Rx, beam sweeping can be optimized and performed via multiple Rx panels simultaneously, which in turn will optimize/reduce L3 measurement delay in FR2-1.

It is clear that L3 measurement is widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay. This could impact UE power consumption negatively. Therefore, activating/deactivating multi-Rx (for fast beam sweeping in L3 measurements) shall depend on the scenario and when the corresponding conditions apply.

Observation 1: L3 measurement is widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay, whereas UE power consumption can be significant.

In the last meeting, RAN4 discussed the conditions to apply the enhancement of FR2-1 SSB based L3 measurement delay reduction. Some companies proposed to reuse the same conditions defined in R18 multi-Rx simultaneous reception WI for R19 L3 measurement delay reduction when UE support multi-Rx. However, if the same conditions specified in Rel-18 multi-Rx are applied, UE multi-Rx would be always activated for any L3 measurements whenever GBBR is configured, regardless which RRM scenario or UE current conditions.

Observation 2: If the same conditions specified in Rel-18 multi-Rx are applied to R19 L3 measurement delay reduction, UE multi-Rx would be always activated for any L3 measurements whenever GBBR is configured (i.e., regardless which RRM scenario or UE current conditions).

In our view, these conditions (i.e., for R18 multi-Rx) are not applicable to R19 L3 measurement enhancement. This was also agreed in RAN4#112 meeting as captured in [2].

Issue 1-2-1: FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature Agreement in online session: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception.

In the last meeting RAN4 also made the following agreement as captured in the WF [3].

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF: Agreement in online session: For scenario 1/2/3/4: - RAN4 to consider conditions for triggering FBS for L3 measurement, including active condition and de-active conditions. - (De-)Activating multi-Rx for L3 measurements (intra/inter-frequency) is depending on the criteria/threshold(s) and UE current conditions. - The detailed criteria/condition(s) is FFS. - Meanwhile, RAN4 agree that the criteria/threshold(s) are configured by RRC signaling. FFS the further details. For Scenario 5/7: - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch). - For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Regarding the FFS part, RAN4 needs to discuss these conditions taking into account the power consumption resulting from activating multi-Rx for L3 measurements. Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

Proposal 1: Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

RAN4 also discussed whether to consider HST in this WI (i.e., high mobility). It is known that HST feature is discussed under a separate WI and therefore it can be precluded from the discussion on FR2-1 SSB based L3 measurement delay reduction. However, low/medium speed mobility of UEs should be considered as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

Proposal 2: On UE mobility status, RAN4 to consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

UE power consumption due to activating multiple panels for simultaneous DL reception is an important aspect to discuss and consider in RAN4. If UE fulfills the conditions to enable FR2-1 SSB based L3 measurement delay reduction by activating multi-Rx, UE should also have the capability to indicate to the NW whenever UE needs to deactivate multi-Rx, for example, due to overheating resulting from activating multiple panels for simultaneous reception for long time.

Proposal 3: RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

In the last meeting RAN4 agreed to apply the enhancement of using multi-Rx for L3 measurement delay reduction on the following scenarios: 1) Intra-frequency measurement with/without gap 2) Inter-frequency measurement with/without gap 3) Handover, and 4) RRC Re-establishment/RRC Connection Release with Redirection. Therefore, RAN4 need to discuss how to apply this enhancement on the agreed scenarios. A smaller RX beam sweeping factor X can be used for aforementioned measurements when UE support R19 multi-Rx for L3 measurement delay reduction, where X can be FFS.

Proposal 4: On the solutions to apply L3 measurement delay reduction, RAN4 to discuss a smaller RX beam sweeping factor X for the agreed scenarios in the last meeting. X can be FFS.

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: L3 measurement is widely used in the specs for different RRM purposes/procedures, thus it is not expected to activate multi-Rx all the time for those procedures even when it is not necessary to enhance L3 measurements delay, whereas UE power consumption can be significant.

Observation 2: If the same conditions specified in Rel-18 multi-Rx are applied to R19 L3 measurement delay reduction, UE multi-Rx would be always activated for any L3 measurements whenever GBBR is configured (i.e., regardless which RRM scenario or UE current conditions).

Proposal 1: Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions:

- UE location (cell centre or cell edge)

- UE mobility (stationary or moving)

- Both above

Proposal 2: On UE mobility status, RAN4 to consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction.

Proposal 3: RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

Proposal 4: On the solutions to apply L3 measurement delay reduction, RAN4 to discuss a smaller RX beam sweeping factor X for the agreed scenarios in the last meeting. X can be FFS.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple.

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.