3GPP TSG-RAN WG4 Meeting #113	R4-2419771

Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia

Title:	Simulation results for PRS Bandwidth Aggregation

Agenda item:	5.22.2.5

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including PRS/SRS bandwidth aggregation:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

PRS/SRS bandwidth aggregation has been studied in the FS_NR_pos_enh2 study item by RAN4 with company contributions and outcome of the study being captured in TR 38.859 [2].  Based on Rel-16 and Rel-17 link level simulation assumptions for RSTD and PRS-RSRP, respectively, in [3] and [4], RAN4 has agreed link level simulation assumptions for PRS bandwidth aggregation in [5].

We have provided initial preliminary simulation results on PRS RSTD with bandwidth aggregation in [6] and [7].  In this contribution we provide further updated simulation results for PRS RSTD and UE Rx-Tx time difference versus [10].

Simulation Assumptions

Updated simulation assumptions for PRS RSTD are captured in [8]. The general parameters and PRS configurations as listed in Table-1 and Table-2 of [8] are used in the simulation study. No RF/BB impairments are included in the simulation.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the PRS RSTD errors in Tc, and 90-percentile CDF of the UE Rx-Tx time difference errors in Tc for given Es/Iot for three cells [5]. Limiting performance is indicated in yellow marked cells.

FR1, AWGN Channel

RSTD results

The PRS RSTD error in Tc for AWGN channel in FR1, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells are listed in Table 1, Table 2, and Table 3.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -16 | 16 | -16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 4 | -2 | 2 | -2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |

Table 1: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -16 | 16 | -16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 4 | -2 | 2 | -2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |

Table 2: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-3,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | -32 | 32 | -32 | 32 |
|  |  |  |  |  |  | 3 | -32 | 32 | -32 | 32 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 272 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 132 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |

Table 3: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for AWGN channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 4, Table 5, and Table 6.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 16 | 16 | 16 |
|  |  |  |  |  |  | 3 | 16 | 16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | 4 | 4 | 4 |
|  |  |  |  |  |  | 4 | 2 | 2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |

Table 4: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 16 | 16 | 16 |
|  |  |  |  |  |  | 3 | 16 | 16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | 4 | 4 | 4 |
|  |  |  |  |  |  | 4 | 2 | 2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |

Table 5: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-3,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | 16 | 16 | 16 |
|  |  |  |  |  |  | 3 | 16 | 16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 272 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |

Table 6: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

FR1, TDL-A Channel

RSTD results

The PRS RSTD error in Tc for TDL-A channel in FR1, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells are listed in Table 7, Table 8, and Table 9.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -9 | 10 | -11 | 9 |
|  |  |  |  |  |  | 3 | -16 | 15 | -19 | 17 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -16 | 15 | -15 | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -7 | 7 |
|  |  | 272 | 16 | 1 | 4 | 2 | -7 | 7 | -7 | 6 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -16 | 14 | -16 | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -10 | 7 |
|  |  | 132 | 16 | 1 | 4 | 2 | -7 | 7 | -7 | 7 |
|  |  |  |  |  |  | 3 | -6 | 5 | -6 | 6 |

Table 7: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -9 | 10 | -11 | 9 |
|  |  |  |  |  |  | 3 | -16 | 15 | -18 | 17 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -16 | 15 | -15 | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -7 | 7 |
|  |  | 272 | 16 | 1 | 4 | 2 | -7 | 7 | -7 | 6 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -17 | 14 | -17 | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -10 | 7 |
|  |  | 132 | 16 | 1 | 4 | 2 | -7 | 7 | -8 | 7 |
|  |  |  |  |  |  | 3 | -6 | 5 | -6 | 6 |

Table 8: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | -9 | 9 | -9 | 9 |
|  |  |  |  |  |  | 3 | -15 | 16 | -16 | 17 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | -17 | 15 | -16 | 16 |
|  |  |  |  |  |  | 3 | -7 | 7 | -7 | 7 |
|  |  | 272 | 16 | 1 | 2 | 2 | -7 | 7 | -7 | 7 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | -17 | 15 | -16 | 16 |
|  |  |  |  |  |  | 3 | -7 | 7 | -8 | 7 |
|  |  | 132 | 16 | 1 | 2 | 2 | -7 | 7 | -7 | 7 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |

Table 9: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for TDL-A channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 10, Table 11, and Table 12.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 36 | 37 | 37 |
|  |  |  |  |  |  | 3 | 41 | 42 | 43 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 30 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 272 | 16 | 1 | 4 | 2 | 30 | 30 | 29 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 30 | 29 | 35 |
|  |  |  |  |  |  | 3 | 30 | 30 | 31 |
|  |  | 132 | 16 | 1 | 4 | 2 | 29 | 29 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |

Table 10: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-6,-13,-13).

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 36 | 37 | 37 |
|  |  |  |  |  |  | 3 | 41 | 42 | 43 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 30 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 272 | 16 | 1 | 4 | 2 | 30 | 30 | 29 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 30 | 29 | 35 |
|  |  |  |  |  |  | 3 | 30 | 30 | 31 |
|  |  | 132 | 16 | 1 | 4 | 2 | 29 | 29 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |

Table 11: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | 36 | 37 | 36 |
|  |  |  |  |  |  | 3 | 41 | 42 | 42 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | 31 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 272 | 16 | 1 | 2 | 2 | 30 | 30 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 27 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | 31 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 132 | 16 | 1 | 2 | 2 | 29 | 30 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 27 |

Table 12: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

FR2, AWGN channel

RSTD results

The PRS RSTD error in Tc for AWGN channel in FR2, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells, respective, are listed in Table 13, Table 14, and Table 15.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | -2 | 2 | -2 | 2 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 13: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | -2 | 2 | -2 | 2 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 14: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 132 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 2 |
|  |  | 128 | 8 | 1 | 2 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |

Table 15: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for AWGN channel in FR2, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 16, Table 17, and Table 18.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | 2 | 2 | 2 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 16: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | 2 | 2 | 2 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 17: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
|  |  | 128 | 8 | 1 | 2 | 2 | 2 | 2 | 2 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 18: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

FR2, TDL-C channel

RSTD results

The PRS RSTD error in Tc for TDL-C channel in FR2, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells, respective, are listed Table 19, Table 20, and Table 21.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -38 | 29 | -38 | 28 |
|  |  |  |  |  |  | 3 | -34 | 32 | -45 | 30 |
|  |  | 132 | 16 | 1 | 4 | 2 | -34 | 30 | -39 | 31 |
|  |  |  |  |  |  | 3 | -31 | 28 | -32 | 27 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -47 | 37 | -47 | 23 |
|  |  |  |  |  |  | 3 | -39 | 38 | -39 | 33 |
|  |  | 128 | 8 | 1 | 4 | 2 | -38 | 38 | - 38 | 33 |
|  |  |  |  |  |  | 3 | -39 | 38 | -44 | 38 |

Table 19: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -38 | 28 | -38 | 28 |
|  |  |  |  |  |  | 3 | -34 | 31 | -45 | 29 |
|  |  | 132 | 16 | 1 | 4 | 2 | -34 | 29 | -40 | 30 |
|  |  |  |  |  |  | 3 | -31 | 28 | -32 | 27 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -49 | 36 | -49 | 23 |
|  |  |  |  |  |  | 3 | -39 | 38 | -39 | 32 |
|  |  | 128 | 8 | 1 | 4 | 2 | -38 | 38 | -38 | 33 |
|  |  |  |  |  |  | 3 | -39 | 38 | -44 | 38 |

Table 20: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | -48 | 48 | -49 | 49 |
|  |  |  |  |  |  | 3 | -51 | 50 | -51 | 50 |
|  |  | 132 | 16 | 1 | 2 | 2 | -51 | 50 | -51 | 51 |
|  |  |  |  |  |  | 3 | -49 | 49 | -49 | 39 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | -53 | 53 | -54 | 52 |
|  |  |  |  |  |  | 3 | -52 | 51 | -51 | 51 |
|  |  | 128 | 8 | 1 | 2 | 2 | -51 | 51 | -51 | 51 |
|  |  |  |  |  |  | 3 | -52 | 52 | -52 | 51 |

Table 21: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for TDL-C channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 22, Table 23, and Table 24.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 82 | 86 | 86 |
|  |  |  |  |  |  | 3 | 38 | 38 | 39 |
|  |  | 132 | 16 | 1 | 4 | 2 | 33 | 33 | 38 |
|  |  |  |  |  |  | 3 | 24 | 24 | 24 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 33 | 39 | 39 |
|  |  |  |  |  |  | 3 | 35 | 35 | 35 |
|  |  | 128 | 8 | 1 | 4 | 2 | 29 | 24 | 29 |
|  |  |  |  |  |  | 3 | 50 | 50 | 50 |

Table 22: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 82 | 86 | 86 |
|  |  |  |  |  |  | 3 | 38 | 38 | 39 |
|  |  | 132 | 16 | 1 | 4 | 2 | 33 | 33 | 38 |
|  |  |  |  |  |  | 3 | 24 | 24 | 24 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 33 | 39 | 39 |
|  |  |  |  |  |  | 3 | 35 | 35 | 35 |
|  |  | 128 | 8 | 1 | 4 | 2 | 29 | 24 | 29 |
|  |  |  |  |  |  | 3 | 50 | 50 | 50 |

Table 23: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | 83 | 83 | 83 |
|  |  |  |  |  |  | 3 | 33 | 44 | 44 |
|  |  | 132 | 16 | 1 | 2 | 2 | 33 | 33 | 33 |
|  |  |  |  |  |  | 3 | 24 | 25 | 24 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | 33 | 33 | 33 |
|  |  |  |  |  |  | 3 | 48 | 48 | 48 |
|  |  | 128 | 8 | 1 | 2 | 2 | 48 | 48 | 48 |
|  |  |  |  |  |  | 3 | 63 | 63 | 63 |

Table 24: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

Discussion

From the PRS BW aggregated RSTD and UE Rx-Tx time difference simulation results reported above, we observed:

- in general, increasing the PRS BW by aggregating PFLs, improves the accuracy of the positioning measurements.

- accuracy of RSTD measurement increases with the increase in the number of measurement samples (i.e., using 4 measurement samples rather than 2 measurement samples) for TDL channels.

- in general Es/IoT (-6, -13, -13) dB and (-3, -13, -13) dB yields the same results for the current simulation setup. These results imply that Es/IoT (-6, -13, -13) dB and (-3, -13, -13) dB both detect the same first path.

Thus, following observations are made:

PRS BW aggregation improves the measurement accuracy.

The RSTD/UE Rx-Tx time difference measurement accuracy improves with the increase in the number of measurement samples.

Following proposal is made.

Include simulation results in section 3 in yellow marked cells (absolute numbers) into the simulation summary for PRS BW aggregation.

Conclusion

In this contribution we have provided revised simulation results for PRS RSTD error and also UE Rx-Tx time difference error without RF/BB impairments for PRS BW aggregation based on simulation assumptions in [8].

Following observations are made.

- PRS BW aggregation improves the measurement accuracy.

- The RSTD/UE Rx-Tx time difference measurement accuracy improves with the increase in number of measurement samples.

- Following proposal is made.

- Include simulation results in section 3 in yellow marked cells (absolute numbers) into the simulation summary for PRS BW aggregation.

- References

| [1] | RP-233382, “Revised WID on Expanded and Improved NR Positioning,” 3GPP TSG RAN#102. |
| --- | --- |
| [2] | T. 38.859, “Study on Expanded and Improved NR Positioning (Release 18),” V18.0.0. |
| [3] | R4-2002287, “Updated link level simulation assumption for RSTD and RSRP,” Huawei. |
| [4] | R4-2211061, “Simulation assumption for PRS-RSRPP,” Huawei. |
| [5] | R4-2321461, “Simulation assumptions for PRS measurement accuracy for PRS/SRS BW aggregation,” Huawei, HiSilicon. |
| [6] | R4-2401874, “Simulation results for PRS Bandwidth Aggregation,” Nokia. |
| [7] | R4-2405886, “Simulation results for PRS Bandwidth Aggregation,” Nokia. |
| [8] | R4-2405673, “Simulation assumptions for PRS measurement accuracy for PRS BW aggregation,” Qualcomm Incorporated. |

[9] R4-2409165, “Simulation results for PRS Bandwidth Aggregation,” Nokia

[10] R4-2416434, “Simulation results for PRS Bandwidth Aggregation,” Nokia