3GPP TSG-RAN WG4 Meeting #113                                                               R4-2419907

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.20.4

Source:	Samsung

Title:	Way Forward for [113][306] NR_duplex_evo_General

Document for:	Approval

# Introduction

This document is provided for capturing agreements and way forward on the thread [306] NR_duplex_evo_general based on the discussion in RAN4#113.

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

### Sub-topic 1-1: SBFD applicability in NR band list

##### Issue 1-1-1: SBFD operation support in NR-U bands

Agreement:

- SBFD operation shall not be applied in NR-U bands, i.e., n46, n96 and n102.

##### Issue 1-1-3: Specification implementation of operating bands support for SBFD

Agreement:

- The following TP is adopted for clause 5.2 in TS 38.104

  - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

- FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

##### Issue 1-1-4: Specification implementation of BS channel bandwidth support for SBFD

Agreement:

- The following note is introduced in Table 5.3.2-1 and Table 5.3.5-1 in TS 38.104:

  - In the bands applicable to SBFD operation, the channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise.

### Sub-topic 1-2: Transmission bandwidth configuration for SBFD

##### Issue 1-2-2: Restriction to use the same SCS for DL and UL subbands

Conclusion:

- The restriction of SCS values for UL and DL subbands:

  - It is RAN4 common understanding the same SCS value should be used for UL and DL subbands

  - The restriction if any should be given in RAN2 spec, rather than in RAN4 spec.

##### Issue 1-2-3: Symbols for SBFD DUD configuration

Agreement:

- Update the symbols as follows:

  - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

  - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

  - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU, UD and DUD case, expressed in resource blocks.

##### Issue 1-2-4: Illustrative figures for transmission bandwidth configuration for SBFD

Agreement:

- The following figures are adopted in RAN4 requirement to illustrate transmission bandwidth configuration for SBFD:

### Sub-topic 1-3: DL/UL subband configurations support for SBFD

##### Issue 1-3-1: Clarification on declaration and flexibility

Agreement:

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

  - FFS on how to capture this in the specification.

  - This restriction shall at least be applied to FR1 WA BS

  - FFS other BS classes

    - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations.

      - FFS how to capture this in the specifications

##### Issue 1-3-2: DL/UL subband configurations support for SBFD

Agreement:

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

    - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

  - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

    - FFS different sizes for specific UL subband for DUD and DU/UD.

    - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth.

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

##### Issue 1-3-6: Other restriction on DL subband size

Way Forward:

- FFS the following restriction on DL subband size:

  - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

  - FR2 minimum CBW to support SBFD is 100MHz.

### Sub-topic 1-4: Other general issues for SBFD requirements

##### Issue 1-4-1: BS RF Specification structure for SBFD requirements

Agreement:

- Draft CR’s endorsed against 38.104 does not imply that a new specification could not be agreed.

# Topic #2: PUSCH with resource muting for CLI handling

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way Forward:

- FFS the RF requirement impact, if any, due to UL resource muting introduced in RAN1.

# Topic #3: Potentially new BS RF requirements for SBFD

### Sub-topic 3-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 3-1-1: Transient period requirement applicability on frequency/antenna

Agreement:

- RAN4 shall not define an SBFD-architecture dependent requirement.

- Transient period between SBFD and non-SBFD is applicable to all four cases.

##### Issue 3-1-2: Location of transient period between SBFD and non-SBFD

Way Forward:

- FFS whether the transmitter transient period shall be located within the SBFD symbol or not:

  - FFS it is an implementation issue and no RAN4 standard impact

### Sub-topic 3-2: In-channel adjacent subband leakage ratio requirements

##### Issue 3-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

Agreement:

- New requirement for in-channel adjacent subband leakage ratio should be defined

  - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

### Sub-topic 3-3: In-channel adjacent subband selectivity/blocking requirements

##### Issue 3-3-2: Necessity of in-channel adjacent subband blocking requirement

Agreement:

- New requirement for in-channel adjacent subband blocking should be defined

  - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation.

  - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement.

  - The requirement level shall be driven by feasibility consideration.

##### Issue 3-3-1: Necessity of in-channel adjacent subband selectivity requirements

Agreement:

- No necessity to introduce the new requirement for in-channel adjacent subband selectivity.

### Sub-topic 3-4: Inter-sector and inter-site interference

##### Issue 3-4-1: Necessity of new requirement for inter-sector and inter-site interference

Agreement:

- No necessity to introduce the new requirement for inter-sector interference.