3GPP TSG-RAN WG4 Meeting # 112bis											R4-2420103

Orlando, US, 18th – 22nd November, 2024

Title: 	WF on RRM requirements for XR_Ph3

Agenda Item:	7.25.3

Source: 	Nokia

Document for:	Approval

# Topic #1: Scenarios for measurement skipping

## Issue 1-1: Deployment scenarios

< Agreement>:

- EN-DC and NE-DC for DCI-based solution

  - Is NOT supported for measurement gap skipping by the WID

<Way forward>:

- Further discussion needed for

  - FR2-2

  - NR-DC

## Issue 1-2: Types of measurements to be considered

<Agreement>:

- The following measurement types are considered for XR measurement skipping

  - Intra-frequency

  - Inter-frequency

  - LTE-inter- RAT (NR-EUTRAN FDD/TDD)

- The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions

<Way forward>:

- Option 1: Skipping of measurements without gaps with scheduling restrictions are supported

  - Option 1a: Support skipping of measurements without gaps

  - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band

- Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

  - Option 2a: Measurement skipping does not apply for measurements without gaps

  - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

## Issue 1-4: L1 measurements with scheduling restrictions and procedures

<Way forward>:

- Option 1: Support skipping of L1 measurements and procedures.

  - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

  - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

  - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

- Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

  - Option 2a: Measurement skipping does not apply for L1 measurements and procedures

  - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

## Issue 1-5: L1/L2 triggered mobility measurements and procedures

<Way forward>:

- Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

- Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

- Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping.

- Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

# Topic #2: Measurements with gaps

## Issue 2-1: General approach for defining requirements for measurements with gaps

<Agreement>:

- For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

## Issue 2-2: Types of measurement gaps

<Way forward>:

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 |  |  |  |
| Type 2 |  |  |  |
| NCSG |  |  |  |
| MUSIM |  |  |  |
| Positioning |  |  |  |
| Rel 18 concurrent gap |  |  |  |
| PreConfigured |  |  |  |

# Topic #3: XR skipping requirements impact

## Issue 3-1: How to capture impact of measurement skipping

<Agreement>:

- Reuse existing measurement accuracy requirements.

- RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping

  - For the delay requirements, FFS whether and how to extend the delay

<Way forward>:

Proposals related to measurement delay requirements:

  - Option 1: Proposals without distinction of XR and legacy requirements

    - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended.

    - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

    - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

  - Option 2: Proposals with distinction of conditions where measurement impact is not needed

    - Option 2a:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

    - Option 2b:

      - Apply legacy measurement delay and accuracy requirements for the following condition:

        - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI

      - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

## Issue 3-2: Principles for measurement delay calculation

<Way forward>:

- Option 1: Options based on scaling factor

  - Option 1a: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

  - Option 1b: Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

  - Option 1c: introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor.

  - Option 1d: pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor).

- Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

  - Option 2a: dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

## Issue 3-3: Conditions for measurement skipping

<Way forward>:

- Option 1: max number of occasions cancelled during a measurement period shall be defined.

- Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

- Option 3: discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL (e.g., MGL≥3 ms, due to low gain with MG skipping),

  - maximum MGL (e.g., MGL≤6 ms),

  - maximum MGRP (e.g., MGRP≤320 ms to avoid too long delays), etc.

- Option 4: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

# Topic #4: Time offset for measurement skipping processing

## Issue 4-1: Time offset requirement for measurement gap skipping

<Agreement>:

- Baseline feature 5 ms

- Optional capability 3 ms, FFS if any condition is needed

# Topic #5: Need/feasibility of UE assistance information

## Issue 5-1: General on UAI

<Way forward>:

- Companies are tasked to discuss the need and feasibility of UAI, based on the following options

- Proposals related to measurement occasions

  - Option 1: Define UAI related to measurement occasions

    - Option 1a: number of occasions which can be skipped

    - Option 1b: number of measurement occasions that are needed for measurement

    - Option 1c: UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 1d: patterns of gaps positions of MG that can be skipped

    - Option 1e: study how UE assistance information/UE capability is reported when DRX is configured.

    - Option 1f: Maximum number of consecutive MGs that can be skipped

  - Option 2:

    - Option 2a: Do not define UAI related to measurement occasions.

    - Option 2b: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution

    - Option 2c: RAN4 will not specify any XR-related requirement, based on XR UAI.

  - Option 3: The information about gaps not used for measurement are reported as UE capability.

  - Option 4: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions.

- Proposals related to channel conditions

  - Option 1: For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

# Topic #6: Additional signalling aspects

## Issue 6-1: Semi-static schemes

<Way forward>:

  - Option 1: Follow RAN1 working assumption and consider DCI only.

  - Option 2: also consider RRC/semi-static approach.

    - Option 2a: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

    - Option 2b: RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion.

    - Option 2c: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

    - Option 2d: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

    - Option 2e: In addition to DCI solution, it is proposed to introduce semi-static schemes.

  - Option 3:

    - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported).

    - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

## Issue 6-2: Skipping indication on frequency domain

<Way forward>:

- Proposals

  - Proposal 1: For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2:

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).