3GPP TSG-RAN WG4 Meeting # 113					       			               R4-2420121

Orlando, US, November 18-22, 2024

Agenda item:			5.1.2

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR network energy saving

Document for:	Approval

- Topic #1: Core requirements maintenance

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell.

Agreement:

RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases:

- When servingCellMO is configured

- When servingCellMO is not configured

RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell.

Sub-issue 2: Definition on intra-frequency or inter-frequency

- Proposals

  - Option 1: Intra-frequency or inter-frequency depends on whether servingCellMO is configured. (Nokia)

    - If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement.

    - In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

  - Option 2: Inter-frequency measurement based on existing definition. (CMCC, Apple, Vivo, Huawei, ZTE)

Sub-issue 3: Whether MG is needed.

- Proposals

  - Option 1: Whether to apply the requirement of inter-frequency measurement with MG or without MG follow the legacy principle and there is no spec impact. (CMCC, Vivo, Huawei, ZTE)

  - Option 2: Treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell. (Apple)

Issue 1-1-2: EPRE conditions updating

- Proposals

  - Option 1: When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.  (Ericsson)

  - Option 2: Specify the threshold of Y as the value not smaller than 30dBm. A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. (ZTE)

Agreement: Conclude the number without [] in the next meeting.

Issue 1-1-3: CHO requirements maintenance

Agreement:

Discussed in the CR directly

Issue 1-1-4: intra-band NCCA

Agreement: The issue is closed.