3GPP TSG-RAN WG4 Meeting #113												 		    R4-2420213

Orlando, FL, US, 18th – 22nd, 2024

Agenda item:			5.1.2

Source:	MediaTek.

Title:	WF on RRM requirements for NR_MG _enh2

Document for:	Approval

# Topic #1: Concurrent gaps with NCSG

### Sub-topic 1-1: Rel-18 UE behavior for deactivated SCell measurements with NCSG

- Background:

< Agreement >: Agreement from RAN4 meeting#111: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case. Agreement from RAN4 meeting#112: - When UE switches measurement between NCSG and Type 2 MG, no additional processing delay is expected.

Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association?

  - Further discuss in the next meeting.

Issue 1-1-2: When the measurement of the measurement object is related to a deactivated SCell, and after this SCell is activated, and when UE is configured with the concurrent measurement gap and NCSG, what is the expected UE behaviour?

  - Further discuss in the next meeting

# Topic #2: NeedForGap

### Sub-topic 2-1: DRX in NFG operation

Issue 2-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements?

- Background:

  - Agreement from RAN4#112 meeting:

    - Interruptions are not allowed during DRX ON duration

      - Exclude the time extended due to drx-inactivityTimer

        - if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration.

- Proposal:

  - Option 1:

    - Revise the value of the time period before and after the DRX-ON cycle for which the presence of SMTC allows interruptions during the DRX-ON period to 1 ms.

------- End of Document -------