3GPP TSG-RAN WG4 Meeting #113	R4-2420465

Orlando, USA, November 18th – 22nd, 2024

Title: 	WF on requirements for NR IoT NTN bands

Agenda Item:	6.17, 6.18, 6.19

Source: 	Moderator (Inmarsat)

Document for:	Approval

# Topic #1: NR-NTN S-band

### Sub-topic 1-1: General Requirements and Coexistence Aspects

#### Issue 1-1-1: Regional Applicability

Agreement:

  - No note is needed in clause 5.2 operating band of TS 38.108 and TS 38.101-5 to clarify regional applicability of band n252.

#### Issue 1-1-2: TN-NTN Isolation Distance

- Offline Agreement:

  - The TN-NTN coexistence assumption of 1.5 km separation between NTN UE and edge of the TN coverage from TR 38.863 has been used to guarantee the minimal performance on both TN and NTN networks. Nevertheless, this might not represent a typical deployment, and an NTN UE might still be able to operate within this isolation distance

    - Revision of the general TN-NTN coexistence assumptions is not in scope of this WI

#### Issue 1-1-3: UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL

- Tentative Agreement:

    - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2

      - FFS: -30 dBm/MHz for relaxed value.

        - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.”

      - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification

    - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable

    - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

#### Issue 1-1-4: Protection of n70 and n66

Agreement:

  - For new S-band n252, the following spurious emission requirements for UE-to-UE coexistence can be specified for addressing the UE-to-UE coexistence with band n70, n66.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n252 | NR Band n70 | FDL_low | - | FDL_high | NA | NA | X |
|  | NR Band n66 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE X: The co-existence between n252 and band 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

### Sub-topic 1-2: UE RF

Sub-topic description: General UE RF Requirements

#### Issue 1-2-1: SS Raster

Tentative agreement:

- For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B and Case C could be introduced.

#### Issue 1-2-2: UE TX-RX Separation

Agreement:

- Specify the flexible Tx-Rx separation for n252 with the wording aligned with n255 and n256 in 38.101-5.

### Sub-topic 1-3: SAN RF

Sub-topic description: SAN RF requirements for band n252

#### Issue 1-3-1: SAN SS Raster

Tentative agreement:

- For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B and Case C could be introduced.

# Topic #2 IoT-NTN S-band

### Sub-topic 2-1: UE RF

Sub-topic description: UE RF requirements

#### Issue 2-1-1: UE TX-RX Separation

Agreement:

- Specify the variable Tx-Rx frequency separation for Band B252

- Discuss the applicability of variable Tx-Rx frequency separation for other IoT NTN bands under other agendas.

#### Issue 2-1-2: UE Maximum Output Power for Cat M1 and NB1/NB2

Agreement:

  - The MOP of this band will be aligned with the agreements in other Rel-19 NTN related WIs.

# Topic #3: NR-NTN L-bands

### Sub-topic 3-1: General Aspects and Regulatory Input

Sub-topic description: Regulatory aspects

#### Issue 3-1-1: General Approach for Capturing Additional Regulatory Radiated Emission Requirements (e.g. ITU, ETSI, FCC)

- Tentative Agreement:

  - Further study how to capture radiated regulatory requirements in the specification(s), in such a way that ensures compliance to regulations in all operational scenarios, but does not create unnecessary implementation burden.

    - (E.g. basic FR1 OTA emission requirements, additional conversion to conducted requirements with reference antenna gain or scaling factor, etc)

#### Issue 3-1-3: ETSI additional blocking Requirements

Agreement:

  - For two new NR NTN L bands, referring to the conclusions in Rel-18 for band 253, do not define the additional in-band / out-of-band blocking requirements in Rel-19 until RAN4 gets clear information from ETSI.

#### Issue 3-1-4: Protection of Radio Astronomy stations – Duplexer dependency

Agreement:

  - The requirements to protect Radio Astronomy service operation are independent with the duplexer implementation.

#### Issue 3-1-5: Requirements and Mechanisms on Protection of Radio Astronomy stations

- Tentative Agreement:

  - Further study mechanisms to implement protection of Radio Astronomy stations, at least the following options could be considered:

    - TX Disable, considering possible impact on backwards compatibility

    - RB restrictions

    - NS

#### Issue 3-1-6: Signalling of proximity to Radio Astronomy stations

- Tentative Agreement:

  - Further study how to determine proximity of NTN UE to Radio Astronomy stations