3GPP TSG-RAN WG4 Meeting #113	R4-2420525

Orlando, United States, November 18 – 22, 2024

Title: 	WF on solution to enable HPUE max transmit power in DL CA

Agenda Item:	9.2

Source: 	Qualcomm Inc.

Document for:	Approval

## Issue 1-1: Output power

Capture in TS 38.101-1:

As an exception, HPUE applicability notes in 5.5A do not restrict power class indication for single configured UL CC with DL CA. UE can indicate power class for the single configured UL CC with DL CA as specified in Table 6.2.1-1 and if UE supports UL MIMO in this carrier, UE can indicate power class for the CA configuration as specified in Table 6.2D.1-1.

## Issue 1-2: Receiver requirements

Capture in TS 38.101-1:

When UE supports higher power class than default power class for a CA configuration with a single UL CC with DL CAand applicability note for the supported power class is not present for this configuration in clause 5.5A.

- if the corresponding higher power class MSD is not specified, reference sensitivity and exceptions for reference sensitivity of default power class shall be verified with output power limited to default power class

- otherwise, the higher power class reference sensitivity and exceptions for reference sensitivity shall be verified with the power class of CA configuration supported by the UE.

## Issue 1-3: Other agreements

- UEs already on the market shall not be retested after the HPUE MSD is defined.

- UEs already under design shall not be required to meet the HPUE MSD, i.e. a 6 month transition period shall be applied to the newly introduced requirements in issue 1-1 and 1-2.

- Further consider how to reflect above two bullets in the specification and inform RAN5 with LS.

- The changes can be reverted/removed in Rel-19 if new MSD rules are agreed.

- In the task report to RAN, inform RAN and RAN5 about the agreements and progress on the new MSD rules discussed under Rel-19 NR_ENDC_RF_Ph4 WI.

- Further discussion on UE signaling for DL CA with 1 UL

- FFS whether and how to update the HPUE applicability notes in clause 5.5A, e.g. change “applicable” to “specified”.

- FFS whether and how to make updates on configured transmission power.