3GPP TSG-RAN WG4 Meeting #113	R4-2417687

Orlando, US, 18-22 November 2024

Source:	Nokia

Title:	On Rel-19 event-triggered L1 reporting requirements

Agenda item:	7.24.2.1

Document for:	Discussion

Introduction

In this contribution we continue the discussion about L1 event-triggered reporting for LTM.

RAN4 agreed to start the discussion about event-triggered L1 measurement reporting from SSB-based FR1 and FR2-1 L1 measurements for intra-frequency, inter-frequency without gap and inter-frequency with gap. Reporting of CSI-RS measurements is to follow the parallel Rel-19 work. RAN4 also agreed to reuse the Rel-18 accuracy requirements for SSB and the to-be-defined Rel-19 accuracy requirements for CSI-RS for event-triggered L1-reporting.

- Scope: - RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements. - To start RAN4 discussion for SSB based L1 measurement firstly, and check how to apply/extend the agreements for SSB based L1 measurement to accommodate CSI-RS based L1 measurement. - From RAN4 RRM requirement perspective, for FR2-1, RRM requirements for multi-Rx based LTM L1 measurement is not considered in the WI scope. - RRM scope of Event triggered L1 measurement reporting - In addition to intra-frequency measurement and inter-frequency measurement without gap, support gap-based inter-frequency measurement for SSB-based L1 measurement. Further discuss for CSI-RS based L1 measurement. - Both FR1 and FR2-1 are in scope of RRM requirements for Event triggered L1 measurement reporting. - Measurement accuracy: - for SSB based L1 RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R18 LTM L1-RSRP accuracy requirements - for CSI-RS based L1-RSRP event triggered reporting, the reported L1-RSRP measurement result shall meet R19 CSI-RS based L1-RSRP accuracy requirements for LTM, which will be discussed under CSI-RS based L1-RSRP measurement agenda. - Starting point of event triggered L1 reporting delay: - Use the following as starting point. Wording can be discussed in CR. - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting.

In this contribution we focus on SSB-based L1 event-triggered reporting

Event-triggered SSB-based L1 reporting

SSB based measurement delay is defined in section 9.14 and 9.15 for intra- and inter-frequency measurements in LTM, respectively. Measurement delay needs to be taken into account in the reporting delay as this is the time it may take for the UE to realize the condition for triggering the report is met. For intra-frequency measurements, without DRX, the measurement delay is defined as:

FR1 without support of RTD>CP capability: max(TReport, ceil(M*P)*TSSB_NBC)

FRI1 with support of RTD>CP capability: max(TReport, ceil(M*P)*TSSB_NBC*NLayer)

FR2 without support of RTD>CP capability: max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC)

FR2 with support of RTD>CP capability: max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC)

As the report is event triggered, TReport can be ignored in the delay. However, RAN2 has agreed to also support event-triggered periodic reporting. In this case for the first report TReport is to be ignored, and for the following periodic reports it shall be taken into account. Otherwise, the measurement period can in our view be reused as the baseline.

RAN4 to define requirements for event-triggered and event-triggered periodic L1 reporting delay.

For event-triggered eporting delay for a single report, reuse the Rel-18 LTM intra- and inter-frequency measurement delay (with and without gaps), where TReport is not taken into account.

For event-triggered periodic reporting delay, reuse the Rel-18 LTM intra- and inter-frequency measurement delay (with and without gaps) without TReport taken into account for the first report, and for the following reports with TReport taken into account.

RAN2 has agreed that TTT (time to trigger) operates only based on a timer, similar to L3 event triggered reporting. In the existing L3 reporting requirements or test cases TTT is not taken into account. RAN4 should discuss whether there is a need to take TTT into account in L1 event triggered reporting requirements. However, we do not think the current RAN1/2 agreements are sufficient to make any conclusions about this in RAN4 yet.

RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available.

While the starting point of event triggered reporting delay was agreed, RAN4 did not yet agree on the ending point of the delay.

RAN2 has agreed to reuse the legacy scheduling request procedure for reporting resource allocation:

The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In our view, this agreement would mean that the reporting is very similar to L3 event-triggered reporting, and hence the same ending point for the reporting can be reused. The ending point for L3 event-triggered reporting delay is the time the UE starts sending the report over the air interface. The uncertainty of UE acquiring UL resources for the report is not taken into account in the delay. With the given RAN2 agreement, we think we can reuse this approach as well.

Reuse the ending point of event-triggered L3 reporting delay for event-triggered L1 reporting delay: the point when the UE starts to transmit the measurement report over the air interface.

The accuracy of the event-triggered L1 report also needs to be discussed by RAN4. RAN1 has agreed to use L1-RSRP as the reporting metric, and RAN2 has agreed to include the measured L1-RSRP value in the report. Hence, we think the existing L1-RSRP measurement accuracy requirements can be reused.

Reuse the existing L1-RSRP measurement accuracy requirements for event-triggered L1 reporting.

In addition to the above requirements, RAN4 eventually needs to discuss the impact of event-triggered reporting to other LTM requirements. Rel-18 LTM discussion focused on the assumption of periodic and network-controlled reporting, and the requirements were written accordingly. For example, cell switch delay requirements assume a recent L1 or L3 report to be present for the requirements to apply. With event-triggered reporting there may not be reports available regularly, and the network is not aware when the report will be triggered and sent. It is also still unclear whether event becoming fulfilled may lead to one or more (periodic) event-triggered reports. In any case, we think RAN4 will need to discuss whether the introduction of event-triggered L1 reporting has an impact at least on known TCI state condition for early TCI state activation and cell switch, and the applicability of cell switch delay requirements

RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as:
- Known TCI state condition for early TCI state activation and cell switch
- Applicability of cell switch delay requirements

Even further, one benefit of event-triggered reporting is that it gives a direct indication to the network about a candidate cell being a potential target for cell switch. Hence, RAN4 may discuss whether event-triggered reporting condition becoming fulfilled may also be considered as a trigger for some of the preparation steps for LTM. We think it would be reasonable that reporting condition becoming fulfilled would trigger the UE to:

- Perform early/fast ASN.1 decoding and validity check, if not done before based on some other trigger.

- Perform DL synchronization for an active TCI state for the candidate cell for which the report is triggered, if DL synchronization is not valid at the time of triggering the report.

Event-triggered L1 report or reporting condition becoming fulfilled triggers fast ASN.1 decoding.

Event-triggered L1 report or reporting condition becoming fulfilled triggers DL synchronization for the candidate cell’s active TCI states, if not valid at the time of report triggering.

RAN4 should also discuss whether LTM L1 measurement delay requirements are impacted by event triggered reporting. Earlier RAN4 has agreed that after TCI state activation, the UE is allowed to prioritize L1 measurements on the candidate cells with active TCI states. As proposed by some companies in the last meeting, we also think similar approach could be taken with event-triggered L1 reporting, where after triggering the report the UE can prioritize the L1 measurements on the candidate cells fulfilling the reporting condition. This way the L1 measurement period after the report may become shorter, which may impact for example DL synchronization based on TCI state activation.

RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cells fulfilling the reporting condition.

In the last meeting there was also discussion about the UE capability of supported number of events, as well as the coordination between LTM and MIMO event-triggered L1 reporting. We think these issues can be discussed towards the end of the work items, when the details of both reporting types are clearer.

Discuss the number of supported events at the end of the WI.

Discuss the coordination between LTM and MIMO events at the end of the WIs.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: RAN4 to define requirements for event-triggered and event-triggered periodic L1 reporting delay.

Proposal 2: For event-triggered eporting delay for a single report, reuse the Rel-18 LTM intra- and inter-frequency measurement delay (with and without gaps), where TReport is not taken into account.

Proposal 3: For event-triggered periodic reporting delay, reuse the Rel-18 LTM intra- and inter-frequency measurement delay (with and without gaps) without TReport taken into account for the first report, and for the following reports with TReport taken into account.

Proposal 4: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available.

Proposal 5: Reuse the ending point of event-triggered L3 reporting delay for event-triggered L1 reporting delay: the point when the UE starts to transmit the measurement report over the air interface.

Proposal 6: Reuse the existing L1-RSRP measurement accuracy requirements for event-triggered L1 reporting.

Proposal 7: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements

Proposal 8: Event-triggered L1 report or reporting condition becoming fulfilled triggers fast ASN.1 decoding.

Proposal 9: Event-triggered L1 report or reporting condition becoming fulfilled triggers DL synchronization for the candidate cell’s active TCI states, if not valid at the time of report triggering.

Proposal 10: RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cells fulfilling the reporting condition.

Proposal 11: Discuss the number of supported events at the end of the WI.

Proposal 12: Discuss the coordination between LTM and MIMO events at the end of the WIs.