3GPP TSG-RAN WG4 Meeting # 113											R4-2418267

Orlando, US, 18th ‒ 22nd November, 2024

Agenda item:			5.1.2

Source:	Moderator (Samsung)

Title:	Topic summary for [113][209] NR_MIMO_evo_DL_UL

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for maintenance for Rel-18 NR MIMO evolution for downlink and uplink:

- 5.25	NR MIMO evolution for downlink and uplink

- 5.25.1 RRM core and performance requirements

Recommended issues to online discussion:

Issue 1-2

Issue 1-1

Topic #1: Core requirements maintenance

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418548 | Apple | Observation #1: For multi-DCI multi-TRP operation with 2TA the UE has gradual timing adjust requirements applicable to each TAG. Observation #2: For multi-DCI multi-TRP operation with 2TA the UE has MTTD requirements applicable to pair of TAGs. Observation #3: While the UE is applying gradual timing adjustment for each TAG, the TTD for the pair of TAGs could exceed the MTTD requirement. Observation #4: Similar issue with legacy gradual timing adjustment and MTTD for CA also needs to be discussed. Proposal #1: For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs. |
| R4-2419099 | vivo | Observation 1 Although mDCI m-TRP with 2-TA is the pre-requisite UE capability for RTD>CP L1 measurement capability in R18 MIMO, it does not mean UE can only perform L1 measurement for RTD>CP case when more than one coresetPoolIndex is configured. L1 measurements with RTD>CP will provide measurement results for gNB to decide whether to configure/activate more than one coresetPoolIndex. Observation 2 For FR2-1, RTD>CP case is already agreed to be tested, with 2-TAGs and 2 coresetPoolIndex-r16 configured, as captured in TS A.5.4.1.2. Observation 3 Existing measurement restriction between SSBs for the inter-cell case may also apply to the case when RTD>CP Proposal 1 RAN4 to further introduce measurement restriction and scheduling restriction in FR2-1 for the RTD>CP with the following scenarios: - Measurement restriction: CSI-RS+SSB - Measurement restriction: CSI-RS+CSI-RS - Scheduling restriction: SSB + DL/UL - Scheduling restriction: CSI-RS + DL/UL Proposal 2 RAN4 not to revert the agreement on introducing FR2-1 UE capability of supporting RTD>CP for R18 mDCI mTRP with 2-TA, which is agreed in RAN4 #104bis. With existing R18 MRTD/MTTD requirements and test cases, RAN4 further solve identified issues for supporting such feature in R18 maintenance. Proposal 3 RAN4 not to add additional requirement applicability for 9.5 and 9.13 regarding RTD>CP, because they are already clarified in 9.5.2 and 9.13.2. |
| R4-2419284 | Ericsson | Observation 1: How to handle TTD becoming larger than MTTD may be general RRM issue not only concerning Rel-18 MIMO. However, no solution or behaviour specified from Rel-15. |

Open issues summary

Sub-topic 1-1: Core requirements

Issue 1-1: Whether to add the condition to check MTTD before applying gradual timing adjustment?

- Proposals

  - Option 1: (Apple)

    - For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs.

- Recommended WF

  - Discuss option 1.

Moderator’s suggestion: The situation is Option 1 is not only in Rel-18 MIMO, two TAG. As we discussed in previous meeting and also mentioned in the contribution, it is similar in multiple TAGs in CA in Rel-15. It is suggested to be discussed and aligned with Rel-15.

@ Ericsson: It is observation#1 in R4-2419284. I didn’t list option here. Please check whether moderator’s suggestion is fine.

Issue 1-2: Applicability for 9.5 and 9.13 regarding RTD>CP and measurement/scheduling restriction in FR2-1 for RTD>CP

- Proposals

  - Option 1: (vivo)

    - RAN4 not to add additional requirement applicability for 9.5 and 9.13 regarding RTD>CP

    - RAN4 to further introduce measurement restriction and scheduling restriction in FR2-1 for the RTD>CP with the following scenarios:

      - Measurement restriction: CSI-RS+SSB

      - Measurement restriction: CSI-RS+CSI-RS

      - Scheduling restriction: SSB + DL/UL

      - Scheduling restriction: CSI-RS + DL/UL

- Recommended WF

  - Discuss option 1.

Sub-topic 1-2: CRs

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418559 | Apple | CR on gradual timing adjustment for 2TA |
| R4-2418886 | Nokia | CR corrections of RRM core requirements for NR MIMO Evo |
| R4-2419100 | vivo | CR on R18 MIMO RRM requirements |

Topic #2: Performance requirements maintenance

Sub-topic 1-2: CRs

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418887 | Nokia | CR corrections of RRM performance requirements for NR MIMO Evo |