3GPP TSG-RAN WG4 Meeting # 113					       			       			R4-2418268

Orlando, US, November 18-22, 2024

Agenda item:			5.1.2

Source:	Moderator (Huawei)

Title:	Topic summary for [113][210] Netw_Energy_NR

Document for:	Information

# Introduction

This topic summary includes RRM core/perf requirements for Rel-18 network energy saving (5.27.1).

Recommended Topics to be treated online (in order of decreasing priority):

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

Issue 1-1-2: EPRE conditions updating

Issue 1-1-3: CHO requirements maintenance

# Topic #1: Adaptation of common signal/channel transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418071 | Nokia, Nokia Shanghai Bell | Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies. Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133. Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell. Observation #4: The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC. - If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement. In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency. Observation #5: Existing NES-based condition handover delay assumes that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. Observation #6: For the case DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index, the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command. The NES-based conditional handover delay shall be defined considering the possible channel variation when DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index. The NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-X command after TEvent_DU + Tidentify_intra_with_index. If the condition of NES-based CHO is not met when receiving the DCI 2-9 command after TEvent_DU + Tidentify_intra_with_index., - Tevent_DU is defined as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until “a condition exists at the measurement reference point after receiving DCI 2-9 which will trigger the NES-based conditional handover”, and - Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter. |
| R4-2418073 | Nokia, Nokia Shanghai Bell | Corrections for inter-band SSB-less SCell activation TCs |
| R4-2418424 | CMCC | Observation 1: Technically, it is feasible to treat this case as intra-frequency measurement, but spec impact is expected. Observation 2: If strictly follow the existing definition of intra-frequency and inter-frequency measurement, the case belongs to inter-frequency measurement. Whether to apply the requirement of inter-frequency measurement with MG or without MG depends on whether UE supports of interFrequencyMeas-NoGap. Proposal 1: For the case of the neighbour cell(s) is on the carrier of SSB-less SCell and the SSB from neighbour cell(s) can be contained in the active BWP of SSB-less SCell, the measurement for such neighbour cell(s) can be treated as inter-frequency measurement based on existing definition. Whether to apply the requirement of inter-frequency measurement with MG or without MG depends on whether UE supports of interFrequencyMeas-NoGap. |
| R4-2418573 | Apple | Proposal 1: to agree with RAN2 that “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO. Proposal 2: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell. Proposal 3: open to clarify the NES case in the current condition for definition of inter-frequency measurement without MG, as following: A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE, or - the SSB of neighbor cells is completely contained in the active BWP of SSB-less SCell of the UE - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15]. Proposal 4: for intra-band FR1 NCCA case, the side condition of RTD for SSB-less SCell activation shall be defined as: - The RTD between the target SSB-less intra-band NCCA SCell and the collocated reference serving cell is within CP where CP is corresponding to the max SCS between reference cell and target SCell. Proposal 5: for intra-band FR1 NCCA case, the side condition of power imbalance for SSB-less SCell activation shall be defined as: - The EPRE difference at the UE is smaller than or equal to [6] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Proposal 6: As an alternative to proposal 4 and 5, RAN4 to not consider SSB-less SCell operation for intra-band NCCA in R18. |
| R4-2418668 | Huawei, HiSilicon | Proposal 1: Measurement on neighbour cells on carrier of SSB-less SCell is regarded as“inter-frequency measurement”. There is no spec impact about the intra-f and inter-f definition. Proposal 2: Measurement on neighbour cells on carrier of SSB-less SCell with or without gap can follow the legacy principle and there is no spec impact. |
| R4-2419086 | vivo | Proposal 1: From RAN4 perspective, configuring CSI-RS based servingCellMO for inter-band SSB-less SCell is an additional optimization, but it is not supported in R18, since the WI is closed. Proposal 2: RAN4 to agree the following: - Intra-frequency/Inter-frequency definition: - Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition. - No spec impact in terms of inter- or intra- frequency, and with or without gap. Proposal 3: For the delay requirement on NES-based CHO, RAN4 to keep the current definition of TEvent_DU and measurement delay Tmeasure unchanged Proposal 4: RAN4 to revise the starting point of TCHO_execution as follows: TCHO_execution is the UE execution preparation time for conditional handover. For NES-based conditional handover, - It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9. - Otherwise, it starts after UE successfully decodes DCI 2-9. |
| R4-2419092 | vivo | CR for conditional handover requirements on network energy saving |
| R4-2419261 | ZTE Corporation, Sanechips | Proposal 1: Specify the threshold of Y as the value not smaller than 30dBm. Proposal 2: A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. Proposal 3: To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, this MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell. - For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig. Proposal 4: Define the neighbour cell measurement as inter-frequency measurement since no ServingCellMo configured at the SSB-less SCell frequency. Proposal 5: Whether the neighbour cell measurement is performed with gap or without gap, follow the legacy rule specified Clause 9.3.1 for inter-frequency. Proposal 6: To move forward, we prefer to specify two sets of requirements respectively assuming single and separate chains, where the requirements of inter-band case are largely reused for the separate chains assumption, and the requirements of intra-band contiguous case are largely reused for the single chain assumption. Two optional UE capabilities refers to the two cases. If neither of them supported by the UE, then UE does not support the SSB-less SCell activation for intra-band non-contiguous CA. |
| R4-2419273 | ZTE Corporation, Sanechips | CR on core part of R18 NES |
| R4-2419274 | ZTE Corporation, Sanechips | CR on perf part of R18 NES |
| R4-2419285 | Ericsson | - When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling. - RAN4 to send LS to RAN2 informing about disadvanges of not configuring the servingCellMO for inter-band SSB-less SCell. In the LS, the disadvantages to be listed as following. - measurement delay is longer for inter-frequency measurement; and - number of cells UE can measure on inter-frequncy layer are lower than intra-freq layer. |
| R4-2419286 | Ericsson | CR to TS 38.133 on core requirement maintenance for NES |
| R4-2419389 | Nokia, Nokia Shanghai Bell, Huawei, CMCC | 38.133 CR on handover delays for NES-based CHO |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1 Core/Perf maintenance

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

Background:

RAN4 WF in RAN4#112 R4-2414102 Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell FFS:  Intra-frequency/Inter-frequency definition: o Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition.  No spec impact in terms of inter- or intra- frequency, and with or without gap. RAN2 agreements in RAN2#127bis Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. 4 Postpone the discussion on intra-band SSB-less SCell.

Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell.

- Proposals

  - Option 1: Agree with RAN2 that “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO. (Apple, Vivo, ZTE)

  - Option 2: servingCellMO can be configured (Ericsson, Nokia?)

    - RAN4 to send LS to RAN2 informing about disadvanges of not configuring the servingCellMO for inter-band SSB-less SCell. In the LS, the disadvantages to be listed as following.

      - measurement delay is longer for inter-frequency measurement; and

      - number of cells UE can measure on inter-frequncy layer are lower than intra-freq layer.

Sub-issue 2: Definition on intra-frequency or inter-frequency

- Proposals

  - Option 1: Intra-frequency or inter-frequency depends on whether servingCellMO is configured. (Nokia)

    - If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement.

    - In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

  - Option 2: Inter-frequency measurement based on existing definition. (CMCC, Apple, Vivo, Huawei, ZTE)

Sub-issue 3: Whether MG is needed.

- Proposals

  - Option 1: Whether to apply the requirement of inter-frequency measurement with MG or without MG follow the legacy principle and there is no spec impact. (CMCC, Vivo, Huawei, ZTE)

  - Option 2: Treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell. (Apple)

- Moderator:

Based on moderator’s understanding, sub-issue 1/2/3 should be discussed. If ServingCellMO will not be configured for inter-band SSB-less, then it shall be defined as inter-frequency. Per majority views, following WF is recommended for discussion in this meeting.

- Recommended WF:

- Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell.

  - Option 1

- Sub-issue 2: Definition on intra-frequency or inter-frequency

  - Option 2

- Sub-issue 3: Whether MG is needed.

  - Option 1

Issue 1-1-2: EPRE conditions updating

Background:

RAN4 WF in RAN4#112 R4-2414102 Issue 1-1-1: Power difference conditions Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30]

- Proposals

  - Option 1: When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling.  (Ericsson)

  - Option 2: Specify the threshold of Y as the value not smaller than 30dBm. A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. (ZTE)

- Recommended WF:

  - Discuss whether further updating is needed.

Issue 1-1-3: CHO requirements maintenance

- Proposals

  - Option 1: Nokia

- The NES-based conditional handover delay shall be defined considering the possible channel variation when DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index.

- The NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-X command after TEvent_DU + Tidentify_intra_with_index.

- If the condition of NES-based CHO is not met when receiving the DCI 2-9 command after TEvent_DU + Tidentify_intra_with_index.,

- Tevent_DU is defined as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until “a condition exists at the measurement reference point after receiving DCI 2-9 which will trigger the NES-based conditional handover”, and

- Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter.

  - Option 2:  Vivo

For the delay requirement on NES-based CHO, RAN4 to keep the current definition of TEvent_DU and measurement delay Tmeasure unchanged

RAN4 to revise the starting point of TCHO_execution as follows:

TCHO_execution is the UE execution preparation time for conditional handover.

For NES-based conditional handover,

- It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9.

- Otherwise, it starts after UE successfully decodes DCI 2-9.

- Recommended WF:

  - Discuss CR (R4-2419092, R4-2419389) directly and try to conclude in this meeting.

Issue 1-1-4: intra-band NCCA

- Proposals

  - Option 1: Apple

    - for intra-band FR1 NCCA case, the side condition of RTD for SSB-less SCell activation shall be defined as:

      - The RTD between the target SSB-less intra-band NCCA SCell and the collocated reference serving cell is within CP where CP is corresponding to the max SCS between reference cell and target SCell.

    - for intra-band FR1 NCCA case, the side condition of power imbalance for SSB-less SCell activation shall be defined as:

      - The EPRE difference at the UE is smaller than or equal to [6] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell.

    - As an alternative to proposal 4 and 5, RAN4 to not consider SSB-less SCell operation for intra-band NCCA in R18.

  - Option 2: To move forward, we prefer to specify two sets of requirements respectively assuming single and separate chains, where the requirements of inter-band case are largely reused for the separate chains assumption, and the requirements of intra-band contiguous case are largely reused for the single chain assumption. Two optional UE capabilities refers to the two cases. If neither of them supported by the UE, then UE does not support the SSB-less SCell activation for intra-band non-contiguous CA. (ZTE)

- Recommended WF:

  - Suggest no further discussion on this issue.

### Sub-topic 1-2 CR handling

SSB-less Core

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2419273 | CR on core part of R18 NES | ZTE Corporation, Sanechips | Discuss the CR |
| R4-2419286 | CR to TS 38.133 on core requirement maintenance for NES | Ericsson | Pending on issue 1-1-2 |

SSB-less Perf

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2418073 | Corrections for inter-band SSB-less SCell activation TCs | Nokia, Nokia Shanghai Bell | Discuss the CR |
| R4-2419274 | CR on perf part of R18 NES | ZTE Corporation, Sanechips | Discuss the CR and whehter new TC is needed. |

CHO Core

| Tdoc | Title | Source | Recommendation |
| --- | --- | --- | --- |
| R4-2419389 | 38.133 CR on handover delays for NES-based CHO | Nokia, Nokia Shanghai Bell, Huawei, CMCC | Discuss the CR |
| R4-2419092 | CR for conditional handover requirements on network energy saving | vivo | Discuss the CR |