3GPP TSG-RAN WG4 Meeting #113										                     R4-2418271

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.1.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [113][213] NR_ENDC_RF_Ph4

Document for:	Information

Introduction

This email thread discusses the RRM requirements of WI on R19 NR_ENDC_RF_Ph4.

Online handling

(Online) Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE

Topic #1:	RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2419506 | Ericsson | Observation 1: No CR work is needed for core part to capture the interruption requirements at SRS antenna port switching. |

Open issues summary

Sub-topic 1: RRM requirements of WI on R19 NR_ENDC_RF_Ph4

(Online) Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE

Background

The below agreement was achieved in RAN4#112bis meeting.

<Agreement> Issue 1-1: Interruption requirements at SRS antenna switching for 6Rx capable UE Agreement: - The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed. - This can be revisited if the existing requirement cannot be applied to any SRS antenna switching pattern agreed in RAN1.

In RAN1#118b, it is agreed that 3T6R UE reuses the principles used for 4T8R.

Agreement To support 3T6R antenna switching for a 3TX UE, - The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’, - Reuse the principles used for 4T8R, i.e., - For 4T8R 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources, transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or - For 4T8R 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports. Note: This is not meant to be a text proposal to be captured for the specifications.

- Proposals

  - Option 1(Ericsson): No CR work is needed for core part to capture the interruption requirements at SRS antenna port switching

- Recommended WF

The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no CR work is needed for core requirements.