3GPP TSG-RAN WG4 Meeting # 113											R4-2418279

Orlando, US, 18th ‒ 22nd November, 2024

Agenda item:			7.19.4

Source:	Moderator (Samsung)

Title:	Topic summary for [113][221] NR_MIMO_Ph5_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for first part of Rel-19 RRM for NR MIMO Phase 5:

- 7.19	NR MIMO Phase 5

- 7.19.3 RRM core requirements

  - 7.19.3.1 Enhancement for UE-initiated/event-driven beam management

Recommended issues to online discussion:

Issue 1-1

Issue 1-3

Issue 1-2

Issue 1-4

Issue 1-5

Issue 1-6

Issue 1-7

Issue 1-10

Issue 1-8

Issue 1-9

Topic #1: RRM requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417516 | Qualcomm Technologies Ireland | Observation 1: The WID targets FR2, hence FR1 does not need to be considered for defining the measurement report delay. Proposal 1: RAN4 to reuse the measurement periods as already defined for L1-RSRP measurements for FR2, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS Observation 2: RAN4 to analyze further whether UE-initiated / event-driven beam management can also be applied for FR2 HST in case the highSpeedMeasFlagFR2-r17 is set. Proposal 2: No additional accuracy requirements are needed for L1-RSRP measurements (Option 1 in Issue 1-12). Observation 3: Not all options in Issue 1-5 are still applicable after it has been agreed in RAN4 #112 that the end point for the reporting delay is the transmission of the [first PUCCH]. Observation 4: For event-triggered periodic reporting of L3 measurements delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH and the case of no UL resources are excluded from the reporting delay. Proposal 3: Delay uncertainty till the [first PUCCH] can be sent after the event has been identified by the UE and the case of no UL resources should not be included in the measurement reporting delay for UE-initiated/event-triggered beam reporting. Observation 5: Tgap is the time between the occurrence of the event over the air and the availability of the first periodic CSI-RS resource for measuring. Observation 6: In the definition of the test case itself when RRM performance requirements are defined, the test can be defined in such a way that the delay uncertainty Tgap is not counted. Observation 7: In Rel-17, measurement requirements for inter-cell beam management have only been introduced for SSB based L1-RSRP but not for CSI-RS based L1-RSRP. Proposal 4: RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. Proposal 5: The existing conditions for known and unknown TCI state can be reused for UE-initiated/event-triggered beam management. Proposal 6: The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. Proposal 7: No additional time window based filtering or other filtering should be applied by the UE for the L1-RSRP measurements, independent whether a time window is configured or not. Observation 8: In case that no filtering is applied in the UE, the measurement period would simply extend by a factor of M based on its configured value. Proposal 8: RAN4 should not define additional requirements for the window-based observation whether the event has occurred M times. |
| R4-2417689 | Nokia | Proposal 1: Requirements should be defined for both the basic functionality, i.e., without filtering, and the case with filtering (i.e., timer and counter M). Proposal 2: The definition of the starting and ending point of reporting delay shall be the same for the scenarios with filtering and without filtering. Proposal 3: For the basic event-evaluation functionality (without time window), reuse existing L1-RSRP measurement period as a baseline while defining measurement reporting delay requirements for UE-initiated/event-driven beam management. Proposal 4: Reporting periodicity (Treport) shall not be considered for the event-triggered reporting delay. Observation 1: An event condition is met when the number of Event-2 instances for at least one same new beam is greater than or equal to M within a time window. Observation 2: The length of the measurement period will depend on the time window within which M or greater than M Event-2 instances of the same beam need to occur. Observation 3: It is possible to have cases where the number of Event-2 instances within a time window exceed M. Proposal 5: L1-RSRP measurement period for UEIBM equals the time window where the number of event instances (≥ M) for one same beam have occurred to determine that the event triggering condition has been met. Proposal 6: The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event. Proposal 7: Existing unified TCI state switching delay requirements shall be reused for event triggered measurement reporting based TCI state switching Proposal 8: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM Proposal 9: Reuse legacy measurement accuracy requirements for measurement reports with UEIBM. Proposal 10: Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. Observation 4: No baseline RAN4 requirements defined for CSI-RS based L1-RSRP measurements for inter-cell beam management. Proposal 11: RAN4 not to define requirements for CSI-RS based L1-RSRP measurements for inter-cell beam management. Proposal 12: RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. |
| R4-2417954 | Xiaomi | Observation 1: For legacy L3 event triggered reporting, it is specified in 38.331 that the measurement result which triggers L3 reporting will be included in the L3 report. Proposal 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. Proposal 2: If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams. Proposal 3: For measurement time, prefer to consider the impact of triggering window defined in RAN1. Proposal 4: Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. Proposal 5: No new accuracy requirements for L1 measurements are required. |
| R4-2418109 | Samsung | Proposal 1: In the event triggered measurement reporting delay, the L1 evaluation time is included at least. However, as figure shown above, it cannot reuse one-shot identify time like L3 event triggered measurement reporting delay. RAN4 to discuss and specify the evaluation time inside of event triggered measurement reporting delay. Proposal 2: For event trigger reporting delay, RAN4 need to consider the time window and M defined in RAN1. Need further RAN1 progress. Proposal 3: Need further RAN1 progress for RSs relationship between current beam and new beams. Proposal 4: For inter-cell mTRP in FR2, the beam management is only based on SSB L1-RSRP. RAN4 do not specify CSI-RS based L1-RSRP measurement for additional PCI Cell (inter-cell). Proposal 5: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy. Proposal 6: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM. Proposal 7: No enhancement for fast beam sweeping. Proposal 8: Focus on core requirements firstly, FFS on accuracy requirements. Proposal 9: Define separate capabilities for MIMO and mobility. |
| R4-2418440 | CMCC | Proposal 1: for reporting delay for SSB based UE-initiated/event-driven beam reporting, it is proposed to take measurement period defind for Rel-18 LTM as baseline - in detail, the reporting delay for SSB bsed UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM . Proposal 2: for reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused. Proposal 3: for UE-initiated/event-driven beam reporting, it is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M). Proposal 4: it is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M). Proposal 5: it is proposed to follow the approach for L3 measurement report delay to add clarification in the spec that the beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. In this case, no need to differentiate mode A and mode B. Proposal 6: for UE-initiated/event-driven beam reporting, except reporting delay requirements, it is proposed to define reporting accuracy requirements. - The reporting accuracy requirement is that the reported L1 measurement contained in event triggered beam reports shall meet the accuracy requirements - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account. |
| R4-2418554 | Apple | Proposal 1: Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , TL1-RSRP_Measurement_Period_SSB_CDP for SSB inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based with following modifications: Remove Treport Remove considerations for HST enhancements/ scenarios M = 1 (further discuss if M is needed) further discuss if P can be the same or modified for simplification Proposal 2: Event triggered measurement delay is defined without any filtering (as single shot measurement) and without time window to trigger as baseline. Proposal 3: The total reporting delay for event triggered beam reporting is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH. Proposal 4: It is reasonable expectation that NW will ensure that the UL resources for beam report indication or request are configured with shorter periodicity that the measurement period. Observation 1: CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell. Proposal 5: CSI-RS based L1-RSRP measurement for new beam is defined based on legacy L1-RSRP measurement without distinguishing inter-cell or intra-cell. The requirements are applicable to both. Proposal 6: No need to define new TCI state switching delay requirements. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching. Observation 2: No impact of UE triggered beam report to known/unknown conditions for TCI state. Observation 3: The known/unknown condition would be determined by the same criteria as legacy. Proposal 7: No impact of UE triggered beam report to known/unknown conditions for TCI state. The known/unknown condition would be determined by the same criteria as legacy. Observation 4: Faster RX, TX beam refinement is not in scope of this WI. Proposal 8: Use legacy assumptions for beam sweeping for requirements for UE/ event triggered beam reporting. Proposal 9: Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM. No new accuracy requirements are necessary. Proposal 10: Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. Proposal 11: Do not combine capabilities across different L1 measurements. |
| R4-2418655 | Huawei, HiSilicon | Observation 1: RAN4 shall focus the measurement behaviour/performance of UE-initiated/event-driven beam management. Observation 2: When the periodicity of current beam and new beam are different, one over/under estimated result due to instant channel fading may leads to several wrongly counted Event instance, which will mislead NW configuration. Proposal 1: For event instance determination for event 2, event instance determination shall be based on new results from both current and new beam, which means one measurement result shall not be used for multiple Event instance counting. Proposal 2: RAN4 to wait for further RAN1 agreement on evaluation periodicity and FFS whether to consider relaxed measurement consider the length of evaluation window. Observation 3: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell. Proposal 3: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. Proposal 4: The existing known conditions for TCI state switching can cover the case that L1-RSRP reporting triggered by UEIBM. |
| R4-2419101 | vivo | Proposal 1 Remove square bracket based on RAN1 latest agreements: - For event triggered measurement reporting delay, ending point is the point when the UE transmits a first PUCCH triggered by the event. Proposal 2 To discuss the requirements for event-triggered L1 reporting, it is proposed to firstly discuss the scenario where UE needs not to consider updating the L1 reporting content based on latest results after first PUCCH is sent. - That means to firstly consider the case that, the condition which will trigger the event is remained fixed during the time UE performs L1 measurements. - Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion. Proposal 3 To discuss the requirements for event-triggered L1 reporting, it is proposed to firstly discuss the scenario where, during the time UE performs L1 measurements for evaluating the event, - No change on the indicated TCI state - No change on the to-be-measured RS of the new beam. Proposal 4 RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured. Proposal 5 Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions. |
| R4-2419267 | ZTE Corporation, Sanechips | Proposal 1: Based on the approved measurement reporting delay, RAN4 needs to define an evaluation period as the upper bound of the measurement reporting delay. The evaluation period is depend on the trigger-event detection and exact event. Proposal 2: For event triggered UEIBM, the evaluation period is equivalent to the measurement period. Proposal 3: Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point. Proposal 4: Need to decide whether to define L1-RSRP requirements for unknown cell. Proposal 5: Reuse existing unified TCI state switching delay requirements for UEIBM. Proposal 6: For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. Proposal 7: RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. Proposal 8: Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1. - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering. - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible. Proposal 9: Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement. |
| R4-2419295 | Ericsson | Proposal 1: Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS. Proposal 2: RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2. Proposal 3: RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. Proposal 4: SSB based L1-RSRP measurement period for the SSB configured for event triggered measurement is given by following table. Configuration; TL1-RSRP_event triggered_measurement_Period_SSB (ms). non-DRX; ceil(M*P)*TSSB). DRX cycle ≤ 320ms; ceil(M*P)*max(TDRX,TSSB). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length.. Proposal 5: CSI-RS based L1-RSRP measurement period for the CSI-RS configured for event triggered measurement period is given by following table. Configuration; TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms). non-DRX; ceil(M*P)*TCSI-RS). DRX cycle ≤ 320ms; ceil(M*P) *max(TDRX,TCSI-RS). DRX cycle > 320ms; ceil(M*P)*TDRX. Note 1: TCSI-RS is the periodicity of the CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length.. Proposal 6: RAN4 not to consider counter or timer in the event triggered L1 reporting as L3 measurements did not consider that. That means M1=1 in RAN4 requirements. Proposal 7: Event evaluation period for a SSB or CSI-RS is same as their respective measurement period Proposal 8: RAN4 to discuss components and criteria for defining TTime to first UL channel. Proposal 9: Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel. Proposal 10: Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching. Proposal 11: RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements. Proposal 12: Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP. Proposal 13: RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. |
| R4-2419745 | MediaTek inc. | Proposal 1: Reuse legacy measurement period for event triggered beam reporting. - SSB based L1-RSRP measurement for both intra-cell and inter-cell. - CSI-RS based L1-RSRP measurement for intra-cell. - Descope CSI-RS based L1-RSRP measurement for Inter-cell case. Proposal 2: RAN4 to consider the number of samples is equal to one in L1-RSRP measurement for event triggered beam reporting. Proposal 3: RAN4 at least to consider the uncertainty time to UL resource on 1st PUCCH since this is the earliest time for NW to know UE is ready to send event triggered beam reporting. Proposal 4: RAN4 do not consider beam sweeping enhancement on UE event triggered reporting since it is out of scope in this WI. Proposal 5: RAN4 to reuse the legacy L1-RSRP accuracy requirement for UE-initiated/event-driven beam management. Proposal 6: Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency. Proposal 7: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. Proposal 8: Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. |

Open issues summary

Sub-topic 1-1:

Issue 1-1: How to define Event triggered measurement reporting delay?

< Agreement in last meeting>

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a [first PUCCH] triggered by the event.

- “[first PUCCH]” can be updated based on further RAN1/2 agreement.

Discuss

- Proposals

  - Option 1: (vivo)

    - Remove square bracket of [first PUCCH]

- Recommended WF

  - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

  - Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay?

- Proposals

  - Option 1: (Qualcomm, Apple, Ericsson, MediaTek)

    - RAN4 to reuse the measurement periods as already defined for L1-RSRP measurements for FR2, i.e., TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS

    - Option 1a: (Qualcomm)

      - FFS on HST scenarios

    - Option 1a: (Apple)

      - Remove HST scenario

      - Remove TReport

      - M = 1 (FFS: whether M is needed or not)

      - FFS: whether P can be the same or modified for simplification

  - Option 2: (Nokia)

    - For without time window case, reuse existing L1-RSRP measurement period as a baseline while defining measurement reporting delay requirements for UE-initiated/event-driven beam management.

  - Option 3: (Xiaomi)

    - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

  - Option 4: (Samsung)

    - In the event triggered measurement reporting delay, the L1 evaluation time is included at least. However, as figure shown above, it cannot reuse one-shot identify time like L3 event triggered measurement reporting delay.

    - RAN4 to discuss and specify the evaluation time inside of event triggered measurement reporting delay.

Option 4a: (ZTE)

RAN4 needs to define an evaluation period as the upper bound of the measurement reporting delay. The evaluation period is depend on the trigger-event detection and exact event.

      - For the basic solution(without time window case), the evaluation period can be expressed as Tevaluasion period = 1*TRS.

      - For the optional solution(with time window and counter case), the evaluation period can be expressed as Tevaluasion period = minimum(N, M*TRS).

  - Option 5: (CMCC)

    - Take measurement period defined for Rel-18 LTM as baseline

    - The reporting delay for SSB based UE-initiated/event-driven beam reporting is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index , which are specified for L1-RSRP measurement for Rel-18 LTM.

- Recommended WF

To discuss options:

  - Option 1 (Qualcomm, Nokia (If without time window case), Apple, ZTE(If without time window case), Ericsson, MediaTek):

    - For SSB based L1-RSRP measurement, serving cell

Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

In legacy Rel-15 L1-RSRP,

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8

- 	P …

For companies supported option 1, discuss on: M

For companies supported option 1, discuss on N, N is related to whether fast beam sweeping is supported in other issue and whether to support HST FR2

For companies supported option 1, discuss on P, whether P can be the same as legacy or not

For CSI-RS based,

Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

In legacy Rel-15 requirements: M, P, N are different for periodic/semi-persistent CSI-RS resources, etc

Discuss on whether same M, P, N can be reused or not.

For SSB based L1-RSRP measurements of configured measurement resources from a cell with different PCI in addition to serving cell (PCI indicated in additionalPCI-r17)

Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Discuss on whether same M, P, N can be reused or not.

  - Option 2: (Xiaomi)

    - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams

  - Option 3: (Samsung)

    - In the event triggered measurement reporting delay, the L1 evaluation time is included at least. However, as figure shown above, it cannot reuse one-shot identify time like L3 event triggered measurement reporting delay. RAN4 to discuss and specify the evaluation time inside of event triggered measurement reporting delay.

    - Need further RAN1 progress for RSs relationship between current beam and new beams.

  - Option 4: (CMCC)

    - Take measurement period defined for Rel-18 LTM as baseline

    - TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index

    - TSSB_time_index is the definition in L3 measurement

Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements apply if the actual RTD between serving cell and neighbor cell is not larger than CP. |  |

Option 5: (Huawei)

    - For event instance determination for event 2, event instance determination shall be based on new results from both current and new beam, which means one measurement result shall not be used for multiple Event instance counting.

    - RAN4 to wait for further RAN1 agreement on evaluation periodicity and FFS whether to consider relaxed measurement consider the length of evaluation window.

- Recommended WF

  - TBA

Moderator’s suggestion:

- Please notice the objective in WID is “targeting FR2 and sTRP with intra- and inter-cell beam management”. Let’s focus FR2 requirements as it is the scope.

- Let’s use same terminology will be agreed in group when discussing it. (It can be discussed and aligned in Adhoc discussion)

- @Ericsson, in proposals in R4-2419295, they are based on the FR1 requirements. According to first bullet, to check whether it is fine in option 1. (Similar idea of proposals but in FR2).

- Ericsson comment: We think FR1 and FR2 are in the scope. Targeting FR2 does not mean FR2 only as per the WID. Our understanding is, they are applicable to both FR1 and FR2.

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2?

Case 1: no time window based

Case 2: with time window based, M is configurable

- Proposals

  - Option 1: (Qualcomm)

    - No additional time window based filtering or other filtering should be applied by the UE for the L1-RSRP measurements, independent whether a time window is configured or not.

  - Option 2: (Nokia, Xiaomi, Samsung, CMCC, ZTE)

    - RAN4 to specify requirements to cover both cases of no time window based and time window based

    - Option 2a: (Nokia, Xiaomi, Samsung, ZTE)

      - Requirements may be different for case 1 and case 2, RAN4 to discuss

    - Option 2b: (CMCC)

      - Same reporting requirements are applied to case 1 and case 2.

    - Option 2c: (Nokia)

      - L1-RSRP measurement period for UEIBM equals the time window where the number of event instances (≥ M) for one same beam have occurred to determine that the event triggering condition has been met.

    - Option 2d: (Samsung)

      - Need further RAN1 progress

  - Option 3: (Apple, vivo, Ericsson, MediaTek)

    - Only define event triggered measurement delay based on without time window

- Recommended WF

  - TBA

Issue 1-4: Reporting delay requirements

- Proposals

  - Option 1: (Qualcomm)

    - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0;

Option 2:

  - Option 2: (Nokia)

    - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

  - Option 3: (Xiaomi)

    - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

  - Option 4: (Samsung)

    - Need further RAN1 progress for RSs relationship between current beam and new beams.

  - Option 5: (CMCC)

    - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on

  - Option 6: (Apple)

    - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay

  - Option 7: ( MediaTek)

    - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS

    - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period

  - Option 8: (Ericsson)

    - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel

    - RAN4 to discuss components and criteria for defining TTime to first UL channel.

- Recommended WF

  - TBA

Issue 1-5: Whether to update L1 report which is measured in L1 report?

- Proposals

  - Option 1: (Xiaomi)

    - It needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

  - Option 2: (vivo)

    - RAN4 to specify the requirements to consider the condition which will trigger the event is remained fixed during the time UE performs L1 measurements.

    - Whether UE shall update the L1 report content based on latest L1 measurement results, or shall even cancel the report in some cases, is up to RAN1/2 discussion.

- Recommended WF

  - TBA

Issue 1-6: Conditions when UE performs L1 measurements for evaluating the event?

- Proposals

  - Option 1: (vivo)

    - No change on the indicated TCI state

    - No change on the to-be-measured RS of the new beam.

- Recommended WF

  - TBA

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM

- Proposals

  - Option 1: (Qualcomm, Nokia, Samsung, MediaTek)

    - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

  - Option 2: (CMCC)

    - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

  - Option 3: (Apple, Huawei)

    - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

  - Option 4: (ZTE)

    - RAN4 needs to identify whether additional clarifications on the following points are necessary:

      - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

      - The prerequisite on L3 measurement on NSC.

      - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

  - Option 5: (Ericsson)

    - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

    - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

- Recommended WF

  - TBA

Moderator’s observation: In Rel-17 unified TCI state framework, there is no “neighbour cell” in L1 measurement, only the definition of “a cell with different PCI in addition to serving cell (PCI indicated in additionalPCI-r17)”. Let’s use the same terminology in R17

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

- Proposals

  - Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei)

    - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

  - Option 2: (vivo)

    - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

  - Option 3: (ZTE)

    - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

- Recommended WF

  - TBA

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

- Proposals

  - Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

  - Option 2: (Samsung)

    - No unknown case is applied if the TCI states switching is based on UEIBM.

  - Option 3: (ZTE)

    - Need to decide whether to define L1-RSRP requirements for unknown cell

- Recommended WF

  - TBA

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

- Proposals

  - Option 1: (Nokia)

    - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

  - Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek)

    - Reuse current beam sweeping factor for UE/event triggered beam reporting

- Recommended WF

  - TBA

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria?

- Proposals

  - Option 1: (Nokia)

    - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

  - Option 2: (Apple)

    - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

  - Option 3: (Ericsson)

    - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP.

  - Option 4: (MediaTek)

    - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

- Recommended WF

  - TBA

Moderator’s: @ option 3, the WID is “targeting FR2 and sTRP”, need further clarification for option 3 in discussion.

Ericsson comment:

RAN1#116 agreement:

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

Issue 1-12: Compatibility of capabilities with LTM

- Proposals

  - Option 1: (Nokia)

    - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

  - Option 2: (Samsung, Apple)

    - Do not combine capabilities across different L1 measurements.

  - Option 3: (Ericsson)

    - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

  - Option 4: (MediaTek)

    - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

    - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

- Recommended WF

  - TBA

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

- Proposals

  - Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek)

    - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

  - Option 2: (Samsung)

    - Focus on core requirements firstly, FFS on accuracy requirements.

  - Option 3: (CMCC)

    - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

    - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

  - Option 4: (ZTE)

    - Discuss the accuracy for the two trigger-event solutions respectively.

      - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

      - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

- Recommended WF

  - TBA