3GPP TSG-RAN WG4 Meeting #113 	R4-2420150

Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5228 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR to 38133 for RRM core part for expanded and improved NR positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The core requirements need a clean up |  |  |  |  |  |  |  |  |
| Summary of change: |  | The big CR includes: - RAN4#113 endorsed CRs, - RAN4#113 editorial corrections to the endorsed CR in R4-2416884, - RAN4#112-bis big CR (R4-2416884). The RAN4#113 endorsed CRs: - General: R4-2419762, Big CR to 38133 for RRM core part for expanded and improved NR positioning, Ericsson. - RedCap: R4-2419352, CR to 38.133 on corrections to core requirements for RedCap positioning, Ericsson. R4-2420133, CR on requirements for RedCap positioning, Huawei, HiSilicon. - BW aggregation: R4-2420134, CR to 38.133 on corrections to core requirement for bandwidth aggregation for positioning measurements, Ericsson. R4-2420135, CR on requirements for PRS BWA, Huawei, HiSilicon. R4-2420136, Draft CR on RRM core requirements for PRS BW aggregation, OPPO. - CPP: R4-2420137, CR to 38.133 on corrections to core requirements for CPP, Ericsson. R4-2420138, Draft CR on RRM core requirements for CPP measurements, OPPO. - LPHAP: R4-2419354, CR to 38.133 on corrections to LPHAP core requirements, Ericsson. The RAN4#112-bis endorsed big CR R4-2416884 is based on the following endorsed CRs from RAN4#112-bis: - RedCap: R4-2416885, draftCR on requirements for RedCap positioning, Huawei, HiSilicon. R4-2416321, draftCR to 38.133 on corrections to core requirements for RedCap positioning, Ericsson. - BW aggregation: R4-2416886, Draft CR on core requirements for SRS BW aggregation, CATT. R4-2416887, draftCR on requirements for PRS BWA, Huawei, HiSilicon. - SL: R4-2416888, Draft CR on core requirements for SL positioning, CATT. R4-2416889, Draft CR 38133 on remaining core issues for SL positioning, Ericsson. - CPP: R4-2416890, draftCR on RRM requirements for CPP, Huawei, HiSilicon. R4-2416891,draftCR to 38.133 on corrections to core requirements for CPP,Ericsson. - LPHAP: - The following changes are included in this big CR from RAN4#113: (R4-2420136): Corrections to the following issues: 1) It was agreed to replace Teffect as Tmargin when defining PRS measureement period with BW aggregation. However when Tmagin=0, the total period for measuring aggregated PRS resource, e.g.Taggregated_RSTD, will be wrong. 2) For the interruption requirements at SRS BW aggregation, the victim cell should be indicated by posSRS-BWA-AffectedBandList-r18 regardless of per-FR MG capability. (R4-2420135): Update the margin for switching to account for all PFL combinations and single PFLs (re-using Tmargin which already exists in the requirements) for Idle and Inactive. The changes are same as those for Connected. (R4-2420134): 1) Core requirement for PRS aggregation is updated based on the agreements reached in RAN4#112bis. 2) Clause numbers used in 9.9.1.1 to refer to the accuracy requirement defined for RSTD and UE Rx-Tx measurements based on PRS aggregation are updated. (R4-2420133): Update the definition of Lavailable_PRS,i, Nhop,i,j and NrepPRS based on agreements in RAN4#112 and RAN4#112-bis for Idle and Inactive. The changes are same as those for Connected requirements. (R4-2420137): Clause numbers referenced in CPP core requirement for IDLE and INACTIVE modes are rectified. (R4-2420138): Correcting issues: 1) If a one-shot time window is configured for CPP measurment, the measurement delay should not only consider the length of CPP time window, but also MGL in RRC CONNECTED mode or 10ms window in RRC IDLE/INACTIVE mode. 2) The requirements when UE is configured with time window and capable of measureing both legacy RSTD/UE Rx-Tx measurement and RSCP/RSCPD are not clear in some clauses. 3) Other typos and wording issues. (R4-2419354): Measurement capability for PRS-RSRP and PRS-RSRPP measurements in Clause 4.5.3.3 and Clause 4.5.4.3 respectively is updated to indicate measurement capability in RRC_IDLE mode. (R4-2419352): Clause numbers in core requirement for RedCap positioning are corrected. The following changes are included in this big CR from RAN4#112bis: (R4-2420138): The current requirements on N_(hops,effect) for RedCap FH are based on PRS repetition, but it is not aligned with the current definition of PRS repetition, and it does not work with PRS muting option 2. Summary of change: Update the requirements on N_(hops,effect) for RedCap FH. (R4-2416321): RRC state clarified. (R4-2416886): When UE performs aggregated SRS transmission for positioning, UE may cause interruptions on other serving cells. The interruption requirements on serving CC(s) due to the guard periods before and after the aggregated SRS transmission and the aggregated SRS transmission should be defined. Summary of change: Define interruption requirements on serving CC(s) for SRS bandwidth aggregation positioning measurements. (R4-2416887): 1) Update the margin for switching to account for all PFL combinations and single PFLs (re-using Tmargin which already exists in the requirements). 2) Made some editorial corrections. (R4-2416888): Replace the UE feature group numbers with corresponding signallings in core requirements for SL positioning. (R4-2416889): Removed []. Editorial corrections. (R4-2416890): 1) Define measurement period requirements when aperiodic time window is configured. 2) Align the structure of clause 5.6.7.5 with other clauses for CPP measurements. (R4-2416891): Clause numbers for RSCPD and RSCP measurement report mapping and accuracy requirements are updated to the correct clause numbers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements need a clean up. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (R4-2420136): 9.9.2.10, 9.9.4.9, 8.2.2.2.21. (R4-2420135): 4.5.2.6, 5.6.2.6, 5.6.4.6. (R4-2420134): 9.9.1.1. (R4-2420133): 4.6.2.6, 4.6.3.6, 5.6A.4.6, 5.6A.5.6, 5.6A.6.6. (R4-2420137): 4.5.5.2, 5.6.7.4. (R4-2419352): 4.5.5.5, 5.6.7.5, 5.6.8.5, 9.9.7.5, 9.9.8.5. (R4-2419354): 4.5.3.3, 4.5.4.3. (R4-2419352):4.6.2.4, 5.6A.4.2, 5.6A.5.2, 5.6A.6.2, 5.6A.7.2, 9.9A.1.1, 9.9A.2.2, 9.9A.1.2, 9.9A.2.5.2, 9.9A.3.1, 9.9A.3.2, 9.9A.3.4, 9.9A.3.5.1, 9.9A.4.2, 9.9A.5.1, 9.9A.5.2, 9.9A.5.4. (R4-2416885): 9.9A.2.6.1, 9.9A.3.6.1, 9.9A.4.8. (R4-2416321): 5.6A.4.6, 5.6A.5.6, 5.6A.6.6. (R4-2416886): 8.2.2.2.21. (R4-2416887): 4.5.2.6, 5.6.2.6, 5.6.4.6, 9.9.2.10, 9.9.4.9. (R4-2416888): 12A.2.5, 12A.4.5, 12A.6.5, 12A.7.5. (R4-2416889): 12A.2, 12A.3, 12A.4, 12A.5, 12A.6. (R4-2416890): 4.5.5.5, 5.6.7.5, 5.6.8.5, 9.9.7.5, 9.9.8.5. (R4-2416891): 4.5.5.4, 5.6.7.2, 5.6.7.4, 5.6.7.5, 5.6.8.4, 9.9.7.2, 9.9.7.4, 9.9.8.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## --- Start of Change #0 ---

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[4]	3GPP TS 38.215: "NR; Physical layer measurements".

[5]	3GPP TS 38.533: "NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)".

[6]	3GPP TS 38.211: "NR; Physical channels and modulation”.

[7]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[8]	3GPP TS 38.212 "NR; Multiplexing and channel coding".

[9]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer".

[10]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[11]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[12]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[13]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[14]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[15]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[16]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[17]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity", Stage 2.

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[20]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[21]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[22]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[23]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[24]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA); Overall description".

[25]	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[26]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[27]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[28]	Void.

[29]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[30]	3GPP TS 25.302: "Services provided by the Physical Layer".

[31]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and Next Generation Radio Access; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[32]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[33]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[34]	3GPP TS 37.355: "LTE Positioning Protocol (LPP) ".

[35]	3GPP TS 38.455	: "NG-RAN; NR Positioning Protocol A (NRPPa) ".

[36]	3GPP TS 37.106: "User Equipment (UE) requirements for shared spectrum channel access".

[37]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP) ".

[38]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[39]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA): Physical layer procedures"

[40]	3GPP TS 38.521-2: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[41]	3GPP TS 38.108: “NR; Satellite Access Node radio transmission and reception”.

[42]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[43]	3GPP TS 38.101-5: "NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[44]	3GPP TS 23.273: “5G System (5GS) Location Services (LCS); Stage 2”.

## --- End of Change #0 ---

## --- Start of Change #1 ---

#### 4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequestedPFL-List in NR-DL-TDOA-RequestLocationInformation.

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in clause 4.5.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,total= TRSTD, aggr+ TRSTD,non-aggr + Tmargin

Where:

-	TRSTD, aggr is the total measurement period for aggregated measurements as defined in this clause, and

-	TRSTD,non-aggr is the total measurement period for non-aggregated measurements as defined in this clause, and

-	When both TRSTD, aggr  and TRSTD,non-aggr are non-zero, then Tmargin=Tmargin'; otherwise Tmargin=0, where Tmargin'Tmargin=maxTeffect,j, and the maximum Teffect,j is across all the PFL combination(s) and non-aggregated PFL(s) configured for positioning measurementsTmargin.

TRSTD,non-aggr is zero if every resource set on every PFL is linked for aggregation to at least one other resource set on another PFL. Otherwise, TRSTD,non-aggr is as defined in clause 4.5.2.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L,

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replace by L-1*Tmargin'.

TRSTD,aggr is zero if no resource sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TRSTD,aggr is defined as

TRSTD,aggr= m=1MTRSTD,aggr,m+ M-1*Tmargin'maxTeffect,aggr,m

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m as defined in this clause,

-	TRSTD,aggr,m is the measurement period for PRS RSTD measurement in PFL combination m as specified below.

TRSTD,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_IDLE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRx,TEG,m is the Rx TEG specific scaling factor:

-	NRx,TEG,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,m = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,m = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN or prs-durationOfThreePRS-BWA-ProcessingSymbolsN specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, defined in TS 38.304 [1], clause 7.1.

-	TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

If the following conditions are met, the time TRSTD,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Note:	No separate requirement on non-aggregated measurement based on TRSTD,non-aggr or on aggregated measurement based on TRSTD,aggr is applied when both TRSTD, aggr and TRSTD,non-aggr are non-zero.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 4.5.2.6.

When PRS-RSRPP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 4.5.2.6.

Longer RSTD measurement period is expected when there are collisions between PRS resources in any of the PFLs configured for aggregation and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.5.2.6 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23A.2.

## --- End of Change #1 ---

## --- Start of Change #1a ---

#### 4.5.3.3	Measurement Capability

UE PRS-RSRP measurement capability in RRC_IDLE state is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

## --- End of Change #1a ---

## --- Start of Change #1b ---

#### 4.5.4.3	Measurement Capability

UE PRS-RSRPP measurement capability in RRC_IDLE state is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

## --- End of Change #1b ---

## --- Start of Change #1c ---

#### 4.5.5.2	Requirements Applicability

The requirements in clause 4.5.5 apply for periodic and triggered RSTD and DL RSCPD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

-	DL RSCPD related side conditions given in clause 10.1.43.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

## --- End of Change #1c ---

## --- Start of Change #2 ---

#### 4.5.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. The UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For RSTD and DL RSCPD measurements performed by the UE in RRC_IDLE state, the measurement reporting delay excludes all of the following:

-	additional delay caused by other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by unavailability of UL resources to transmit the measurement report,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.23.3. The reported DL RSCPD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.43.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2, for each measured DL PRS resource. The DL RSCPD measurements performed and reported according to this section shall meet the DL RSCPD measurement accuracy requirements in clause 10.1.43.2, for each measured DL PRS resource.

## --- End of Change #2 ---

## --- Start of Change #3 ---

#### 4.5.5.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] with nr-DL-PRS-RSCPD-Request, the UE shall be able to measure multiple (up to the UE capability specified in clause 4.5.5.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4].

When LMF does not configure measurement time window(s):

-	When a single PFL is configured, requirements in clause 4.5.2.5 apply to both DL RSTD and DL RSCPD.

-	When multiple PFLs are configured, the UE performs DL RSCPD measurement on a single PFL that is common between the reference TRP and the target TRP, and requirements in clause 4.5.2.5 apply to both DL RSTD and DL RSCPD.

When LMF configures measurement time window(s), but UE does not support supportOfRSCPD-MeasurementInTimeWindow:

-	The UE performs DL RSCPD measurement on the indicated PFL by the network. The requirements in clause 4.5.2.5 apply to both DL RSTD and DL RSCPD measurements.

When LMF configures measurement time window(s), and UE supports supportOfRSCPD-MeasurementInTimeWindow but does not support supportOfLegacyMeasurementInTimeWindow:

-	The requirements in the clause 4.5.2.5 apply to DL RSTD measurement.

-	The requirements in clause 4.5.5.5 apply to DL RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

When LMF configures measurement time window(s), and UE supports supportOfRSCPD-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow:

-	The requirements in clause 4.5.5.5 apply to DL RSTD measurement and DL RSCPD measurement.

If a one-shot time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 4.5.5.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the one-shot time window during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=min⁡(Lwindow,10 ms)+ Ti,

and the UE shall be able to measure multiple DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total,with one shot window defined as:

TRSTD,Total, with one shot window=TRSCPD with RSTD,Total+maxTeffect,i+TRSTD,Total

Where:

-	the start of the measurement is the start of the one-shot time window, and

-	Lwindow is the length of the one-shot window, and

-	Teffect,i and Ti are defined in the following of this clause, and

-	TRSTD,Total is defined in clause 4.5.2.5.

The requirements with one-shot window assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

If a periodic time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 4.5.5.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the time window during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=i=1LTRSCPD with RSTD,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSCPD with RSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSCPD with RSTD,i= Kcarrier_PRS* NRx,TEG,i*NRxBeam,i*NPRS,islotN'*Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation.

-	NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_IDLE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,i is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the unmuted PRS resources in the indicated resources sets that are not fully overlapped with other higher-priority DL signals/channels and overlapped the time window(s) are considered if PFL i is associated with the time window are considered, otherwise only the unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Nsample is the number of PRS RSTD samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, Twindow, the least common multiple between the time window periodicity Twindow TPRS,i, and the DRX cycle length TDRX, defined in TS 38.304 [1], clause 7.1, if PFL i is associated with the time window, otherwise Tavailable_PRS,i=LCMTPRS,i,TDRX,

-	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i, and when calculating TPRS,i, only the PRS resources in the indicated resources sets and overlapped with the indicated time window(s) are considered if PFL i is associated with the time window,

-	Twindow is the maximum periodicity of the indicated time window(s).

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

-	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	{N,T} is the UE capability combination per band for RRC_IDLE state where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34],  T (ms) corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], and T-N>0 is the time required to process duration N of DL PRS symbols already buffered in memory, for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot in RRC_IDLE state as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 specified in TS 37.355 [34].

If the following conditions are met, the time TRSTD,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the RSCPD with RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSCPD with RSTD and PRS-RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSCPD with RSTD and PRS-RSRPP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer RSCPD with RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.5.5 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD and DL RSCPD measurements are being performed, then the UE shall continue and complete the on-going RSTD and DL RSCPD measurements after the cell selection is completed. The RSTD and DL RSCPD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the measurement period then the UE shall continue the RSTD and DL RSCPD measurement in the RRC_CONNECTED state. The RSTD and DL RSCPD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2.

The UE shall meet the DL-RSCPD measurement accuracy requirements in clause 10.1.43.2.

## --- End of Change #3 ---

## --- Start of Change #3a ---

#### 4.6.2.4	Measurement Reporting Requirements

The measurement reporting delay shall satisfy the requirements defined in clause 4.6.2.4, with the exception of the applicable measurement accuracy requirements, which are specified below.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.1, for each measured DL PRS resource by 1 Rx RedCap UE without RX FH.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2, for each measured DL PRS resource by 2 Rx RedCap UE without RX FH.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.2, for each measured DL PRS resource by 2 Rx RedCap UE with RX FH.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.2, for each measured DL PRS resource by 1 Rx RedCap UE with RX FH.

## --- End of Change #3a ---

## --- Start of Change #3b ---

#### 4.6.2.6	Measurement Period Requirements with RX FH

The requirements in this clause apply when a RedCap UE is requested by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 4.5.2.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single time window.

-	Lavailable_PRS,i=j=1JNhop,i,j*Lper-hop,i,jNhop,i*Lper-hop,i, where

-	Nhop,i,j is the number of hops that UE can do within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 4.6.2.6-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a time window instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the time window instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 4.6.2.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i ,

where

-	Nhop,max,i is the maximum number of Rx hops signaled as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i,

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

where NrepPRS is the number of PRS repetitions within the time window occasion, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in Table 4.6.2.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap ,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34],

Nhop,i,j is number of hops within a single time window as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

## --- End of Change #3b ---

## --- Start of Change #3c ---

#### 4.6.3.6	Measurement Period Requirements with RX FH

When physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the UE shall be able to measure multiple (up to the UE capability specified in 4.6.3.3) PRS-RSRP measurements, defined in TS38.215 [4], during the measurement period TPRS-RSRP,Total defined in Clause 4.6.3.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS-RSRP measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i ,

where,

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 4.6.3.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 4.6.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 4.6.3.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i	,

where

-	Nhop,max,i is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability for positioning frequency layer i as specified in [34].

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i.

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 4.6.3.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap	,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34].

Nhop,i,j is number of hops within a single time window as defined above.

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

## --- End of Change #3c ---

## --- Start of Change #4 ---

#### 5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequestedPFL-List [34].

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,total= TRSTD, aggr+ TRSTD,non-aggr + Tmargin

Where:

-	TRSTD, aggr is the total measurement period for aggregated measurements as defined in this clause, and

-	TRSTD,non-aggr is the total measurement period for non-aggregated measurements as defined in this clause, and

-	When both TRSTD, aggr  and TRSTD,non-aggr are non-zero,  then Tmargin=Tmargin'; otherwise Tmargin=0, where Tmargin'Tmargin=maxTeffect,j, andthe maximum Teffect,j is across all the PFL combination(s) and non-aggregated PFL(s) configured for positioning measurementsTmargin.

TRSTD,non-aggr is zero if every resource set on every PFL is linked for aggregation to at least one other resource set on another PFL. Otherwise, TRSTD,non-aggr is as defined in clause 5.6.2.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L,

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replace by L-1*Tmargin'.

TRSTD,aggr is zero if no resource sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TRSTD,aggr is defined as

TRSTD,aggr= m=1MTRSTD,aggr,m+ M-1*Tmargin'maxTeffect,aggr,m     ,

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m as defined in this clause,

-	TRSTD,aggr,m is the measurement period for PRS RSTD measurement in PFL combination m as specified below.

TRSTD,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m ,

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_INACTIVE,

-	Kcarrier,aggr=1 if the UE supports parallelPRS-MeasRRC-Inactive-r17,

-	Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier,aggr=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier,aggr=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,aggr,m is a scaling factor for PRS measurements with multiple Rx TEGs

-	NRx,TEG,aggr,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,aggr,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2 specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, where TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m.

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements are transmitted by the TRP using single Tx chain as defined in clause 5.1.6.5.3 in TS 38.214 [26].

If PRS resources in one or more of the aggregated PFLs in PFL combination m are dropped because of collision with other signals, TRSTD,aggr,m can be longer than specified above.

The time TRSTD,Total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No separate requirement on non-aggregated measurement based on TRSTD,non-aggr or on aggregated measurement based on TRSTD,aggr is applied when both TRSTD, aggr and TRSTD,non-aggr are non-zero.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period. When PRS-RSRPP is configured for DL-TDOA, RSTD and PRS-RSRPP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6.2 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the RSTD measurement period, then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23A.2.

## --- End of Change #4 ---

## --- Start of Change #5 ---

#### 5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequestedPFL-List [34].

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.4.3) UE Rx-Tx time difference measurements, defined in TS 38.215 [4], during the measurement period TUERxTx,Total defined as:

TUERxTx,total= TUERxTx, aggr+ TUERxTx,non-aggr + Tmargin

Where:

-	TUERxTx, aggr is the total measurement period for aggregated measurements as defined in this clause, and

-	TUERxTx,non-aggr is the total measurement period for non-aggregates measurements as defined in this clause, and

-	Tmargin=Tmargin' is maxTeffect,j, if both TUERxTx, aggr  and TUERxTx,non-aggr are non-zero, and Tmargin= 0 otherwise, where Tmargin'=maxTeffect,j and Teffect,j is the Teffect from both PFL combination(s) and non-aggregated PFL(s) which are configured for positioning measurementTmargin.

TUERxTx,non-aggr is zero if every resource set on every PFL is linked for aggregation with at least one other resource set on another PFL. Otherwise, TUERxTx,non-aggr is as defined in clause 5.6.4.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L,

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replace by L-1*Tmargin'.

TUERxTx,aggr is zero if no resourse sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TUERxTx,aggr is defined as

TUERxTx,aggr= m=1MTUERxTx,aggr,m+ M-1*Tmargin'maxTeffect,aggr,m    ,

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m as defined in this clause,

-	TUERxTx,aggr,m is the measurement period for UE Rx-Tx time difference measurement in PFL combination m as specified below.

TUERxTx,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_INACTIVE, and is defined as:

-	Kcarrier,aggr=1 if the UE supports parallelPRS-MeasRRC-Inactive-r17,

-	Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier,aggr=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier,aggr=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,aggr,m is a scaling factor for PRS measurements with multiple Rx TEGs, and is defined as:

-	NRx,TEG,aggr,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,aggr,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2 specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, where TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m.

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements are transmitted by the TRP using single Tx chain as defined in clause 5.1.6.5.3 in TS 38.214 [26].

If PRS resources in one or more of the aggregated PFLs in PFL combination m are dropped because of collision with other signals, TRSTD,aggr,m can be longer than specified above.

The time TUERxTx,total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message are delivered from LMF to the UE via LPP [34].

Note:	No separate requirement on non-aggregated measurement based on TUERxTx,non-aggr or on aggregated measurement based on TUERxTx,aggr is applied when both TUERxTx, aggr and TUERxTx,non-aggr are non-zero.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the UE Rx-Tx time difference measurement period then the UE Rx-Tx time difference measurement period can be longer.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

If during UE Rx-Tx time difference measurement period PRS resources overlap with other DL signals/channels then the UE Rx-Tx time difference measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period. When PRS-RSRPP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRPP are performed over the same measurement period.

The requirements in clause 5.6.4 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25A.2.

## --- End of Change #5 ---

## --- Start of Change #6 ---

#### 5.6.7.2	Requirements Applicability

The requirements in clause 5.6.7 apply for periodic and triggered reporting of RSCPD with RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

-	RSCPD related side conditions given in clause 10.1.43.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

## --- End of Change #6 ---

## --- Start of Change #7 ---

#### 5.6.7.4	Measurement Reporting Requirements

The measurement reporting delay shall satisfy the requirements defined in clause 5.6.2.4, and the applicable measurement report mapping and measurement accuracy requirements are specified below.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.23.3.

The reported RSCPD measurement values contained in the measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.43.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2, for each measured DL PRS resource.

The RSCPD measurements performed and reported according to this section shall meet the RSCPD measurement accuracy requirements in clause 10.1.43.2, for the measured DL PRS resource.

## --- End of Change #7 ---

## --- Start of Change #8 ---

#### 5.6.7.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message with nr-DL-PRS-RSCPD-Request from the LMF via LPP [34], when LMF configures measurement time window(s) for a PFL, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.7.3) DL RSTD and RSCPD measurements, defined in TS 38.215 [4], during the time window configured to UE via nr-DL-PRS-MeasurementTimeWindowsConfig.TRSCPD with RSTD,Total

When LMF does not configure measurement time window(s):

-	When a single PFL is configured, requirements in clause 5.6.2.5 apply to both RSCPD and RSTD measurements.

-	When multiple PFLs are configured for legacy measurements, the UE performs RSCPD measurement on a single PFL that is common between the reference TRP and the target TRP. The requirement in clause 5.6.2.5 apply to both RSTD and RSCPD measurements.

When LMF configures measurement time window(s), but UE does not support supportOfRSCPD-MeasurementInTimeWindow:

-	The UE performs RSCPD measurement on the indicated PFL by the network. The requirements in clause 5.6.2.5 apply to both RSTD and RSCPD measurements.

When LMF configures measurement time window(s), but UE does not support supportOfLegacyMeasurementInTimeWindow but supports supportOfRSCPD-MeasurementInTimeWindow:

-	The requirements in the clause 5.6.2.5 apply to RSTD measurements.

-	The requirements in clause 5.6.7.5 apply to RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

When LMF configures measurement time window(s), and UE supports supportOfRSCPD-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow:

-	The requirements in Clause 5.6.7.5 apply to DL RSTD measurement and DL RSCPD measurement.

If a one-shot time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.7.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the one-shot time window during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=min⁡(Lwindow,10 ms)+Ti	,

and the UE shall be able to measure multiple DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total,with one shot window defined as:

TRSTD,Total, with one shot window=TRSCPD with RSTD,Total+maxTeffect,i+TRSTD,Total

Where:

-	the start of the measurement is the start of the one-shot time window, and

-	Lwindow is the length of the one-shot window, and

-	Teffect,i and Ti are defined in the following of this clause, and

-	TRSTD,Total is defined in clause 5.6.2.5.

The requirements with one-shot window assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

If a periodic time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.8.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=i=1LTRSCPD with RSTD,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSCPD with RSTD,i is the measurement period for PRS RSTD with RSCPD measurement in positioning frequency layer i as specified below:

TRSCPD with RSTD=Kcarrier_PRS* NRx,TEG,i*NRxBeam,i*NPRS,islotN'*LavailablePRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

-	Kcarrier_PRS, NRx,TEG,i, NRxBeam,i, NPRS,islot, N, N’, Nsample, Teffect,i and Tlast,i are defined in clause 5.6.2.5

-	LavailablePRS,i is the time duration of available PRS in the positioning frequency layer to be measured during TPRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	When periodic time window(s) are configured by the LMF and PFL i is associated with the configured time window, TavailablePRS,i=LCMTPRS,i,TDRX, Twindow, the least common multiple between TPRS,i , the DRX cycle length TDRX and Twindow being the maximum periodicity of the indicated time window(s). TPRS.i is defined in clause 5.6.2.5.

-	When periodic time window(s) are configured by the LMF and PFL i is not associated with the configured time window(s), TavailablePRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX .  TPRS,i is defined in clause 5.6.2.5.

-	LavailablePRS,i and TPRS,i are calculated by only considering the PRS resources in the indicated resources sets overlapping with the indicated time window(s).

The time TRSCPD with RSTD starts from the first time window (TDL RSCPD) configured by LMF within DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

If the DRX cycle is reconfigured during the measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSCPD with RSTD and PRS-RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSCPD with RSTD and PRS-RSRPP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for the PFL during the measurement period, the measurement period can be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6.7 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSCPD together with RSTD measurements are being performed, then the UE shall continue and complete the on-going RSCPD and RSTD measurements after the cell re-selection is completed. The measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period then the UE shall continue the RSCPD and RSTD measurements in the RRC_CONNECTED state. The measurement period can be longer.

## --- End of Change #8 ---

## --- Start of Change #9 ---

#### 5.6.8.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

This measurement reporting delay excludes the delay caused by any of the following:

-	delay caused by other LPP signalling on the DCCH.

-	delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

-	delay caused due to lack of UL resources for UE to send the measurement report.

-	delay required by SDT for reporting the measurement using SDT resouces.

-	delay required for transition to RRC_CONNECTED state for report the measurement in RRC_CONNECTED.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.25.3.

The DL RSCP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.44.3.

The UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25.2.

The DL RSCP measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.44.2.

## --- End of Change #9 ---

## --- Start of Change #10 ---

#### 5.6.8.5	Measurement Period Requirements

When the physical layer receives the last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with nr-UE-RSCP-Request and configuring a measurement time window via nr-DL-PRS-MeasurementTimeWindowsConfig, subject to UE capabilities supportOfRSCP-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the time window only.

When LMF does not configure measurement time window(s):

-	When a single PFL is configured, requirements in clause 5.6.4.5 apply to both RSCP and UE Rx-Tx measurements.

-	When multiple PFLs are configured for legacy measurements, the UE performs RSCP measurement on a single PFL that is common between the reference TRP and the target TRP. The requirement in clause 5.6.4.5 apply to both RSCP and UE Rx-Tx measurements.

When LMF configures measurement time window(s), but UE does not support supportOfRSCP-MeasurementInTimeWindow:

-	The UE performs RSCP measurement on the indicated PFL by the network. The requirement in clause 5.6.4.5 apply to both UE Rx-Tx and RSCP measurements.

When LMF configures measurement time window(s), but UE does not support supportOfLegacyMeasurementInTimeWindow but supports supportOfRSCP-MeasurementInTimeWindow:

-	The requirements in the clause 5.6.4.5 apply to UE Rx-Tx measurement.

-	The requirements in clause 5.6.8.5 apply to RSCP measurement for the PRS resource(s) that have occasions only within the measurement time window.

When LMF configures measurement time window(s), and UE supports supportOfRSCP-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow:

-	The requirements in Clause 5.6.8.5 apply to UE Rx-Tx measurement and DL RSCP measurement.

If a one-shot time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the one-shot time window during the measurement period TDL RSCP with UERxTx,Total defined as:

TDL RSCP with UERxTx,Total=min⁡(Lwindow,10 ms)+Ti	,

and the UE shall be able to measure multiple UE Rx-Tx measurements, defined in TS 38.215 [4], during the measurement period TUERxTx,Total,with one shot window defined as:

TUERxTx,Total,with one shot window=TDL RSCP with UERxTx,Total+maxTeffect,i+TUERxTx,Total

Where:

-	the start of the measurement is the start of the one-shot time window, and

-	Lwindow is the length of the one-shot window, and

-	Teffect,i and Ti are defined in the following of this clause, and

-	TUERxTx,Total is defined in clause 5.6.4.5.

The requirements with one-shot window assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

If a periodic time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the measurement period TDL RSCP with UERxTx,Total defined as:

TRSCP with UERxTx,Total=i=1LTDL RSCP with UERxTx,i+ L-1*maxTeffect,i ,

where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i.

TDL RSCP with UERxTx,i is the measurement period for DL RSCP with UE Rx-Tx measurement in positioning frequency layer i as specified below:

TDL RSCP with UERxTx,i=Kcarrier_PRS*NRxTx,TEG,i*NRxBeam,i*NPRS,islotN'LavailablePRS,iN*Nsample-1*Teffect,i+Tlast,i,

where:

-	Kcarrier_PRS, NRxTx,TEG,i, NRxBeam,i, NPRS,islot, N, N’, Nsample, Teffect,i and Tlast,i are defined in clause 5.6.4.5.

-	DL-RSCP performed during TDL RSCP with UERxTx is a single sample measurement where DL RSCP and UE Rx-Tx measurements are performed on the same PFL.

-	LavailablePRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	When periodic time window(s) are configured by the LMF and PFL i is associated with the configured time window, TavailablePRS,i= LCMTPRS,i,TDRX, Twindow, the least common multiple between TPRS,i , TDRX and Twindow being the maximum periodicity of the indicated time window(s).

-	When periodic time window(s) are configured by the LMF and PFL i is not associated with the configured time window(s), TavailablePRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX .

-	LavailablePRS,i and TPRS,i are calculated by only considering the PRS resources in the indicated resources sets overlapping with the indicated time window(s).

The time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within DRX cycle containing the DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period then the UE shall continue the DL RSCP measurement and shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the measurement period then the UE shall restart the DL RSCP and UE Rx-Tx time difference measurements after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration LavailablePRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the measurement period, then the measurement period can be longer.

If during the measurement period, PRS resources overlap with other DL signals/channels then the measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the DL RSCP with UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

When PRS-RSRPP is configured for multi-RTT, the DL RSCP with UE Rx-Tx time difference measurements and PRS-RSRPP measurements are performed over the same measurement period.

The requirements in clause 5.6.8 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the DL RSCP with UE Rx-Tx measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in table 7.1.2-2 during the DL RSCP with UE Rx-Tx measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the measurement period, then:

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The DL RSCP with UE Rx-Tx time difference measurement period may be restarted in such case.

## --- End of Change #10 ---

## --- Start of Change #11 ---

### 5.6A.4	RSTD measurements for RedCap

#### 5.6A.4.1		Introduction

The requirements in clause 5.6A.4.5 shall apply provided the RedCap UE has received NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] requesting the RedCap UE to measure and report DL RSTD measurements defined in TS 38.215 [4] without FH via nr-DL-PRS-RxHoppingRequest.

The requirements in clause 5.6A.4.6 shall apply provided the RedCap UE has received NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] requesting the UE to measure and report DL RSTD measurements defined in TS 38.215 [4] with FH via nr-DL-PRS-RxHoppingRequest.

#### 5.6A.4.2	Requirements applicability

The requirement in clause 5.6A.4.5 apply for periodic and triggered RSTD measurements, provided:

PRS-RSTD related side conditions given in clause 10.1A.16.2.1 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

PRS-RSTD related side conditions given in clause 10.1.23.2 for FR1 and FR2 are fulfilled, for a corresponding band, for 2 Rx RedCap UE.

The requirement in clause 5.6A.4.6 apply for periodic and triggered RSTD measurements, provided:

PRS-RSTD related side conditions given in clause 10.1A.16.2.2 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

PRS-RSTD related side conditions given in clause 10.1A.16.2.2 for FR1 and FR2 are fulfilled, for a corresponding band, for 2 Rx RedCap UE.

#### 5.6A.4.3	Measurement Capability

The UE PRS RSTD measurement capability in RRC_INACTIVE state is as indicated by the UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355 [34].

#### 5.6A.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. If the UE supports reporting of NR positioning measurements via SDT, the UE may be able to report the measurements while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For RSTD measurements performed by the UE in RRC_INACTIVE state, with and without FH, the measurement reporting delay excludes all of the following:

-	additional delay caused other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by unavailability of UL resources to transmit the measurement report,

-	any transmission delay needed by SDT,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.23.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.1, for each measured DL PRS resource by 1 Rx RedCap UE without FH.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.2, for each measured DL PRS resource by 1 Rx RedCap UE with FH.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.1, for each measured DL PRS resource by 2 Rx RedCap UE without FH.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.2, for each measured DL PRS resource by 2 Rx RedCap UE with FH.

#### 5.6A.4.5	Measurement Period Requirement without RX FH

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message from the LMF via LPP [34], and UE is not configured to perform measurement with RX FH, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6A.4.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1×maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i= Kcarrier_PRS_RedCap× NRx,TEG,i×NRxBeam,i×NPRS,islotN'×Lavailable_PRS,iN×Nsample-1×Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1 or UE has only 1Rx branch,

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2, if positioning frequency layer i is in FR2 or the UE has 2Rx branches, and the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation.

-	NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS_RedCap is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS_RedCap = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,i is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Nsample is the number of PRS RSTD samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i×Tavailable_PRS,i ,

where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX

- When UE is configured with RAN eDRX_INACTIVE ≤ 10.24s:

TDRX is calculated as T defined in TS 38.304 [1] when CN eDRX_INACTIVE ≤ 10.24s.

TDRX is calculated as max(Tinside, Toutside), where Tinside and Toutside of the CN PTW as defined in TS 38.304 [1].

- When UE is configured with RAN eDRX_INACTIVE > 10.24s:

TDRX is calculated as max(TDRX_RAN, TDRX_CN), TDRX_RAN and TDRX_CN are DRX cycles with the RAN PTW and CN PTW defined in TS 38.304 [1].

Otherwise, TDRX is the DRX cycle of the UE in the serving cell.

-	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

-	TperPRS with muting=Nmuting×TperPRS, is the PRS periodicity with muting per PRS resource,

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS×Lmuting, where

-	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	{N,T} is the UE capability combination per band for RRC_INACTIVE state where N is the duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34],  T (ms) corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], and T-N (>0) is the time required to process duration N of DL PRS symbols already buffered in memory, for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

-	N’ is UE capability for number of DL PRS resources that it can process in a slot in RRC_INACTIVE state as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 specified in TS 37.355 [34].

If the following conditions are met, the time TRSTD,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with RAN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	RAN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOAProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

If eDRX_INACTIVE cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSTD and PRS-RSRPP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6A.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS_RedCap changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.6A.4.5 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the RSTD measurement period, then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

#### 5.6A.4.6	Measurement Period Requirement with RX FH

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 5.6A.4.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in clause 5.6A.4.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i	,

where

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 5.6A.4.6-1, and RRTFH is the RF retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 5.6A.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table5.6A.4.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i ,

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i,

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 5.6A.4.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap ,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34],

Nhop,i,j is number of hops within a single time window as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

### 5.6A.5	PRS-RSRP measurements for RedCap

#### 5.6A.5.1		Introduction

The requirements in clause 5.6A.5.5 shall apply provided the RedCap UE has received NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4] without FH via nr-DL-PRS-RxHoppingRequest. And the UE is capable of supporting the PRS-RSRP measurement in RRC_INACTIVE state.

The requirements in clause 5.6A.5.6 shall apply provided the RedCap UE has received NR-DL-AoD-RequestLocationInformation message from the LMF via LPP [34] requesting the UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4] with FH via nr-DL-PRS-RxHoppingRequest. And the UE is capable of supporting the PRS-RSRP measurement in RRC_INACTIVE state.

#### 5.6A.5.2	Requirements applicability

The requirements in clause 5.6A.5.5 apply for periodic and triggered PRS-RSRP measurements, provided:

PRS-RSRP related side conditions given in clause 10.1A.17.2 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

PRS-RSRP related side conditions given in clause 10.1.24.2 are met for a corresponding Band, for 2Rx RedCap UE.

The requirements in clause 5.6A.5.6 apply for periodic and triggered PRS-RSRP measurements, provided:

PRS-RSRP related side conditions given in clause 10.1A.17.2 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

PRS-RSRP related side conditions given in clause 10.1.24.2 are met for a corresponding Band, for 2Rx RedCap UE.

#### 5.6A.5.3	Measurement Capability

UE PRS-RSRP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

#### 5.6A.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE is ready to transmit the measurement report over the air interface. If the UE supports reporting of NR positioning measurements via SDT, the UE may be able to report the measurements while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the measurement report.

For PRS-RSRP measurements performed by the UE in RRC_INACTIVE state, the measurement reporting delay excludes all of the following:

-	any delay caused other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH which is equal to 2 × TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by no UL resources for UE to send the measurement report,

-	any transmission delay needed by SDT,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

The reported PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.24.3.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requirements specified in the clause 10.1A.17.2, for each measured DL PRS resource by 1 Rx RedCap UE without FH.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requirements specified in the clause 10.1A.17.2, for each measured DL PRS resource by 1 Rx RedCap UE with FH.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requirements specified in the clause 10.1A.17.2, for each measured DL PRS resource by 2 Rx RedCap UE without FH.

The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requirements specified in the clause 10.1A.17.2, for each measured DL PRS resource by 2 Rx RedCap UE with FH.

#### 5.6A.5.5	Measurement Period Requirements without RX FH

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], and UE is not configured to perform measurement with RX FH, the UE shall be able to measure multiple (up to the UE capability specified in clause 5.6A.5.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1×maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=Kcarrier_PRS_RedCap×NRxBeam,i×NPRS,islotN'Lavailable_PRS,iN×Nsample-1×Teffect,i+Tlast,iWhere:

-	Kcarrier_PRS_RedCap is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1 or UE has only 1Rx branch,

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2, if positioning frequency layer i is in FR2 or the UE has 2Rx branches, and the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation.

-	NRxBeam,i equals to 8, otherwise.

-	Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of PRS-RSRP measurement samples and

-	Nsample= 1, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamplesrequestedDL-PRS-ProcessingSamples [34], and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise

-	Tlast,i = Ti + Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

-	Teffect,i= TiTavailable_PRS,i×Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and the DRX cycle length TDRX

- When UE is configured with RAN eDRX_INACTIVE ≤ 10.24s:

TDRX is calculated as T defined in TS 38.304 [1] when CN eDRX_INACTIVE ≤ 10.24s.

TDRX is calculated as max(Tinside, Toutside), where Tinside and Toutside of the CN PTW as defined in TS 38.304 [1].

When UE is configured with RAN eDRX_INACTIVE > 10.24s:

TDRX is calculated as max(TDRX_RAN, TDRX_CN), TDRX_RAN and TDRX_CN are DRX cycles with the RAN PTW and CN PTW defined in TS 38.304 [1].

Otherwise, TDRX is the DRX cycle of the UE in the serving cell.

-	TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

-	TDRX is the DRX cycle length.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting×TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS×Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

When UE is configured with DRX_cycle, the time TPRS-RSRP,total starts from the first DRX cycle containing the DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

If the following conditions are met, the time TPRS-RSRP, total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation messages are delivered from LMF to the UE via LPP [34].

-	UE is configured with RAN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	RAN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOAProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation messages are delivered from LMF to the UE via LPP [34].

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 5.6A.4.5.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 5.6A.6.5.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state.

The requirements in clause 5.6A.5.5 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If the DRX cycle is reconfigured during the PRS-RSRP measurement period, then the PRS-RSRP measurement period can be longer.

If the eDRX_INACTIVE cycle is reconfigured during the PRS-RSRP measurement period, then the PRS-RSRP measurement period can be longer.

If cell reselection occurs while PRS-RSRPP measurement is being performed, then the UE shall continue and complete the on-going PRS-RSRP measurement after the cell selection is completed. The PRS-RSRP measurement period can be longer.

If the UE’s RRC state changes from the RRC_INACTIVE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

#### 5.6A.5.6	Measurement Period Requirement with RX FH

When physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the UE shall be able to measure multiple (up to the UE capability specified in 5.6A.5.3) PRS-RSRP measurements, defined in TS38.215 [4], during the measurement period TPRS-RSRP,Total defined in clause 5.6A.5.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS-RSRP measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i ,

where

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in a time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 5.6A.5.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 5.6A.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table5.6A.5.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i ,

where

-	Nhop,max,i is the maximum number of Rx hops as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability for positioning frequency layer i as specified in [34],

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i,

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window,  which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 5.6A.5.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap ,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34],

Nhop,i,j is number of hops within a single time window as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34],

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

### 5.6A.6	UE Rx-Tx time difference measurements for RedCap

#### 5.6A.6.1	Introduction

The requirements in clause 5.6A.6.5 shall apply, provided the RedCap UE has received NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4] without FH via nr-DL-PRS-RxHoppingRequest.

The requirements in clause 5.6A.6.6 shall apply, provided the RedCap UE has received nr-Multi-RTT-ReqstLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4] with FH via nr-DL-PRS-RxHoppingRequest.

#### 5.6A.6.2	Requirements Applicability

The requirements in clause 5.6A.6.5 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

- UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.18.2.2 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25.2 are met for a corresponding band, for 2Rx RedCap UE.

-	SRS is configured on the PCell.

The requirements in clause 5.6A.6.6 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

- UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.18.2.4 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.18.2.3 are met for a corresponding band, for 2RX RedCap UE.

-	SRS is configured on the PCell.

#### 5.6A.6.3	Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

#### 5.6A.6.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

This measurement reporting delay excludes the delay caused by any of the following:

-	delay caused by other LPP signalling on the DCCH.

-	delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

-	delay caused due to lack of UL resources for UE to send the measurement report.

-	delay required by SDT for reporting the measurement using SDT resouces.

-	delay required for transition to RRC_CONNECTED state for report the measurement in RRC_CONNECTED.

The UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1A.18.2.2.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1A.18.2.1, for each measured DL PRS resource by 1 Rx RedCap UE without FH.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement measurement accuracy requirements in clause 10.1A.18.2.4, for each measured DL PRS resource by 2 Rx RedCap UE without FH.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement measurement accuracy requirements in clause 10.1A.18.2.3, for each measured DL PRS resource by 1 Rx RedCap UE with FH.

The UE Rx-Tx time difference measurement performed and reported according to this section shall meet the UE Rx-Tx time difference measurement measurement accuracy requirements in clause 10.1A.25.x.x.z, for each measured DL PRS resource by 2 Rx RedCap UE with FH.

#### 5.6A.6.5	Measurement Period Requirements without RX FH

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], and UE is not configured to perform measurement with RX FH, UE shall be able to measure multiple (up to the UE capability specified in clause 5.6A.6.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total defined in 5.6.4.5.

If the RRC state transion occurs from RRC_INACTIVE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the UE Rx-Tx time difference measurement period then the UE Rx-Tx time difference measurement period can be longer.

If during UE Rx-Tx time difference measurement period PRS resources overlap with other DL signals/channels then the UE Rx-Tx time difference measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

When PRS-RSRPP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRPP measurements are performed over the same measurement period.

The requirements in clause 5.6.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25.2.

#### 5.6A.6.6	Measurement Period Requirements with RX FH

When physical layer receives NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 5.6A.6.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4], during the measurement period TUE RxTx,  Total defined in clause 5.6.4.5 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,jNhop,i×Lper-hop,i ,

where

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 5.6A.6.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 5.6A.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table5.6A.6.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz in FR1, and 60kHz, 120kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i ,

where

-	Nhop,max,i is the maximum number of Rx hops as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i ,

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i,

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 5.6A.6.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap ,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34],

Nhop,i,j is number of hops within a single time window as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34],

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

## --- End of Change #11 ---

## --- Start of Change #11a ---

#### 5.6A.7.2	Requirements applicability

The requirements in clause 5.6A.7.5 apply for periodic and triggered PRS-RSRPP measurements, provided:

-	PRS-RSRPP related side conditions given in clause 10.1A.19.2 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

-	PRS-RSRPP related side conditions given in clause 10.1.38.2 are met for a corresponding Band, for 2Rx RedCap UE.

The requirements in clause 5.6A.7.6 apply for periodic and triggered PRS-RSRPP measurements, provided:

-	PRS-RSRPP related side conditions given in clause 10.1A.19.2 for FR1 are fulfilled, for a corresponding band, for 1 Rx RedCap UE.

-	PRS-RSRPP related side conditions given in clause 10.1A.38.2 are met for a corresponding Band, for 2Rx RedCap UE.

## --- End of Change #11a ---

## --- Start of Change #12 ---

##### 8.2.2.2.21	Interruptions at NR SRS bandwidth aggregation for positioning

The requirements in this clause are applicable to SRS transmission for positioning with bandwidth aggregation, when SRS resource configured in a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured in an UL active BWP of a CC with PUSCH/PUCCH.

SRS resource sets across two or three CCs are aggregated for positioning when UE is configured with linkage information SRS-PosResourceSetLinkedForAggBWList specified in TS 37.355 [34], provided that:

-	the aggregated SRS transmission satisfy all the conditions defined in clause 6.2.1.4.2 in TS 38.214 [26];

-	the aggregated SRS transmission and the guard period before and after the SRS transmission are not colliding with any other UL transmission or/and DL reception with higher priority in victim cells defined in TS 38.214 [26];

-	the aggregated SRS transmission plus guard period before and after does not collide with any NR L3 or L1 measurements on victim cells.

For interruption caused by SRS transmission for positioning with bandwidth aggregation, the victim cell is identified based on the entry number of the band indicated by UE in posSRS-BWA-AffectedBandList-r18 regardless of per-FR MG capability.

NOTE 1: UE may indicate no other bands in the band combination are affected by the SRS carrier switching, in which case, only the band with the aggregated SRS transmissions is affected.

NOTE 2: If the capability is not indicated, interruption is allowed on all serving cells.

For each SRS transmission, interruption is allowed on any active serving cell on the band as indicated in posSRS-BWA-AffectedBandList-r18, during the guard periods before and after aggregated SRS transmission and the aggregated SRS transmission duration with up to X OFDM symbols, where X equals to (Y+Z+1). Interruption length of one extra OFDM symbol is added to cover the impact of RTD.

The supported guard period is reported by UE in guardPeriod in NR-UL-SRS-Capability [34] and the total duration Y of guard periods before and after aggregated SRS transmission is converted into number of OFDM symbols in table 8.2.2.2.21-1.

Table 8.2.2.2.21-1: Total duration Y of guard periods before and after aggregated SRS transmission (OFDM symbols)

|  | Guard period (s) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 0 | 30 | 100 | 140 | 200 |
| 15 | 0 | 1 | 3 | 4 | 6 |
| 30 |  | 2 | 6 | 8 | 12 |
| 60 |  | 4 | 12 | 16 | 23 |
| 120 |  | 7 | 23 | 32 | 45 |

The duration of aggregated SRS transmission is given by nrofSymbols in corresponding SRS-Resource configurations [34]. This duration in number of nrofSymbols OFDM symbols is converted to Z OFDM symbols in victim cells according to the SCS configurations among the aggressor cell and victim cell(s). Z is specified in table 8.2.2.2.21-2, table 8.2.2.2.21-3 and table 8.2.2.2.21-4 for nrofSymbols equal to {1, 2, 4} respectively with different SCS configurations.

Table 8.2.2.2.21-2: Duration Z of aggregated SRS transmission with nrofSymbols = 1 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 1 | 1 | 1 | 1 | 1 | 1 |
| 30 | 2 | 1 | 1 | 1 | 1 | 1 |
| 60 | 4 | 2 | 1 | 1 | 1 | 1 |
| 120 | 8 | 4 | 2 | 1 | 1 | 1 |

Table 8.2.2.2.21-3: Duration Z of aggregated SRS transmission with nrofSymbols = 2 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 2 | 1 | 1 | 1 | 1 | 1 |
| 30 | 4 | 2 | 1 | 1 | 1 | 1 |
| 60 | 8 | 4 | 2 | 1 | 1 | 1 |
| 120 | 16 | 8 | 4 | 2 | 1 | 1 |

Table 8.2.2.2.21-4: Duration Z of aggregated SRS transmission with nrofSymbols = 4 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 | 480 | 960 |
| 15 | 4 | 2 | 1 | 1 | 1 | 1 |
| 30 | 8 | 4 | 2 | 1 | 1 | 1 |
| 60 | 16 | 8 | 4 | 2 | 1 | 1 |
| 120 | 32 | 16 | 8 | 4 | 1 | 1 |

## --- End of Change #12 ---

## --- Start of Change #12a ---

#### 9.9.1.1	General Aspects of Gap-based Measurement

For gap-based RSTD, RSTD based on aggregated PRS resources from multiple PFLs, PRS-RSRP, UE Rx-Tx time difference, UE Rx-Tx time difference based on aggregated PRS resources and aggregated SRS resources from multiple PFLs, PRS-RSRPP, DL RSCPD reported with RSTD, DL RSCP reported with UE Rx-Tx time difference  measurements, the requirements in clauses 9.9.2.5, 9.9.2.10, 9.9.3.5, 9.9.4.5, 9.9.4.9, 9.9.6.5, 9.9.7, 9.9.8 apply provided:

-	the UE is configured or pre-configured with measurement gaps or configured with concurrent measurement gaps

-	all positioning frequency layers are measured or associated with only one per-UE measurement gap, or

-	for the UE supporting independentGapConfigPRS-r17, all positioning frequency layers in the same FR are measured or associated with only one per-FR measurement gap in the corresponding FR.

-	if the measurement gap is pre-configured, the gap must be activated throughout the measurement period, and

-	if concurrent measurement gaps are configured, one of the gap combinations specified in clause 9.1.8.2 is configured, and

-	if the UE does not support independentGapConfigPRS-r17, the configured or pre-configured gap used to perform the PRS measurements must be of per-UE type, and

-	No active BWP switching occurs during the measurement gaps for PRS measurement.

All measurement requirements specified in clauses 9.9.2.5, 9.9.2.10, 9.9.3.5, 9.9.4.5, 9.9.4.9, 9.9.6.5, 9.9.7, and 9.9.8 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

UE is only required to measure PRS resources that are fully or partially overlapped with measurement gaps, and the requirements in clauses 9.9.2.5, 9.9.2.10, 9.9.3.5, 9.9.4.5, 9.9.4.9, 9.9.6.5, 9.9.7, and 9.9.8 are applicable to PRS resources that are fully or partially overlapped with measurement gaps.

A PRS resource is considered to be fully (partially) overlapped with measurement gaps if all (some) of its instances are overlapped with a measurement gap occasion. A PRS resource instance is considered to be overlapped with measurement gap occasion if the minimum number of unmuted repetitions of the instance taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD is fully covered by the MGL excluding RF switching time, where the minimum number is given in the accuracy requirements in clause 10.1.23, 10.1.23A, 10.1.24, 10.1.25, 10.1.25A, and 10.1.38 for RSTD, RSTD based on aggregated PRS resources from multiple PFLs, PRS-RSRP, UE Rx-Tx time difference and UE Rx-Tx time difference based on aggregated PRS resources and aggregated SRS resources from multiple PFLs,  PRS-RSRPP, respectively.

When UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

If a positioning measurement gap is configured via PosGapConfig and activated by MAC CE, the measurement requirements in clauses 9.9.2.5, 9.9.2.10, 9.9.3.5, 9.9.4.5, 9.9.4.9, 9.9.6.5, 9.9.7, and 9.9.8 apply provided that no other MGs are configured, and only one measurement gap configured via PosGapConfig is activated.

When UE supports musim-GapPreference-r17 and if the configured aperiodic MUSIM gap collides with the measurement gap associated with PRS measurement, where MUSIM gap collision rule in section 9.1.10.4 is applied, longer measurement period for the PRS measurement is expected than the requirements as defined in clause 9.9.2.5, 9.9.2.10, 9.9.3.5, 9.9.4.5, 9.9.4.9, 9.9.6.5, 9.9.7, and 9.9.8.

## --- End of Change #12a ---

## --- Start of Change #13 ---

#### 9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation

When UE physical layer receives last of NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] with a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List, the UE shall be able to perform RSTD measurement by aggregating PRS resources from multiple PFLs (up to the UE capability specified in clause 9.9.2.3) defined in TS 38.215 [4], during the measurement period TRSTD_aggregate, Total defined as:

TRSTD_aggregate, Total = Tnon_aggregate_RSTD + Taggregate_RSTD + Tmargin,

where,

Tnon_aggregate_RSTD is the total measurement period for RSTD measurement on PFLs that do not contain PRS resources for aggregation.

Calculation of Tnon_aggregate_RSTD is based on clause 9.9.2.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replace by L-1*T'margin.

Tnon_aggregate_RSTD is equal to zero if UE is not configured to perform non-aggregated measurements by the LMF.

If UE is configured to perform positioning measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation, Tmargin= T´margin is delay margin to account for delay between RSTD measurement performed by UE on PFLs that contain PRS resources for aggregation and RSTD measurement performed by UE on PFLs that do not contain PRS resources for aggregation.

If UE is only configured to perform measurements on PFLs that contain PRS resources for aggregation or if UE is only configured to perform measurements on PFLs that do not contain PRS resources for aggregation, then Tmargin = 0.

T'margin= max(Teffect,i), when both aggregated PFLs and non-aggregated PFLs are configured for positioning measurement.

If UE is capable of performing latency reduced positioning measurements and is configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 2 shall be considered in calculations of Taggregate_RSTD and Tnon-aggregate_RSTD.

If UE is not capable to perform latency reduced positioning measurements or is not configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 4 shall be considered in calculations of Taggregate_RSTD and Tnon-aggregate_RSTD.

Taggregate_RSTD, total measurement period for RSTD measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as:

Taggregate_RSTD=i=1GTRSTD_aggregate,i +(G-1)*T'marginmaxTeffect_agg,i,

where

i is the index of effective PFL, corresponding to the group of PFLs containing linked PRS resource sets to be aggregated for RSTD measurement,

G denotes the number of effective PFLs that UE is configured to perform aggregated measurements on. G is configured by LMF and is provisioned to UE via nr-DL-PRS-AggregationInfo.

TRSTD_aggregate,i is the measurement period for PRS RSTD measurement in the effective PFL i as specified below:

TRSTD_aggregate,i=kmultiTEG_agg,i×CSSFPRS_agg,i×ceil( Kp,PRS_agg,i)×NRxBeam,i×NPRS_agg,islotNagg'Lavailable_PRS_agg,iNagg×Nsample_agg-1×Teffect_agg,i+Tlast_agg,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor.

In FR1, NRxBeam,i = 1.

In FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing effective PFL i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_agg,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in effective PFL i as defined in clause 9.1.5.2.

kmultiTEG_agg,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG_agg,i = 1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

Otherwise,

kmultiTEG_agg,i = NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEGagg,i=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure aggregated PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS_agg,i is a scaling factor for effective PFL i to be measured within the associated measurement gap pattern, which is defined as Kp,PRS_agg,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS_agg,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS_agg,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the effective PFL, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable = 0.

NPRS_agg,islot is the maximum number of DL PRS resources in effective PFL i configured in a slot.

Lavailable_PRS_agg,i is the time duration of available PRS in the effective PFL i to be measured during Tavailable_PRS_agg,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_agg,i, only the PRS resources unmuted and fully or partially overlapped with MG and satisfying the conditions for PRS BW aggregation are considered.

Nsample_agg is the number of PRS RSTD measurement samples.

Tlast_agg,i is the measurement duration for the last PRS RSTD sample in the effective PFL i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe_agg, Tlast_agg,i = Tagg,i + MGL. Otherwise, Tlast_agg,i = Tagg,i + Tavailable_PRS_agg,i,

Teffect_agg,i is the periodicity of the PRS RSTD measurement in the effective PFL i defined as:

Teffect_agg,i = Tagg,iTavailable_PRS_agg,i×Tavailable_PRS_agg,i

where,

Tavailable_PRS_agg,i= LCMTPRS_agg,i,MGRPi, the least common multiple between TPRS_agg,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the effective PFL i.

TPRS_agg,i is the periodicity of DL PRS resources meeting the bandwidth aggregation conditions with muting on effective PFL i. If more than one PRS periodicities are configured in effective PFL i, the least common multiple of PRS periodicities Tper_aggPRS with muting among all DL PRS resource sets in the effective PFL is used to derive TPRS_agg,i, where,

Tper_aggPRS with muting=Nmuting×Tper_aggPRS, is the PRS periodicity with muting per PRS resource configured for aggregation,

Tper_aggPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS_agg×Lmuting, where TmutingPRS_agg is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS_agg,i, only the PRS resources fully or partially covered by the MG and PRS resources that satisfy the conditions for PRS BW aggregation are considered.

{Nagg, Tagg,i} is UE capability combination per band to aggregate PRS resources from multiple PFLs within an effective PFL where Nagg is a duration of DL PRS symbols in ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 in TS 37.355 [34] processed every Tagg,i ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to maximumOfTwoAggregatedDL-PRS-Bandwidth or maximumOfThreeAggregatedDL-PRS-Bandwidth in TS 37.355 [34]

Nagg' is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2 specified in TS 37.355 [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273] [44], the time TRSTD_aggregate,Total starts from the first MG instance aligned with DL PRS resources in the assistance data for aggregation after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34] including a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the time TRSTD_aggregate,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the UE shall perform the aggregated RSTD measurement in each reporting period and activate the location report at the time when the periodic timer expires.

When PRS-RSRP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 9.9.2.10.

When PRS-RSRPP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 9.9.2.10.

If during the measurement period of one or more effective PFLs, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD_aggregate,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for aggregated PRS resources, if the PRS resources to be aggregated are across two sampling duration of Nagg within duration Lavailable_PRS_agg,i.

The measurement requirements do not apply, if time span of the instance of the PRS resources to be aggregated (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability Nagg.

The requirements in clause 9.9.2.10 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which aggregated PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while aggregated RSTD measurements are being performed, then the UE shall continue and complete the on-going aggregated RSTD measurements. The aggregated RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23A.2.

## --- End of Change #13 ---

## --- Start of Change #14 ---

#### 9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation

When UE physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequestedPFL-List, the UE shall be able to perform UE Rx-Tx time difference measurement by aggregating PRS resources from multiple PFLs (up to the UE capability specified in clause 9.9.2.3) defined in TS 38.215 [4], during the measurement period TUE_RxTx_aggregated, Total defined as:

TUE_RxTx_aggregated, Total = Tnon-aggregate_RxTx + Taggregated_RxTx + Tmargin,

where,

Tnon_aggregate_RxTx is the total measurement period for UE Rx-Tx time difference measurement on PFLs that do not contain PRS resources for aggregation.

Calculation of Tnon_aggregate is based on clause 9.9.4.5 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replaced by L-1*T'margin.

Tnon_aggregate_RxTx is equal to zero if UE is not configured to perform non-aggregated measurements by the LMF.

Taggregate_RxTx is the total measurement period for UE Rx-Tx time difference measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26].

If UE is configured to perform measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation, Tmargin =T'margin is delay margin to account for delay between Rx-Tx measurement performed by UE on PFLs that contain PRS resources for aggregation and Rx-Tx measurement performed by UE on PFLs that do not contain PRS resources for aggregation.

If UE is only configured to perform measurements on PFLs that contain PRS resources for aggregation or if UE is only configured to perform measurements on PFLs that do not contain PRS resources for aggregation, then Tmargin = 0.

T'margin= max(Teffect,i), by considering both aggregated PFLs and non-aggregated PFLs configured for positioning measurement.

If UE is capable of performing latency reduced positioning measurements and is configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 2 shall be considered in calculations of Taggregate_RxTx and Tnon-aggregate_RxTx.

If UE is not capable to perform latency reduced positioning measurements or is not configured to perform latency reduced positioning measurement by LMF via reducedDL-PRS-ProcessingSamples-r17 [34], then Nsample = 4 shall be considered in calculations of Taggregate_RxTx and Tnon-aggregate_RxTx.

Taggregate_RxTx, total measurement period for UE Rx-Tx time difference measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as:

Taggregate_RxTx=i=1GTRxTx_aggregate,i +(G-1)*T'marginmaxTeffect_agg,i,

where

i is the index of effective PFL, corresponding to the group of PFLs containing linked PRS resource sets to be aggregated for UE Rx-Tx time difference measurement,

G denotes the number of effective PFLs that UE is configured to perform aggregated measurements on. G is configured by LMF and is provisioned to UE via nr-DL-PRS-AggregationInfo.

TRxTx_aggregate,i is the measurement period for UE Rx-Tx time difference measurement in the effective positioning frequency layer i as specified below:

TRxTx_aggregate,i=kmultiTEG_agg,i×CSSFPRS_agg,i×ceil( Kp,PRS_agg,i)×NRxBeam_agg,i×NPRS_agg,islotNagg'Lavailable_PRS_agg,iNagg×Nsample_agg-1×Teffect_agg,i+Tlast_agg,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor.

In FR1, NRxBeam,i = 1.

In FR2, NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing effective positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-Multi-RTT-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_agg,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in effective positioning frequency layer i as defined in clause 9.1.5.2.

kmultiTEG_agg,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i = 1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation;

Otherwise,

kmultiTEG,i = NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG,i=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure aggregated PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 [34] in NR-Multi-RTT-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS_agg,i is a scaling factor for effective positioning frequency layer i to be measured within the associated measurement gap pattern, which is defined as Kp,PRS_agg,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS_agg,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS_agg,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the effective positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable = 0.

NPRS_agg,islot is the maximum number of DL PRS resources in effective positioning frequency layer i configured in a slot.

Lavailable_PRS_agg,i is the time duration of available PRS in the effective positioning frequency layer i to be measured during Tavailable_PRS_agg,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS_agg,i, only the PRS resources unmuted and fully or partially overlapped with MG and satisfying the conditions for PRS BW aggregation are considered.

Nsample_agg is the number of UE Rx-Tx time difference measurement samples.

Tlast_agg,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the effective positioning frequency layer i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe_agg, Tlast_agg,i = Tagg,i + MGL. Otherwise, Tlast_agg,i = Tagg,i + Tavailable_PRS_agg,i,

Teffect_agg,i is the periodicity of the UE Rx-Tx time difference measurement in the effective positioning frequency layer i defined as:

Teffect_agg,i = Tagg,iTavailable_PRS_agg,i×Tavailable_PRS_agg,i

where,

Tavailable_PRS_agg,i= LCMTPRS_agg,i,MGRPi, the least common multiple between TPRS_agg,i and MGRPi. MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the effective PRS frequency layer i.

TPRS_agg,i is the periodicity of DL PRS resources meeting the bandwidth aggregation conditions with muting on effective positioning frequency layer i. If more than one PRS periodicities are configured in effective positioning frequency layer i, the least common multiple of PRS periodicities Tper_aggPRS with muting among all DL PRS resource sets in the effective positioning frequency layer is used to derive TPRS_agg,i, where,

Tper_aggPRS with muting=Nmuting×Tper_aggPRS, is the PRS periodicity with muting per PRS resource configured for aggregation,

Tper_aggPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS_agg×Lmuting, where TmutingPRS_agg is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note: For the purpose of calculating TPRS_agg,i, only the PRS resources fully or partially covered by the MG and PRS resources that satisfy the conditions for PRS BW aggregation are considered.

{Nagg, Tagg,i}is UE capability combination per band to aggregate PRS resources from multiple PFLs within an effective PFL where Nagg is a duration of DL PRS symbols in ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsN or prs-durationOfThreePRS-BWA-ProcessingSymbolsN  in TS 37.355 [34] processed every Tagg,i ms corresponding to prs-durationOfTwoPRS-BWA-ProcessingSymbolsT or prs-durationOfThreePRS-BWA-ProcessingSymbolsT in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to maximumOfTwoAggregatedDL-PRS-Bandwidth or maximumOfThreeAggregatedDL-PRS-Bandwidth in TS 37.355 [34]

Nagg' is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot specified in TS 37.355 [34].

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements satisfy all the conditions specified in clause 5.1.6.5.3 in TS 38.214 [26].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273] [44], the time TUE_RxTx_aggregated,Total starts from the first MG instance aligned with DL PRS resources in the assistance data for aggregation after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message are delivered from LMF to the physical layer of UE via LPP [34] including a request to perform measurement by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequested.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the time TUE_RxTx_aggregated,Total starts from the first MG instance aligned with DL PRS resources in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the UE shall perform the aggregated UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

When PRS-RSRP is also reported by UE together with UE RX-Tx measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 9.9.4.9.

When PRS-RSRPP is also reported by UE together with UE Rx-Tx measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 9.9.4.9.

If during the measurement period of one or more effective positioning frequency layers, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TUE_RxTx_aggregated,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

The requirements in clause 9.9.4.9 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which aggregated PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE continues the UE Rx-Tx time difference measurement, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the UE Rx-Tx time difference measurement after the SRS reconfiguration is complete. If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

When a serving cell change occurs during the measurement period, the UE shall continue and complete the UE Rx-Tx time difference measurement provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx time difference measurement.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the UE Rx-Tx measurement period, then:

-	UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The UE Rx-Tx time difference measurement period may be restarted in such case.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25A.2.

## --- End of Change #14 ---

## --- Start of Change #15 ---

#### 9.9.7.2	Requirements Applicability

The requirements in clause 9.9.7 apply for periodic and triggered reporting of RSCPD with RSTD measurements, provided:

-	PRS-RSTD related side conditions given in clause 10.1.23 for FR1 and FR2 are fulfilled, for a corresponding Band.

-	RSCPD related side conditions given in clause 10.1.43.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

## --- End of Change #15 ---

## --- Start of Change #16 ---

#### 9.9.7.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the periodic measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.23.3.

The reported RSCPD measurement values contained in the measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1.43.3.

The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1.23, for the measured DL PRS resource.

The RSCPD measurements performed and reported according to this section shall meet the RSCPD measurement accuracy requirements in clause 10.1.43.2, for the measured DL PRS resource.

## --- End of Change #16 ---

## --- Start of Change #17 ---

#### 9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message with nr-DL-RSCPD-Request from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.7.3) DL RSTD and RSCPD measurements, defined in TS 38.215 [4], during the time window configured to UE via nr-DL-PRS-MeasurementTimeWindowsConfig , the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) RSTD and DL RSCPD measurements, defined in TS 38.215 [4], during the time window only.

When LMF does not configure measurement time window(s):

-	When a single PFL, requirements in clause 9.9.2.5 apply to both RSCPD and RSTD measurements.

-	When multiple PFLs are configured for legacy measurements, the UE performs RSCPD measurement on a single PFL that is common between the reference TRP and the target TRP. The requirement in clause 9.9.2.5 apply to both RSTD and RSCPD measurements.

When LMF configures measurement time window(s), but UE does not support supportOfRSCPD-MeasurementInTimeWindow:

-	The UE performs RSCPD measurement on the indicated PFL by the network. The requirement in clause 9.9.2.5 apply to both RSTD and RSCPD measurements.

When LMF configures measurement time window(s), but UE does not support supportOfLegacyMeasurementInTimeWindow but supports supportOfRSCPD-MeasurementInTimeWindow:

-	The requirements in the clause 9.9.2.5 apply to RSTD measurements.

-	The requirements in clause 9.9.7.5 apply to RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

When LMF configures measurement time window(s), and UE supports supportOfRSCPD-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow:

-	The requirements in Clause 9.9.7.5 apply to DL RSTD measurement and DL RSCPD measurement.

If a one-shot time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.7.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the one-shot time window during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=min⁡(Lwindow,MGL)+Ti	,

and the UE shall be able to measure multiple DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total,with one shot window defined as:

TRSTD,Total, with one shot window=TRSCPD with RSTD,Total+maxTeffect,i+TRSTD,Total ,

where:

-	the start of the measurement is the start of the one-shot time window, and

-	Lwindow is the length of the one-shot window, and

-	Teffect,i and Ti are defined in the following of this clause, and

-	TRSTD,Total is defined in clause 9.9.2.5, and

-	MGL is the measurement gap length.

The requirements with one-shot window assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

If a periodic time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.7.3) RSTD and DL RSCPD measurements, defined in TS 38.215 [4], during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=i=1LTRSCPD with RSTD,i+ L-1*maxTeffect,i ,

where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSCPD with RSTD,i is the measurement period for PRS RSTD with RSCPD measurement in positioning frequency layer i as specified below:

TRSCPD with RSTD,i=kmultiTEG,i×CSSFPRS,i×ceil( Kp,PRS,i)×NRxBeam,i×NPRS,islotN'Lavailable_PRS,iN×Nsample-1×*Teffect,i+Tlast,i ,

where:

DL-RSCPD measurement performed during TRSCPD with RSTD,i is a single sample measurement. The DL-RSCPD measurement of reference TRP and the target TRP is performed on the same PFL.

NRxBeam,i, CSSFPRS,i, kmultiTEG,i,  NPRS,islot, N, N’, Teffect,i, and Tlast,i are defined in clause 9.9.2.5.

Nsample = 1 or 2 or 4 as defined in clause 9.9.2.5.

LavailablePRS,i is the time duration of available PRS in the positioning frequency layer to be measured during TavailablePRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

When periodic time window(s) are configured by the LMF and PFL i is associated with the configured time window, TavailablePRS,i=LCM(Tprs,i, MGRP, Twindow), the least common multiple between Tprs,i , MGRP and Twindow, Twindow being the maximum periodicity of the indicated time window(s).

When the periodic time window(s) are configured by the LMF and the PFL i is not associated with the configured time window,  TavailablePRS,i=LCM(Tprs,i, MGRP).

Tprs,i and MGRP are defined in clause 9.9.2.5.

LavailablePRS,i and Tprs,i are calculated by only considering the PRS resources in the indicated resources sets overlapping with both the MG and the indicated time window(s).

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,i,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule.

Requirements do not apply if Navailable =0.

Otherwise, for a window W of duration max(TPRS,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273] [44], the time TRSCPD with RSTD starts from the first time window (TRSCPD) configured by LMF within MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the time TRSCPD with RSTD starts from the first time window (TRSCPD) configured by LMF within MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the UE shall perform the RSCPD with RSTD measurement in each reporting period within the time window(TRSCPD) configured by LMF within MG instance and activate the location report at the time when the periodic timer expires.

If during the measurement period, the MG pattern is reconfigured or time window (TRSCPD) for RSCPD measurement is reconfigured, the measurement period can be longer.

When PRS-RSRP is also configured to UE, RSCPD with RSTD and RSRP are performed over the same measurement period.

When PRS-RSRPP is also configured to UE, RSCPD with RSTD and RSRPP are performed over the same measurement period.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSCPD with RSTD,i within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration LavailablePRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9.7 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD and RSCPD measurements are being performed together by UE, then the UE shall continue and complete the on-going RSTD and RSCPD measurements. The measurement period for RSCPD with RSTD measurement can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23. The UE shall also meet the RSCPD measurement accuracy requirements in clause 10.1.43.2.

## --- End of Change #17 ---

## --- Start of Change #18 ---

#### 9.9.8.2	Requirements Applicability

The requirements in clause 9.9.8 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25 are met for a corresponding band.

- 	DL RSCP related side conditions given in clause 10.1.44.2 for FR1 and FR2 are fulfilled, for a corresponding Band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

## --- End of Change #18 ---

## --- Start of Change #19 ---

#### 9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference

When the physical layer receives the last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] with nr-UE-RSCP-Request and configuring a measurement time window via nr-DL-PRS-MeasurementTimeWindowsConfig, subject to UE capabilities supportOfRSCP-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow, the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the time window only.

When LMF does not configure measurement time window(s):

-	When a single PFL is configured, requirements in clause 9.9.4.5 apply to both RSCP and UE Rx-Tx measurements.

-	When multiple PFLs are configured for legacy measurements, the UE performs RSCP measurement on a single PFL that is common between the reference TRP and the target TRP. The requirement in clause 9.9.4.5 apply to both RSCP and UE Rx-Tx measurements.

When LMF configures measurement time window(s), but UE does not support supportOfRSCP-MeasurementInTimeWindow:

- The UE performs RSCP measurement on the indicated PFL by the network. The requirement in clause 9.9.4.5 apply to both UE Rx-Tx and RSCP measurements.

When LMF configures measurement time window(s), but UE does not support supportOfLegacyMeasurementInTimeWindow but supports supportOfRSCP-MeasurementInTimeWindow:

The requirements in the clause 9.9.4.5 apply to UE Rx-Tx measurement.

The requirements in clause 9.9.8.5 apply to RSCP measurement for the PRS resource(s) that have occasions only within the measurement time window.

When LMF configures measurement time window(s), and UE supports supportOfRSCP-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow:

-	The requirements in Clause 9.9.8.5 apply to UE Rx-Tx measurement and DL RSCP measurement.

If a one-shot time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the one-shot time window during the measurement period TDL RSCP with UERxTx,Total defined as:

TDL RSCP with UERxTx,Total=min⁡(Lwindow,MGL)+Ti	,

and the UE shall be able to measure multiple UE Rx-Tx measurements, defined in TS 38.215 [4], during the measurement period TUERxTx,Total,with one shot window defined as:

TUERxTx,Total,with one shot window=TDL RSCP with UERxTx,Total+maxTeffect,i+TUERxTx,Total

Where:

-	the start of the measurement is the start of the one-shot time window, and

-	Lwindow is the length of the one-shot window, and

-	Teffect,i and Ti are defined in the following of this clause, and

-	TUERxTx,Total is defined in clause 9.9.4.5, and

-	MGL is the measurement gap length.

The requirements with one-shot window assume scaling factors equal 1 for the PFLs, for which the one-shot time window is configured.

If a periodic time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the measurement period TRSCP with UERxTx,Total defined as:

TDL RSCP with UERxTx,Total=i=1LTDL RSCP with UERxTx,i+ L-1*maxTeffect,i ,

where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i.

TDL RSCP with UERxTx,i is the measurement period for DL RSCP with UE Rx-Tx measurement in positioning frequency layer i as specified below:

TDL RSCP with UERxTx,i=kmultiTEG,i*CSSFPRS,i*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'LavailablePRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

- DL RSCP performed during TDL RSCP with UERxTx is a single sample measurement where DL RSCP and UE Rx-Tx measurements are performed on the same PFL.

- NRxBeam,i, CSSFPRS,i, kmultiTEG,i, Nsample, NPRS,islot, N, N’, Teffect,i, and Tlast,i are defined in clause 9.9.4.5.

- Nsample = 1 or 2 or 4 as defined in clause 9.9.4.

- Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,i,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule.

Requirements do not apply if Navailable =0.

Otherwise, for a window W of duration max(TPRS,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

-	Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer, to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

-	When periodic time window(s) are configured by the LMF and PFL i is associated with the configured time window, Tavailable_PRS,i=LCM(Tprs,i, MGRP, Twindow), the least common multiple between Tprs,i , MGRPMGRP , and Twindow, Twindow being the maximum periodicity of the indicated time window(s).

-	When periodic time window(s) are configured by the LMF and the PFL i is not associated with the configured time window, Tavailable_PRS,i=LCM(Tprs,i, MGRP).

-	Tprs,i and MGRPMGRP are defined in clause 9.9.8.5.

-	-Lavailable_PRS,i and Tprs,i are calculated by only considering the PRS resources in the indicated resources sets overlapping with both the MG and the indicated time window(s).

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273] [44], the time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the UE shall perform the DL RSCP with UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

The DL RSCP with UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of the positioning frequency layer, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TDL RSCP with UERxTx within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

When PRS-RSRPP is configured for multi-RTT, RSCP with UE Rx-Tx time difference measurements and PRS-RSRPP measurements are performed over the same measurement period.

The requirements in clause 9.9.8 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or SCell addition or release does not cause SRS reconfiguration during the measurement period, UE shall continue and complete the DL RSCP and UE Rx-Tx time difference measurements, and the measurement period requirements apply.

When PSCell or SCell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.

When a serving cell change occurs during the measurement period, the UE shall restart DL RSCP and UE Rx-Tx time difference measurements.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in table 7.1.2-2 during the measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the measurement period, then:

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The DL RSCP with UE Rx-Tx time difference measurement period may be restarted in such case.

## --- End of Change #19 ---

## --- Start of Change #19a ---

#### 9.9A.1.1	General Aspects of Gap-based Measurement

For gap-based RSTD, PRS-RSRP, UE Rx-Tx time difference, and PRS-RSRPP measurements, the requirements in clauses 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 apply provided:

-	the RedCap UE is configured or pre-configured with measurement gaps or configured with concurrent measurement gaps

-	all positioning frequency layers are measured or associated with only one per-UE measurement gap, or

-	for the RedCap UE supporting independentGapConfigPRS-r17, all positioning frequency layers in the same FR are measured or associated with only one per-FR measurement gap in the corresponding FR.

-	if the measurement gap is pre-configured, the gap must be activated throughout the measurement period, and

-	if concurrent measurement gaps are configured, one of the gap combinations specified in clause 9.1.8.2 is configured, and

-	if the RedCap UE does not support independentGapConfigPRS-r17, the configured or pre-configured gap used to perform the PRS measurements must be of per-UE type, and

-	No active BWP switching occurs during the measurement gaps for PRS measurement, and

All measurement requirements specified in clause 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

RedCap UE is only required to measure PRS resources that are fully or partially overlapped with measurement gaps, and the requirements in clause 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 are applicable to PRS resources that are fully or partially overlapped with measurement gaps.

A PRS resource is considered to be fully (partially) overlapped with measurement gaps if all (some) of its instances are overlapped with a measurement gap occasion. A PRS resource instance is considered to be overlapped with measurement gap occasion if the minimum number of unmuted repetitions of the instance taking into account nr-DL- PRS-ExpectedRSTD-Uncertainty and nr-DL-PRS-ExpectedRSTD is fully covered by the MGL excluding RF switching time, where

-	 the minimum number is given in the accuracy requirements without FH for 2Rx RedCap UEs in clause 10.1.23.2 for RSTD measurement,  10.1.24.2 for PRS-RSRP measurement, 10.1.25.2 for UE Rx-Tx measurement, and 10.1.38.2 for PRS-RSRPP measurement,

-	the minimum number is given in the accuracy requirements without FH for 1Rx RedCap UEs in clause 10.1A.16.2.1 for RSTD measurement,  10.1A.17.2 for PRS-RSRP measurement, 10.1A.18.2.2 for UE Rx-Tx measurement, and 10.1A.19.2 for PRS-RSRPP measurement,

-	the minimum number is given in the accuracy requirements with FH for 2Rx RedCap UEs in clause 10.1A.16.2.2 for RSTD measurement,  10.1A.17.2 for PRS-RSRP measurement, 10.1A.18.2.3 for UE Rx-Tx measurement, and 10.1A.19.2 for PRS-RSRPP measurement,and

-	 the minimum number is given in the accuracy requirements with FH for 1Rx RedCap UEs in clause 10.1A.16.2.2 for RSTD measurement,  10.1A.17.2 for PRS-RSRP measurement, 10.1A.18.2.4 for UE Rx-Tx measurement, and 10.1A.19.2 for PRS-RSRPP measurement.

When RedCap UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when RedCap UE is configured with measurement for single positioning request.

If a positioning measurement gap is configured via PosGapConfig and activated by MAC CE, the measurement requirements in clause 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 apply provided that no other MGs are configured, and only one measurement gap configured via PosGapConfig is activated.

#### 9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH

The requirements for RSTD, PRS-RSRP, UE Rx-Tx time difference, and PRS-RSRPP measurement without measurement gaps specified in clauses 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 shall apply provided that:

-	Positioning frequency layer to be measured is within an active BWP,

-	UE is configured with PPW, and the PPW for the active BWP containing the positioning frequency layer to be measured is activated,

-	No active BWP switching occurs during the measurement period specified in clauses 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5,

-	PRS is within PPW and does not overlap with other signals/channels of higher priority,

-	for PPW type 1A/1B, the PPW does not overlap with any symbol for SSB-based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement on any CC or for SSB based RRM measurement on any MOs that are measured outside measurement gaps,

-	for PPW type 2, PRS does not overlap with any symbol for SSB-based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement on any CC or for SSB based RRM measurement on any MOs that are measured outside measurement gaps,

-	max∣ΔT∣≤ THR, where

-	∆T is the time difference between the start of a slot containing PRS from the neighbor cell/TRP and the start of the closest slot from the serving cell;

-	the range of ∆T is determined by the expected RSTD and expected RSTD uncertainty in the assistance data;

-	THR is the threshold as reported in UE capability prs-MeasurementWithoutMG-r17.

-	SCS of PRS within PPW and SCS of DL active BWP are the same.

All measurement requirements specified in clauses 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

The UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

The UE is only required to measure PRS resources that are unmuted and fully or partially overlapped with PPW, and the requirements in clauses 9.9A.2, 9.9A.3, 9.9A.4 and 9.9A.5 are applicable to PRS resources that are unmuted and fully or partially overlapped with PPW.

A PRS resource is considered to be fully (partially) overlapped with PPW if all (some) of its instances are overlapped with a PPW occasion. A PRS resource instance is considered to be overlapped with PPW occasion if the minimum number of unmuted repetitions of the instance taking into account Rx time difference between serving and non-serving cell is fully covered by the PPW , where

-	the minimum number is given in the accuracy requirements without FH for 2Rx RedCap UEs in clause 10.1.23.2 for RSTD measurement,  10.1.24.2 for PRS-RSRP measurement, 10.1.25.2 for UE Rx-Tx measurement, and 10.1.38.2 for PRS-RSRPP measurement, and

-	the minimum number is given in the accuracy requirements without FH for 1Rx RedCap UEs in clause 10.1A.16.2.1 for RSTD measurement, 10.1A.17.2 for PRS-RSRP measurement, 10.1A.18.2.2 for UE Rx-Tx measurement, and 10.1A.19.2 for PRS-RSRPP measurement.

When UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

## --- End of Change #19a ---

## --- Start of Change #20 ---

### 9.9A.2	RSTD measurements for RedCap

#### 9.9A.2.1	Introduction

The requirements in clause 9.9A.2 shall apply provided the RedCap UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the RedCap UE to measure and report DL RSTD measurements defined in TS 38.215 [4].

The requirements in clause 9.9A.2.5 shall apply provided the RedCap UE does not support, or support but not configured, to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated via NR-DL-AoD-ProvideCapabilities.

The requirements in clause 9.9A.2.6 shall apply provided the RedCap UE supports and is configured to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated via NR-DL-AoD-ProvideCapabilities.

#### 9.9A.2.2	Requirements Applicability

The requirements in clause 9.9A.2.5 apply for periodic and triggered RSTD measurements without FH, provided:

-	PRS-RSTD related side conditions for 2 Rx RedCap UE given in clause 10.1.23.2 for FR1 and FR2 are fulfilled, for a corresponding Band, or

-	PRS-RSTD related side conditions for 1 Rx RedCap UE given in clause 10.1A.16.2.1 for FR1 are fulfilled, for a corresponding Band.

The requirements in clause 9.9A.2.6 apply for periodic and triggered RSTD measurements with FH, provided:

-	PRS-RSTD related side conditions for 2 Rx RedCap UE given in clause 10.1A.16.2.2 for FR1 and FR2 are fulfilled, for a corresponding Band. or

-	PRS-RSTD related side conditions for 1 Rx RedCap UE given in clause 10.1A.16.2.2 for FR1 are fulfilled, for a corresponding Band.

#### 9.9A.2.3	Measurement Capability

RedCap UE PRS RSTD measurement capability is as indicated by the RedCap UE in NR-DL-TDOA-ProvideCapabilities, according to TS 37.355[34].

#### 9.9A.2.4	Measurement Reporting Requirements

The requirements in clause 9.9.2.4 shall apply except for RSTD measurement accuracy. The RSTD measurements performed and reported according to this section shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2 for each measured DL PRS resource.

#### 9.9A.2.5	Measurements Period Requirements without FH

##### 9.9A.2.5.1	Measurements Period Requirements without FH with MG

When physical layer receives last of NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i ,

where

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=kmultiTEG,i*CSSFPRS_RedCap,i*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam,i is the RedCap UE Rx beam sweeping factor. In FR1 (for 2 Rx or 1 Rx RedCap UE), NRxBeam,i = 1;

and in FR2 (for 2 Rx RedCap UE only), NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

CSSFPRS_RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the

sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, Tlast,i = Ti +MGL. Otherwise, Tlast,i = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273] [44], the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the time TRSTD,Total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the UE shall perform the RSTD measurement in each reporting period and activate the location report at the time when the periodic timer expires.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSTD and RSRPP are performed over the same measurement period.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD,Total within measurement gaps due to collisions with other signals; otherwise, the measurement period can be longer.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9A.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.

##### 9.9A.2.5.2	Measurements Period Requirements without FH without MG

When physical layer receives last of NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD_wo_gap,i+L-1*maxTeffect,i, if any of the positioning frequency layers are in Case 1, or

TRSTD,Total=maxTRSTD_wo_gap,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2,

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TRSTD,Total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TRSTD_wo_gap,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below.

TRSTD_wo_gap,i=kmultiTEG,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1 (for 2 Rx or 1 Rx RedCap UE), NRxBeam,i = 1; and in FR2 (for 2 Rx RedCap UE only), NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

kmultiTEG,i is the scaling factor for measurement of same PRS resource with multiple Rx TEGs.

kmultiTEG,i=1 if UE is not supported measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 or not requested by LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation;

otherwise,

kmultiTEG=NTEG,i, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, and

kmultiTEG=NTEG,ikTEG,simul,i if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs.

Where

NTEG,i is the number of Rx TEGs with which UE is requested to measure a PRS resource indicated via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] in NR-DL-TDOA-RequestLocationInformation, and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability, and

kTEG,simul,i is the number of Rx TEGs UE can measure simultaneously which is reported via measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i,

only the PRS resources unmuted and fully or partially overlapped with PPW are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then  Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

Tavailable_PRS,i= LCMTPRS,i,PPWRPi, the least common multiple between TPRS,i and PPWRPi.

PPWRPi is the repetition periodicity of the PRS processing window applicable for measurements in the positioning frequency layer i.

TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity,

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the conditions for PRS measurement outside measurement gaps as defined in clause 9.9.1.2 are considered.

N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2.

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [34].

The time TRSTD_wo_gap,i starts from the first instance of the activated PPW for measurement of positioning frequency layer i aligned with a DL PRS resource(s) in the assistance data after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP [34].

When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSTD and RSRPP are performed over the same measurement period.

If during the measurement period of one or more positioning frequency layers, the PPW is re-configured or reactivated, the measurement period can be longer.

The measurement requirements in this clause apply, provided no PRS symbols are dropped during the measurement period TRSTD_wo_gap,i within PPW due to collisions with other signals; otherwise, the measurement period can be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9A.2 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements. The RSTD measurement period can be longer. The UE shall meet the RSTD measurement accuracy requirements in clause 10.1A.16.2.1.

If any positioning frequency layer is in Case 2, the requirements in this clause apply provided that the PPWL corresponding to the positioning frequency layer is larger than (T2+X) ms.

The requirements in this clause apply provided that a single positioning frequency layer is configured for measurement in each PPW.

##### 9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW

If the RedCap UE is configured with both MG applicable to positioning measurement and PPW, the UE shall measure positioning frequency layer i within

-	MG, if PRS resources on positioning frequency layer i are overlapped with MG, or

-	PPW, if PRS resources on positioning frequency layer i are overlapped with PPW.

The measurement period TRSTD,Total is defined as:

TRSTD,Total=TRSTD,Total,MG+ TRSTD,Total,PPW+ Tguard,

Where

-	TRSTD,Total,MG is defined in clause 9.9A.2.5.1 and includes all positioning frequency layers to be measured within MG,

-	TRSTD,Total,PPW is defined in clause 9.9A.2.5.2 and includes all positioning frequency layers to be measured within PPW,

-	Tguard is the maximum Teffect defined in clauses 9.9A.2.5.1 and 9.9A.2.5.2 among all PFLs.

The requirements apply provided the following conditions are satisfied:

-	MGs and PPWs do not overlap in time;

-	Each PFL in the assistance data can be measured completely (all PRS resources) either within MG or within activated PPW.

#### 9.9A.2.6	Measurements Period Requirements with FH

##### 9.9A.2.6.1	Measurements Period Requirements with FH with MG

When UE physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in clause 9.9A.2.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

-	Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j

where,

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 9.9A.2.6.1-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.2.6.1-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 9.9A.2.6.1-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34],

Nhop,i,j is number of hops within a single MG occasion as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34],

BWoverlap BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

### 9.9A.3	PRS-RSRP measurements for RedCap

#### 9.9A.3.1	Introduction

The requirements in clause 9.9A.3 shall apply provided the RedCap UE has received a message from LMF via LPP [34] requesting the RedCap UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4].

The requirements in clause 9.9A.3.5 shall apply provided the RedCap UE does not support, or support but not configured, to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated in NR-DL-AoD-ProvideCapabilities.

The requirements in clause 9.9A.3.6 shall apply provided the RedCap UE supports and is configured to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated in NR-DL-AoD-ProvideCapabilities.

#### 9.9A.3.2	Requirements applicability

The requirements in clause 9.9A.3.5 apply for periodic and triggered PRS-RSRP measurements without FH, provided:

-	PRS-RSRP related side conditions for 2 Rx RedCap UE given in clause 10.1.24.2 for FR1 and FR2 are met for a corresponding Band, or

-	PRS-RSRP related side conditions for 1 Rx RedCap UE given in clause 10.1A.17.2 for FR1 are met for a corresponding Band.

The requirements in clause 9.9A.3.6 apply for periodic and triggered PRS-RSRP measurements with FH, provided:

-	PRS-RSRP related side conditions for 2 Rx RedCap UE given in clause 10.1.24.2 for FR1 and FR2 are met for a corresponding Band, or

-	PRS-RSRP related side conditions for 1 Rx RedCap UE given in clause 10.1A.17.2 for FR1 are met for a corresponding Band.

#### 9.9A.3.3	Measurement Capability

UE PRS-RSRP measurement capability is as indicated by the UE in NR-DL-AoD-ProvideCapabilities according to TS 37.355 [34].

#### 9.9A.3.4	Measurement Reporting Requirements

The requirements in clause 9.9.3.4 shall apply except for PRS-RSRP measurement accuracy. The PRS-RSRP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clauses 10.1A.17.2.

#### 9.9A.3.5	Measurements Period Requirements without FH

##### 9.9A.3.5.1	Measurement Period Requirements without FH with MG

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the RedCap UE shall be able to measure multiple (up to the RedCapUE capability specified in clause 9.9A.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where

i is the index of positioning frequency layer,

L is total number of positioning frequency layers,

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=CSSFPRS_RedCap,i*ceil( Kp,PRS,i)*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFPRS_RedCap, i is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1A.5.2,

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

For a window W of duration max(TPRS,i,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioining frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including both dropped and non-dropped instances of the associated measurement gap within the window, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 (for 2 Rx or 1 Rx RedCap UE) and if positioning frequency layer i is in FR2 (for 2 Rx RedCap UE only), NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the PRS resources unmuted and fully or partially overlapped with MG are considered.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time, if not all PRS resources to be measured are available in the same measurement gap occasion during Tavailable_PRS,i, otherwise Tlast,i = Ti + MGL,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

MGRPi is the measurement gap repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

When PRS-RSRP measurements are configured for DL-AoD, except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273] [44], the time TPRS-RSRP,total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the time TPRS-RSRP,total starts from the first MG instance aligned with a DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273] [44], the UE shall perform the PRS-RSRP measurement in each reporting period and activate the location report at the time when the periodic timer expires.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9.4.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource:

if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of one or more positioning frequency layers, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TPRS-RSRP, total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

The requirements in clause 9.9A.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The PRS-RSRP measurement period can be longer. The UE shall meet the PRS-RSRP measurement accuracy requirements in clause 10.1A.17.2.

##### 9.9A.3.5.2	Measurement Period Requirements without FH without MG

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 9.9A.3.3) PRS-RSRP measurements as defined in TS 38.215 [4] without measurement gap, on configured positioning frequency layer i, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i if any of the positioning frequency layers are in Case 1, or

TPRS-RSRP, total=maxTPRS-RSRP,i+Tuncertainty,i, if all the positioning frequency layers are in Case 2,

Where,

i is the index of positioning frequency layer, and

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i, and

Tuncertainty,i is the time from the start of the first PPW occasion for positioning frequency layer i to the start of measurement period TPRS-RSRP, total.

A positioning frequency layer is in Case 1 if UE reports ppw-durationOfPRS-Processing1-r17 for the band containing the positioning frequency layer, and a positioning frequency layer is in Case 2 if UE reports ppw-durationOfPRS-Processing2-r17 for the band containing the positioning frequency layer.

TPRS-RSRP,i is the measurement period for PRS-RSRP measurement in positioning frequency layer i as specified below.

TPRS-RSRP,i=NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

where

Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 (for 2 Rx or 1 Rx RedCap UE). If positioning frequency layer i is in FR2 (for 2 Rx RedCap UE only), NRxBeam,i is equal to the value reported by the UE in supportedLowerRxBeamSweepingFactor-FR2 if the UE supports the capability for the band containing positioning frequency layer i, and the LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation. NRxBeam,i is equal to 8, otherwise.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i,

only unmuted PRS resource instances that meet the applicability conditions and fully or partially overlapped with PRS processing window are considered, if positioning frequency layer i is in Case 1, or

only the PRS resources unmuted and fully or partially overlapped with the first (PPWL-T2) ms of PPW are considered, if positioning frequency layer i is in Case 2, where PPWL is the PPW length and T2 corresponds to ppw-durationOfPRS-ProcessingSymbolsT2.

NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to ppw-durationOfPRS-ProcessingSymbolsN  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponding to ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2,

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by ppw-maxNumOfDL-PRS-ResProcessedPerSlot in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the active BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

Tlast  is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time.

If positioning frequency layer i is in Case 1 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = Ti +PPWL, else

if positioning frequency layer i is in Case 2 and all of the PRS resources to be measured are available in the same PPW occasion during Tavailable, then Tlast,i = PPWL;

otherwise, Tlast,i = Ti + Tavailable_PRS,i.

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

Ti	corresponds to ppw-durationOfPRS-ProcessingSymbolsT  in TS 37.355 [34] if positioning frequency layer i is in Case 1, or corresponds to the sum of ppw-durationOfPRS-ProcessingSymbolsT2 and ppw-durationOfPRS-ProcessingSymbolsN2 in TS 37.355 [34] if positioning frequency layer i is in Case 2

Tavailable_PRS,i= LCMTPRS,i,PPWRPi is the least common multiple between TPRS,i and PPWRPi,

TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

PPWRPi is the PRS processing window repetition period in positioning frequency layer i.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

Note:	For the purpose of calculating TPRS,i, only the PRS resources that meet the applicability conditions and fully or partially covered by the PRS processing window are considered.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,i starts from the first PRS processing window instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 9.9A.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 9.9A.4.

If CSSF changes during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 9.9A.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If during the measurement period of one or more positioning frequency layers, the PPW is reconfigured or reactivated, the PRS-RSRP measurement period can be longer.

If any positioning frequency layer is in Case 2, the requirements in this clause apply provided that the PPWL corresponding to the positioning frequency layer is larger than (T2+X) ms.

The requirements in this clause apply provided that a single positioning frequency layer is configured for measurement in each PPW.

##### 9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW

If the RedCap UE is configured with both MG applicable to positioning measurement and PPW, the UE shall measure positioning frequency layer i within

-	MG, if PRS resources on positioning frequency layer i are overlapped with MG, or

-	PPW, if PRS resources on positioning frequency layer i are overlapped with PPW.

The measurement period TPRS-RSRP, total is defined as:

TPRS-RSRP, total=TPRS-RSRP,Total,MG+ TPRS-RSRP,Total,PPW+ Tguard,

Where

-	TPRS-RSRP,Total,MG is defined in clause 9.9A.3.5.1 and includes all positioning frequency layers to be measured within MG,

-	TPRS-RSRP,Total,PPW is defined in clause 9.9A.3.5.2 and includes all positioning frequency layers to be measured within PPW,

-	Tguard is the maximum Teffect defined in clauses 9.9A.3.5.1 and 9.9A.3.5.2 among all PFLs.

The requirements apply provided the following conditions are satisfied:

-	MGs and PPWs do not overlap in time;

-	Each PFL in the assistance data can be measured completely (all PRS resources) either within MG or within activated PPW.

#### 9.9A.3.6	Measurements Period Requirements with FH

##### 9.9A.3.6.1	Measurements Period Requirements with FH with MG

When the UE physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure PRS-RSRP measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCapUE capability specified in clause 9.9A.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms as defined clause 9.9A.3.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

-	Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j

where,

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 9.9A.3.6.1-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.3.6.1-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 9.9A.3.6.1-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

-	BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34],

Nhop,i,j is number of hops within a single MG occasion as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34],

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

### 9.9A.4	UE Rx-Tx time difference measurements for RedCap

#### 9.9A.4.1	Introduction

The requirements in this clause apply for RedCap UE, when the UE received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP specified in TS 37.355 [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

If UE does not support RX FH for PRS measurement as indicated by UE capability NR-DL-PRS-ProcessingCapability, or if LMF does not request UE to use RX FH for PRS measurement as indicated via NR-Multi-RTT-RequestLocationInformation in the measurement request, the requirements in clause 9.9A.4.5, 9.9A.4.6 and 9.9A.4.7 shall apply for applicable configurations.

If UE supports RX FH for PRS measurement as indicated by UE capability NR-DL-PRS-ProcessingCapability, and LMF requests UE to use RX FH for PRS measurement as indicated via NR-Multi-RTT-RequestLocationInformation in the measurement request, the requirements in clause 9.9A.4.8 shall apply.

#### 9.9A.4.2	Requirements Applicability

The requirements in clause 9.9A.4 apply for periodic and triggered UE Rx-Tx time difference measurements, provided:

-	UE Rx-Tx time difference measurement related side conditions given in clause 10.1A.18.2 are met for a corresponding band.

-	SRS is configured on at least one of the PCell, PSCell and SCell.

-	The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

#### 9.9A.4.3	Measurement Capability

UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

#### 9.9A.4.4	Measurement Reporting Requirements

The requirements in clause 9.9.4.4 shall apply except that the UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1A.18.2.

#### 9.9A.4.5	Measurement Period Requirements without FH with MG

The requirements in clause 9.9.4.5 shall apply.

#### 9.9A.4.6	Measurement Period Requirements without FH without MG

The requirements in clause 9.9.4.6 shall apply.

#### 9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW

The requirements in clause 9.9.4.8 shall apply.

#### 9.9A.4.8	Measurements Period Requirements with FH

The requirements in this clause apply when a RedCap UE is requested by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 9.9A.4.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples. Otherwise, Nsample= 4.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap.

-	Lavailable_PRS,i=j=1JNhop,i,j*Lper-hop,i,j, where

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 9.9A.4.8-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetunTimeFR1 or rf-RxRetunTimeFR2 via NR-DL-PRS-ProcessingCapability specified in TS 37.355 [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.4.8-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15kHz, 30kHz, 60kHz in FR2, 120kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in TS 37.355 [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the MG occasion excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in table 9.9A.4.8-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop-1*BWoverlap

where

-	BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in TS 37.355 [34],

Nhop,i,j is number of hops within a single MG occasion as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in TS 37.355 [34],

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in TS 37.355 [34].

## --- End of Change #20 ---

## --- Start of Change #20a ---

#### 9.9A.5.1	Introduction

The requirements in this clause apply for RedCap UE, when the UE received NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more PRS-RSRPP measurements defined in TS 38.215 [4].

If UE does not support FH for PRS measurement as indicated by UE capability NR-DL-AoD-ProvideCapabilities, or if LMF does not request UE to use FH for PRS measurement as indicated via NR-DL-AoD-RequestLocationInformation in the measurement request, the requirements in clause 9.9A.5.5, 9.9A.5.6 and 9.9A.5.7 shall apply for applicable configurations.

If UE supports FH for PRS measurement as indicated by UE capability NR-DL-AoD-ProvideCapabilities, and LMF requests UE to use FH for PRS measurement as indicated via NR-DL-AoD-RequestLocationInformation in the measurement request, the requirements in clause 9.9A.5.8 shall apply.

#### 9.9A.5.2	Requirements Applicability

The requirements in clause 9.9A.5 apply for periodic and triggered PRS-RSRPP measurements, provided:

-	PRS-RSRPP related side conditions given in clause 10.1A.19.2 are met for a corresponding Band.

--- unchanged clauses ---

#### 9.9A.5.4	Measurement Reporting Requirements

This requirement assumes that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes the delay uncertainty caused by inserting the measurement report into the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

The reported PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.1A.19.3.

The PRS-RSRPP measurement accuracy for all measured PRS resources shall be fulfilled according to the accuracy requriements specified in the clauses 10.1A.19.2.

## --- End of Change #20a ---

## --- Start of Change #21 ---

# 12A	NR Sidelink Measurements for Positioning

## 12A.1	Introduction

Clause 12A contains requirements for UE capable of V2X sidelink or 5G ProSe operation, which is also capable of performing SL positioning measurements defined in TS 38.215 [4], including SL RSTD, SL PRS-RSRP, SL Rx-Tx time difference, SL PRS-RSRPP measurements, SL AoA, and SL RTOA, provided that:

- 	The SL-PRS are received on NR PC5 interface within a single sidelink BWP on a single carrier,

-	The UE is in any cell selection state or the UE is inside NG-RAN coverage while configured for SL positioning operation on a sidelink carrier, which is dedicated to only sidelink operation, and configured with only a PCell on WAN carrier, and

-	The measuring UE is the location target UE or an anchor UE, and

-	The UE is not required to monitor PSCCH, which is associated with SL-PRS in the same slot, outside the SL-DRX active time.

NOTE 1:	Any cell selection state refers to a UE that is out of network coverage and is not associated with a serving cell on any carrier as defined in TS 38.304 [1].

NOTE 2:	When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all the requirements specified in clause 9 assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrup the SL positioning measurements in order to meet the measurement requirements specified in clause 9.

NOTE 3: When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all relevant requirements related to its WAN operation, assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrup the SL positioning measurements or SL-PRS transmissions in order to meet the measurement requirements related to its WAN operation.

Prior to performing SL-PRS based measurements, the target UE may need to perform the discovery procedure to discover anchor UEs according to TS 38.305 [22].

## 12A.2	SL RSTD measurements

### 12A.2.1	Introduction

The requirements in clause 12A.2 apply for SL RSTD measurements of the first and additional paths.

The requirements in clause 12A.2 shall apply provided the UE has received a RequestLocationInformation message from LMF or another UE via SLPP specified in TS 38.355 [37] requesting the UE to measure and report SL RSTD measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.2.2	Requirements Applicability

The requirements in clause 12A.2 apply for periodic, aperiodic, and triggered RSTD measurements, provided:

-	SL RSTD related side conditions given in clause 10.4A.2.2 for FR1 are fulfilled, for a corresponding Band.

### 12A.2.3	Measurement Capability

UE SL RSTD measurement capability is as indicated by the UE in:

SL-TDOA-ProvideCapabilities, according to TS 38.355 [37].

### 12A.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.4A.2.1.

The SL RSTD measurements performed and reported according to this section shall meet the SL RSTD measurement accuracy requirements in clause 10.4A.2.2, for each measured SL-PRS resource.

### 12A.2.5	Measurements Period Requirements

When the UE physical layer receives the last of SL-TDOA-ProvideAssistanceData and SL-TDOA-RequestLocationInformation from LMF or another UE via SLPP specified in TS 38.355 [37], the UE shall be able to perform multiple SL RSTD measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in clause 12A.2.3), with each SL RSTD measurement based on SL-PRS from the reference UE and SL-PRS from another anchor UE, as defined in TS 38.215 [4]. The SL RSTD measurement shall be performed during the measurement period TSL RSTD,total defined as:

TSL RSTD,Total=s=1STSL RSTD,effect,s  ,

where

S is the number of samples per measured link, defined below:

S = 1 for SL-PRS bandwidth>48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs, and

for each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities reported by UE via maxNumOfActiveSL-PRS-ResourcesInOneSlot-r18 and maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 specified in TS 38.331 [2], TSL RSTD,effect,s is defined as:

TSL RSTD,effect,s=ts+1-ts , for s<S, where ts+1 and ts are the beginning of the first slot of SL-PRS sample s+1 and SL-PRS sample s, respectively,

TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RSTD measurement,

ΔSLproc is the processing time reported by the UE via minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 specified in TS 38.331 [2].

A UE may drop one or more SL PRS measurement samples if the number of active slots or the number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities reported by UE via sl-PRS-CommonProcCapabilityPerBand-r18 specified in TS 38.331 [2]. For a single-sample measurement, the whole measurement may not be performed.

If the synchronization reference source changes during TSL RSTD,total at the measuring UE, while the UE is performing the SL RSTD measurement, then the UE shall restart the SL RSTD measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RSTD,total.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL RSTD,total at the UE transmitting SL-PRS for the SL RSTD measurement.

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL RSTD,total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL RSTD,total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

## 12A.3	SL PRS-RSRP measurements

### 12A.3.1	Introduction

The requirements in clause 12A.3 apply for SL PRS-RSRP measurements and for SL PRS-RSRP path measurements of the first and additional paths.

The requirements in clause 12A.3 shall apply provided the UE has received a RequestLocationInformation message from LMF or another UE via SLPP specified in TS 38.355 [37] requesting the UE to measure and report SL PRS-RSRP measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.3.2	Requirements Applicability

The requirements in clause 12A.3 apply for periodic, aperiodic, and triggered SL PRS-RSRP measurements, provided:

-	SL PRS-RSRP related side conditions given in clause 10.4A.3.2 for FR1 are fulfilled, for a corresponding Band.

### 12A.3.3	Measurement Capability

UE SL PRS-RSRP measurement capability is as indicated by the UE in:

-	SL-TDOA-ProvideCapabilities, SL-RTT-ProvideCapabilities, SL-AOA-ProvideCapabilities, or SL-TOA-ProvideCapabilities, according to TS 38.355 [37].

### 12A.3.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.4A.3.1.

The SL PRS-RSRP measurements performed and reported according to this section shall meet the SL PRS-RSRP measurement accuracy requirements in clause 10.4A.3.2, for each measured SL-PRS resource.

### 12A.3.5	Measurements Period Requirements

When the physical layer receives the last of:

-	SL-TDOA-ProvideAssistanceData and SL-TDOA-RequestLocationInformation, or

-	SL-AOA-ProvideAssistanceData and SL-AOA-RequestLocationInformation, or

-	SL-TOA-ProvideAssistanceData and SL-TOA-RequestLocationInformation, or

-	SL-RTT-ProvideAssistanceData and SL-RTT-RequestLocationInformation,

from LMF or another UE via SLPP specified in TS 38.355 [37], and the UE is configured to perform SL PRS-RSRP measurement together with the corresponding mandatory measurement (SL RSTD, SL AoA/ZoA, SL RTOA, and SL Rx-Tx, respectively), the UE shall be able to perform multiple SL PRS-RSRP measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in clause 12A.3.3), as defined in TS 38.215 [4].

The SL PRS-RSRP measurement for the measured anchor UE SL-PRS shall be performed during the measurement period of the corresponding mandatory measurement, with which the SL PRS-RSRP measurement is configured, i.e., during:

TSL RSTD,total  defined in clause 12A.2.5, for SL PRS-RSRP configured together with SL RSTD,

TSL Rx-Tx,total  defined in clause 12A.4.5, for SL PRS-RSRP configured together with SL Rx-Tx,

TSL AoA,total  defined in clause 12A.6.5, for SL PRS-RSRP configured together with SL AoA/ZoA, or

TSL RTOA,total  defined in clause 12A.7.5, for SL PRS-RSRP configured together with SL RTOA.

## 12A.4	SL Rx-Tx measurements

### 12A.4.1	Introduction

The requirements in clause 12A.4 apply for SL Rx-Tx measurements of the first and additional paths.

The requirements in clause 12A.4 shall apply provided the UE has received SL-RTT-RequestLocationInformation from LMF or another UE via SLPP specified in TS 38.355 [37] requesting the UE to measure and report SL Rx-Tx time difference measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.4.2	Requirements Applicability

The requirements in clause 12A.4 apply for periodic, aperiodic, and triggered SL Rx-Tx time difference measurements, provided:

-	SL Rx-Tx time difference related side conditions given in clause 10.4A.4.2 for FR1 are met for a corresponding Band.

-	The actual time difference between the corresponding SL-PRS transmission and reception used to derive the measurement is no larger than 160 ms.

### 12A.4.3	Measurement Capability

SL Rx-Tx time difference measurement capability is as indicated by the UE in SL-RTT-ProvideCapabilities according to TS 38.355 [37].

### 12A.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.4A.4.1.

The SL Rx-Tx time difference measurements performed and reported according to this section shall meet the SL Rx-Tx time difference measurement accuracy requirements in clause 10.4A.4.2, for each measured SL-PRS resource.

### 12A.4.5	Measurement Period Requirements

When the UE physical layer receives SL-RTT-ProvideAssistanceData message and SL-RTT-RequestLocationInformation message from LMF or another UE via SLPP specified in TS 38.355 [37], the UE shall be able to perform multiple SL Rx-Tx time difference measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in clause 12A.4.3), as defined in TS 38.215 [4]. For each individual SL-PRS resource measured by a UE, the SL Rx-Tx time difference measurement is performed during TSL Rx-Tx,total defined as:

TSL Rx-Tx,total=s=1STSL Rx-Tx,effect,s+Tuncertain ,

where,

S is the number of samples for a single SL Rx-Tx measurement defined below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth≤48 PRBs, and

for SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities reported by UE via maxNumOfActiveSL-PRS-ResourcesInOneSlot-r18 and maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 specified in TS 38.331 [2], TSL Rx-Tx,effect,s is defined as:

TSL Rx-Tx,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the start of the s-th and (s+1)-th slot of SL-PRS samples s and SL-PRS samples s+1, respectively,

TSL Rx-Tx,effect,s=Tdur,s+ΔSLproc , for s = S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of SL Rx-Tx measurement,

ΔSLprocis the processing time indicated by UE via  minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 specified in TS 38.331 [2] of the UE performing the SL Rx-Tx time difference measurement.

Tuncertain is defined as below:

If the UE reports the transmission timestamp of a SL PRS as defined in TS 38.215 [4], and the SL PRS transmission occurs after the SL PRS reception used to derive the measurement, Tuncertain is the additional time delay from the SL PRS reception until the actual SL PRS transmission.

Otherwise, Tuncertain=0.

A UE may drop one or more SL PRS measurement samples if the number of active slots or the number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities reported by UE via sl-PRS-CommonProcCapabilityPerBand-r18 specified in TS 38.331 [2]. For a single-sample measurement, the whole measurement may not be performed.

If the synchronization reference source changes during TSL Rx-Tx,total at the measuring UE, while the measuring UE is performing the SL Rx-Tx time difference measurement, then the measuring UE shall restart the SL Rx-Tx time difference measurement and shall send the measurement report during a measurement period, which can be longer than TSL Rx-Tx,total.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL Rx-Tx,total at the UE transmitting SL-PRS for the SL Rx-Tx measurement.

The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL Rx-Tx,total. Otherwise, the measurement period can be longer.

The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL Rx-Tx,total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

## 12A.5	SL PRS-RSRPP measurements

### 12A.5.1	Introduction

The requirements in clause 12A.5 shall apply provided the UE has received SL-TDOA-RequestLocationInformation or SL-AOA-RequestLocationInformation or SL-TOA-RequestLocationInformation or SL-RTT-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL PRS-RSRPP measurements defined in TS 38.215 [4].

### 12A.5.2	Requirements Applicability

The requirements in clause 12A.5 apply for periodic and triggered SL PRS-RSRPP measurements, provided:

-	SL PRS-RSRPP related side conditions given in clause 10.4A.5.2 for FR1 are met for a corresponding Band.

### 12A.5.3	Measurement Capability

SL PRS-RSRPP measurement capability is as indicated by the UE in SL-TDOA-ProvideCapabilities, SL-RTT-ProvideCapabilities, SL-AOA-ProvideCapabilities, or SL-TOA-ProvideCapabilities according to TS 38.355 [37].

### 12A.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.4A.5.1.

The SL PRS-RSRPP measurements performed and reported according to this section shall meet the SL PRS-RSRPP measurement accuracy requirements in clause 10.4A.5.2, for each measured SL-PRS resource.

### 12A.5.5	Measurement Period Requirements

When the physical layer receives

-	SL-TDOA-ProvideAssistanceData message and SL-TDOA-RequestLocationInformation message, or

-	SL-AOA-ProvideAssistanceData message and SL-AOA-RequestLocationInformation message, or

-	SL-TOA-ProvideAssistanceData message and SL-TOA-RequestLocationInformation message, or

-	SL-RTT-ProvideAssistanceData message and SL-RTT-RequestLocationInformation message,

from LMF or another UE via SLPP specified in TS 38.355 [37], and the UE is configured to perform SL PRS-RSRPP measurement together with the corresponding mandatory measurement (SL RSTD, SL AoA/ZoA, SL RTOA, and SL Rx-Tx, respectively), the UE shall be able to perform multiple SL PRS-RSRPP measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in clause 12A.5.3), as defined in TS 38.215 [4].

The SL PRS-RSRPP measurement for the measured anchor UE SL-PRS shall be performed during the measurement period of the corresponding mandatory measurement, with which the SL PRS-RSRPP measurement is configured, i.e., during:

-	TSL RSTD,total  defined in clause 12A.2.5, for SL PRS-RSRPP configured together with SL RSTD,

-	TSL Rx-Tx,total  defined in clause 12A.4.5, for SL PRS-RSRPP configured together with SL Rx-Tx,

-	TSL AoA,total  defined in clause 12A.6.5, for SL PRS-RSRPP configured together with SL AoA/ZoA, or

-	TSL RTOA,total  defined in clause 12A.7.5, for SL PRS-RSRPP configured together with SL RTOA.

## 12A.6	SL AoA measurements

### 12A.6.1	Introduction

The requirements in clause 12A.6 apply for SL AoA measurements of the first and additional paths.

The requirements in clause 12A.6.5 shall apply for azimuth angle of arrival (A-AoA) and zenith angle of arrival (Z-AoA) first path measurement, provided the UE has received SL-AoA-RequestLocationInformation from LMF or another UE via SLPP specified in TS 38.355 [37] requesting the UE to measure and report SL AoA measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.6.2	Requirements Applicability

The requirements in clause 12A.6 apply for periodic, aperiodic, and triggered SL AoA measurements, provided:

-	Conditions defined in clause 7.3E of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	SL AoA related side conditions given in clause B.4A.1 for FR1 are met for a corresponding Band.

### 12A.6.3	Measurement Capability

SL AoA measurement capability is as indicated by the UE in:

SL-AoA-ProvideCapabilities according to TS 38.355 [37].

### 12A.6.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL AoA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.4A.6.1.

### 12A.6.5	Measurement Period Requirements

When the UE physical layer receives the last of SL-AoA-ProvideAssistanceData message and SL-AoA-RequestLocationInformation message from LMF or another UE via SLPP specified in TS 38.355 [37], the UE shall be able to measure multiple SL AoA measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in 12A.6.3), as defined in TS 38.215 [4]. The SL AoA measurement shall be performedduring the measurement period TSL AoA,total defined as:

TSL AoA,total=s=1STSL AoA, effect,s ,

where,

S is the number of samples for the SL AoA measurement, defined as below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth ≤ 48 PRBs, and

for each SL-PRS sample s, which is received within a slot where the UE receives SCI and the associated SL-PRS is within its capabilities reported by UE via maxNumOfActiveSL-PRS-ResourcesInOneSlot-r18 and maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 specified in TS 38.331 [2]. TSL AoA,effect,s is defined as below,

TSL AoA,effect,s=ts+1-ts  for s < S, where ts and ts+1 are the beginning of the slots of SL-PRS sample s and SL-PRS sample s+1, respectively,

TSL AoA,effect,s=Tdur,s+ΔSLproc for s = S,

Tdur,s is the duration of slot carrying SL-PRS sample s of the SL AoA measurement,

ΔSLproc is the processing time indicated by UE minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 specified in TS 38.331 [2] of the UE performing SL AoA measurement.

A UE may drop one or more SL PRS measurement samples if the number of active slots or the number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities reported by UE via sl-PRS-CommonProcCapabilityPerBand-r18 specified in TS 38.331 [2]. 	For a single-sample measurement, the whole measurement may not be performed.

If the synchronization reference source of the measuring UE changes during TSL AoA,total, while the UE is performing the SL AoA measurements, then the measuring UE shall continue performing the SL AoA measurement after the synchronization reference source change, while meeting the requirements in this clause.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL AoA,total at the UE transmitting SL-PRS for the SL AoA measurement.

The requirements in this clause apply provided that no SL-PRS symbols for the SL AoA measurement are dropped due to e.g., the selection or reselection of synchronization reference source according to clause 12.4 during the measurement period. Otherwise, the measurement period can be extended.

The requirements in this clause, apply provided that reception of slots containing SL-PRS for the SL AoA measurement is not interrupted e.g., due to network coverage change. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be extended.

## 12A.7	SL RTOA measurements

### 12A.7.1	Introduction

The requirements in clause 12A.7 apply for SL RTOA measurements of the first and additional paths.

The requirements in clause 12A.7.5 shall apply provided the UE has received SL-RTOA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL RTOA measurements defined in TS 38.215 [4].

### 12A.7.2	Requirements Applicability

The requirements in clause 12A.7 apply for periodic, aperiodic, and triggered SL RTOA measurements, provided:

-	Conditions defined in clause 7.3E of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	SL RTOA related side conditions given in clause B.4A.1 for FR1 are met for a corresponding Band.

### 12A.7.3	Measurement Capability

SL RTOA measurement capability is as indicated by the UE in SL-RTOA-ProvideCapabilities according to TS 38.355 [37].

### 12A.7.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL RTOA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses 10.4A.7.1.

### 12A.7.5	Measurement Period Requirements

When the UE physical layer receives the last of SL-RTOA-ProvideAssistanceData message and SL-RTOA-RequestLocationInformation message from LMF or another UE via SLPP specified in TS 38.355 [37], the UE shall be able to measure multiple SL RTOA measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in 12A.7.3), as defined in TS 38.215 [4]. The SL RTOA measurement shall be performed during TSL RTOA,total defined as:

TSL RTOA,total=s=1STSL RTOA, effect,s ,

where,

S is the number of samples for the SL RTOA measurement, defined as below:

S = 1 for SL-PRS bandwidth > 48 PRBs,

S = 4 for SL-PRS bandwidth ≤ 48 PRBs, and

for SL-PRS sample s, which is received within a slot where the UE receives SCI, provided that the associated SL-PRS is within its capabilities reported by UE via maxNumOfActiveSL-PRS-ResourcesInOneSlot-r18 and maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 specified in TS 38.331 [2]. TSL RTOA,effect,s is defined as below,

TSL RTOA,effect,s=ts+1-ts, for s<S, where ts and ts+1 are the beginning of the slots of SL-PRS sample s and SL-PRS sample s+1, respectively,

TSL RTOA,effect,s=Tdur,s+ΔSLproc for s = S,

Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RTOA measurement,

ΔSLproc is the processing time indicated by UE via minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 specified in TS 38.331 [2] of the UE performing SL RTOA measurement.

If the synchronization reference source of the measuring UE changes during TSL RTOA,total, while the UE is performing the SL RTOA measurements, then the measuring UE shall restart the SL RTOA measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RTOA,total.

The requirements in this clause do not apply, when the synchronization reference source changes during TSL RTOA,total at the UE transmitting SL-PRS for the SL RTOA measurement.

The requirements in this clause apply provided that no SL-PRS symbols for the SL RTOA measurement that are dropped due to e.g., the selection or reselection of synchronization reference source according to clause 12.4 during the measurement period. Otherwise, the measurement period can be extended. The requirements in this clause apply, provided that reception of slots containing SL-PRS for the SL RTOA measurement is not interrupted due to network coverage change. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

## --- End of Change #21 ---