# 3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417733

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5002 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RRM_enh2-Perf) CR on test cases of PUCCH SCell activation and deactivation delay requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The single PUCCH SCell activation/deactivation dealy and interruption requirements in the corresponding test cases are not complete. - For PUCCH SCell activation/deactivation with Multiple SCells test cases, the delay and interruption requirements for other SCells are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the single PUCCH SCell activation/deactivation delay and interruption requirements in the corresponding test cases. - Add the delay and interruption requirements for other SCells in the PUCCH SCell activation/deactivation with Multiple SCells test cases. - Some typo corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases of PUCCH SCell activation and deactivation delay and interruption requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.3.6, A.6.5.3.7, A.6.5.3.8, A.6.5.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1

A.6.5.3.6.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell activation and deactivation times are within the requirements stated in clause 8.3, when the PUCCH SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.6.1-1 below. The test parameters are given in Tables A.6.5.3.6.1-2 and cell-specific parameters in A.6.5.3.6.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Cell 1 is the PCell in primary Timing Advance Group (pTAG) and Cell 2 is the PUCCH SCell in the secondary Timing Advance Group (sTAG). Before the test starts the UE is connected to Cell 1, but is not aware of Cell 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

The test consists of two sub tests. In Test 1, UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG. In Test 2, the TimeAlignmentTimer of sTAG expires before receiving the activation command.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the PUCCH SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. In Test 1, the UE shall be able to report valid CSI on for the activated PUCCH SCell on PUCCH SCell at latest in slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3∗Ttarget_PL−RS)+TCSI_Reporting_afterNR slot lengtℎn+THARQ+Tactivation_time+ X + TCSI_ReportingNR slot length, as defined in clause 8.3. In Test 2, the UE shall be able to report valid CSI for the activated PUCCH SCell on PUCCH SCell at latest in slot n+THARQ+Tdelay_PUCCH_SCellNR slot lengtℎ, as defined in clause 8.3.

Any PCell interruption due to activation of PUCCH SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell in a slot m+THARQ+3msNR slot lengtℎ, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot lengtℎ to m+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the PUCCH SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued.

Table A.6.5.3.6.1-1: known FR1 SCell activation test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.6.1-2: General test parameters for known FR1 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| TimeAlignmentTimer | ms | 500 | Test 1 and Test 2 |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.6.1-3: Cell specific test parameters for known FR1 SCell activation case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  |  | - |  |  |
|  | Config 3 |  | 10 |  |  | - |  |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  |  | - |  |  |
|  | Config 3 |  | 5 |  |  | - |  |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | - |  |  | 5 |  |  |
|  | Config 3 |  | - |  |  | 10 |  |  |
| CSI reporting offset for SCell | Config 1,2 | slot | - |  |  | 2 |  |  |
|  | Config 3 |  | - |  |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |

A.6.5.3.6.2	Test Requirements

In Test1，during T2 the UE shall start sending CSI reports for PUCCH SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3∗Ttarget_PL−RS)+TCSI_Reporting_afterNR slot lengtℎn+THARQ+Tactivation_time+ X + TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3. In Test2，during T2 the UE shall start sending CSI reports for PUCCH SCell with non-zero CQI index at latest in a slot n+THARQ+Tdelay_PUCCH_SCellNR slot lengtℎ, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for PUCCH SCell at latest in a slot m+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during PUCCH SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during PUCCH SCell deactivation shall not happen outside the slot m+1+THARQNR slot lengtℎ to m+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.18.

All of the above test requirements shall be fulfilled in order for the observed PUCCH SCell activation delay and PUCCH SCell deactivation delay to be counted as correct. The rate of correct observed PUCCH SCell activation delay and PUCCH SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3∗Ttarget_PL−RS)+TCSI_Reporting_afterNR slot lengtℎn+THARQ+Tactivation_time+ X + TCSI_ReportingNR slot length in Test 1 or in slot n+THARQ+Tdelay_PUCCH_SCellNR slot lengtℎ in Test 2 as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

A.6.5.3.7.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell activation and deactivation times are within the requirements stated in clause 8.3, when the PUCCH SCell in FR1 is unknown by the UE at the time of activation. In this test, UE shall support cross PUCCH group CSI reporting capability csi-ReportingCrossPUCCH-Grp-r16.

The supported test configurations are shown in Table A.6.5.3.7.1-1 below. The test parameters are given in Table A.6.5.3.7.1-2 and cell-specific parameters in Table A.6.5.3.7.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Cell 1 is the PCell in primary Timing Advance Group (pTAG) and Cell 2 is the PUCCH SCell in the secondary Timing Advance Group (sTAG). Both cells have constant signal levels throughout the test.

There are two sub tests in this section.

For Test 1 (valid TA case), UE is provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment for sTAG.

For Test 2 (invalid TA case), TimeAlignmentTimer of sTAG expires before UE receives the activation command

Before the test starts the UE is connected to PCell, but is not aware of PUCCH SCell. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell becomes configured on NR. During T1 the PUCCH SCell is powered off and UE is not aware of PUCCH SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI on PUCCH SCell for the activated PUCCH SCell at latest in

slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3∗Ttarget_PL−RS)+TCSI_Reporting_afterNR slot lengtℎn+THARQ+Tactivation_time+X+TCSI_ReportingNR slot length for Test 1, or

slot n+THARQ+Tdelay_PUCCH_SCellNR slot lengtℎ  for Test 2

Note: Tdelay_PUCCH_SCell = Tactivation_time + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS) + TCSI_reporting_after, as defined in clause 8.3,

Any PCell interruption due to activation of PUCCH SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruptionn+1+THARQ+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2TXTactivation_time+X+TCSI_ReportingTdelay_PUCCH_SCell.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the PUCCH SCell in a slot m+THARQ+3msNR slot lengtℎ, as defined in clause 8.3, and the starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot lengtℎ to m+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the PUCCH SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued.

Table A.6.5.3.7.1-1: unknown FR1 PUCCH SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.7.1-2: General test parameters for unknown FR1 PUCCH SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated PUCCH secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| TimeAlighmentTimer | ms | 500 | Identify the whether TA information is valie or not |
| T1 | s | [7] | During this time the PCell shall be known and the PUCCH SCell configured and detected. |
| T2 | s | [1] | During this time the UE shall activate the PUCCH SCell. |
| T3 | s | [1] | During this time the UE shall deactivate the PUCCH SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.7.1-3: Cell specific test parameters for unknown FR1 PUCCH SCell activation case, 160ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | N/A |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | N/A |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | N/A |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting (CQI index non 0) periodicity for PUCCH SCell | Config 1,2 | slot | N/A |  |  | 5 |  |  |
|  | Config 3 |  | N/A |  |  | 10 |  |  |
| CSI reporting (CQI index non 0) offset for PUCCH SCell | Config 1,2 | slot | N/A |  |  | 4 |  |  |
|  | Config 3 |  | N/A |  |  | 6 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  | N/A | -104 | -104 |
|  | Config 3 |  | -101 |  |  | N/A | -101 | -101 |
|  |  | dB | 17 |  |  | -infinity | 17 | 17 |
|  |  | dB | 17 |  |  | -infinity | 17 | 17 |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  | -infinity | -87 | -87 |
|  | Config 3 |  | -84 |  |  | -infinity | -84 | -84 |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  | N/A | -87 | -87 |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  | N/A | -58.96 | -58.96 |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  | N/A | -52.87 | -52.87 |
| Propagation condition |  | - | AWGN |  |  | N/A | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |

A.6.5.3.7.2	Test Requirements

During T2, as defined in clause 8.3, the UE shall start sending CSI reports for PUCCH SCell on PUCCH SCell with non-zero CQI index at latest in

a slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3∗Ttarget_PL−RS)+TCSI_Reporting_afterNR slot lengtℎn+THARQ+Tactivtion_time+TCSI_ReportingNR slot lengtℎ for Test 1. Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

a slot n+THARQ+Tdelay_PUCCH_SCellNR slot lengtℎ  for Test 2.

During T3 the UE shall stop sending CSI reports for PUCCH SCell at latest in a slot m+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

During T2 interruption of PCell during PUCCH SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during PUCCH SCell deactivation shall not happen outside the slot m+1+THARQNR slot lengtℎ to m+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.18.

All of the above test requirements shall be fulfilled in order for the observed PUCCH SCell activation delay and PUCCH SCell deactivation delay to be counted as correct. The rate of correct observed PUCCH SCell activation delay and PUCCH SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot  n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing),   3∗Ttarget_PL−RS)+TCSI_Reporting_afterNR slot lengtℎn+THARQ+Tactivtion_time+TCSI_ReportingNR slot lengtℎ  and in a slot n+THARQ+Tdelay_PUCCH_SCellNR slot lengtℎ for Test 1 and Test 2, respectively, as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command

A.6.5.3.8.1	Test Purpose and Environment

The purpose of this test is to verify the SCell activation and deactivation delay requirements for PUCCH SCell with multiple SCells specified in clause 8.3.13 and 8.3.15, when one configured deactivated known PUCCH SCells in FR1 and one configured unknown SCell in FR1 by the UE at the time of activation.

The supported test configurations are defined in table A.6.5.3.8.1-1 below. The test parameters are given in table A.6.5.3.8.1-2 and cell-specific parameters in A.6.5.3.8.1-3 below.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three NR carriers. All Cells has constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCC), but is not aware of Cell 2 (PUCCH SCell) and Cell 3(SCell). The UE is monitoring the Cell 1 (PCC). The UE shall be continuously scheduled in the Cell 1 throughout the whole test. PCC, SCC of Cell 2 and SCC of Cell 3 are on different FR1 bands.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell (Cell 2) and SCell (Cell 3) become configured on radio channel 2 and 3 respectively.

A MAC message for activation of UCCH SCell (Cell 2) and SCell (Cell 3) is sent by the test equipment 100ms after the RRC message, in a slot # denoted m. The point in time at which the MAC message for activation of PUCCH SCell (Cell 2) and SCell (Cell 3) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 and Cell 3 are increased to same level as for Cell 1. The UE shall be able to report valid CSI on PCell for the activated PUCCH SCell (Cell 2) at latest in slot m+THARQ+Tdelay_multiple_SCells_PUCCH_SCellNR slot lengtℎm+THARQ+Tdelay_multiple_SCells_otℎer_SCellNR slot lengtℎ  and be able to report valid CSI on PCell for the activated SCell  (Cell 3) at latest in slot m+THARQ+Tdelay_multiple_SCells_otℎer_SCellNR slot lengtℎ as defined in clause 8.3.13 provided the PUCCH SCell can be successfully detected on the first attempt.

For Cell 2 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the PUCCH SCell activation has been completed. For Cell 3 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the DL SCell activation has been completed.

Any PCell interruption due to activation of SCells (including one SCell configured with PUCCH) shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where  Ninterruption is the interruption length given in section 8.2.

Time period T3 starts when a MAC message for deactivation of the SCells (Cell 2 and Cell 3), sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCells (Cell 2 and Cell 3) at latest in slot n+THARQ+3msNR slot lengtℎ  as defined in clause 8.3. The starting point of PCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot lengtℎ to n+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell and DL SCell, respectively.

The test equipment verifies the PUCCH SCell activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received. The test equipment verifies the DL SCell activation time by counting the slots from the time when the DL SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the PUCCH SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued. The test equipment verifies the DL SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for DL SCell is discontinued.

Table A.6.5.3.8.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.8.1-2: General test parameters for unknown FR1 SCell activation case with 2 deactivated SCells, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three NR radio channel (1, 2, 3) are used for this test |
| Active Cell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated PUCCH SCell |  | Cell 2 | Configured deactivated PUCCH SCell on NR RF channel number 2 which is an inter-band CC to PCC of Cell 1; |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated SCell on NR RF channel number 3 which is an inter-band CC to PCC of Cell 1 and SCC of Cell 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell 2 timing offset to cell1 | s | 0 |  |
| Cell 3 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Time alignment error between cell3 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.8.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  | Unit | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T2 | T3 | T2 | T3 |
| SSB ARFCN |  |  |  | Freq2 |  | Freq3 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |  |
| DL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  |  |  |
| DL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  |  |  |
| UL initial BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.0.1 |  |  |  |
| UL dedicated BWP configuration |  | Config 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | Config 2,5 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3,6 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| RMC CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | Config 2,5 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3,6 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  | Config 2,5 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3,6 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |  |
|  |  | Config 3,6 |  | 30kHz |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1,4 |  | CSI-RS.1.1 FDD |  |  |  |
|  |  | Config 2,5 |  | CSI-RS.1.1 TDD |  |  |  |
|  |  | Config 3,6 |  | CSI-RS.2.1 TDD |  |  |  |
| reportConfigType |  | Config 1-6 |  | periodic |  |  |  |
| reportQuantity |  | Config 1-6 |  | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity |  | Config 1,2,4,5 | slot | 5 |  | N/A |  |
| CSI reporting offset |  | Config 3,6 | slot | 10 |  | N/A |  |
|  |  | Config 1,2,4,5 |  | 2 |  | N/A |  |
|  |  | Config 3,6 |  | 4 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104 |  |  |  |
| Note2 |  | Config 1,2,4,5 | dBm/SCS | -104 |  |  |  |
|  |  | Config 3,6 |  | -101 |  |  |  |
|  |  |  | dB | 17 |  |  |  |
|  |  |  | dB | 17 |  |  |  |
| SS-RSRPNote3 |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |
|  |  | Config 3,6 |  | -84 |  |  |  |
| SCH_RP Note 3 |  |  | dBm/15 kHz | -87 |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| IoNote3 | Config 1,2,4,5 |  | dBm/ 9.36MHz | -58.96 |  |  |  |
|  | Config 3,6 |  | dBm/ 38.16MHz | -52.87 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |

A.6.5.3.8.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.  Whether CSI report in slot (m+k) was interrupted is checked by monitoring ACK/NACK sent in PCell in slot (m+k). And the UE shall be able to report valid CSI for the activated PUCCH SCell (Cell 2) at latest in slot m+THARQ+Tdelay_multiple_SCells_PUCCH_SCellNR slot lengtℎTHARQ+Tdelay_multiple_SCells_otℎer_SCellNR slot lengtℎ  and report valid CSI for the activated SCell (Cell 3) at latest in slot m+THARQ+Tdelay_multiple_SCells_otℎer_SCellNR slot lengtℎ as defined in clause 8.3.13. And the PCell interruption due to activation of SCells (including one SCell configured with PUCCH) shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in section 8.2.

During T3, the UE shall carry out deactivation of the the SCells (including one SCell configured with PUCCH) at latest in slot n+THARQ+3msNR slot lengtℎ  as defined in clause 8.3. And the starting point of PCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot lengtℎ to n+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX

A.6.5.3.9.1	Test Purpose and Environment

The purpose of this test is to verify that the PUCCH SCell and DL SCell activation and deactivation times are within the requirements stated in clause 8.3.13 and clause 8.3.15, when the PUCCH SCell in FR1 and DL SCell in FR1 are unknown to the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.9.1-1 below. The test parameters are given in Tables A.6.5.3.9.1-2 and cell-specific parameters in A.6.5.3.9.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three NR carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell 2 (PUCCH SCell) and Cell 3(DL SCell). The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test. PCC, SCC of Cell 2 and SCC of Cell 3 are on different bands. The primary PUCCH group contain Cell 1 and the secondary PUCCH group contains Cell 2 and Cell 3.

At the beginning of T1 the UE receives an RRC message by which the PUCCH SCell (Cell 2) and DL SCell (Cell 3) becomes configured on radio channel 2 and 3 respectively. The UE starts monitoring the SCC1(Cell 2 CC) and SCC2(Cell 3 CC). The test equipment sends a MAC message for activation of the PUCCH SCell and DL SCell simultaneously.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated PUCCH SCell at latest in slot n+ THARQ+Tdelay_multiple_SCells_PUCCH_SCellNR slot lengtℎ, and report valid CSI in PCell for the activated DL SCell at latest in slot n+THARQ+Tdelay_multiple_SCells_otℎer_SCellNR slot lengtℎ, as defined in clause 8.3.13. In this test case, both valid TA and invalid TA cases shall be tested.

Test for case when UE has valid TA: the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running, and Tdelay_multiple_SCells_PUCCH_SCell = Tdelay_multiple_SCells_PUCCH_SCell = Tactivation_time_multiple_scells + max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS) + TCSI_reporting_after.

Test for case when UE do not have valid TA: Tdelay_multiple_SCells_PUCCH_SCell = Tactivation_time_multiple_scells + max ((TFirst_available_CSI + TCSI_processing), (T1+T2+T3), 3*Ttarget_PL-RS) + TCSI_reporting_after.

Tdelay_multiple_SCells_other_SCell   = Tactivation_time_multiple_scells +TCSI_Reporting.

Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario as specified in section 8.3.7

For Cell 2 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the PUCCH SCell activation has been completed. For Cell 3 activtion, the UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the DL SCell activation has been completed.

Any PCell interruption due to activation of PUCCH SCell or DL SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.13, where Ninterruption is the interruption length given in clause 8.2.2.2.7.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell abd DL SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.15, and the starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot lengtℎ to m+1+THARQ+3msNR slot lengtℎ, as defined in clause 8.3.15.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell and DL SCell, respectively.

The test equipment verifies the PUCCH SCell activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received. The test equipment verifies the DL SCell activation time by counting the slots from the time when the DL SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the PUCCH SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued. The test equipment verifies the DL SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for DL SCell is discontinued.

Table A.6.5.3.9.1-1: unknown FR1 PUCCH SCell and DL SCell activation test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode, for both PUCCH SCell and DL SCell |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode, for both PUCCH SCell and DL SCell |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode, for both PUCCH SCell and DL SCell |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.3.9.1-2: General test parameters for unknown FR1 PUCCH SCell and DL SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three NR radio channel (1, 2, 3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated PUCCH SCell |  | Cell 2 | Configured deactivated PUCCH secondary cell on NR RF channel number 2 |
| Configured deactivated DL SCell |  | Cell 3 | Configured deactivated DL secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | For both Cell 2 and Cell 3 |
| Cell 2/Cell 3 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1; cell3 and cell1; cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 0.1 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.9.1-3: Cell specific test parameters for unknown FR1 PUCCH SCell and DL SCell activation case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 and Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  |  | - |  |  |
|  | Config 3 |  | 10 |  |  | - |  |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  |  | - |  |  |
|  | Config 3 |  | 5 |  |  | - |  |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | - |  |  | 5 |  |  |
|  | Config 3 |  | - |  |  | 10 |  |  |
| CSI reporting offset for SCell | Config 1,2 | slot | - |  |  | 2 |  |  |
|  | Config 3 |  | - |  |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |

A.6.5.3.9.2	Test Requirements

The test requirements defined in clause A.6.5.3.8.2 shall apply to this test case.

<End of Change 1>