3GPP TSG-RAN WG4 Meeting # 113											                 R4-2418259

Orlando, US, 18th – 22nd November, 2024

Agenda item:			4.5.1

Source:	Moderator (Xiaomi)

Title:	Topic summary for [113][201] Maintenance_up_to_R16

Document for:	Information

Introduction

This document provides summary for RRM related Tdocs submitted to the following AI

4.5.1		Rel-15/16 RRM requirements 	[WI code]

Please kindly take following notes for Tdoc handling in this topic thread.

- Open issues are based on Discussion papers.

- Based on Chair’s guidance, all CRs in this email thread will be first handled in NWM flagging procedure which will be triggered separately.

- Tdocs that are withdrawn or revised in the Tdocs list will not be handled in the summary document or the NWM flagging procedure.

- Cat-A CRs will not be handled in the summary document or the NWM flagging procedure.

Topic #1: R15 NR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418369 | Nokia | - Active TCI state list update delay is to define time after which the UE can be scheduled using PDCCH on a newly activated target PDSCH TCI state. The delay is defined from the time the UE receives the TCI state activation MAC-CE until the UE receives PDCCH scheduling the PDSCH with the target PDSCH TCI state. - The delay from the time the UE receives PDCCH scheduling the PDSCH with the target PDSCH TCI state is taken into account in DCI based TCI state switch delay requirement. - The technical meaning of the current Rel-15 requirement shall not be changed as this impact actual implementations. - Proposal 1: If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH to schedule scheduling the PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3. |
| R4-2418540 | Apple | - While the UE is applying gradual timing adjustment, the TTD for the pair of TAGs or between PCell-PSCell could exceed the MTTD requirement - In certain cases there might be an impact to UL transmission if the MTTD is exceeded. - For NR-DC, EN-DC and NE-DC if asynchronous mode is supported there might not be issue with UL transmission if TTD exceeds MTTD. - For NR-DC, EN-DC and NE-DC if asynchronous mode is not supported there would be an impact to UL transmission if TTD exceeds MTTD. - For NR CA if TTD exceed MTTD, there would be an impact to UL transmission. Proposal #1:The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases: Inter-band CA Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation Intra-band EN-DC if UE doesn’t support asynchronous operation Inter-band NE-DC if UE doesn’t support asynchronous operation Inter-band NR-DC if UE doesn’t support asynchronous operation |
| R4-2418375 | Nokia | - Define the additional MIB reading delay, assuming that the signal quality in the target cell is sufficient for successful MIB reception on the first attempt. - Allow UE additional 10 ms delay for target cell MIB reading. |
| R4-2419429 | Ericsson | - Proposal 1:If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of 10 ms. |

Open issues summary

Sub-topic 1-1: MAC CE based active TCI state list update delay

- Proposals

  - Option 1 (Nokia):

    - Proposal 1: If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH to schedule scheduling the PDSCH with the new target TCI state at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

- Recommended WF

  - Discuss the option

Sub-topic 1-2: Gradual timing adjustment for CA and DC

- Proposals

  - Option 1 (Apple):

    - Proposal 1:The gradual timing adjustment is applied in the until timing error is within ±Te or the MRTD requirement depending on the configuration is not exceeded in the following cases:
       Inter-band CA
       Inter-band EN-DC with spectrum overlapping between LTE and NR, and UE doesn’t support asynchronous operation
       Intra-band EN-DC if UE doesn’t support asynchronous operation
       Inter-band NE-DC if UE doesn’t support asynchronous operation
       Inter-band NR-DC if UE doesn’t support asynchronous operation

- Recommended WF

  - Discuss the option

Sub-topic 1-3: NR-E-UTRAN HO delay

- Proposals

  - Option 1 (Nokia):

    - Define the additional MIB reading delay, assuming that the signal quality in the target cell is sufficient for successful MIB reception on the first attempt.

    - Allow UE additional 10 ms delay for target cell MIB reading

  - Option 1a (Ericsson):

    - Proposal 1:If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of 10 ms.

- Recommended WF

  - Discuss the option. Option 1 and option 1a are the same.

Topic #2: R16 NR_2step_RACH-Perf

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418637 | Huawei, HiSilicon, Starpoint | Observations 1: In WF [1], transmit power of first MsgA PUSCH is defined as the 3μ+2 dB power shift to first MsgA PRACH power, rather than the absolute power of 3μ+2 dBm. Proposal 1: Defining transmit power of first MsgA PUSCH power by specifying its power offset relative to first MsgA PRACH. Observations 2: msgA-DeltaPramble can only be even value as specified in 38.331. Proposal 2: Set msgA-DeltaPramble = 2dB in MsgA RMCs. Proposal 3: Specify the power offset of first MsgA PUSCH to first MsgA PRACH in test requirements as follows: - For FDD test configurations, power offset is -3dB; - For TDD test configurations, power offset is 0dB; Proposal 3: Set ra-PreambleIndex = 40 in 2-step CFRA RMCs to align with 4-step CFRA RMCs. |

Open issues summary

Sub-topic 2-1: 2-step RACH RRM test case

- Proposals

  - Option 1 (Huawei):

    - Proposal 1: Defining transmit power of first MsgA PUSCH power by specifying its power offset relative to first MsgA PRACH.

    - Proposal 2: Set msgA-DeltaPramble = 2dB in MsgA RMCs.

    - Proposal 3: Specify the power offset of first MsgA PUSCH to first MsgA PRACH in test requirements as follows:

    - For FDD test configurations, power offset is -3dB;

    - For TDD test configurations, power offset is 0dB;

    - Proposal 4: Set ra-PreambleIndex = 40 in 2-step CFRA RMCs to align with 4-step CFRA RMCs.

- Recommended WF

  - Discuss the option

Topic #3: R16 NR_RRM_enh-Core

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2419004 | Ericsson | Proposal 1: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns. |

Open issues summary

Sub-topic 3-1: Clarification on mandatory gap patterns

- Proposals

  - Option 1 (Ericsson):

    - Proposal 1: RAN4 to send LS to RAN2 to clarify that UE is not allowed to report ‘0’ for one pattern in supportedGapPatterns-NRonly-r16 if UE reports ‘1’ to support the same pattern in Rel-15 supportedGapPatterns.

- Recommended WF

  - Discuss the option