3GPP TSG-RAN WG4 Meeting # 113														R4-2418262

Orlando, US, 18th – 22nd November, 2024

Agenda item:			5.1.2

Source:	Moderator (vivo)

Title:	Topic summary for [113][204] FR2_multiRx

Document for:	Information

# Introduction

This topic summary covers following agenda for FR2 multi-Rx chain DL reception WI.

5.10	Requirement for NR FR2 multi-Rx chain DL reception

5.10.1	RRM core and performance requirements

Recommendation of issues for online discussion:

For Topic #1:

Issue 1-1: Conditions/applicability of multi-Rx operation for fast beam sweeping

For Topic #2:

Issue 2-1: Test setup for dual TCI state switching for m-DCI

Issue 2-2: Test requirements for measurement and scheduling restriction TC

Issue 2-3: AoA setup for multi-Rx

# Topic #1: Core requirements maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417956 | Xiaomi | Proposal 1: Starting point of fast beam sweeping is that UE is configured with group-based beam reporting (GBBR) report and UE is activated with dual TCI states. Proposal 2: Ending point of fast beam sweeping can be initiated by NW or UE. |
| R4-2418537 | Apple | Observation 1: Neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation. Proposal 1: It is proposed to adopt Option 2, i.e., in “3.6.19 Applicability of requirements for multi-Rx operation in FR2-1”, add a note “Note: The requirements related to the support of [reducedRxBeamNum] is not applicable when the UE has sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.” |
| R4-2417922 | BeammWave | Corrections to fast beam sweeping capability |
| R4-2418464 | CMCC | CR on RLM for multi-RX operation |
| R4-2418538 | Apple | CR on multi-RX core performance maintenance |
| R4-2418763 | vivo | CR on core part requirements maintenance for multi-Rx |
| R4-2418783 | OPPO | CR on RRM requirements for multi-Rx |
| R4-2419163 | OPPO | CR on RRM requirements for multi-Rx |
| R4-2419277 | ZTE Corporation, Sanechips | CR on core part of R18 multi-Rx RLM&BFD&CBD |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

### Sub-topic 1-1: General

#### Issue 1-1: Conditions/applicability of multi-Rx operation for fast beam sweeping

In TS38.133 v18.7.0, the condition of multi-Rx operation for fast beam sweeping is specified as follows.

N=2, 4, or 6 for FR2-1 for UE supporting fastBeamSweepingMultiRx-r18 according to the conditions in clause 3.6.19.

In clause 3.6.19 in TS38.133 v18.7.0, the applicability requirements for multi-Rx operation in FR2-1 are as follows.

The requirements related to the support of fastBeamSweepingMultiRx-r18 is applicable when the network configures the UE with a CSI report containing groupBasedBeamReporting-v1710.

Proposals are further provided to modify/enhance the conditions or applicability.

- Proposals

  - Option 1: (Apple)

    - In “3.6.19 Applicability of requirements for multi-Rx operation in FR2-1”, add a note

      - Note: The requirements related to the support of [reducedRxBeamNum] is not applicable when the UE has sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.

  - Option 2: (Xiaomi)

    - Starting point of fast beam sweeping is that UE is configured with group-based beam reporting (GBBR) report and UE is activated with dual TCI states.

    - Ending point of fast beam sweeping can be initiated by NW or UE.

- Recommended WF

  - Draw conclusion in this meeting.

## Recommendations on CRs

| Tdoc number | Source | Title | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2417922 | BeammWave | Corrections to fast beam sweeping capability | Merged | All CRs are focused on requirements for fast beam sweeping and scheduling/measurement restriction relaxation by changing UE capabilities and other changes. CR 8763 covers most of the sections that need to be revised. 8783 and 9163 seems the same. |
| R4-2418464 | CMCC | CR on RLM for multi-RX operation | Merged |  |
| R4-2418783 | OPPO | CR on RRM requirements for multi-Rx | Merged |  |
| R4-2419163 | OPPO | CR on RRM requirements for multi-Rx | Merged |  |
| R4-2418763 | vivo | CR on core part requirements maintenance for multi-Rx | Revised |  |
| R4-2419277 | ZTE Corporation, Sanechips | CR on core part of R18 multi-Rx RLM&BFD&CBD | Revised | For CBD requirements only. Others are merged into 8763. |
| R4-2418538 | Apple | CR on multi-RX core performance maintenance | Return to | Depending online discussions. |

# Topic #2: Performance requirements maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2419287 | Ericsson | - Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be - TCI states before TCI state switching: - For CORESETPoolIndex 0 - PDCCH TCI state: TCI state 0 (probe 0, SSB0) - PDSCH TCI states: TCI state 0 (probe 0, SSB0) - For CORESETPoolIndex 1 - PDCCH TCI state: TCI state 1 (probe 1, SSB1) - PDSCH TCI states: TCI state 1 (probe 1, SSB1) - TCI states after TCI state switching: - For CORESETPoolIndex 0 - PDCCH TCI state: TCI state 0 (probe 0, SSB0) - PDSCH TCI states: TCI state 3 (probe 0, SSB3) - For CORESETPoolIndex 1 - PDCCH TCI state: TCI state 1 (probe 1, SSB1) - PDSCH TCI states: TCI state 2 (probe 2, SSB2) |
| R4-2417684 | Nokia | Observation 1: Since three probes are to be used during the test case for TCI state switching delay in m-DCI, two RS (RS1 and RS3) will be transmitted from same probe. Observation 2: RAN4 agreement does not clarify if RS1 and RS3 will be transmitted with same power and at the same time from the same probe. Observation 3: Test cannot verify accurately if the UE has performed time/frequency synchronization when it receives the command to switch to PDSCH QCL’d to RS3 if RS1 and RS3 are transmitted with same power and at the same time from the same probe. Observation 4: When three probes are used to transmit four RS, the test probe transmitting more than one RS should emulate different DL transmit beams by transmitting different RS with different power and delay Proposal 1: When using the same probe for RS1 and RS3, the reference signals (RS1 and RS3) should be transmitted with a different power (5 dB difference) and with a time delay (difference of 25% of CP length). Observation 5: Test case does not indicate that UE has PDSCH during T1. |
| R4-2417907 | MediaTek inc. | (NR_FR2_multiRX_DL-Perf) CR on measurement restriction relaxation TC |
| R4-2418764 | vivo | CR on AoA setup for multi-Rx |
| R4-2419288 | Ericsson | maintenance CR for test cases for multi-rx |
| R4-2417685 | Nokia | CR for Rel-18 multi-Rx AoA setup |
| R4-2417686 | Nokia | CR for Rel-18 multi-Rx test case corrections |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Test setup

Sub-topic description:

Open issues and candidate options before f2f meeting:

#### Issue 2-1: Test setup for dual TCI state switching for m-DCI

- Proposals (Nokia)

  - P1 (Ericsson): Distinguish PDCCH and PDSCH TCI states clearly in the m-DCI TCI state switching test case. The TCI states and associated probes and SSBs should be

    - TCI states at the beginning of the test:

      - For CORESETPoolIndex 0

        - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

        - PDSCH TCI states: TCI state 0 (probe 0, SSB0)

      - For CORESETPoolIndex 1

        - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

        - PDSCH TCI states: TCI state 1 (probe 1, SSB1)

    - TCI states at the end of the test:

      - For CORESETPoolIndex 0

        - PDCCH TCI state: TCI state 0 (probe 0, SSB0)

        - PDSCH TCI states: TCI state 3 (probe 0, SSB3)

      - For CORESETPoolIndex 1

        - PDCCH TCI state: TCI state 1 (probe 1, SSB1)

        - PDSCH TCI states: TCI state 2 (probe 2, SSB2)

  - P2: (Nokia)

    - When using the same probe for RS1 and RS3, the reference signals (RS1 and RS3) should be transmitted with a different power (5 dB difference) and with a time delay (difference of 25% of CP length)

  - P3: (Nokia)

    - Test case does not indicate that UE has PDSCH during T1

- Recommended WF

  - Further discuss the proposals.

#### Issue 2-2: Test requirements for measurement and scheduling restriction TC

- Proposals

  - P1: (MTK)

    - In the test requirement, change the delay to Y + 80 slots + 80 slots.

- Recommended WF

  - Discuss if the proposal is agreeable.

#### Issue 2-3: AoA setup for multi-Rx

In TS38.133 v18.7.0, the AoA setup for multi-Rx are specified with brackets as follows.

A.3.15.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D

The angular separation between the directions (AoA1 and AoA2) of the 2 active probes is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and [shall not be changed for each test iteration].

A.3.15.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D

The angular offset between the directions of the AoA pair (AoA2, AoA3) is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 and [shall not be changed for each test iteration].

- Proposals

  - P1: (vivo)

    - In AoA setup 5, the angular separation between the directions (AoA1 and AoA2) of the 2 active probes shall not be changed for each test iteration.

    - In AoA setup 6, the angular offset between the directions of the AoA pair (AoA2, AoA3) shall not be changed for each test iteration.

  - P2: (Nokia)

    - In AoA setup 6, the UE positioning shall be such that the UE passes both spherical coverage requirements.

- Recommended WF

  - Discuss if the proposals are agreeable.

## Recommendations on CRs

| Tdoc number | Source | Title | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2417907 | MediaTek inc. | (NR_FR2_multiRX_DL-Perf) CR on measurement restriction relaxation TC | Return to |  |
| R4-2418764 | vivo | CR on AoA setup for multi-Rx | Return to |  |
| R4-2417685 | Nokia | CR for Rel-18 multi-Rx AoA setup | Return to |  |
| R4-2419288 | Ericsson | maintenance CR for test cases for multi-rx | Return to | In which CR the test for m-DCI will be captured depending on online progress. |
| R4-2417686 | Nokia | CR for Rel-18 multi-Rx test case corrections | Return to |  |

---EoD---