3GPP TSG-RAN WG4 Meeting # 113												      R4-2418266

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda item:			   5.1.2

Source:	Moderator (MediaTek Inc.)

Title:	Topic summary for [113] [208] NR_Mob_enh2

Document for:	Information

Introduction

This document is the email discussion summary for [113] [208] NR_Mob_enh2 with the following topics covered.

- Topic 1: LTM – Core part (AI 5.23.1)

  - The proposals related to LTM core part requirements are captured here.

- Topic 2: Improvement on SCell/SCG setup delay - Core part (AI 5.23.1)

  - The proposals related to Improvement on SCell/SCG setup delay core part requirements are captured here.

- Topic 3: Performance Part (AI 5.23.1)

  - The proposals related to performance part are captured here.

Recommended Online issues:

Part 1: LTM

Issue 1-2-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay and PDCCH-order RACH delay

Issue 1-3-1-2: Conditions of Tfirst-RS =0 in cell switch delay

Issue 1-5-1: Answer to Q1

Issue 1-5-3: Answer to Q3

Issue 1-5-2: Answer to Q2

Issue 1-6-1: Whether and how to reply RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM (R2-2409360/ R4-2417508)

Issue 1-4-1: Capability for fast RRC decoding

Issue 1-4-2: Capability for RACH-less LTM cell switch

Issue 1-4-3: Clarification on LTM L1-RSRP related capabilities

Issue 1-3-1-1: T/F tracking when TRS as QCL source in cell switch delay

Issue 1-3-2-1: Which cell(s) TLTM-RRC-processing = 0 apply to when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability?

Issue 1-3-3-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report?

Issue 3-1-1: Test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2)

Part 2: Improvement on SCell/SCG setup delay:

Issue 2-1-1: Mis-alignment between RAN2 and RAN4 spec when measReselectionCarrierListNR is not configured

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay

Topic #1: LTM – Core part (AI 5.23.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417712 | CATT | Proposal 1: RAN4 to remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements. Proposal 2: RAN4 to confirm the understanding from RAN2 in question 1. Proposal 3: RAN4 to reply RAN2 that it is possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells. |
| R4-2417897 | MediaTek Inc. | Observation 1: In early TCI state activation requirements, it is not precise to capture the requirement based on whether SSB is within the BWP or whether the SSB is to be measured with MG. Proposal 1: In early TCI state activation requirements, differentiate the cases based on whether the SSB is to be measured within or outside gap. Proposal 2: Remove the sentence “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.” in early TCI state activation requirements. Proposal 3: In PDCCH order delay requirements: - differentiate the cases based on whether the SSB is to be measured within or outside gap. - Remove the sentence “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.” Proposal 4: The following agreement is only applicable to intra-frequency: - Extend the condition for Tfirst-RS = 0 “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480ms” in FR1 for UE not supporting/configured with L1 measurement. Proposal 5: Open to discuss how to optimize the known TCI state definition in R19 considering L1 report can be triggered when leaving condition is met. Not to further optimize known TCI state definition in R18. Proposal 6: Not to extend cell switch delay requirements to FR2 without L1 report and remove the following in known TCI state definition for cell switch: [- The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or] [- The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or] Proposal 7: When the configured candidate cells are more than number of candidates that UE supports early RRC decoding and validity check, we are open to further optimize this scenario only if a simple and clear rule can be agreed. Otherwise, we prefer not to optimize further. Proposal 8: Send LS to RAN2 to inform them that 45-3a/45-4a (MAC-CE activated joint LTM TCI states) are not the prerequisite FG of 39-6 “Fast processing of LTM candidate cell RRC configuration”. Proposal 9: For Q1, confirm RAN2’s understanding that if the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations. Proposal 10: For Q2, reply RAN2 that: - No. But if the number of serving cell(s) + SpCell/SCell(s) in decoded LTM candidate configuration(s) exceeds maxNumberStoredConfigCells, UE will not store the decoded configurations and still needs extra time to perform RRC decoding and validity check again during cell switch. This leads to a waste of UE efforts and power, and not preferred in RAN4. - RAN4 suggests RAN2 adding a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration to avoid potential waste of UE efforts and power. Proposal 11: For Q3, reply RAN2 that: Yes. RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding. |
| R4-2418462 | CMCC | Proposal 1: it is proposed to update Tfirst-ssb to cover TRS in cell switch delay requirements. |
| R4-2418503 | ZTE Corporation, Sanechips | Proposal 1: Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell. Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case, the definiation of Tfirst-SSB should be applied to both inter-frequency L1 measurement with MG and inter-frequency L3 measurement with MG as LTM can be triggered by L1 or L3 measurement. Proposal 3: When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed. Observation 1: for FR2 without L1 measurement report, cell switch delay could be reduced compared with legacy handover when TCI state is activated. Proposal 4: cell switch delay requirements applicable to FR2 without L1 measurement report with the following conditions - The target TCI state in the cell switch command is activated not more than [1280] ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or - The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every [1280] ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB |
| R4-2418594 | Apple | Proposal 1: answer to Q1 is YES, i.e. UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations under this condition. Proposal 2: answer to Q2: it is feasible for UE to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, NW has to assume UE may not have done early decoding, i.e. TLTM-RRC-processing during cell switch cannot be 0 as minimum requirements. Proposal 3: if NW expects UE can successfully perform early ASN.1 decoding, number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations is expected NOT to exceed the UE reported maxNumberStoredConfigCells. Observation 1: in RAN4 design inter-frequency L1 measurement with gap is a per UE feature. Observation 2: in RAN4 design inter-frequency L1 measurement without gap can only happen on serving bands. Observation 3: the case, where SSBs of LTM candidate cells on which the UE can perform inter-frequency L1 measurements are limited to the BCs in which the UE indicates support of inter-frequency L1 measurements, does NOT exist from RAN4 measurement design point of view. Proposal 4: the new UE capability interFreqL1-OnlyBC is unnecessary from RAN4 measurement design point of view. |
| R4-2418661 | Huawei, HiSilicon | Proposal 1: No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time. Proposal 2: In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. This is applicable to UE supporting inter-frequency L1 measurement with MG. Proposal 3: SSB based T/F tracking is sufficient for LTM cell switch delay. Suggest not to define requirements of TRS based enhancement. Proposal 4: In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if the time gap between early RACH transmission and cell switch command is larger than 160ms and the L3 measurement interval is larger than 160 ms. Proposal 5: In case the configured candidate cells are more than UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion. Proposal 6: Not to extend cell switch delay requirements to FR2 without L1 measurement report. |
| R4-2418858 | China Telecom | Proposal 1: Not to extend cell switch delay requirements to FR2 without L1 measurement report. Proposal 2: - UE does not need to know the number of SpCells and SCells. - The number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations could not exceed the UE reported maxNumberStoredConfigCells |
| R4-2419096 | vivo | Observation 1 From the perspective of RAN1/2 spec, LTM L1 measurements on current serving cell SSBs are configured by LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, while legacy L1 measurements on any current serving cell are configured by CSI-ResourceConfig under csi-MeasConfig. Observation 2 For R18 MIMO, rxTimingDiff-r18 is introduced as a UE capability for indicating support of RTD>CP between any two downlink timing reference signals within a CC, based on which UE will be able to perform L1-RSRP measurement so as to enable 2-TA m-TRP operation. The capability is per-FSPC, and needs pre-requisite of either multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 from RRM requirement perspective. Observation 3 For R18 Mobility, multiCellL1-measRTD-greaterThan-CP-r18 is introduced as a UE capability for indicating support of RTD>CP between any two cells on the same frequency layer or within the same active BWP, on which UE will be able to perform L1-RSRP measurement so as to enable LTE operations including early DL/UL sync. The capability is per-BC, and needs pre-requisite of one of intraFreqL1-MeasConfig-r18, ltm-InterFreqMeasGap-r18 or interFreqSSB-L1-MeasWithoutGaps-r18. Observation 4 Both rxTimingDiff-r18 and multiCellL1-measRTD-greaterThan-CP-r18 are for the scenario of RTD>CP, and are for L1-RSRP measurement. However, pre-requisites and scenario details for these two features are different. Hence, UE may not always support both of them at the same time. Proposal 1 Although same UE behaviour on L1-RSRP measurement for current serving cell is assumed from RRM requirements perspective, no matter it is configured by either CSI-ResourceConfig under csi-MeasConfig or LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, it is still proposed to capture LTM L1 measurement on serving cell in clause 9.14 of TS 38.133, so as to better differentiate UE support of either rxTimingDiff-r18 or multiCellL1-measRTD-greaterThan-CP-r18. Observation 5 UE needs to meet the legacy serving L1 measurement requirements, including L1 measurements on cell with different PCI, as long as the total number of resources to measure does not exceed the UE capability beamManagementSSB-CSI-RS. There is no restriction on the number of layers and/or cells for legacy L1 measurements for serving cell(s). Proposal 2 RAN4 to clarify that in RAN4 FG 39-3-1 maxFreqLayersL1-Meas-r18, only serving SSB frequency layers configured with LTM L1 measurements are counted. It does not include serving SSB frequency layers on which only legacy L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig, are to be performed. Observation 6 If Proposal 2 is agreeable, UE performs legacy serving cell L1 measurements on any SSB frequency layer that is configured by CSI-ResourceConfig under csi-MeasConfig, but only performs LTM L1 measurement on frequency layers that are configured with LTM L1 measurements on candidate cells. Proposal 3 RAN4 to clarify that in RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, which option below is the common understanding: - Option 1: ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not. - Option 2: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps. - Option 3: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some serving cells, while only serving cells with either intra-f LTM L1 measurements and/or inter-f LTM L1 measurements without gaps are counted. Proposal 4 RAN4 to clarify that in RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18, which option below is the common understanding: - Option 1: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not. - Option 2: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP. Proposal 5 RAN4 to clarify that in RAN4 FG 39-3-6 supportedMaxSSB-L1-Meas-r18, which option below is the common understanding: - Option 1: ‘SSB resources of serving cells and neighbouring cells’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not. - Option 2: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps. - Option 3: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP. Proposal 6 To better capture UE capabilities related to the max number of layers/cells/resource UE can measure under LTM configurations, LTM related serving cell L1 measurement shall be captured under 9.14. Observation 7 In existing TS 38.133, in clause 9.5.4.1, the description of ‘when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure’ has not restricted whether it only applies the to the corresponding frequency layer where only intra-f LTM L1 measurement happens, or to all frequency layer configured for serving cell L1 measurements across all CCs. Observation 8 No R18 RRM requirements are specified for the case R18 LTM and R17 inter-cell L1 measurement for ICBM is configured simultaneously on the same SSB frequency in FR2-1. Observation 9 For FR2-1, RAN4 has agreed to define sharing factor between all serving cell L1 measurements (including LTM L1 and legacy L1) and neighbour cell(s) LTM L1 measurement, considering different UE Rx beams could be used. It shall be understood as measurement relaxation to legacy serving cell L1 measurements if LTM L1 measurements on neighbour cell(s) are configured, and PL1_sharing would be enough to capture this impact. Proposal 7 Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements if counted as LTM L1 measurement, in 9.14. The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1. Proposal 8 Update the known condition for candidate cell SSB, candidate cell TCI and LTM target cell with the following: - A L1-RSRP report is valid if the reported L1-RSRP value corresponding to one of the valid codepoints for L1-RSRP in Table 10.1.6.1-1, or any codepoints other than DIFFRSRP_15 in Table 10.1.6.1-2. Proposal 9 Remove the FFS and confirm the following - In early candidate cell’s TCI state activation delay for known TCI state case: For UE supporting inter-frequency L1 measurement with gap, Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if UE supports inter-frequency L1 measurement with MG. - In PDCCH ordered RACH delay, when additional time for T/F tracking is needed: For UE supporting inter-frequency L1 measurement with gap, TSSB is the time to the first SSB occasion overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH, plus 2ms (SSB processing time), if UE supports inter-frequency L1 measurement with MG. |
| R4-2419432 | Ericsson, Qualcomm Incorporated | - When one or more of TCI state activation commands are received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. where TTF is mentioned in the below table. TTF; Comments. FR1 intra-frequency cell, known and unknown TCI state; max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn); Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n.. FR1 inter-frequency cell without MG, known and unknown TCI state; max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB; Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity.. FR1 inter-frequency cell with MG, known and unknown TCI state; TFirstMG + (M-1) *MGRP+MGL; TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity. FR2 intra-frequency cell, known TCI state; max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB; Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. FR2 inter-frequency cell without MG, known TCI state; max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB; Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. FR2 inter-frequency cell with MG, known TCI state; TFirstMG + (M-1) *MGRP+MGL; TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity. - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between completion of TCI activation and cell switch command is larger than 480ms. - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected. - We suggest modifying the TCI known condition to following The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met: - The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or - The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or - RAN4 to confirm to RAN2 that NW do not need to provide the total cells information in the LTM candidate configuration and inform RAN2 about the RAN4 agreements on how the fast processing is expected to work. - Till the total number of cells to be fast RRC decoded does not reach UE capability maxNumberStoredConfigCells-r18 and the number of LTMCandidateConfigs fast decoded does not reach maxNumberConfigs-r18, TLTM_RRC-processing =0 applies to the LTM candidates with early TCI activation or early PDCCH order RACH. Otherwise, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) till the UE capability reach maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - RAN4 to define the following UE capability: - RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell. - N = {[1], 2, …, 7}, if not reported, N=8. - Granularity: Per UE |
| R4-2417681 | Nokia | Proposal 1: Reflect the interFreqL1-OnlyInBC-r18 capability in the 38.133 requirements for inter-frequency with and without gap Proposal 2: In PDCCH ordered RACH delay, TSSB = 0 for FR1 if the time from reception of TCI state activation MAC-CE is not longer than TCI state activation delay + 160 ms. Proposal 3: RAN4 not to define requirements for activation of TCI states for more than one candidate cell in the same slot in Rel-18, unless a reasonable scenario can be agreed by the group. Proposal 4: A target TCI state in TCI state activation command is known if the UE has sent an L1-RSRP report for the RS of the TCI state within 1280 ms (i.e. under current known TCI state conditions), even if the UE is not required to measure the target cell at the time of TCI state activation due to an earlier TCI state activation for another candidate cell. Proposal 5: Proposal: RAN4 to further clarify the intent of the agreement regarding PL-RS measurement maintenance. Proposal 6: The condition for Tfirst-RS = 0 “The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].” applies also for the case when the UE is configured with L1 measurements. Proposal 7: When Tfirst-RS != 0, Tfirst-RS is the time to the first SSB or TRS transmission associated to the target DL TCI state. Observation 1: No need to consider TCI state activation or PDCCH order for more than one cell in one slot in the TLTM_RRC-processing = 0 conditions. Observation 2: If the number of configured candidate cells is larger than UE capability to pre-process, the UE can pre-process the first maxNumberConfigs-r18 candidate cell configurations in the LTM RRC configuration. Proposal 8: TLTM-RRC-processing = 0, if the UE supports ltm-FastProcessingConfig-r18 capability, and: - The number of candidate cells in the LTM candidate cell configuration does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and candidate cells does not exceed maxNumberStoredConfigCells-r18, or - The following conditions are fulfilled for the target cell: - The target cell is listed as one of the first Min(maxNumberConfigs-r18, maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells in the LTM candidate cell configuration and the UE has not received TCI state activation command or PDCCH order for any of the candidate cells, or - The target cell is one of the most recent Min(maxNumberConfigs-r18, maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells for which TCI state is activated or PDCCH order is received, and - UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and/or - UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3]. Proposal 9: Reply to RAN2 questions in R2-2409377 as follows:  Question 1 reply: Yes, this is the correct understanding.  Question 2 reply: No, the UE does need to know. RAN4 capability is about the number of candidate configurations, not about the cells inside the configuration.  Question 3 reply: Yes, it is up to the network to decide how many serving and candidate cells to configure. As long as the number of LTM candidate configurations and serving cells does not exceed maxNumberStoredConfigCells and the number of LTM candidate configurations does not exceed maxNumberConfigs, the UE shall be able to decode all LTM candidate configurations after receiving the RRC configuration |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

- 1.2.1 Sub-topic 1-1 Early Sync

Issue 1-1-1: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay and PDCCH-order RACH delay

Current spec: TCI state activation delay: - Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when the target SSB is within active BWP, upon receiving PDSCH carrying MAC-CE activation command in slot n; - Tfirst-SSB is the time to first SSB occasion overlapping with MGL after slot n + THARQ +3Nslotsubframe,µ + 2ms if the SSB needs to be measured with MG, upon receiving PDSCH carrying MAC-CE activation command in slot n; - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. PDCCH-order RACH: - TSSB = Tfirst-SSB_RACH + TSSB-proc, otherwise, where Tfirst-SSB_RACH is the time to the first SSB occasion, after 1slot from the end of the slot of the PDCCH, when SSB is within active BWP - Tfirst-SSB_RACH is the time to the first SSB occasion, overlapped with MGL after 2ms and 1slot from the end of the slot of the PDCCH when SSB is outside active BWP - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.

- Proposals

  - Proposal 1 (CATT, MTK):

    - RAN4 to remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

      - Proposal 1a (MTK):

        - Differentiate the cases based on whether the SSB is to be measured within or outside gap.

  - Proposal 2 (ZTE):

    - In early candidate cell’s TCI state activation delay for known TCI state case, the definition of Tfirst-SSB should be applied to both inter-frequency L1 measurement with MG and inter-frequency L3 measurement with MG as LTM can be triggered by L1 or L3 measurement.

  - Proposal 3 (Huawei, vivo):

    - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG. This is applicable to UE supporting inter-frequency L1 measurement with MG.

- Recommended WF

  - Further discuss how to determine T/F tracking performed within MG or not in early candidate cell’s TCI state activation delay and PDCCH-order RACH delay:

    - Option 1 (MTK): Pending on whether the SSB is needed to be measured within MG.

    - Option 2 (ZTE): T/F tracking performed within MG for both inter-frequency L1 measurement with MG and inter-frequency L3 measurement with MG as LTM can be triggered by L1 or L3 measurement.

    - Option 3 (Huawei, vivo):  T/F tracking performed within MG for UE supporting inter-frequency L1 measurement with MG.

Issue 1-1-2: Whether to consider early TCI state activation for multiple cells at the same time

- Proposals

  - Proposal 1 (ZTE, Huawei, Nokia):

    - No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

  - Proposal 2 (Ericsson, QC)

    - When one or more of TCI state activation commands are received at slot n, UE shall be able to finish the TCI state activation within slot n+ THARQ +3Nslotsubframe,µ+TTF + TSSB-proc. where TTF  is mentioned in the below table.

|  | TTF | Comments |
| --- | --- | --- |
| FR1 intra-frequency cell, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) | Tfirst-SSB_TCIn is the time for first SSB associated to TCI state n. |
| FR1 inter-frequency cell without MG, known and unknown TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity. |
| FR1 inter-frequency cell with MG, known and unknown TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |
| FR2 intra-frequency cell, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell without MG, known TCI state | max (Tfirst-SSB_TCI1, Tfirst-SSB_TCI2 .., Tfirst-SSB_TCIn) + (M-1)*TSSB | Tfirst-SSB_TCIn is the time for first SSB reception associated to TCI state n. M is the number of cells to activate the TCI states TSSB is the SSB burst periodicity |
| FR2 inter-frequency cell with MG, known TCI state | TFirstMG + (M-1) *MGRP+MGL | TFirstMG is the time to start of first MG after slot n+THARQ+3ms, MGL is measurement gap length M is the number of cells to activate the TCI states MGRP is the MG repetition periodicity |

- Recommended WF

  - If there is no consensus during the meeting, based on the  RAN4#112’s guidance, following will be agreed.:

RAN4#112 Issue 1-2-1: Whether to consider early TCI state activation for multiple cells at the same time < Agreement>: - Interested companies can have further offline discussion. If no further progress, the following will be agreed in the Nov meeting. - No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

Issue 1-1-3: The value of TSSB in PDCCH ordered RACH delay

RAN4#111 < Agreement>: - In cell switch delay requirement: - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between completion of TCI activation and cell switch command is larger than [160ms], or - the time gap between early RACH transmission and cell switch command is larger than [160ms]. - In PDCCH order RACH delay requirement: - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view.

- Proposals

  - Proposal 1 (Nokia):

    - In PDCCH ordered RACH delay, TSSB = 0 for FR1 if the time from reception of TCI state activation MAC-CE is not longer than TCI state activation delay + 160 ms.

- Recommended WF

In moderator’s understanding, the proposal is to revert the highlighted agreement above. Moderator suggests the proponent to discuss with other companies at first. If the proposal is not the majority view, suggest not to discuss this issue online or at ad hoc.

  - Collect more views from other companies at first.

    - Sub-topic 1-2 L1-RSRP Measurement

Issue 1-2-1: Spec organization

- Proposals

  - Option 1 (vivo):

    - Although same UE behaviour on L1-RSRP measurement for current serving cell is assumed from RRM requirements perspective, no matter it is configured by either CSI-ResourceConfig under csi-MeasConfig or LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, it is still proposed to capture LTM L1 measurement on serving cell in clause 9.14 of TS 38.133, so as to better differentiate UE support of either rxTimingDiff-r18 or multiCellL1-measRTD-greaterThan-CP-r18.

    - To better capture UE capabilities related to the max number of layers/cells/resource UE can measure under LTM configurations, LTM related serving cell L1 measurement shall be captured under 9.14.

    - Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements if counted as LTM L1 measurement, in 9.14. The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1.

- Recommended WF

  - Collect more views from other companies.

Sub-topic 1-3 Cell switch delay requirements for Pcell/PSCell

T/F tracking: Tfirst-RS and Tmargin

Issue 1-3-1-1: T/F tracking when TRS as QCL source in cell switch delay

- Proposals

  - Option 1 (CMCC, Nokia):

    - it is proposed to update Tfirst-ssb to cover TRS in cell switch delay requirements.

    - Option 1a (Nokia): When Tfirst-RS != 0,  Tfirst-RS is the time to the first SSB or TRS transmission associated to the target DL TCI state.

  - Option 2 (Huawei):

    - SSB based T/F tracking is sufficient for LTM cell switch delay. Suggest not to define requirements of TRS based enhancement.

- Recommended WF

  - Need more discussion.

Issue 1-3-1-2: Conditions of Tfirst-RS =0 in cell switch delay

RAN4#112 <Agreement>: - For cell switch with L1 measurement: Extend the condition for Tfirst-RS = 0 “the time gap between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than [160 ms]” from the agreed 160 ms, to TCI state activation delay stated in section 8.25.3 + 160 ms. - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between completion of TCI activation and cell switch command is larger than 480ms - Note: This is not to extent 160ms to 480ms directly. Longer L3 measurement delay may be expected in the 480ms duration. - Discuss the wording in CR. - FFS the following assumption - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - the time gap between early RACH transmission and cell switch command is larger than [160ms] and the L3 measurement interval is larger than 160 ms.

- Proposals

  - Proposal 1 (MTK): The following agreement is only applicable to intra-frequency:

    - Extend the condition for Tfirst-RS = 0 “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480ms” in FR1 for UE not supporting/configured with L1 measurement.

  - Proposal 2 (Ericsson, QC)

    - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

      - the time gap between completion of TCI activation and cell switch command is larger than 480ms.

      - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

  - Proposal 3 (Huawei):

    - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if the time gap between early RACH transmission and cell switch command is larger than 160ms and the L3 measurement interval is larger than 160 ms.

  - Proposal 4 (Nokia):

    - The condition for Tfirst-RS = 0 “The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].” applies also for the case when the UE is configured with L1 measurements.

- Recommended WF

  - Need more discussion.

Conditions of Early ASN.1 decoding and validity/compliance check

Issue 1-3-2-1: Which cell(s) TLTM-RRC-processing = 0 apply to when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability?

- Proposals

  - Proposal 1 (ZTE, Huawei, Ericsson, QC, Nokia):

    - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

      - Huawei: The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

  - Proposal 2 (MTK):

    - Open to further optimize this scenario only if a simple and clear rule can be agreed. Otherwise, we prefer not to optimize further.

- Proposal 3 (Nokia):

  - If the number of candidate cell configurations exceeds UE pre-processing capability and the UE has not received TCI state activation command or PDCCH order for any of the candidate cells before receiving the cell switch command, T_LTM_RRC-processing = 0, if the target cell is listed as one of the first Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells in the LTM candidate cell configuration.

- Recommended WF

  - Recommend follow the majority view and agree on:

    - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

      - Further discuss the applicable condition:

        - The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion.

Cell Switch without L1 measurement report in FR2?

Issue 1-3-3-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report?

- Proposals

  - Option 1 (MTK, Huawei, CTC):

    - Not to extend cell switch delay requirements to FR2 without L1 report and remove the following in known TCI state definition for cell switch:

      - [-	The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or]

      - [-	The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

    - MTK: Open to discuss how to optimize the known TCI state definition in R19 considering L1 report can be triggered when leaving condition is met. Not to further optimize known TCI state definition in R18.

  - Option 2 (ZTE):

    - cell switch delay requirements applicable to FR2 without L1 measurement report with the following conditions

      - The target TCI state in the cell switch command is activated not more than [1280] ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or

      - The target TCI state in the cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every [1280] ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB.

  - Option 3 (Ericsson, QC): Modifying the TCI known condition to following (For both FR1 and FR2)

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	The TCI state is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state is ≥ -3dB; where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same; or

-	The TCI state is activated before the reception of the cell switch command (where the TCI state is considered activated if the activated TCI state and target TCI state in the cell switch command are same or the SSB associated to target TCI state in cell switch command and the SSB associated to activated TCI state are same) and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or

  - Option 4 (Nokia):

    - A target TCI state in TCI state activation command is known if the UE has sent an L1-RSRP report for the RS of the TCI state within 1280 ms (i.e. under current known TCI state conditions), even if the UE is not required to measure the target cell at the time of TCI state activation due to an earlier TCI state activation for another candidate cell.

- Recommended WF

  - Need more discussion.

Known cell definition

Issue 1-3-4-1: Whether and how to update the known condition for candidate cell SSB, candidate cell TCI and LTM target cell?

- Proposals

  - Proposal 1 (vivo):

    - Update the known condition for candidate cell SSB, candidate cell TCI and LTM target cell with the following:

      - A L1-RSRP report is valid if the reported L1-RSRP value corresponding to one of the valid codepoints for L1-RSRP in Table 10.1.6.1-1, or any codepoints other than DIFFRSRP_15 in Table 10.1.6.1-2.

- Recommended WF

  - Collect more views from other companies.

PL-RS

Issue 1-3-5-1: Clarification on agreed “Conditions of no extra time for PL-RS measurement in cell switch delay” in RAN4#112

RAN4#112 Issue 1-4-2-1: Conditions of no extra time for PL-RS measurement in cell switch delay <Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. - Further discuss the wording in the CR next meeting.

Moderator’s understanding on the condition:

| RS of the DL/joint TCI state indicated in cell switch command | PL-RS |
| --- | --- |
| SSB#X | Should be Type D QCL-d with SSB#X |
| TRS#Y which is Type D QCL-d with SSB#X |  |

- Proposals

  - Proposal 1 (Nokia):

    - RAN4 to further clarify the intent of the agreement regarding PL-RS measurement maintenance.

- Recommended WF

Moderator suggests offline discussion at first. If the proponent thinks it is necessary to clarify online after offline discussion, please let me know.

  - Recommended offline discussion at first.

- 1.2.4 Sub-topic 1-4 UE feature

Issue 1-4-1: Capability for fast RRC decoding

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-6 | Fast processing of LTM candidate cell RRC configuration | 1. Indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and Scell(s) in LTMCandidateConfig(s) for MCG and SCG, that UE can store the configurations. 2. Indicates the maximum number of LTMCandidateConfigs that UE can support fast processing . | 45-3a or 45-4a in RAN1 feature list |

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 45-3a | MAC-CE activated joint LTM TCI states | 1. Supported QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of MAC-CE activated joint LTM TCI states per candidate cell 3. Maximum number of MAC-CE activated joint LTM TCI states across candidate cells and serving cells | 45-3 |
| 45-4a | MAC-CE activated DL/UL LTM TCI states | 1. Supported QCL source RS for MAC-CE activated DL/UL LTM TCI states 2. Maximum number K1 of MAC-CE activated DL TCI states per candidate cell 3. Maximum number K2 of MAC-CE activated UL TCI states per candidate cell 4. Maximum number of MAC-CE activated DL TCI states across all candidate cells and serving cells 5. Maximum number of MAC-CE activated UL TCI states across all candidate cells and serving cells | 45-4 |

- Proposals

  - Proposal 1 (MTK):

    - Send LS to RAN2 to inform them that 45-3a/45-4a (MAC-CE activated joint LTM TCI states) are not the prerequisite FG of 39-6 “Fast processing of LTM candidate cell RRC configuration”.

- Recommended WF

  - Need more discussion.

Issue 1-4-2: Capability for RACH-less LTM cell switch

- Proposals

  - Proposal 1 (Ericsson, QC): RAN4 to define the following UE capability:

    - RACH-less LTM cell switch can be conducted to one of ‘N’ cells to which the UE most recently transmitted the ‘PDCCH-order PRACH’ except for the cell configured as SCell.

      - N = {[1], 2, …, 7}, if not reported, N=8.

      - Granularity: Per UE

- Recommended WF

  - Need more discussion.

Issue 1-4-3: Clarification on LTM L1-RSRP related capabilities

| Index | Feature group | Components | Moderator’s understanding |
| --- | --- | --- | --- |
| 39-3-1 | Number of frequency layers for L1-RSRP measurement | - The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement 2. The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps | Component 1: If NW only configures L1 measurement on serving cell but not on any neighbor cell on the same frequency layer, this frequency layer will not be counted in the measurement requirement, no matter L1 measurement on serving cell is configured in LTM L1 measurement configuration or serving cell’s L1 measurement configuration. |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | Including all the serving cells configured with SSB based L1-RSRP measurement |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps 2. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | Component 1: including SSB resources from serving cell and neighbor cell(s). |
| 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | Including the serving cell(s) with no intra-frequency neigbor cell configured with SSB based L1-RSRP measurement |

- Proposals

  - Proposal 1 (vivo):

    - RAN4 to clarify that in RAN4 FG 39-3-1 maxFreqLayersL1-Meas-r18, only serving SSB frequency layers configured with LTM L1 measurements are counted. It does not include serving SSB frequency layers on which only legacy L1 measurements configured by CSI-ResourceConfig under csi-MeasConfig, are to be performed.

    - RAN4 to clarify that in RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, which option below is the common understanding:

      - Option 1: ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not.

      - Option 2: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps.

      - Option 3: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some serving cells, while only serving cells with either intra-f LTM L1 measurements and/or inter-f LTM L1 measurements without gaps are counted.

    - RAN4 to clarify that in RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18, which option below is the common understanding:

      - Option 1: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

      - Option 2: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

    - RAN4 to clarify that in RAN4 FG 39-3-6 supportedMaxSSB-L1-Meas-r18, which option below is the common understanding:

      - Option 1: ‘SSB resources of serving cells and neighbouring cells’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

      - Option 2: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with intra-f LTM L1 measurements or inter-f LTM L1 measurements without gaps.

      - Option 3: ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and some SSBs from serving cells, which provides the timing reference for the neighbour cells if UE does not support RTD>CP.

- Recommended WF

  - Collect more views from other companies.

- 1.2.5 Sub-topic 1-5 RAN2 LS on the fast RRC processing for LTM

MaxNumberStoredConfigCells indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTM candidate configurations and Scell(s) in LTM candidate configurations for MCG and SCG, that UE can store the configurations. MaxNumberConfigs-r18 represents the maximum number of LTM candidate configuration for which the UE can perform early ASN.1 decoding and validity check

LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 OPTIONAL, -- Need N attemptLTM-Switch-r18 ENUMERATED {true} OPTIONAL, -- Cond LTM-MCG ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ... } LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ... } RAN2 is discussing whether to introduce an IE to indicate “the number of SpCells and SCells” in LTM-Candidate-r18

Issue 1-5-1: Answer to Q1

Q1: As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations?

- Proposals

  - Option 1 (CATT, MTK, Apple, Nokia):

    - Yes

- Recommended WF

  - Reply “Yes” to Q1 in RAN2 LS on the fast RRC processing for LTM.

Issue 1-5-2: Answer to Q2

Q2: Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration?

- Proposals

  - Option 1 (MTK, Apple, CTC, Ericsson, QC, Nokia): No

Additional information to include in the reply LS

    - Option 1a (MTK):

      - No. But if the number of serving cell(s) + SpCell/SCell(s) in decoded LTM candidate configuration(s) exceeds maxNumberStoredConfigCells, UE will not store the decoded configurations and still needs extra time to perform RRC decoding and validity check again during cell switch. This leads to a waste of UE efforts and power, and not preferred in RAN4.

      - RAN4 suggests RAN2 adding a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration to avoid potential waste of UE efforts and power.

    - Option 1b (Apple):

      - it is feasible for UE to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, NW has to assume UE may not have done early decoding, i.e. TLTM-RRC-processing during cell switch cannot be 0 as minimum requirements

    - Option 1c (Ericsson, QC):

      - RAN4 to confirm to RAN2 that NW do not need to provide the total cells information in the LTM candidate configuration and inform RAN2 about the RAN4 agreements on how the fast processing is expected to work.

    - Option 1d (Nokia):

      - RAN4 capability is about the number of candidate configurations, not about the cells inside the configuration.

- Recommended WF

  - Reply “No” to Q2 in RAN2 LS on the fast RRC processing for LTM.

  - Further discuss what additional information to include in the reply LS:

    - Option 1a (MTK):

      - No. But if the number of serving cell(s) + SpCell/SCell(s) in decoded LTM candidate configuration(s) exceeds maxNumberStoredConfigCells, UE will not store the decoded configurations and still needs extra time to perform RRC decoding and validity check again during cell switch. This leads to a waste of UE efforts and power, and not preferred in RAN4.

      - RAN4 suggests RAN2 adding a new field in LTM-Candidate IE to indicate the number of serving cells configured by this LTM candidate configuration to avoid potential waste of UE efforts and power.

    - Option 1b (Apple):

      - it is feasible for UE to perform early ASN.1 decoding of an LTM candidate configuration without knowing the number of SpCells and SCells within the configured LTM candidate configurations. However, if the actual number of cells within the configured LTM candidate configuration exceeds the corresponding UE capability, NW has to assume UE may not have done early decoding, i.e. TLTM-RRC-processing during cell switch cannot be 0 as minimum requirements

    - Option 1c (Ericsson, QC):

      - RAN4 to confirm to RAN2 that NW do not need to provide the total cells information in the LTM candidate configuration and inform RAN2 about the RAN4 agreements on how the fast processing is expected to work.

    - Option 1d (Nokia):

      - RAN4 capability is about the number of candidate configurations, not about the cells inside the configuration.

Issue 1-5-3: Answer to Q3

Q3: Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

- Proposals

  - Option 1 (CATT, MTK, Nokia):

    - Yes

      - Option 1a (MTK): Yes. RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding.

      - Option 1b (Nokia): it is up to the network to decide how many serving and candidate cells to configure. As long as the number of LTM candidate configurations and serving cells does not exceed maxNumberStoredConfigCells and the number of LTM candidate configurations does not exceed maxNumberConfigs, the UE shall be able to decode all LTM candidate configurations after receiving the RRC configuration

  - Option 2 (Apple):

    - Conditionally No

      - if NW expects UE can successfully perform early ASN.1 decoding, number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations is expected NOT to exceed the UE reported maxNumberStoredConfigCells.

  - Option 3 (CTC):

    - No

- Recommended WF

  - Need more discussion.

- 1.2.6 Sub-topic 1-6 RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM

Background Question 2 in RAN2 LS R2-2404014 Question 2: The above features, 45-1 and 45-1a, from RAN1 and related RAN4 features (39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, 39-3-6) are defined per BC for both intra-frequency and inter-frequency measurements. RAN2 would like check with RAN1/4 for which BC (e.g. BC of current serving cells, BC including current serving cells and cell to be measured or something else) these capabilities are to be considered for L1 intra-frequency and inter-frequency LTM measurements?. RAN4 Reply in R4-2410303 Reply to the Question 2: RAN4 discussed the following options. Option 1 is supported by majority companies in RAN4, but RAN4 cannot reach consensus on option 1. Meanwhile, RAN4 discussed the compromised option in Option 3, and the feasibility and details are up to RAN2. - Option 1: The BC granularity is BC of serving cells. - Option 2: The BC granularity is BC including serving cells and candidate cells to be measured. - Option 3: The UE capability is based on option 1 and introduce additional UE capability to accommodate option 2. From RAN4 perspective, the UE capabilities can be implemented into the specification based on RAN2 conclusion..

Issue 1-6-1: RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM (R2-2409360/ R4-2417508)

- Proposals

  - Proposal 1 (Apple):

    - the new UE capability interFreqL1-OnlyBC is unnecessary from RAN4 measurement design point of view.

  - Proposal 2 (Nokia):

    - Reflect the interFreqL1-OnlyInBC-r18 capability in the 38.133 requirements for inter-frequency with and without gap.

- Recommended WF

  - Need more discussion.

- 2. Topic #2: Improvement on SCell/SCG setup delay - Core part (AI 5.23.1)

- 2.1 Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418503 | ZTE Corporation, Sanechips | Observation 2: when measReselectionCarrierListNR is not present, there maybe disalignment between RAN2 and RAN4. Proposal 5: RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not present. UE will repoet any measurement results if available or UE will not repoet any measurement results. |
| R4-2419735 | Nokia | If needed, send an LS to RAN2 to fix the specification with the following RAN4 interpretation of RAN2#125bis (R2-2404006): If not configured, the UE reports what is has - “If not configured” refers to the ValidityDuration being configured / not configured - “What it has” refers to UE having performed measurements, which have previously been configured. The current RAN4 requirements are well reviewed by all the companies are they shall remain unchanged. Any ambiguity should be resolved in RAN2. |

- 2.2 Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

- 2.2.1 Sub-topic 2-1

Issue 2-1-1: Mis-alignment between RAN2 and RAN4 spec when measReselectionCarrierListNR is not configured

TS 38.331 clause 5.7.10.3 1> if the reselectionMeasurementReq is included in the UEInformationRequest: 2> if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig; 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig; 2> else: 3> if measReselectionCarrierListNR is present in VarMeasReselectionConfig: 4> set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig; 3> else: 4> set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available; 38.133 clause 4.3 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

Moderator’s understanding on RAN4 and RAN2 spec:

|  | measReselectionValidityDuration configured |  | measReselectionValidityDuration not configured |  |
| --- | --- | --- | --- | --- |
|  | measReselectionCarrierListNR configured | measReselectionCarrierListNR not configured | measReselectionCarrierListNR configured | measReselectionCarrierListNR not configured |
| R4 | Check validity of measurement results on the carriers within measReselectionCarrierListNR | ? Ambiguity? Or not specified in R4? | not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18. May report the measurement results (Moderator’s view: on the carriers configured in measReselectionCarrierListNR-r18? Spec is not crystal clear.) | ? Not specified in R4? |
| R2 | Check validity of measurement results on the carriers within measReselectionCarrierListNR | Validity check on all the measurement results | Report measurement results on the carriers within measReselectionCarrierListNR without validity check | Report all the measurement results without validity check |
|  | align | ? | Align? | ? |
|  | Case 1 | Case 2 | Case 3 | Case 4 |

Agreement in RAN2#125bis (R2-2404006) For reselection measurements: Confirm that the freq is optionally configured by the network, applicable to UE availability indication and UE reporting (doesn’t impact the actual measurements). If not configured, the UE reports what is has.

- Proposals

  - Option 1 (ZTE):

    - when measReselectionCarrierListNR is not present, there maybe disalignment between RAN2 and RAN4.

    - RAN4 should confirm what the UE behavior is if measReselectionCarrierListNR is not present. UE will repoet any measurement results if available or UE will not repoet any measurement results.

  - Option 2 (Nokia):

    - If needed, send an LS to RAN2 to fix the specification with the following RAN4 interpretation of RAN2#125bis (R2-2404006): If not configured, the UE reports what is has

      - “If not configured” refers to the ValidityDuration being configured / not configured

      - “What it has” refers to UE having performed measurements, which have previously been configured.

    - The current RAN4 requirements are well reviewed by all the companies are they shall remain unchanged. Any ambiguity should be resolved in RAN2.

- Recommended WF

It seems there is different understanding on the part which is dis-aligned between RAN2 and RAN4 spec in the proposals.

In moderator’s understanding, if measReselectionCarrierListNR is not configured, it is equivalent to all the carriers configured in RAN2. But RAN4 may assume measReselectionCarrierListNR always configured, and result case 2-4 not well addressed in spec.

Moderator suggests the proponents to align the understanding at first.

  - Identify the part that RAN2 and RAN4 have misalignment.

  - Further discuss how to resolve the dis-alignment part:

    - Option 1 (ZTE): align RAN4 spec with RAN2

    - Option 2 (Nokia): ask RAN2 to align with RAN4

- 3. Topic #3:  Performance Part (AI 5.23.1)

- 3.1 Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417712 | CATT | Proposal 4: RAN4 need to discuss if the understanding of ‘If a UE has already passed the test for R16 EMR and R18 EMR tests, no matter how the validity time of the measurement results is defined, the UE shall be able to complete valid reporting because its reporting function has been verified to be good’ is right. Proposal 5: If the above understanding is assumed, then the current R18 test cases design is enough. |
| R4-2417898 | MediaTek Inc. | Observation 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), UE may fail to transmit RACH to the target cell on the first RACH occasion after PDCCH order RACH delay as the RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG. Proposal 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3. Proposal 2: In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case, i.e., 80slots+80slots. |
| R4-2418503 | ZTE Corporation, Sanechips | Observation 3: different from non-EMR, EMR has already completed testing on reporting in Rel-16 and UE need to pass the test case defined in Rel-16 first. Proposal 6: the test cases with valid reporting for FR1 and FR2 maybe not needed for the case when the UE is configured with EMR measurements. |
| R4-2418663 | Huawei, HiSilicon | Observation: The new test for EMR based UE reporting valid measurement within X could not verify whether UE correctly executed validity check correctly. Proposal 1: Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement. |
| R4-2419096 | vivo | Observation 10 Since R17, supporting joint TCI configuration is the pre-requisite for supporting separate TCI. Observation 11 According to RAN1 spec, the meaning of configuring ‘QCL-D’ and UL TCI to UE would be ambiguous in FR1. Proposal 10 In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18. |
| R4-2419735 | Nokia | Add Test case for Idle mode fast CA/DC eEMR measurement for FR1 with valid reporting |

- 3.2 Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

- 3.2.1 Sub-topic 3-1 LTM

Issue 3-1-1: Test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2)

- Proposals

  - Proposal 1 (MTK):

    - In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

- Recommended WF

  - Need more discussion.

Issue 3-1-2: Test cases for SSB based L1-RSRP measurement in FR1 – intra-f & inter-f without gap (A.6.6.26.1 & A.6.6.28.1.3)

- Proposals

  - Proposal 1 (MTK): In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case, i.e., 80slots+80slots.

- Recommended WF

  - Need more discussion.

Issue 3-1-3: TCI state configurations

- Proposals

  - Proposal 1 (vivo):

    - In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

- Recommended WF

  - Need more discussion.

- 3.2.2 Sub-topic 3-2 Improvement on SCell/SCG setup delay

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay

- Proposals

  - Option 1 (ZTE, Huawei):

    - Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement.

  - Option 2 (CATT):

    - RAN4 need to discuss if the understanding of ‘If a UE has already passed the test for R16 EMR and R18 EMR tests, no matter how the validity time of the measurement results is defined, the UE shall be able to complete valid reporting because its reporting function has been verified to be good’ is right.

    - If the above understanding is assumed, then the current R18 test cases design is enough.

  - Option 3 (Nokia):

    - Add Test case for Idle mode fast CA/DC eEMR measurement for FR1 with valid reporting.

- Recommended WF

  - Need more discussion.