3GPP TSG-RAN WG4 Meeting #113											                R4-2418290

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.27.4

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [113][232] IoT_NTN_Ph3

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for R19 IoT (Internet of Things) NTN (non-terrestrial network) phase 3, including the following topics covered

- Topic#1: RRM core requirements (AI 7.27.3)

Recommended issues for online discussion:

- Issue 1-1-1

# Topic #1: RRM core requirements (AI 7.27.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2419633 | Nokia | Proposal 1: If the UE performs CB-MSG3 transmission, the value of NTA_Ref is to be considered equal to zero, unless otherwise agreed by RAN1. |
| R4-2418436 | CMCC | Proposal 1: Except for the case of NB-IoT with 15kHz SCS, the current timing requirement can be applied when it is the first transmission for CB-msg3, and the NTA_Ref for NPRACH can be assume as 0 if there is no other procedure for NTA determination introduced by other WG. |
| R4-2418903 | Ericsson | Observation 1: RRM requirements may be impacted by S&F mode, in a same manner as introducing requirements during the discontinuous coverage in Rel-18 IoT NTN. Observation 2: The Impacted RRM requirements may include the following, depending on the specifics of S&F mode scenarios: - Keep/suspend/relax/resume measuring SSBs and monitoring paging or not. - Cell-reselection for cells (the serving cell and/or neighbour cells)in S&F mode. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 1-1: CB-msg3 EDT enhancement

#### Issue 1-1-1: RRM impact from CB-msg3 EDT

Background:

- RAN2 agreement in RAN2#127bis

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

RAN4’s feedback:

For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view.

Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any.

Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

LS R4-2414114 agreed in RAN4#112

RAN4’s feedback:

For Note 1, RAN4 assumed that N_TA is equal to 0, if there is no other procedure for N_TA determination introduced by other WGs. RAN4 will follow the conclusion of the other working groups in the future, if any.

For Note 2, RAN4 has already agreed that UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN respectively in [R4-2414114].

LS R4-2416918 agreed in RAN4#112bis

### 7.20A.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Current timing requirement in 36.133

Proposals:

- Proposal 1 (Nokia): If the UE performs CB-MSG3 transmission, the value of NTA_Ref is to be considered equal to zero, unless otherwise agreed by RAN1.

- Proposal 2 (CMCC): Except for the case of NB-IoT with 15kHz SCS, the current timing requirement can be applied when it is the first transmission for CB-msg3, and the NTA_Ref for NPRACH can be assume as 0 if there is no other procedure for NTA determination introduced by other WG.

Moderator’s Note:

- In  RAN4#112bis, RAN4 agreed to assume N_TA is equal to 0, if there is no other procedure for N_TA determination introduced by other WGs (as Note 1 in LS R4-2416918). Besides, RAN4 has agreed that the current timing requirement apply to the Msg3 transmission without Msg1/Msg2 (as Note 2 in LS R4-2416918).

- The moderator's interpretation is that the "N_TA" mentioned in the agreement pertains to N_TA_Ref as specified in clause 7.20A.2 and 7.24A.2 of TS 36.133. Consequently, there is no need for further discussion for N_TA_Ref = 0 unless other procedure for N_TA determination introduced by other WG.

Recommended WF:

- Clarify the “N_TA” mentioned in the agreement R4-2416867 refers to “N_TA_Ref” as specified in clause 7.20A.2 and 7.24A.2 of TS 36.133.

- FFS whether and how to add clarification for CB-MSG3 transmission in the timing requirement definition.

### Sub-Topic 1-2: RRM impact from other objectives

Background:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

….

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

The latest WID RP-242397

#### Issue 1-2-1: Store&Forward (S&F) satellite operation

Background:

- Observation provided for Store&Forward (S&F) satellite operation in R4-2418903.

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418903 | Ericsson | Observation 1: RRM requirements may be impacted by S&F mode, in a same manner as introducing requirements during the discontinuous coverage in Rel-18 IoT NTN. Observation 2: The Impacted RRM requirements may include the following, depending on the specifics of S&F mode scenarios: - Keep/suspend/relax/resume measuring SSBs and monitoring paging or not. - Cell-reselection for cells (the serving cell and/or neighbour cells)in S&F mode. |

Moderator’s Note: RAN4 is not included for this objective in WID.

Recommended WF: There is no need for further discussion in this meeting.