3GPP TSG-RAN WG4 Meeting #113                                                               R4-2419593

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.20.4

Source:	Moderator (Samsung)

Title:	Topic summary for [113][306] NR_duplex_evo_General

Document for:	Information

# Introduction

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved [RP-234035], with WID further revised in the follow-up RAN plenary [RP-240789]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”.

Over RAN4#113, there was extensive discussion on SBFD general aspects and potential new requirements for SBFD operation for FR1 and FR2-1, which results in the progress captured in the approved WF [R4-2416584].

This document is provided for the moderator summary of thread [306] on Rel-19 work item on evolution of NR duplex operation (SBFD) for general aspect and potentially new requirements for SBFD, in which the following highlighted agenda items are supposed to be covered specifically:

- Evolution of NR duplex operation: Sub-band full duplex (SBFD) [NR_duplex_evo] - General aspects (including RAN4 aspects for SBFD system parameters) [NR_duplex_evo-Core] - BS RF requirements [NR_duplex_evo-Core] - Potentially new requirements for SBFD operation for FR1 and FR2-1 [NR_duplex_evo-Core] - Modification of existing Tx requirements for FR1 and FR2-1 [NR_duplex_evo-Core] - Modification of existing Rx requirements for FR1 and FR2-1 [NR_duplex_evo-Core] - RRM core requirements [NR_duplex_evo-Core] - RRM requirements for UE-to-UE CLI handling [NR_duplex_evo-Core] - RRM impacts for SBFD operation [NR_duplex_evo-Core] - Moderator summary and conclusions

# Topic #1: General aspects (including RAN4 aspects for SBFD system parameters)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417786 | Tejas Network Limited | Proposal 1: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested, and no restrictions should apply to other FR1 BS classes or FR2 BS. Proposal 2: The UL subband size relative to the channel bandwidth can range from 20% to 40%. Observation 1: the 30 kHz SCS is used in FR1 to balance coverage and throughput, and SBFD is supported in TDD operation, allowing it to be employed based on UE requirements. Observation 2: The guardband size for SBFD will vary depending on the type of base station Proposal 3: Define the SBFD RBs requirement for all classes of base stations. |
| R4-2417845 | CATT | Proposal 1: For SBFD, at least one DL SB BW should be larger than (SSB BW + sync raster step size). Proposal 2: FR2 minimum CBW to support SBFD is 100MHz. Proposal 3: No restriction on the UL SB size ratio in the core requirements. Proposal 4: Flexibility of the UL SB transmission center frequency should be allowed that it may not be the center frequency of UL SB. Proposal 5: No restriction on maximum/minimum number of RBs for guard band in RAN4 specification. |
| R4-2418024 | Nokia | Observation 1: Alt-2 is basically just option 2b without any additional restrictions. Observation 2: Alt-1 will solve the issue of different guard bands for different manufacturer implementations. Observation 3: There are multiple reasons why RAN4 should not allow too much flexibility. Observation 4: Additional manufacturer declarations are not desired, and they should be avoided. Proposal 1: RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work. Proposal 2: NR-U bands should be excluded from SBFD supported bands. Proposal 3: If needed to distinguish, define lower and upper DL sub-bands the following way: • NRB,SBFD,DL1: Transmission bandwidth configuration for SBFD lower DL subband for DUD configuration, expressed in resource blocks. • NRB,SBFD,DL2: Transmission bandwidth configuration for SBFD upper DL subband for DUD configuration, expressed in resource blocks. Proposal 4: RAN4 to agree on Alt-1 as the configuration method with fixed guard bands. Proposal 5: One DL subband size to be declared and tested. Proposal 6: Apply Alt-1 also for FR1 MR BS subband configuration. Proposal 7: More flexibility for subband configuration can be considered for FR1 LA BS. Proposal 8: Subband configurations for FR2-1 is still FFS. Proposal 9: For UD/DU configuration, RAN4 to apply Alt-1 with modification of placing the UL subband to either bottom for UD or top for DU. Otherwise configuration follows Alt-1. |
| R4-2418097 | Samsung | Proposal 1: SBFD operation shall not be applied in NR-U bands. Proposal 2: RAN4 shall adopt the following text proposal in Clause 5.2: =================== Start of Text Proposal =================== ## 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation. NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31, n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1. Subband full duplex (SBFD) is designed to operate in all NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96 and n101, and all NR operating bands in FR2 which are defined in Table 5.2-2. =================== End of Text Proposal =================== Proposal 3: RAN4 shall introduce the following note in Table 5.3.5-1: - “Note 9: Subband full duplex (SBFD) can be applied to all supported channel bandwidth values except the ones smaller than 20MHz in band n39 and ones smaller than 40MHz in other TDD bands except bands n34, n39, n46, n51, n53, n54, n96 and n101.” Proposal 4: For SBFD DUD case, - the transmission bandwidth configurations for two SBFD DL subbands are denoted by NRB,SBFD,DL,1 and NRB,SBFD,DL,2 - the guardbands between SBFD UL and DL subbands are denoted by NRB,SBFD,GB,1 and NRB,SBFD,GB,2 Proposal 5: The existing symbol explanation for NRB,SBFD,DL and NRB,SBFD,GB can be updated and the new ones for DUD case can be added as: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks. - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU or UD case, expressed in resource blocks. - NRB,SBFD,GB,1: The size of guardband between SBFD UL subband and the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. - NRB,SBFD,GB,2: The size of guardband between SBFD UL subband and the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. Observation 1: The following issues are identified in the illustrative figures for SBFD DUD, DU and UD patterns: - Transmission bandwidth configuration for SBFD NRB,SBFD shall be used instead of NRB - In SBFD DUD case, the values of NRB,SBFD,DL,1 and NRB,SBFD,DL,2 are used instead, which can be different - In SBFD DUD case, the two values of NRB,SBFD,GB,1 and NRB,SBFD,GB,2 are used instead, which can be different - WGuard is not defined which should not be used - “Channel Edge” can be added to illustrate channel bandwidth edge, the same as Figure 5.3.1-1 in TS38.104. - “Resource Block” can be added to illustrate the resource block represented by a rectangular, the same as Figure 5.3.1-1 in TS38.104. Proposal 6: To solve the issues in the baseline figure for DUD, DU and UD case, the issues given in Observation 1 shall be solved, and the following revised figure for DUD case is given as example: Proposal 7: To specify DL/UL subband configuration in BS RF requirements, Option-2b is adopted as baseline while Option-1 should be precluded. Observation 3: For restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD), it is possible that UL subband can’t be located at the exact middle of channel bandwidth. Proposal 8: The proponent of restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD) shall solve the issue in the Observation 3. Proposal 9: For the number of DL subband sizes to be declared and tested, - For FR1 WA BS, all DL subband sizes declared to support SBFD shall be tested; - This restriction is not applicable to other FR1 BS classes and FR2 BS. Proposal 10: For UL subband size based on the specific channel bandwidth, the following restriction can be considered - For DUD and DU case, the ratio for UL subband size over channel bandwidth shall be no less than 20% - The restriction can be applied to both FR1 and FR2. Proposal 11: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs. Observation 4: Setting new sub-clauses with a suffix for a new feature is not only a common practice used in UE RF requirement, but also in the specification for network nodes like IAB. Observation 5: In WID of Rel-19 duplex evolution, there is no new specification expected, and TS 38.104 is listed as impacted existing TS by SBFD feature introduced. Proposal 12: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, i.e., - For the SBFD BS RF requirements specified by modifying existing requirements: - Set up new sub-clause with suffix-B - For the new BF RF requirements dedicated to SBFD BS - Set up new sub-clause with suffix-B Proposal 13: The issue of BS RF specification structure (or potentially new specification) is standardization problem for specification implementation, which shall be separated from other technical discussion. |
| R4-2418192 | vivo | Proposal 1：NR operating bands n38, n39, n40, n41, [n46,] n48, n50, n77, n78, n79, n90, [n96, ][n102,] n104 which are defined in Table 5.2-1, can be applied for subband full duplex operation. Observation 1: For NR-U supporting SBFD, these issues need further study: - Granularity of one subband: 20MHz - LBT threshold Proposal 2: Use the same SCS for DL and UL subbands in the illustrative figures for transmission bandwidth configuration for SBFD, while do not retrict different SCS for DL and UL subband. Proposal 3：GB doesn’t have to be in the number of RB when different SCS are configured for UL and DL subbands. Proposal 4: Option 2b (UL subband shall be selected from BS transmission channel bandwidth) can be used as a starting point for subband configuration. - For DUD case, the UL subband should be placed in the middle of channel bandwidth - the number of DL subband sizes to be declared and tested - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - No restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - [20%~] ratio for UL subband size over channel bandwidth - UL subband position can be different for DU and DUD, other than that there is no different restrcitions. - These above restrictions doesn’t apply to FR2. Proposal 5: Option-2 (Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD) is preferred. |
| R4-2418738 | CMCC | Proposal 1: NR-U band could not be supported. Proposal 2: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD. Proposal 3: To keep the flexibility of SBFD, Alt-2 could be used as baseline. Proposal 4: The maximum value of the guard band size should be specified in the spec. |
| R4-2418790 | Ericsson | Observation 1 SBFD configurations shall be captured in Rel-19 RAN4 specification and restricted to a limited set. Proposal 1 To include the following text in the specification (either new specification or TS 38.104, which is still under discussion): subband full duplex can be applied to the following TDD bands given in Table 5.2-1 in TS 38.104: n38, n39, n40, n41, n48, n50, n77, n78, n79, n90, n102, n104 for FR1. Proposal 2 For FR1 and FR2, do not use 60 kHz SCS for SBFD symbols. Proposal 3 The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested. Proposal 4 For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth. Proposal 5 Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband. Proposal 6 Select Option 1 to specify DL/UL subband configuration in BS RF requirements, due to interoperability issues between different vendors. Proposal 7 Reuse the existing channel bandwidths for SBFD UL subbands. Proposal 8 For WA BSs, wider guardbands than existing ones are needed for SBFD. Proposal 9 RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1. Proposal 10 Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below. Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel Proposal 11 RAN4 create new specifications for SBFD-capable BS. |
| R4-2419033 | Huawei, HiSilicon | Proposal 1: The number of the declared DL subband sizes should not be limited to one. Proposal 2: As a compromise, only introduce the following bandwidth from Table 5.3.2-1 for SBFD UL subband configuration: • 10MHz, 20MHz, 30MHz, 40MHz, 50MHz, 60MHz 70MHz, 80MHz and 90MHz. Proposal 3: Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary. |
| R4-2419219 | ZTE Corporation, Sanechips | Proposal 1: propose to clearly clarify whether NR-U bands in FR1 should be supported or not. Proposal 2: SCS for DL and UL sub-band should be the same. Proposal 3: to support the multiple DL sub-band sizes for SBFD BS to accommodate the operators’ deployment request. Proposal 4:all UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured. Proposal 5: propose to guard band between DL and UL sub-band is up to the declaration from the implementation perspective. Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols. |
| R4-2418159 (Moved from 7.20.2) | Charter Communications, Inc | Proposal: Add this note to Table 5.3.2-1: Note: The channel bandwidth for a Subband Full Duplex carrier shall be no less than X = 40 MHz. • For band n39, X = 20MHz is used instead. |
| R4-2418160 (Moved from 7.20.2) | Charter Communications, Inc | Draft CR |
| R4-2418601 (Moved from 6.20.1) | Qualcomm Germany | Proposal 1: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation. Proposal 2: RAN4 to consider fixed number of downlink subband configuration supported for FR1 WA BS. FFS what exact number to be agreed. Proposal 3: RAN4 to include Rel-19 SBFD BS requirements in the legacy BS specs, preferably via additional sub-clauses. |

## Open issues summary

### Sub-topic 1-1: SBFD applicability in NR band list

##### Issue 1-1-1: SBFD operation support in NR-U bands

- [Moderator] Based on the discussion in RAN4#112bis, it is agreed to “FFS the band for NR-U shall be supported”. Accordingly, companies have provided proposals on this issue.

- There is option to clearly indicate SBFD operation shall not be applied in NR-U bands:

  - Option-1 (CMCC, Samsung, Nokia, Ericsson): SBFD operation shall not be applied in NR-U bands.

- Other relevant observations/proposals:

  - Observation 1 (vivo): For NR-U supporting SBFD, these issues need further study:

    - Granularity of one subband: 20MHz

    - LBT threshold

  - Proposal 1 (ZTE): propose to clearly clarify whether NR-U bands in FR1 should be supported or not.

- Moderator Recommendation:

  - Agree on Option-1 to preclude SBFD operation in NR-U bands.

##### Issue 1-1-2: SCS restriction for SBFD operation

- In this meeting, there is proposal to restrict SBFD to be used with 60kHz SCS in both FR1 and FR2-1:

  - Proposal 1 (Ericsson): For FR1 and FR2, do not use 60 kHz SCS for SBFD symbols.

- Moderator Recommendation:

  - Discussion on Proposal 1 firstly.

##### Issue 1-1-3: Specification implementation of operating bands support for SBFD

- [Moderator] Based on the discussion in RAN4#112bis, the following agreement is given for specification implementation for SBFD support for operating bands:

Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

- Proposals:

  - Option-1 (Samsung): TP for clause 5.2

=================== Start of Text Proposal ===================

5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in all NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96 and n101, and all NR operating bands in FR2 which are defined in Table 5.2-2.

=================== End of Text Proposal ===================

  - Option-2 (Ericsson): To include the following text in the specification (either new specification or TS 38.104, which is still under discussion): subband full duplex can be applied to the following TDD bands given in Table 5.2-1 in TS 38.104: n38, n39, n40, n41, n48, n50, n77, n78, n79, n90, n102, n104 for FR1.

  - Option-3 (vivo): NR operating bands n38, n39, n40, n41, [n46,] n48, n50, n77, n78, n79, n90, [n96, ][n102,] n104 which are defined in Table 5.2-1, can be applied for subband full duplex operation.

- Moderator Recommendation:

  - Suggest to use Option 1 (given NR-U band is excluded based on the discussion results from Issue 1-1-1).

##### Issue 1-1-4: Specification implementation of BS channel bandwidth support for SBFD

- [Moderator] Based on the discussion in RAN4#112bis, the following agreement is given for the support channel bandwidth for SBFD operation:

Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead.

- Proposals:

  - Option-1 (Samsung): introduce the following note in Table 5.3.5-1:

“Note 9: Subband full duplex (SBFD) can be applied to all supported channel bandwidth values except the ones smaller than 20MHz in band n39 and ones smaller than 40MHz in other TDD bands except bands n34, n39, n46, n51, n53, n54, n96 and n101.”

  - Option-2 (Charter): Add this note to Table 5.3.2-1:

    - Note: The channel bandwidth for a Subband Full Duplex carrier shall be no less than X = 40 MHz.

      - For band n39, X = 20MHz is used instead.

- Moderator Recommendation:

  - The similarity between two options has been observed, while it is suggested to discuss the note should be added in Table 5.3.5-1 or Table 5.3.2-1.

  - Suggest to collect views from other companies firstly.

### Sub-topic 1-2: Transmission bandwidth configuration for SBFD

##### Issue 1-2-1: Transmission bandwidth configuration support for SBFD

- [Moderator] In RAN4#112, much progress has been achieved on the definition clarification of BS channel bandwidth and BS transmission bandwidth configuration for SBFD

Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. Transmission bandwidth configuration for SBFD Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed

And in RAN4 #112bis, the following WF is provided:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- The following proposals are provided to discuss remaining general issues for transmission bandwidth configuration:

  - Proposal 1 (Tejas): Define the SBFD RBs requirement for all classes of base stations. (Moderator: my interpretation of this proposal is the SBFD transmission bandwidth configuration shall be defined for all kinds of BSs (WA, MR, LA))

- Moderator Recommendation:

  - Discussion the proposal first to see it can be covered by others as the common understanding or not. Further clarification on this proposal could be needed.

##### Issue 1-3-2: Restriction to use the same SCS for DL and UL subbands

- [Moderator] In RAN4 #112bis, the following WF is provided, in which it is discussion to “FFS the restriction of SCS for DL and UL subbands”:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- Based on companies’ proposals, the following options are given for the restriction of SCS for DL and UL subbands

  - Option 1 (ZTE): SCS for DL and UL sub-band should be the same.

  - Option 2 (vivo): Use the same SCS for DL and UL subbands in the illustrative figures for transmission bandwidth configuration for SBFD, while do not restrict different SCS for DL and UL subband.

- Moderator Recommendation:

  - Discussion the proposals first by collecting other companies’ preferences.

##### Issue 1-2-3: Symbols for SBFD DUD configuration

- [Moderator] In RAN4#112, much progress has been achieved on the definition clarification of BS channel bandwidth and BS transmission bandwidth configuration for SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL and guardband between SBFD UL and DL subband NRB,SBFD,GB, as follows:

Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

In RAN4#112bis, the illustrative figures are discussed and the group agree to use the figures from Ericsson [R4-2416134] used as the baseline for further study. In this study, we found it could be needed to “Revisit the DUD figure based on the updated definition of NRB,SBFD,DL”.

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- [Moderator] The following proposals are provided to discuss the necessity of updating the definition for DUD case:

  - Option 1 (Samsung): Update the symbols for DUD case.

    - the transmission bandwidth configurations for two SBFD DL subbands are denoted by NRB,SBFD,DL,1 and NRB,SBFD,DL,2

    - the guardbands between SBFD UL and DL subbands are denoted by NRB,SBFD,GB,1 and NRB,SBFD,GB,2

    - The existing SBFD symbol explanation can be updated:

      - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

      - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

      - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU or UD case, expressed in resource blocks.

      - NRB,SBFD,GB,1: The size of guardband between SBFD UL subband and the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

      - NRB,SBFD,GB,2: The size of guardband between SBFD UL subband and the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - Option 2 (Nokia): If needed to distinguish, define lower and upper DL sub-bands the following way:

    - NRB,SBFD,DL1: Transmission bandwidth configuration for SBFD lower DL subband for DUD configuration, expressed in resource blocks.

    - NRB,SBFD,DL2: Transmission bandwidth configuration for SBFD upper DL subband for DUD configuration, expressed in resource blocks.

- Moderator Recommendation:

  - Further discussion on the above options.

##### Issue 1-2-4: Illustrative figures for transmission bandwidth configuration for SBFD

- [Moderator] The following agreement is provided in last meeting:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

- Follow-up proposals for the corresponding figure and other spec. implementation issue:

  - Observation 1 (Samsung): The following issues are identified in the illustrative figures for SBFD DUD, DU and UD patterns:

    - Transmission bandwidth configuration for SBFD NRB,SBFD shall be used instead of  NRB

    - In SBFD DUD case, the values of NRB,SBFD,DL,1 and NRB,SBFD,DL,2 are used instead, which can be different

    - In SBFD DUD case, the two values of NRB,SBFD,GB,1 and NRB,SBFD,GB,2 are used instead, which can be different

    - WGuard is not defined which should not be used

    - “Channel Edge” can be added to illustrate channel bandwidth edge, the same as Figure 5.3.1-1 in TS38.104.

    - “Resource Block” can be added to illustrate the resource block represented by a rectangular, the same as Figure 5.3.1-1 in TS38.104.

- The figures for SBFD DUD, DU and UD are updated in Ericsson’s contribution:

Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel

Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel

Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

- Moderator Recommendation:

  - Further discussion on the above illustrative figures.

### Sub-topic 1-3: DL/UL subband configurations support for SBFD

##### Issue 1-3-1: Clarification on declaration and flexibility

- [Moderator] For clarification on declaration and flexibility, the following proposal is given:

  - Proposal 1 (Ericsson): The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

- Moderator Recommendation:

  - Further discussion on the above proposal.

##### Issue 1-3-2: DL/UL subband configurations support for SBFD

- [Moderator] Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#112, the following three options with different levels of configuration flexibilities have been provided:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

- Companies’ preference on 3 identified options (based on last meeting’s contributions, and updates if any):

  - Option-1 (with the least flexibility): Nokia, Ericsson

  - Option-2 (with the middle flexibility): CMCC, Samsung, CATT, vivo, Huawei, ZTE, Qualcomm

- Furthermore, by following the procedure given in R4-2418790, the following table is given for Option 1:

  - Proposal 1 (Ericsson): RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1, by (1) Reuse the existing channel bandwidths for SBFD UL subbands, (2) For WA BSs, wider guardbands than existing ones are needed for SBFD.

|  |  | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
| DUD (SCS 30 kHz) | NRB,SBFD,DL | 49/50 | 5857 | 70/71 | 78 | 92 | 106 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 45 | 5 | 5 | 5 | 5 |
|  | D | 40 | 45 | 55 | 60 | 70 | 80 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
| DU/UD (SCS 30 kHz) | NRB,SBFD,DL | 104 | 119 | 146 | 161 | 189 | 217 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 5 | 5 | 5 | 5 | 5 |

- Moderator Recommendation:

  - Still need more discussion on three options.

##### Issue 1-3-3: Additional restriction for Option 2b - DL subband sizes to be declared/tested

- [Moderator] Based on last meeting’s agreement, the following additional restrictions are FFS for Option 2b:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

- [Moderator] For the restriction (the number of DL subband sizes to be declared and tested), the following proposals/observations are provided.

- Among Alt-1 and Al-2:

  - Support Alt-1 (Nokia, Ericsson): One DL subband size to be declared and tested.

  - Support Alt-2 (Tejas, Samsung, CMCC, vivo, Qualcomm, ZTE, Huawei): For FR1 WA BS, all DL subband sizes declared to support SBFD shall be tested;

    - Qualcomm: FFS the number of downlink subband configuration supported for FR1 WA BS

- For the restriction to other FR1 BS classes and FR2 BS:

  - Option 1 (Tejas, Samsung, vivo, Qualcomm): This restriction is not applicable to other FR1 BS classes and FR2 BS.

  - Option 2 (Nokia): Apply Alt-1 also for FR1 MR BS subband configuration. More flexibility for subband configuration can be considered for FR1 LA BS. Subband configurations for FR2-1 is still FFS.

- Moderator Recommendation:

  - The discussion on this issue can be continued if the group see the possibility of adopting Option-2b.

    - Further discussion is still needed on above Alt-1 and Alt-2.

    - Further discussion is still needed on the restriction to other FR1 BS classes and FR2 BS.

##### Issue 1-3-4: Additional restriction for Option 2b - UL subband size based on the specific channel BW

- [Moderator] Based on last meeting’s agreement, the following additional restrictions are FFS for Option 2b:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

- [Moderator] For the restriction (on UL subband size based on the specific channel bandwidth), the following proposals/observations are provided:

  - Option 1 (CATT, ZTE): No restriction on the UL SB size ratio in the core requirements.

  - Option 2 (Tejas): The UL subband size relative to the channel bandwidth can range from 20% to 40%.

  - Option 2a (Samsung, vivo): For DUD and DU case, the ratio for UL subband size over channel bandwidth shall be no less than 20%

  - Option 3 (Huawei): As a compromise, only introduce the following bandwidth from Table 5.3.2-1 for SBFD UL subband configuration:

    - 10MHz, 20MHz, 30MHz, 40MHz, 50MHz, 60MHz 70MHz, 80MHz and 90MHz.

  - Option 4 (Ericsson, Nokia): For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

- For the restriction to other FR1 BS classes and FR2 BS:

  - Option 1 (Samsung)

    - The restriction can be applied to both FR1 and FR2.

- Moderator Recommendation:

  - The discussion on this issue can be continued if the group see the possibility of adopting Option-2b.

    - Further discussion is still needed on above options for the restriction on UL subband size based on the specific channel bandwidth.

##### Issue 1-3-5: Additional restriction for Option 2b - UL subband in the middle of channel BW

- [Moderator] Based on last meeting’s agreement, the following additional restrictions are FFS for Option 2b:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth

- [Moderator] For the restriction (the UL subband should be placed in the middle of channel bandwidth for DUD), the following proposals/observations are provided:

  - Observation 1 (Samsung): For restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD), it is possible that UL subband can’t be located at the exact middle of channel bandwidth.

  - Proposal 1 (Samsung): The proponent of restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD) shall solve the issue in the Observation 1.

  - Proposal 2 (CATT): Flexibility of the UL SB transmission center frequency should be allowed that it may not be the center frequency of UL SB.

  - Proposal 3 (vivo): For DUD case, the UL subband should be placed in the middle of channel bandwidth

  - Proposal 4 (Ericsson, Nokia): Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

- Moderator Recommendation:

  - The discussion on this issue can be continued if the group see the possibility of adopting Option-2b.

    - Further discussion is still needed on the restriction of the UL subband in the middle of channel BW for DUD case.

##### Issue 1-3-6: Other restriction on DL subband size

- [Moderator] Based on CATT’s analysis, the following proposals are given. For 2nd proposal, although it is related to channel bandwidth size, we still put it in this issue, because it is the proposal derived from 1st proposal.

  - Proposal 1 (CATT): For SBFD, at least one DL SB BW should be larger than (SSB BW + sync raster step size).

  - Proposal 2 (CATT): FR2 minimum CBW to support SBFD is 100MHz.

- Moderator Recommendation:

  - Discussion on the above proposals firstly.

##### Issue 1-3-7: Restriction on guardband size for SBFD

- [Moderator] Based on last meeting’s agreement, the following options for restriction on guardband size are identified:

Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

- Companies’ preference (based on this meeting’s contribution):

  - Option 3: CMCC, ZTE

  - Option 3a: Samsung, CATT, ZTE

- The following proposal is given for GB size, while it is related to restricting SCS for UL and DL subbands in Issue 1-3-1.

  - Proposal 3 (vivo): GB doesn’t have to be in the number of RB when different SCS are configured for UL and DL subbands. (Moderator: the proposal is given because of P2)

- Moderator Recommendation:

  - Further discussion on the above options, which are dependent on above issues within this sub-topic.

### Sub-topic 1-4: Other general issues for SBFD requirements

##### Issue 1-4-1: BS RF Specification structure for SBFD requirements

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved. Accordingly, BS RF specification structure for SBFD requirements shall be further discussed in this meeting.

<From RAN4#112> - Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

- Options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option 2 (CMCC, Samsung, vivo, Qualcomm, Huawei): Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

    - Option 2b (CMCC, Samsung): Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

  - Option 3 (Ericsson, Nokia): A new specification for SBFD BS RF requirements

- Furthermore, there are proposals related to how to handle this discussion:

  - Proposal 1 (Samsung): The issue of BS RF specification structure (or potentially new specification) is standardization problem for specification implementation, which shall be separated from other technical discussion.

- Moderator Recommendation:

  - Based on majority view, agree on Option 2 firstly.

##### Issue 1-4-2: CLI handling impact on RF requirement

- [Moderator] Based on RAN4#112 discussion, the follow way forward is achieved.

<From RAN4#112> - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Based on previous meeting’s discussion, there is no agreement to either revisit or confirming no revisit the “FFS” bullet.

- Further proposals are provided:

  - Proposal 1 (Nokia): RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

- Moderator Recommendation:

  - Suggest to deprioritize the discussion on this issue since it is not easily to be concluded, and this will also not directly make any difference for detailed RF requirements discussion till now.

# Topic #2: PUSCH with resource muting for CLI handling

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418052 (submitted in AI 7.20.1) | Qualcomm Incorporated | Observation 1: Resource muting changes the waveform characteristics and could have an impact on MPR. Observation 2: With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.) needs to be revisited. |
| R4-2419034 (submitted in AI 7.20.2.1) | Huawei, HiSilicon | Observation 2: For UL resource muting, there is no transmit quality or MPR requirement impact can be identified for PUSCH as the spec is already support DMRS multiplexing as the data could be mapped according to comb-2 pattern. Further measurements or simulation can be conducted if RAN1 inputs on necessary assumptions is possible. |

[Moderator] It should be noted the above contributions contain the discussion related to the RAN4 requirement impact due to the new waveform for UL resource muting, for which there is no dedicated AI planned for this meeting.

## Open issues summary

### Sub-topic 2-1: RF requirement impact for PUSCH with resource muting

##### Issue 2-1-1: RF requirement impact for PUSCH with resource muting

- [Moderator] The following object is given in the updated WID:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-s-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap with UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability

- [Moderator] Based on moderator’s understanding, it is not clear this issue should be directly discussed in RAN4 or triggered to be discussed in RAN4 by RAN1 LS. The following observations are provided in companies’ contributions:

  - Observation 1 (Qualcomm): Resource muting changes the waveform characteristics and could have an impact on MPR.

  - Observation 2 (Qualcomm): With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  needs to be revisited.

  - Observation 3 (Huawei): For UL resource muting, there is no transmit quality or MPR requirement impact can be identified for PUSCH as the spec is already support DMRS multiplexing as the data could be mapped according to comb-2 pattern. Further measurements or simulation can be conducted if RAN1 inputs on necessary assumptions is possible.

- Moderator Recommendation:

  - Collect companies’ view on the above observations firstly.

# Topic #3: Potentially new BS RF requirements for SBFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417787 | Tejas Network Limited | Observation 1: In general, the transient time should be the same for both full TDD switching and switching related to SBFD, as the total transmit (TX) switching time budget must be consistent for both switching between the complete uplink/downlink (UL/DL) and the SBFD resource blocks (RBs). For the transitions between DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used, since the power amplifier (PA) will always remain on during this transition. Additionally, because the user equipment (UE) operates in half-duplex mode, the transition from non-SBFD DL to SBFD UL should account only for the PA switching time. As a result, no new requirements are anticipated from the base station (BS) side. Proposal 1: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol. |
| R4-2417854 | CATT | Proposal 1: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot. Proposal 2: The following new requirements are not defined for SBFD BS. – In-channel adjacent subband leakage ratio – In-channel adjacent subband selectivity – In-channel adjacent subband blocking |
| R4-2418023 | Nokia | Observation 1: On transient period requirements, that SBFD antenna configuration option 1, 2, 3 mentioned in R4-2416498 were discussed in Rel-18 by RAN1/RAN4 for the purposes of performance evaluation, but no agreement on normative specification impact has been made. Observation 2: The OTA sensitivity requirement does not capture the effects from co-site inter-sector and inter-site interference. Observation 3: In-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment. Observation 4: Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to enable proper operation considering self-interference, co-site inter-sector and inter-site interference. Proposal 1: For introduction of new BS RF requirements for SBFD operation, solution with creating new specification is proposed (option 3). Proposal 2: Agreement on a text proposal on transmitter transient period for SBFD should wait for the discussion on whether SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used. Proposal 3: Potential text proposal on transmitter transient period for SBFD should adhere to the agreement reached in RAN4#112 where four transition cases were identified as shown in Table 1, without detailing gNB antenna configuration options. Proposal 4: RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands. Proposal 5: RAN4 to define in-channel adjacent sub-band RX requirement to cover both selectivity and blocking requirement. |
| R4-2418098 | Samsung | Observation 1: For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected: Expected timing behavior for DL/UL subbands respectively; Location of transient period (if any). Case-A (SBFD symbol to DL symbol); DL subband(s); No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol.; N/A. UL subband; TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period.; Located in TA_offset in SBFD symbol. Case-B (SBFD symbol to UL symbol); DL subband(s); UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset.; Located in guard symbols reserved in SBFD symbols. UL subband; TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all; N/A. Case-C (DL symbol to SBFD symbol); DL subband(s); No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol.; N/A. UL subband; UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset.; Located in guard symbols reserved in non-SBFD DL symbols. Case-D (UL symbol to SBFD symbol); DL subband(s); UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset; Located in TA_offset in non-SBFD UL symbol. UL subband; TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all.; N/A. Observation 2: The location of transient periods should be located in (1) SBFD symbol for Case-A; (2) SBFD symbol(s) reserved as guard symbol(s) for Case-B; (3) Non-SBFD DL symbol(s) reserved as guard symbol(s) for Case-C; (4) Non-SBFD UL symbol for Case-D. Proposal 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. Observation 3: For the cases in which the transient period is located in guard symbols reserved in Non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband. Observation 4: The transient period for SBFD operation is different from TDD operation in terms of: - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements. Observation 5: The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration: Frequency regions for transition; SBFD antenna configuration Option-1 (Same panel area as TDD); SBFD antenna configuration Option-2&3 (Twice panel area as TDD); BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter OFF-to-ON. Proposal 2: Based on the example given in R4-2416498, the text proposal is updated for the clause of transmitter transient period for SBFD: =================== Start of Text Proposal =================== #### 6.4.2B Transmitter transient period for SBFD ##### 6.4.2.1B General Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS. The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1. The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases antenna configuration: Table 6.4.2.1B-1: Transmitter transient period for SBFD Frequency regions for transition; SBFD antenna configuration Option-1 (Same panel area as TDD); SBFD antenna configuration Option-2&3 (Twice panel area as TDD); BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); SBFD RX antenna panel; N/A; BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; SBFD TX antenna panel; SBFD TX antenna panel; BS transmitter OFF-to-ON. For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. =================== End of Text Proposal =================== In-channel adjacent subband requirements Observation 6: All the gNB-to-gNB CLI handling schemes likely to be introduced in RAN1 (including beam nulling, beam pairing and non-transparent UL resource muting) will greatly mitigate the gNB-to-gNB CLI, and make it be hard to have a criterion for a “proper” interference level or in-channel adjacent subband leakage ratio. Observation 7: For system-level evaluation (given a certain level of system performance degradation, e.g., 5%), the required in-channel adjacent subband leakage ratio levels can be different if adopting different inter-gNB CLI handling schemes. Observation 8: It is difficult for RAN4 to agree on a reference scheme for self-interference suppression implemented to derive the potential new requirement in-channel adjacent subband leakage ratio. Proposal 3: There is no necessity to introduce new requirement for in-channel adjacent subband leakage ratio. Observation 9: Considering the RAN1-introduced gNB-gNB CLI-handling mechanism, the co-channel interference from neighboring gNB DL subband transmission could be greatly mitigated. Observation 10: The blocking from co-channel gNB DL subband interference is not as important as adjacent channel gNB DL interference. Proposal 4: There is no necessity to introduce new requirement for in-channel adjacent subband blocking and adjacent subband selectivity, considering that the in-band blocking requirement will be specified anyway. |
| R4-2418602 | Qualcomm Germany | Observation 1: RAN4 has not specified before in-band emission requirements for legacy TDD BSs as it is implementation dependent and different solutions might vary between vendors. Observation 2: The in-channel subband sub-band Tx leakage, selectivity, and blocking depend on the self-interference, inter-site and inter-sector interference considerations, thus, they are implementation dependent. Proposal 1: RAN4 to agree that no new requirements for in-channel subband leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs. |
| R4-2418741 | CMCC | Proposal 1: In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements. Proposal 2: Both sub-band selectivity and blocking requirements should be defined. |
| R4-2418791 | Ericsson | Proposal 1 Place the transient period outside of both SBFD and non-SBFD slots. Proposal 2 RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases. Proposal 3 Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline. Proposal 4 Define a requirement on RX sub-band blocking similar to the in-band blocking requirement and use existing in-band blocking requirement as baseline. Proposal 5 New requirement to capture Inter-sector interference shall be adopted. |
| R4-2419034 | Huawei, HiSilicon | Observation 1: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent. Proposal 1: The location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol. Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio. Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking. |
| R4-2419220 | ZTE Corporation, Sanechips | Proposal 1: for the inter-site scenario, not to define in-channel sub-band leakage and leave this up to the implementation. Proposal 2: if companies have strong preference to define the Tx in-band leakage requirement, then prefer to define it similar as UE in-band emission general requirement which could be applicable for any DL sub-band transmission bandwidth configuration and also UL sub-band transmission bandwidth configuration. Observation 1: increasing Iot level due to BS2BS CLI problem in the DL sub-band region if SBFD BS is capable of receiving the whole carrier instead of UL sub-band only and without considering the BS CLI coordination within the carrier is larger than legacy 25dBc for FR1 ICS requirement and also 14dBcfor FR2 ICS requirement except for FR2 WA scenario. Observation 2: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the increasing IoT level for SBFD BS reception. Proposal 3: not to define in-channel adjacent subband selectivity/blocking requirements if received BS2BS CLI within the carrier manged lower than in-band blocking power level. |
| R4-2419219 (under AI 7.20.1) | ZTE Corporation, Sanechips | Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols. |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Requirements for Transient period between SBFD and non-SBFD

##### Issue 3-1-1: Transient period requirement applicability on frequency/antenna

- [Moderator] In RAN4#112 and #112bis, the following agreement and WF are achieved for the requirement for transient period between SBFD and non-SBFD:

<Agreement in RAN4#112> Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. <Agreement in RAN4#112bis> Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498.

- Observation/Proposal from Samsung:

  - Observation 1 (Samsung): The transient period for SBFD operation is different from TDD operation in terms of:

    - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

    - Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

  - Observation 2 (Samsung): The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

  - Proposal 1 (Samsung): Based on the example given in R4-2416498, the text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

#### 6.4.2B	Transmitter transient period for SBFD

##### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the SBFD TX and RX antenna penal and transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases antenna configuration:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

- Observation/Proposal from Nokia:

  - Observation 1 (Nokia): On transient period requirements, that SBFD antenna configuration option 1, 2, 3 mentioned in R4-2416498 were discussed in Rel-18 by RAN1/RAN4 for the purposes of performance evaluation, but no agreement on normative specification impact has been made.

  - Proposal 1 (Nokia): Agreement on a text proposal on transmitter transient period for SBFD should wait for the discussion on whether SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used.

  - Proposal 2 (Nokia): Potential text proposal on transmitter transient period for SBFD should adhere to the agreement reached in RAN4#112 where four transition cases were identified as shown in Table 1, without detailing gNB antenna configuration options.

- Observation/Proposal from Ericsson:

  - Proposal 1 (Ericsson): RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases.

- Moderator Recommendation:

  - RAN4 agree on the following principle from Ericsson:

    - RAN4 do not define an architect dependent requirement. Transient period between SBFD and non-SBFD is applicable to all four cases.

  - Based on the above principle (if agreed), discuss the revised text proposal from Samsung.

##### Issue 3-1-2: Location of transient period between SBFD and non-SBFD

- [Moderator] In RAN4#112, the following agreement is achieved for “Location of transient period between SBFD and non-SBFD”:

- Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

- Location of transient period between SBFD and non-SBFD:

  - Option 1 (Samsung): No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

  - Option 1a (Ericsson): Place the transient period outside of both SBFD and non-SBFD slots.

  - Option 2 (CATT/Tejas/Huawei/ZTE): The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

- Furthermore, the following observations are provided to support the above options.

  - Observation 1 (Samsung): For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in SBFD symbols |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbols reserved in non-SBFD DL symbols |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

  - Observation 2 (Samsung): The location of transient periods should be located in (1) SBFD symbol for Case-A; (2) SBFD symbol(s) reserved as guard symbol(s) for Case-B; (3) Non-SBFD DL symbol(s) reserved as guard symbol(s) for Case-C; (4) Non-SBFD UL symbol for Case-D.

  - Observation 3 (Tejas): In general, the transient time should be the same for both full TDD switching and switching related to SBFD, as the total transmit (TX) switching time budget must be consistent for both switching between the complete uplink/downlink (UL/DL) and the SBFD resource blocks (RBs). For the transitions between DL-SBFD UL and SBFD-DL, the existing TDD switching time can be used, since the power amplifier (PA) will always remain on during this transition. Additionally, because the user equipment (UE) operates in half-duplex mode, the transition from non-SBFD DL to SBFD UL should account only for the PA switching time. As a result, no new requirements are anticipated from the base station (BS) side.

- Moderator Recommendation:

  - RAN4 can firstly discuss the technical details behind Option 1 and 2, while if no conclusion, consider to send LS to RAN1 to clarify this issue.

### Sub-topic 3-2: In-channel adjacent subband leakage ratio requirements

##### Issue 3-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

- [Moderator] The necessity of the requirement for in-channel adjacent subband leakage ratio requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei): no necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

    - Option 1a (CMCC): In-channel adjacent sub-band leakage ratio should be defined or reflect by other requirements.

  - Option 2 (Nokia/Ericsson): New requirement for in-channel adjacent subband leakage ratio should be defined.

    - Option 2a (Nokia/Ericsson): RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

  - Option 3 (ZTE):

    - for the inter-site scenario, not to define in-channel sub-band leakage and leave this up to the implementation.

    - if companies have strong preference to define the Tx in-band leakage requirement, then prefer to define it similar as UE in-band emission general requirement which could be applicable for any DL sub-band transmission bandwidth configuration and also UL sub-band transmission bandwidth configuration.

- Moderator Recommendation:

  - This issue has been discussed for several meetings without any progress.

  - Since Option 3 is newly proposed, companies are encouraged to discuss this new option.

### Sub-topic 3-3: In-channel adjacent subband selectivity/blocking requirements

##### Issue 2-3-2: Necessity of in-channel adjacent subband blocking requirement

- [Moderator] The necessity of the requirement for in-channel adjacent subband blocking requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei/ZTE): no necessity to introduce new requirement for in-channel adjacent subband blocking.

  - Option 2 (Nokia/CMCC/Ericsson): New requirement for in-channel adjacent subband blocking should be defined.

    - Option 2a (Ericsson): Define a requirement on RX sub-band blocking similar to the in-band blocking requirement and use existing in-band blocking requirement as baseline.

- Moderator Recommendation:

  - This issue has been discussed for several meetings without any progress. Moderator suggest more detailed proposal on Option 2 for how this requirement shall be defined, such as interference level, etc.

  - Considering that the in-band blocking requirement will be specified anyway, can this new requirement can be guaranteed by the in-band blocking requirement specified for SBFD operation?

##### Issue 3-3-1: Necessity of in-channel adjacent subband selectivity requirements

- [Moderator] The necessity of the requirement for in-channel adjacent subband selectivity requirements has not yet been confirmed, based upon previous meeting discussion. In RAN4#112, the following WF is approved:

- Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

- Proposals:

  - Option 1 (Samsung/CATT/Qualcomm/Huawei/ZTE): no necessity to introduce new requirement for in-channel adjacent subband selectivity.

  - Option 2 (Nokia/CMCC): New requirement for in-channel adjacent subband selectivity should be defined.

  - Option 3 (Ericsson): No necessity for selectivity requirement if the in-channel adjacent subband blocking requirement is defined.

- Moderator Recommendation:

  - Discussion on the above issue firstly. Seems the selectivity requirement can be included in the blocking requirement (if any).

### Sub-topic 3-4: Inter-sector and inter-site interference

##### Issue 3-4-1: Necessity of new requirement for inter-sector and inter-site interference

- [Moderator] The following proposal is given in this meeting:

  - Proposal 1 (Ericsson): New requirement to capture Inter-sector interference shall be adopted.

Figure 1: Illustration of inter-sector interference

  - Additional explanation from Ericsson:

    - “In our view, inter-sector interference has been one of the main discussion points since start of SI. There are dedicated tables which take inter-sector islolation to account, e.g., Table 9.2.2.1-1: FR1 WA BS Co-site Inter-sector Co-channel Interference Analysis Summary, row ”Spatial isolation”.”

- Moderator Recommendation:

  - Considering this new requirement is not yet studied in Rel-18, suggest no discussion on this new requirement, considering the limited TU in Rel-19.