3GPP TSG-RAN WG4 Meeting # 113														R4-2419599

Orlando, US, 18th – 22nd November 2024

Agenda item:			7.8.5

Source:	Moderator (Nokia)

Title:	Topic summary for [113][312] NR_IoT_NTN_less_than_5MHz_BSRF

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Summary for contributions submitted under agenda items 7.8.3.3 for SAN RF core requirements of Less than 5MHz for NTN.

List of candidate target of discussion for 1st round and 2nd round:

- 1st round: Discussion and agreement on open issues listed below.

- 2nd round: Continue discussion and agreement on open issues listed below.

# Topic #1: Core requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417811 | CATT | Proposal 1: The -1.9 dB scaling for NTN SAN REFSENS requirements for NR NTN support 3 MHz channel bandwidth could be considered. Observation 1: |
| R4-2417821 | CATT | Proposal 1: DraftCR for TS 38.108, Introduction on RF requirements for SAN supporting less than 5 MHz channel bandwidth. Observation 1: |
| R4-2418363 | ZTE Corporation, Sanechips | Proposal 1: DraftCR to TS38.108 Introduction of less than 5MHz channel bandwidth for FR1-NTN. Observation 1: |
| R4-2419664 | Ericsson France S.A.S | Proposal 1: Regarding the total power dynamic range requirement, the minimum requirement should be calculated according to the transmission bandwidth configuration: specify 11.7dB in Table 6.3.3.2-1, which is the same value as for 3MHz CBW for TN. Proposal 2: For the ACLR requirement, 3 MHz CBW should be added to Tables 6.6.3.2-1 and 6.6.3.2-2, where the ACLR limit should remain the same and the assumed adjacent channel carrier should be an NR of same BW. Proposal 3: For the reference sensitivity requirement for 3 MHz CBW, 1.9dB improvement compared with 5 MHz CBW should be specified, which is the same principle as for TN. Proposal 4: For the ACS requirement, 3 MHz CBW shall be added. The same PREFSENS + 8dB wanted signal mean power as for TN 3 MHz CBW should be specified in Table 7.4.1.2-1. In Table 7.4.1.2-2, the same type of interfering signal and the interfering signal center frequency offset could be reused from 3 MHz BS CBW for TN: ±1.5075 MHz frequency offset for 3 MHz DFT-s-OFDM NR signal with 15 kHz SCS and 15 RBs. Proposal 5: For the OTA reference sensitivity requirement for 3 MHz CBW, 1.9dB improvement could be achieved compared with 5 MHz CBW, i.e. adopt -101.2 - ΔOTAREFSENS. Proposal 6: For the OTA dynamic range requirement for 3 MHz CBW, lower the requirement for 5 MHz CBW by 2.2dB, i.e. adopt -79.3- ΔOTAREFSENS for the wanted signal mean power (dBm) and -90.4- ΔOTAREFSENS (dBm/BWconfig) for the interfering signal mean power. Proposal 7: For the OTA ACS requirement for 3 MHz CBW, the wanted signal mean power should be increased by 2dB compared with 5 MHz or larger CBWs, i.e. EISminSENS + 8 dB. Interfering signal centre frequency offset from the lower/upper SAN RF Bandwidth edge or sub-block edge inside a sub-block gap can be ±1.5075 MHz. Proposal 8: For the OTA in-channel selectivity requirement for 3 MHz CBW, the wanted signal mean power and the interfering signal mean power should be decreased by 2.2dB compared with 5 MHz CBW, i.e. adopt -100.4- ΔminSENS (dBm) for the wanted signal mean power and -94.2 – ΔminSENS (dBm) for the interfering signal mean power. Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: Proposals on core requirements

- Proposals

  - All proposals in R4-2417811 and R4-2419664 have been included in the draft CRs in R4-2417821 and R4-2418363.

- Recommended WF

  - Note R4-2417811 and R4-2419664.

### Sub-topic 1-2

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-2: Draft CRs

- Proposals

  - Draft CRs in R4-2417821 and R4-2418363 have implemented agreements in WFs in previous meeting as well as proposals in R4-2417811 and R4-2419664.

- Recommended WF

  - Endorse R4-2417821 and R4-2418363.

# Topic #2: Test requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2417542 | Nokia | Proposal 1: The necessary changes are provided in the table below. NR SAN RF conformance requirement; Proposal for 3MHz channel bandwidth in FR1-NTN bands. 4.1.2 Acceptable uncertainty of Test System; Add maximum test system uncertainty for 3 MHz in Table 4.1.2.2-1 and Table 4.1.2.2-2.. 4.7.2 Test signal used to build Test Configurations; Add a note in Table 4.7.2-1 to use 3 MHz channel bandwidth if supported.. 6.2 Satellite Access Node output power; No specification impact.. 6.3.2 RE power control dynamic range; No specification impact.. 6.3.3 Total power dynamic range; Refer to TS 38.108, no update on specification.. 6.4 Transmit ON/OFF power; No specification impact.. 6.5 Transmitted signal quality; No specification impact.. 6.6.2 Occupied bandwidth; No specification impact.. 6.6.3 Adjacent Channel Leakage Power Ratio; Requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.5.2-1 and Table 6.6.3.5.2-2.. 6.6.4 Out-of-band emissions; No specification impact.. 6.6.5 Transmitter spurious emissions; No specification impact. 6.7 Transmitter intermodulation; No specification impact.. 7.2 Reference sensitivity level; Add minimum requirements for 3 MHz in Table 7.2.5-1 and Table 7.2.5-2. Scaling from 5 MHz can be used. Use FRC G-FR1-A1-7, same as in TN.. 7.3 Dynamic range; Add minimum requirements for 3 MHz in Table 7.3.5-1. Scaling from 5 MHz can be used. Reuse TN FRC G-FR1-A2-15 (to be introduced to A.2).. 7.4.1 Adjacent Channel Selectivity (ACS); Define ACS requirement similar to TN for 3MHz in Table 7.4.1.5-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.5-2.. 7.4.2 In-band blocking; No specification impact.. 7.5 Out-of-band blocking; No specification impact.. 7.6 Receiver spurious emissions; No specification impact.. 7.7 Receiver intermodulation; No specification impact.. 7.8 In-channel selectivity; Add minimum requirements for 3 MHz in Table 7.8.5-1 and Table 7.8.5-2. Reuse TN FRC G-FR1-A1-20 (to be introduced to A.1).. 9.2 Radiated transmit power; No specification impact.. 9.3 OTA SAN Output power dynamics; No specification impact.. 9.4 OTA output power dynamic range; 1-O follow same requirement as 1-H, no update on specification.. 9.5 OTA transmit ON/OFF Power; No specification impact.. 9.6 OTA transmitted signal quality; No specification impact.. 9.7 OTA unwanted emissions; 1-O follow same requirement as 1-H, no update on specification.. 9.8 OTA Transmitter intermodulation; No specification impact.. 10.2 OTA sensitivity; No specification impact.. 10.3 OTA reference sensitivity level; Similar update as conductive requirements to add 3MHz channel bandwidth.. 10.4 OTA dynamic range; Similar update as conductive requirements to add 3MHz channel bandwidth.. 10.5 OTA In-band selectivity and blocking; Similar update as conductive requirements to add 3MHz channel bandwidth.. 10.6 OTA Out-of-band blocking; No specification impact.. 10.7 OTA receiver spurious emissions; No specification impact.. 10.8 OTA receiver intermodulation; No specification impact.. 10.9 OTA In-channel selectivity; Similar update as conductive requirements to add 3MHz channel bandwidth.. A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3); FRC for 3MHz in-channel selectivity need to be added in Table A.1-1. Reuse TN FRC G-FR1-A1-20. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance.. A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3); FRC for 3MHz needs to be added in Table A.2-1. Reuse TN FRC G-FR1-A2-15. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance.. Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: Proposals on test requirements

- Proposals

  - Option 1: Agree the necessary changes are provided in the table in R4-2417542.

  - Option 2: Revise the necessary changes are provided in the table in R4-2417542.

  - Option 3: Note the necessary changes are provided in the table in R4-2417542.

- Recommended WF

  - Agree the necessary changes are provided in the table in R4-2417542.

| NR SAN RF conformance requirement | Proposal for 3MHz channel bandwidth in FR1-NTN bands |
| --- | --- |
| 4.1.2 Acceptable uncertainty of Test System | Add maximum test system uncertainty for 3 MHz in Table 4.1.2.2-1 and Table 4.1.2.2-2. |
| 4.7.2 Test signal used to build Test Configurations | Add a note in Table 4.7.2-1 to use 3 MHz channel bandwidth if supported. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Refer to TS 38.108, no update on specification. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality | No specification impact. |
| 6.6.2 Occupied bandwidth | No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio | Requirement limits as other channel bandwidth, but assume different adjacent channel carriers in Table 6.6.3.5.2-1 and Table 6.6.3.5.2-2. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact |
| 6.7 Transmitter intermodulation | No specification impact. |
| 7.2 Reference sensitivity level | Add minimum requirements for 3 MHz in Table 7.2.5-1 and Table 7.2.5-2. Scaling from 5 MHz can be used. Use FRC G-FR1-A1-7, same as in TN. |
| 7.3 Dynamic range | Add minimum requirements for 3 MHz in Table 7.3.5-1. Scaling from 5 MHz can be used. Reuse TN FRC G-FR1-A2-15 (to be introduced to A.2). |
| 7.4.1 Adjacent Channel Selectivity (ACS) | Define ACS requirement similar to TN for 3MHz in Table 7.4.1.5-1, and assume same interferer frequency offset values as TN 3 MHz in Table 7.4.1.5-2. |
| 7.4.2 In-band blocking | No specification impact. |
| 7.5 Out-of-band blocking | No specification impact. |
| 7.6 Receiver spurious emissions | No specification impact. |
| 7.7 Receiver intermodulation | No specification impact. |
| 7.8 In-channel selectivity | Add minimum requirements for 3 MHz in Table 7.8.5-1 and Table 7.8.5-2. Reuse TN FRC G-FR1-A1-20 (to be introduced to A.1). |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN Output power dynamics | No specification impact. |
| 9.4 OTA output power dynamic range | 1-O follow same requirement as 1-H, no update on specification. |
| 9.5 OTA transmit ON/OFF Power | No specification impact. |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7 OTA unwanted emissions | 1-O follow same requirement as 1-H, no update on specification. |
| 9.8 OTA Transmitter intermodulation | No specification impact. |
| 10.2 OTA sensitivity | No specification impact. |
| 10.3 OTA reference sensitivity level | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.4 OTA dynamic range | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.5 OTA In-band selectivity and blocking | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| 10.6 OTA Out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | No specification impact. |
| 10.8 OTA receiver intermodulation | No specification impact. |
| 10.9 OTA In-channel selectivity | Similar update as conductive requirements to add 3MHz channel bandwidth. |
| A.1 Fixed Reference Channels for RF Rx requirements (QPSK, R=1/3) | FRC for 3MHz in-channel selectivity need to be added in Table A.1-1. Reuse TN FRC G-FR1-A1-20. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |
| A.2 Fixed Reference Channels for dynamic range (16QAM, R=2/3) | FRC for 3MHz needs to be added in Table A.2-1. Reuse TN FRC G-FR1-A2-15. Rename the FRC to indicate “NTN” can be discussed as Rel-18 maintenance. |

# Recommendations for Tdocs

| Tdoc number | Recommendation | Comments |
| --- | --- | --- |
| R4-2417542 | Noted |  |
| R4-2417811 | Noted |  |
| R4-2417821 | Endorsed |  |
| R4-2418363 | Endorsed |  |
| R4-2419664 | Noted |  |

…