3GPP TSG-RAN WG4 Meeting # 113	R4-2419839

# Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0291 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI17) CR to TS 37.105 - BS spurious receiver protection note [MSR_BSRF_RX] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For BS capable of multi-band operation, if the BS transmit in a band is overlapping to receive frequency range of the other band, e.g. band 5 and band 8, similar as co-existence requirements and co-location requirements, a NOTE is needed to clarify that the requirement is not applied. It is noted that the same change was agreed for 38 series |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new NOTE is added in clause 9.7.6.2.2 and 9.7.6.4.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The not applicable requirement exists for the specific case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.6.2.2, 9.7.6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change >

9.7.6.2.2	Protection of the BS receiver of own or different BS

This requirement shall be applied for FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a OTA AAS BS.

The requirement is a co-location requirement. The power levels are specified at the co-location reference antenna output.

The power sum of any spurious emissions is specified over all supported polarizations of the co-location reference antenna and shall not exceed the limits in table 9.7.6.2.2-1 depending on the declared Base Station class and Band Category.

Table 9.7.6.2.2-1: BS Spurious emissions limits for protection of the BS receiver

| BS-class | Band category | Frequency range | Maximum Level | Measurement Bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- |
| Wide Area BS | BC1 | FUL_low – FUL_high | -117 dBm | 100 kHz |  |
| Wide Area BS | BC2 | FUL_low – FUL_high | -119 dBm | 100 kHz |  |
| Medium Range BS | BC1,BC2 | FUL_low – FUL_high | -112 dBm | 100 kHz |  |
| Local Area BS | BC1,BC2 | FUL_low – FUL_high | -109 dBm | 100 kHz |  |
| NOTE 1: For BS operating in regions where a band is only partially allocated for E-UTRA/NR operations (e.g. band 28), this requirement only applies in the UL frequency range of the partial allocation. NOTE 2: For BS capable of multi-band operation, Table 9.7.6.2.2-1 assumes that the supported operating bands, where the corresponding BS transmit and receive frequency ranges in TS 37.104 [5] Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special protection requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |  |

<Next Change >

9.7.6.4.2	Protection of the BS receiver of own or different BS

This requirement shall be applied for E-UTRA FDD operation in order to prevent the receivers of own or a different BS of the same band being desensitised by emissions from a OTA AAS BS.

The requirement is a co-location requirement. The power levels are specified at the co-location reference antenna output.

The power sum of any spurious emission is specified over all supported polarizations of the co-location reference antenna and shall not exceed the limits in table 9.7.6.4.2-1.

Table 9.7.6.4.2-1: BS Spurious emissions limits for protection of the BS receiver

|  | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| Wide Area BS | FUL_low – FUL_high | - 117 dBm | 100 kHz |  |
| Medium Range BS | FUL_low – FUL_high | - 112 dBm | 100 kHz |  |
| Local Area BS | FUL_low – FUL_high | -109 dBm | 100 kHz |  |
| NOTE 1: For BS operating in regions where a band is only partially allocated for E-UTRA operations (e.g. band 28), this requirement only applies in the UL frequency range of the partial allocation. NOTE 2: For BS capable of multi-band operation, Table 9.7.6.4.2-1 assumes that the supported operating bands, where the corresponding BS transmit and receive frequency ranges in TS 37.104 [5] Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special protection requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

<End of Change >