3GPP TSG-RAN WG4 Meeting #113	R4-2419875

# Orlando, US, 18th  - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0049 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) CR for TS 38.181, Correction on OTA out-of-band blocking requirement for SAN type 2-O |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - ΔfOOB for FUL,high – FUL,low < 4000 MHz for SAN type 2-O is missing, since the FUL,high – FUL,low for SAN type 2-O for n512, n511, and n510 is less than 4000MHz, so 1500MHz ΔfOOB is reasonable. - The out-of-band blocking requirement should be alignment with out-of-band blocking requirement in TS 38.108 according to Endorsed CR R4-2409817 in RAN4#110bis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add ΔfOOB for FUL,high – FUL,low < 4000 MHz for SAN type 2-O in Table 10.6.3.1-2. - Remove [] for 1500MHz ΔfOOB. - Simplification of the out-of-band blocking requirement. - Remove [] for Interferer RMS field-strength. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ΔfOOB for FUL,high – FUL,low < 4000 MHz for SAN type 2-O would be unclear, and out-of-band blocking requirement would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2417839 |  |  |  |  |  |  |  |  |

<Start of Change 1>

10.6.6	Test requirements for SAN type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted signal applies to each supported polarization, under the assumption of polarization match. The interferer shall be polarization matched in-band and the polarization maintained for out-of-band frequencies.

For SAN type 2-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL,low –ΔfOOB  and from FUL,high +ΔfOOB up to 2nd harmonic of the upper frequency edge of the operating band.The ΔfOOB for SAN type 2-O is defined in table 10.6.6-2.

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 10.6.6-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.3 for each SAN channel bandwidth and further specified in annex A.1.

Table 10.6.6-1: OTA out-of-band blocking performance requirement

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.0029 | CW |
| 12750 to FUL,low –ΔfOOB | EISREFSENS + 6 dB | 0.0029 | CW |
| FUL,high +ΔfOOB to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.0029 | CW |

Table 10.6.6-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL,high – FUL,low ≤ 4000 MHz | 1500 |

<End of Change 1>