3GPP TSG-RAN WG4 Meeting #113	 														R4-2420083

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.24.3

Source:	Apple

Title:	Ad-hoc minutes for NR mobility enhancements

Document for:	Approval

# Plan for ad-hoc discussion

Topic #1 General aspects and work plan (5 min)

- Issue 1-1: R19 inter-CU subsequent LTM

Topic #2 Event triggered L1 measurement reporting (35 min)

- Issue 2-3: ending point in event triggered L1 report requirements

- Issue 2-4: periodic report and single report

- Issue 2-6: impact of time-to-trigger (TTT) on RRM

- Issue 2-16: Impact on other LTM requirements

Topic #3 CSI-RS based L1 measurement (90 min)

- Issue 3-4: Categorization of CSI-RS based L1 RSRP measurement

- Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

- Issue 3-8: measurement type

Topic #4 conditional LTM (20 min)

- Issue 1-1-1: The applicable scenarios for conditional LTM

- Issue 1-3-3: The ending point of conditional LTM delay

- Issue 1-2-1: Early UL Synchronization

# Topic #1: General aspects and work plan

## Open issues summary

#### Issue 1-1: R19 inter-CU subsequent LTM

- Proposals:

  - P1: it is proposed that the Rel-18 cell switch delay requirements can be reused for Rel-19 inter-CU subsequent LTM. (CMCC)

- Recommended WF:

  - Discuss if P1 is already covered by agreements in RAN4#112:

Issue 2-1-1: RAN4 scope of inter-CU LTM - Agreement: - Existing R18 LTM related RRM requirements also apply to same procedures in inter-CU scenario. - No RRM impact is foreseen to support inter-CU LTM. RAN4 can revisit this if any issue identified.

Agreement:

- The Rel-18 cell switch delay requirements can be reused for Rel-19 inter-CU subsequent LTM.

- Note: this can be revisited if any critical issue identified.

# Topic #2: Event triggered L1 measurement reporting

## Open issues summary

#### Issue 2-3: ending point in event triggered L1 report requirements

- Candidate options:

  - Option 1: Reuse the ending point of event-triggered L3 reporting delay for event-triggered L1 reporting delay: the point when the UE starts to transmit the measurement report over the air interface. (Nokia, HW, vivo)

  - Option 2: For event triggered reporting delay, end point is defined as time to first special SR transmission. RAN4 to discuss if we need to consider scenario when UE supports normal SR only. (E///)

- Recommended WF:

  - Discuss the options.

Discussion:

E///: there is a new special SR for L1 reporting agreed in RAN2.

Nokia/MTK: this is only one use case.

QC: this can be part of the test configuration.

Agreement:

- Ending point in event triggered L1 report requirements is defined as the point when the UE starts to transmit the first UL transmission to report measurement result over the air interface.

#### Issue 2-4: periodic report and single report

- Candidate options:

  - Option 1: RAN4 requirements shall cover both Event triggered periodic MR and Event triggered MR. (Nokia, MTK, Xiaomi, Apple, HW)

- Recommended WF:

  - Agree on option 1.

Agreement:

- RAN4 requirements shall cover both Event triggered periodic MR and Event triggered MR.

#### Issue 2-6: impact of time-to-trigger (TTT) on RRM

- Candidate options:

  - Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi)

- Recommended WF:

  - Discuss the option.

#### Issue 2-16: Impact on other LTM requirements

- Candidate solutions:

  - Option 1: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: (Nokia, E///)

    - Known TCI state condition for early TCI state activation and cell switch

    - Applicability of cell switch delay requirements

  - Option 2: RAN4 to postpone discussions on the options listed under ‘Issue 1-12: Impact on other LTM requirements’ in the WF (R4-2416862) approved at RAN4#112bis until the group achieves a certain level of detailed requirements for the other components. (QC)

- Recommended WF:

  - Discuss the options.

# Topic #3: CSI-RS based L1 measurement

## Open issues summary

#### Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

- Candidate options:

  - Option 1: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement. e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: (Apple, CATT)

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for L1 measurement, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for L1 measurement, and

      - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement.

  - Option 2: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (Huawei)

  - Option 3: No need to introduce new definition of intra-frequency or inter-frequency for CSI-RS based L1 measurement. CSI-RS based L1 measurement on one candidate cell is intra-frequency L1 measurement if the SSBs of this candidate cell is of the same SSB frequency, SCS and CP type. (vivo)

  - Option 4: A frequency layer is called an intra-frequency measurement if the CSI-RS resources of the frequency layer are within the active BWP. (Ericsson)

  - Option 5: Use intra-frequency / inter-frequency terminology in RAN4 specification for rel-19 CSI-RS measurements. CSI-RS based L1-RSRP measurement is considered to be intra-frequency measurement if it is fully contained within UE channel bandwidth. CSI-RS based L1-RSRP measurement is considered to be inter-frequency measurement if it does not fulfil the intra-frequency definition. (RAN4) (Nokia)

  - Option 6: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: (Samsung)

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

      - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

  - The measurementBW are the same.

  - Option 7: The discussion on whether and how to accommodate the definition of intra-frequency and inter-frequency CSI-RS-based LTM L1-RSRP measurement can be postponed until the group gains more clarity on the overall measurement framework. (Qualcomm)

  - Option 8: How to define CSI-RS based intra-frequency L1 measurement is related about how to configure CSI-RS for LTM measurement which is pending on RAN1/2 progress. (CMCC)

    - If CSI-RS configuration structure for MO (MeasObjectNR) is used, which means L3 CSI-RS resources for a MO have the same CSI-RS center frequency, SCS, bandwidth,

      - In this case, the definition of CSI-RS based L3 measurement can be reused. In detail, a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that SCS/CP type/centre frequency of the CSI-RS resource of the neighbour cell configured for measurement are same as that of the serving cell indicated for measurement.

    - If NZP-CSI-RS-Resource structure is used, it is possible that different CSI-RS resources from the same cell have different SCS, center frequency, BW,

      - In this case, it is difficult to follow the definition of CSI-RS based L3 measurement. It is preferred not to have definition of CSI-RS based intra-frequency L1 measurement (i.e. CSI-RS based L1-RSRP measurement are categorized into measurement within active BWP and measurement outside active BWP).

  - Option 9: Prefer to define the intra-frequency and inter-frequency for CSI-RS based L1 measurement and a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: (CT, ZTE)

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

      - It is applied for SCS = 60KHz

    - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

  - Option 10: If legacy definition of intra-frequency and inter-frequency will be re-used, all the CSI-RS resources configured for the same cell should be with the same frequency location and SCS. RAN4 to discuss whether to limit the CSI-RS configuration. (Xiaomi)

- Recommended WF:

  - Option 1+Option6?

It is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement. e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: (Apple, CATT)

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for L1 measurement, and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for L1 measurement, and

      - It is applied for SCS = 60KHz

    - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement.

Discussion:

Vivo: it is not good to follow R16 logic since the configuration is different. In legacy there is center frequency configured by NW. UE can be configured with multiple BWP, which may have different center frequency.

Nokia: similar comment as vivo. We can consider some minimum UE CBW, e.g. if target CSI-RS is within that, then it is intra.

QC: do we need to consider centre frequency of the CSI-RS? The important thing is whether UE can measure CSI-RS simultaneously.

CMCC: definition depends on how the CSI-RS is configured, which is still being discussed in RAN1/2. We prefer to have same definition of L3 CSI-RS or not having any definition.

OPPO: other groups are interested in definition of intra/inter

Apple: support option 1. There are quite many other procedures that have similar design.

Alternative 1: define intra/inter based on legacy L3 (with some modification)

- support: CMCC, OPPO, Apple, CATT, ZTE

Alternative 2: define intra/inter depending on whether measurement needs gap or not

- support: MTK, E///, Nokia, vivo

- object: OPPO

Alternative 3: not define intra/inter, define measurement with and without gap

- support: CMCC, MTK, E///, vivo, CATT, ZTE

- object: Apple

Agreement:

A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

    - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and

    - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and

      - It is applied for SCS = 60KHz

    - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

#### Issue 3-4: Categorization of CSI-RS based L1 RSRP measurement

- Candidate options:

  - Option 1: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency (Apple, CATT, CT)

  - Option 2: Prefer to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. We can compromise to call “CSI-RS based L1-RSRP measurement within active BWP” intra-frequency if it is the majority view. (MTK)

  - Option 3: How to category CSI-RS based L1-RSRP measurement is pending on whether/how to define CSI-RS based intra-frequency L1 measurement. (CMCC)

    - 3a: If the definition of CSI-RS based intra-frequency L1 measurement is same as that for CSI-RS based intra-frequency L3 measurement, CSI-RS based L1-RSRP measurement are categorized into intra-frequency and inter-frequency.

    - 3b: Otherwise, CSI-RS based L1-RSRP measurement are categorized into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP can be considered.

- Recommended WF:

  - Option 1: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency

Discussion:

MTK: we are not against to define intra/inter-frequency. Question is how to define.

#### Issue 3-8: measurement type

- Candidate options:

  - Option 1: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L1 measurement. (Apple, Huawei, CT)

  - Option 2: Support intra-frequency measurement and inter-frequency measurement and deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) (OPPO)

  - Option 3: Measurement within active BWP, Measurements outside active BWP (Ericsson)

  - Option 4: Start from defining requirements for CSI-RS based L1-RSRP intra-frequency scenario. Not to define the intra-frequency CSI-RS based L1-RSRP measurements with GAP. (Nokia)

  - Option 5: Support both intra-frequency and inter-frequency CSI-RS measurement. (Samsung)

  - Option 6: Not to consider scenarios where a UE needs measurement gap for CSI-RS measurements from LTM candidate cell. (Qualcomm)

- Recommended WF:

  - TBD

MTK/QC: what is the benefit of L1 measurement with gap? Given that SSB based L1 measurement with gap is already supported.

HW: flexibility can be increased.

CMCC: they are separate features. NW may only configure CSI-RS based L1 measurement without configuring SSB based L1 measurement.

CSI-RS based L1 measurement with gap is NOT considered

- Support: QC, MTK, OPPO, vivo, Xiaomi

CSI-RS based L1 measurement with gap is considered

- Support: Samsung, Apple, CMCC, HW, E///, ZTE, CATT

Agreements:

- CSI-RS based L1 measurement without gap is supported.

- FFS on CSI-RS based L1 measurement with gap

# Topic #4: conditional LTM

## General aspects on Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#112bis < Agreement > - Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell - Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.

- Candidate solutions:

- Proposal 1: In the agreed applicable scenario of “PCell switch to an LTM candidate cell that is a serving SCell in MCG”, clarify that if CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell. (Apple)

- Recommended WF

  - CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell.

Agreement:

  - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells.

  - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

## Early synchronization and TCI state activation

Issue 1-2-1: Early UL Synchronization

RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.

- Candidate solutions:

- Proposal 1: Define PDCCH ordered RACH related requirements for conditional intra-CU LTM. (CTC, CATT, E///, ZTE, Nokia)

  - Proposal 1a: R18 LTM PDCCH ordered RACH could be baseline. (CTC, CATT, ZTE)

  - Proposal 1b: Wait for more progress from other WGs. (CATT, Nokia)

- Proposal 2: Define RRM requirements for UE based TA measurement. (CTC, CATT, CMCC)

  - Nokia: Discuss whether to define requirements for UE based TA estimation in LTM and CLTM or not.

- Proposal 3: Define RRM requirements for DL TODA-based UE autonomous TA estimation. FFS on the details and side conditions. (QC)

- Recommended WF

  - Define PDCCH ordered RACH related requirements for conditional intra-CU LTM.

  - R18 LTM PDCCH ordered RACH could be baseline.

  - Define RRM requirements for UE based TA measurement.

Discussion:

    - Define RRM requirements for UE based TA measurement.

Nokia: does it only apply for CLTM?

CTC: it only applies CLTM.

CMCC: we are open for further discussion.

CMCC/vivo: we support to define requirements for UE based TA measurement.

Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT)

Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

## Conditional LTM delay

Issue 1-3-3: The ending point of conditional LTM delay

- Candidate solutions:

- Proposal 1: The ending point of conditional LTM is the time the UE transmits the first UL message on the target cell. (CTC, CMCC, Apple, Samsung, Huawei, Nokia, E///)

- Proposal 1a: (vivo, CATT, ZTE)

  - for RACH-based, it is the time the UE acquires the first available PRACH occasion transmission on the target cell

  - for RACH-less based, it is the time the UE transmits the first uplink transmission on the target cell

- Recommended WF

  - The ending point of conditional LTM is the time the UE transmits the first UL message on the target cell.

Agreement:

The ending point of conditional LTM is the time the UE transmits the first UL transmission on the target cell.