# 3GPP TSG-RAN WG4 Meeting #113	R4-2420084

# Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.22.3

Source:	Ericsson

Title:	Ad-hoc minutes for Netw_Energy_NR_enh WI

Document for:	Approval

# Introduction

This is the ad-hoc minutes for Netw_Energy_NR_enh WI.

##### Topic: [113][224] Netw_Energy_NR_enh_Part1

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

- Proposals

  - Option 1-1: CMCC, Qualcomm, Mediatek, CATT, Samsung, OPPO, vivo, Huawei

    - The same approach in Case 1 can be reused(hybrid fast and slow mode measurement).

      - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell).

  - Option 1-2: Apple

    - The same approach in Case 1 can be reused if the serving cell is unknown.

    - UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

  - Option 2: CATT, China Telecom, ZTE

    - UE follows OD-SSB periodicity ignoring the DRX and measCycleSCell configuration.

  - Option 3: LG, Ericsson

    - UE to combine AO-SSB and OD-SSB at least for the same position within SSB burst or same SSB index(same spatial relation).

  - Option 4: Apple, Huawei

    - UE follows legacy requirements based on measCycleSCell

  - Option 5: Intel

    - The configured measurement cycle dedicated for OD-SSB.

  - Option 6: Nokia

    - UE assumes OD-SSB overrides the regular SSBs.

    - RAN4 needs to discuss how to define the measurement requirement when OD-SSB is activated.

  - Option 7: Xiaomi

    - base on a single SSB period cycle instead of hybrid ones.

- Recommended WF

  - From moderator’s understanding, this issue only focuses on the scenario 1: OD-SSB and always-on SSB are with same frequency, same beam, same/different offsets and different periodicities. Whether and how to apply to other scenarios can be discussed after RAN1’s LS response.

  - Information from RAN1 #119 meeting:

[HIGH] Proposal #2-1 (Q1): Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. RAN1 is discussing whether to introduce other periodicity value(s) for on-demand SSB. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress.

  - Moderator suggests starting the discussion based on the following options:

    - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

      - Option 1-1: The measurement periodicity is OD-SSB periodicity

      - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB

    - Option 2: UE always follow legacy requirements based on measCycleSCell

    - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

[Comments]

Apple: Option 3 is compromise. If the target SCell is known, UE can follow legacy.

OPPO: Option 1. UE shall follow fast measurement based on OD-SSB within window.

CATT: Option 1. Unified approach. Option 3 needs clarification.

Nokia: The assumption is ‘OD-SSB and always-on SSB are with same frequency, same beam, same/different offsets and different periodicities’. Need time to check.

Xiaomi: Option 1. Question 1-2: Last meeting our agreement is not combined. Companies for issue 1-2 need to clarify.

ZTE: Option 1. It’s already a compromise. Option 3 needs the clarification about the known unknown definition.

Ericsson: The intention of equal periodicity is to combine OD-SSB and AO-SSB.

QC: Last meeting, we discussed the slow mode definition. Same understanding as apple to support option 3.

Vivo: Option 1 and 3.

CTC: Option 1.

Huawei: Option 1. Open to issue 1-1 and 1-2, depends on RAN1. Option 3 needs the clarification. NW doesn’t know that.

Apple: OD-SSB intention is to fast SCell activation. If UE already meets known SCell activation requirement, why need to measure OD-SSB fast? Definition of legacy known condition can be reused.

Ericsson: It’s a compromise in Option 1.

CMCC: Option 1. If NW knows it’s a known cell based on meas. reporting, NW doesn’t need to trigger OD-SSB.

OPPO: Not convinced based on Apple comments.

ZTE: What’s the check point?

Huawei: The intention is to avoid frequent meas. The issue is similar as Case 1.

Nokia: There is a validity time period. NW needs to understand UE’s behaviour.

Apple, QC: Why UE has still performed fast meas. after sending the meas. report

Intel: Even it’s known, the meas. period delay is still too long.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

- Background

  - The deactivated SCell measurement requirement in FR1 is as follow.

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |

  - The RAN4 agreement in last meeting is as follow.

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

  - The RAN1 agreement about OD-SSB deactivation is as follow.

RAN1 #118bis Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

- Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

  - Option 1: CMCC, MediaTek, CATT, Nokia, China Telecom, Samsung, ZTE, Apple

    - After the time window, UE fallbacks to legacy requirement based on measCycleSCell until OD-SSB deactivated.

  - Option 2: Qualcomm, LG

    - UE stops the measurement after the measurement time window.

  - Option 3: Apple

    - UE stops the measurement until the cell is becoming unknown

  - Option 4: Xiaomi

    - The measurement cycle which shall be followed by UE during the Tslow is

      - min⁡(Tc2,  measCycleScell)  if

        - only single MAC CE to update OD-SSB received and

        - measCycleSCell indicated

      - min⁡(Tc2,  [160ms] )  if

        - only single MAC CE to update OD-SSB received and

        - measCycleSCell absent

      - min⁡(max⁡(Tc2,Tc3, ….),  measCycleScell)  if

        - multiple single MAC CE to update OD-SSB received and

        - measCycleSCell indicated

      - minmax⁡(Tc2,Tc3, ….,[160ms])  if

        - multiple single MAC CE to update OD-SSB received and

        - measCycleSCell absent

    - The time window in which UE shall perform the slow measurements can be defined as the duration between t1 and t3 (denoted as[t2,t3]). Where in,

      - t2 is time point when receiving MAC CE to update OD-SSB periodicity parameter

      - t3 is time point when NW deactivated OD-SSB

  - Option 5: LG

    - No requirement is defined for slow mode measurement

  - Option 6: OPPO

    - For both Scenario 2 and Scenario 2A, once OD-SSB transmission is triggered, the OD-SSB is assumed to be not deactivated until the SCell activation is completed.

    - UE stops the measurement after the measurement time window.

- Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

  - Option 1: Qualcomm

    - To avoid that the frequency of OD-SSB based measurements increases, the measurement time window should have a periodicity of 5*max(measCycleSCell, DRX cycle).

    - All other always-on SSB or OD-SSB in between two measurement time windows can be ignored by the UE for L3 measurements in deactivated SCell.

  - Option 2: Huawei

    - In order to have clear understanding from NW perspective on how to configure/trigger OD-SSB for deactivated SCell, RAN4 to specify following two conditions for the interval between activated OD-SSB for deactivated SCell measurement:

      - The expected minimum interval between activated OD-SSB for deactivated SCell measurement, which can take exiting measurement period when MeasCycle =160 ms as reference. UE is not required to perform “fast measurement” more frequently than the minimum interval.

      - The expected maximum interval between activated OD-SSB for deactivated SCell measurement, when UE can perform measurement without Cell detection.

  - Option 3: LG

    - If the time is shorter than a threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition

  - Option 4: Ericsson

    - UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter.

- Sub-issue 3 – The length of time window

  - Option 1: CMCC, Qualcomm, Nokia, Apple, Huawei

    - based on the number of OD-SSB samples.

      - Option 1-1(CMCC, Apple): min(m+3Nslotsubframe,μ+1 +T_meas_process + Tidentify_intra_with_index, TOD-SSB_activation)

        - TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication.

      - Option 1-2(Nokia): Tidentify_intra_without_index or Tidentify_intra_with_index

      - Option 1-3(Qualcomm): 5*max(measCycleSCell, DRX cycle)

      - Option 1-4(Huawei): UE shall finish at least one complete measurement

  - Option 2: CATT, Samsung, Ericsson, ZTE

    - based on UE firstly reports the target OD-SSB based deactivated SCell measurement results after receiving the OD-SSB indication.

  - Option 3: LG

- based on the number of OD-SSB samples and UE provided valid Y reports.

- Twindow = Tprocess + Tactual_measure

  - Tprocess = Tmin +Tpossible_SSB

    - where, Tprocess is the processing time, Tmin is OD-SSB activation processing time, and Tpossible_SSB is the time of actual transmitted OD-SSB after OD-SSB activation processing

  - Tacutal_measure = X*TOD_SSB

    - where, Tacutal_measure is actually measured time, X is the number of OD-SSB period, and TOD_SSB is the period of OD-SSB

  - Option 4: OPPO

    - The ending point is defined at the earlier one between SCell activation command reception and 5s.

  - Option 5: Xiaomi

    - from the SCell activation command received to the first MAC CE to update OD-SSB

  - Option 5: Xiaomi

    - max⁡(5*Tc1, [t0,t1] ).

      - t1 is the time point of SCell activation command

      - t2 is the time point of first MAC CE to update OD-SSB after UE receiving Scell activation command,

      - Tc1 is the OD-SSB periodicity

- Sub-issue 4 – how to configure (based on OD-SSB deactivation command) or pre-defined the time window?

  - Option 1: CMCC, Qualcomm, Nokia, CATT, Ericsson, vivo, Huawei

    - The time window is predefined.

  - Option 2: LG

    - In case of explicit OD-SSB deactivation indication, the expiration time or the duration of the time window is configured regardless of OD-SSB deactivation indication

    - In case of implicit OD-SSB deactivation indication, the expiration time or the duration of the time window depends on whether signaling of implicit indication is cell-specific or UE-specific.

  - Option 3: ZTE

    - When the OD-SSB deactivation command received earlier than the ending point of the time window, UE would follow the OD-SSB deactivation command to terminate the measurement.

  - Option 4: OPPO

    - OD-SSB is not allowed to be deactivated during the time window.

  - Option 5: Samsung, OPPO

    - RAN4 defer the discussion until RAN1 achieves the agreements on OD-SSB deactivation.

- Recommended WF

  - From moderator’s understanding, most proposals are not contradicted to each other but based on different angles.

  - Following a unified approach to consider all perspectives, let’s try to discuss the issue based on the following deactivated SCell measurement procedures with the following sub-issues.

    - Note: The purpose of the figure is only for discussion. The figure doesn’t imply any requirement, implementation, delay for the spec.

- Sub-issue 1: How to perform measurement after time window(stage 3) within one time OD-SSB activation/deactivation(reconfiguration)?

[Moderator’s suggestion] Option 1 and 2 are not contradicted.

Option 1 means during Stage 3(in the figure), UE should meet legacy deactivated SCell measurement, but UE is allowed to choose any occasions to perform measurement.

Option 2 focuses on the UE behaviours in Stage 2, it’s a specific UE implementation for option 1 in which UE stops measurement until the last 5 samples within the measCycleSCell coming.

Based on most companies’ proposals, moderator suggests starting the discussion based on the following proposal:

  - During each OD-SSB activation time duration, after the time window, the UE fallbacks to legacy deactivated SCell requirement based on measCycleSCell until OD-SSB is deactivated/reconfiguration.

[Comments]

OPPO: OD-SSB deactivation is not valid. UE expects SCell activation command soon when UE received OD-SSB.

Nokia: When OD-SSB indication transmit is up to NW. UE shall follow the legacy requirement. UE can choose any sample to measure.

Huawei: OD-SSB always transmission is one use case. NW may check UE time to time based on multiple times OD-SSB trigger.

Xiaomi: One shot OD-SSB is typical case from NW for power saving. It will be harm if no OD-SSB deactivation.

Ericsson: UE should fallback to legacy measurement after fast meas.

ZTE: The intention of OD-SSB is for NW power saving. If UE reports good quality, NW can deactivate the OD-SSB. Otherwise, NW needs to keep OD-SSB transmission.

Intel: OD-SSB is per cell. What’s the difference to legacy behaviour if OD-SSB is transmitted?

LG:

QC: There is no deactivation solution in RAN1. We don’t want to tighten the requirement and consider power saving.

Apple: Different assumptions.

Ericsson: UE should not stop measurement.

Intel: What’s the meas. cycle of OD-SSB here?

Apple: Legacy measCycleSCell IE.

Agreement:

After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism.

- Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

[Moderator’s suggestion] From moderators’ understanding, the issue related to the time relation between multiple times OD-SSB activation/deactivation.

Based on most companies’ proposals, moderator suggests starting the discussion based on the following proposal:

  - T2 which is the time interval between two times activated OD-SSBs can be defined based on N1*max(measCycleSCell, DRX cycle) to consider the following aspects:

    - The expected minimum interval for T2 when UE cannot restart the fast mode window.

    - The expected maximum interval for T2 to avoid cell identification and keep the measured SCell as known.

[Comments]

OPPO: Skip the issue. No need to define T2. Continue transmit the OD-SSB.

Apple: Postpone.

- Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined?

[Moderator’s suggestion] From moderator’s understanding, most companies suggest pre-defined the time window as in sub-issue 3. However, the relation between time window and OD-SSB deactivation command should also be considered.

Moderator suggests starting the discussion based on the following options:

Option 1:  The time window is pre-defined based on the number of OD-SSB samples.

    - When the OD-SSB deactivation command received earlier than the ending point of the time window, UE would follow the OD-SSB deactivation command to terminate the measurement.

  - The detail time window duration is as follow.

    - min(T_min +T_processing+T_uncertainy+ Tidentify_intra_with_index, TOD-SSB_activation), if UE needs to perform the Scell identification

    - min(T_min +T_processing+T_uncertainy + Tidentify_intra_without_index, TOD-SSB_activation), otherwise.

    - TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation/reconfiguration indication.

    - T_min is defined in RAN1 based on RAN4 reply LS.

    - T_processing is the UE processing time. The detail value is FFS

    - FFS: T_uncertainy is the the time to the end of the first complete OD-SSB burst

  - The condition to skip the Scell identification is FFS.

Option 2: The ending point of the time window is defined as the moment the UE first reports the target OD-SSB measurement results for deactivated SCell after receiving the OD-SSB indication.

[Comments]

LG: Merge option 1 and option 2. based on the number of OD-SSB samples and UE provided valid Y reports.

vivo: Concern on option 2. The side condition is poor, and UE had to continue measurement till the condition is met.

Apple: Same concern.

MTK: We can have some margin other than go to option 2.

Xiaomi: We need to combine option 1 and option 2.

QC: The timing info. Can come from PCell/reference cell. Skip cell identification.

Ericsson: Option 1 and 2 are similar. The margin is needed.

Huawei:

Nokia:

Agreement:

The time window is based on the number of OD-SSB samples and UE provided one valid report.

- The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy

- UE should have enough number of OD-SSB samples to achieve the measurement requirement.

- The number of OD-SSB is FFS.

- FFS whether UE measurement reporting is needed

##### Topic: [113][225] Netw_Energy_NR_enh_Part2

Issue 1-2-1: L1 requirements at transition

Further discuss based on Alt 1, 3, 5:

- Alt 1: evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. (Apple, CATT, vivo, QC, ZTE, Nokia, LG, Huawei, Xiaomi, MTK, CMCC, OPPO, CTC)

- Alt 3: evaluation/measurement period is no less than the minimum of the period corresponding to the SSB before and after adaptation. (CMCC, OPPO)

- Alt 5: evaluation/measurement period is defined assuming UE combines SSB before and after SSB adaptation with different periodicities(Ericsson)

[Comments]

Ericsson: We can compromise to Alt. 1.

Agreement:

L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.