3GPP TSG-RAN WG4 Meeting # 113	             R4-2420104

# Orlando , US, Nov 18 – 22, 2024

Agenda Item:	7.26.5

Source: 	Moderator (CATT)

Title: 	WF on RRM requirements for NR_NTN_Ph3_Part1

Document for:	Approval

0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

- < Tentative Agreement > will be changed to < Agreement > if no comments are received.

1 Topic #1: General consideration on RRM requirements

    - Sub-topic 1-1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN?

- Background

The related agreement is copied as follows:

RAN4#112: - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - Inter-RAT measurements in idle/inactive/connected mode RAN4#112-bis: Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN Online agreement <Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Since companies had different understandings for the agreement reached in RAN4#112 meeting, and some companies think that inter-RAT (NTN to E-UTRA TN) measurement in idle/inactive are not only defined for (e)RedCap with TN but also defined for normal UE with NTN (TS 38.133 clause 4.2C.2.11), so this issue need to be further revisited.

This issue only about whether to define the related requirements and how to define them for 1Rx/2Rx will be discussed in issue 2-1-1.

<Way Forward>:

  - Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE):

    - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

  - Proposal 2 (vivo):

    - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Issue 1-1-2: The Requirements to be further determined

<Way Forward>:

  - For SDT requirements in RRC_INACTIVE state,

    - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements.

      - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN.

      - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission.

      - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

      - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable)

    - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

  - For MDT requirements in RRC_IDLE and RRC_INACTIVE state

    - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK):

      - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system.

      - Proposal 1a (CATT): Deprioritize to define.

    - Proposal 2 (CMCC):

      - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec.

    - Proposal 3 (ZTE):

      - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

    - Sub-topic 1-2 Features to be considered for (e)RedCap UE with FR1-NTN

Issue 1-2-1: Relaxed measurement

Online agreement

< Agreement >:

- For supported types of relaxations for (e)RedCap UE with FR1-NTN,

  - Only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection.

- RAN4 to further discuss the thresholds and specific relaxation factor.

Issue 1-2-2: eDRX configuration

Online agreement

< Agreement >:

- For earth-moving LEO scenario:

  - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario.

- For quasi-earth fixed LEO scenario:

  - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN.

    - Requirements are applicable for 2.56s and 5.12s eDRX cycle

    - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

Issue 1-2-3: Concurrent gap

Online agreement

< Agreement >:

- Deprioritize parallel measurement gap for (e)RedCap UEs in NTN.

- Come back to this issue at Apr 2025 meeting.

Issue 1-2-4: Multiple SMTCs

- Background

R17 NTN UE capabilities related to 'in parallel' are copied as follows:

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_NTN_solutions | 25-1 | Parallel measurements on multiple SMTC-s for a single frequency carrier | Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier |  | parallelSMTC-r17 | MeasAndMobParametersCommon | FDD only | FR1 only | UE is mandatory to support 2 and can optionally support 4 if the feature is supported | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-2 | Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell | Support of measurements on cells belonging to different satellite as the serving cell in parallel with normal operation (i.e. data/control transmission and/or reception, and L1 measurements) of serving cell without scheduling restrictions. The feature is applicable only when the serving satellite is NGSO. If the serving cell belongs to GSO satellite, the scheduling restriction is not applied on the premise that a mixed type of satellites on the same frequency layer is not supported in this release (Rel-17). |  | parallelMeasurementWithoutRestriction-r17 | BandNR | FDD only | FR1 only | For UEs not able to perform measurements in parallel with normal operation of serving cell scheduling restrictions shall apply. | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-3 | Parallel measurements with multiple measurement gaps | Support of 2 measurement gaps |  | parallelMeasurementGap-r17 | MeasAndMobParametersCommon | FDD only | FR1 only | UE is mandatory to support 1 measurement gaps | Optional with capability signaling |
| 25. NR_NTN_solutions | 25-5 | Parallel measurements on multiple NGSO satellites within a SMTC | Support of simultaneously measurements on target cells belonging to different NGSO satellites within a SMTC |  | maxNumber-NGSO-SatellitesWithinOneSMTC-r17 | BandNR | FDD only | FR1 only | Candidate values for the number of NGSO satellites are 1,2,3, or 4 |  |

Online agreement

<Agreements>:

- RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs.

- For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs?

  - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support

  - Option 2 (HW, Nokia): FFS

- Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs.

- Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period.

- Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

Issue 1-2-5: The number of searchers and CSSF

<Way Forward>:

- Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Issue 1-2-6: FH in PRS measurement for NW verified location requirements

<Way Forward>:

- Not to consider FH in the PRS measurement requirements for NW verified location for (e)RedCap UE with FR1-NTN.

Issue 1-2-7: NCD-SSB

<Way Forward>:

- For the terminology of SSB used in the spec,

  - Proposal 1 (CATT, Ericsson, HW, CMCC): Use the terminology of SSB without differentiating CD-SSB/NCD-SSB.

- For whether to add additional requirement applicability about CD-SSB/NCD-SSB,

  - Option 1 (CATT, Ericsson, HW): Add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

    - The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

  - Option 2 (CMCC): No need to add additional requirement applicability about CD-SSB/NCD-SSB.

Issue 1-2-8: Other features

<Way Forward>:

  - Proposal 1 (CMCC): For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed.

    - Set 1: propose to be optionally supported by Redcap over NTN

      - SMTC adjustment in idle/inactive

      - Location-based measurement report trigger

      - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

      - The number of target LEO satellites the UE can monitor per carrier including serving satellite

- Further discuss the max number

    - Set 2: FFS

      - Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell

      - Parallel measurements on multiple NGSO satellites within a SMTC

2 Topic #2: Reduction in the number of UE Rx branches

    - Sub-topic 2-1 Mobility requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

1. RRC_IDLE/RRC_INACTIVE mode mobility

Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands,

    - RAN4 to reuse the legacy mobility requirements in RRC_IDLE / RRC_INACTIVE state defined for normal UEs with FR1-NTN in high level.

      - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - RAN4 to reuse the same offset of 1Rx RedCap UEs in TN compared to 2Rx (e)RedCap UEs.

- For Qrxlevmin and Qqualmin: -1 dB offset can be reused.

- For the measurements of intra- and inter-frequency NR cells,

- +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline.

- For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked.

- In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria.

- For 1 Rx RedCap by a margin of at least:

- 6 dB in FR1 for reselections based on ranking or

- 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

- 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

    - RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring.

      - 6 NR inter-frequency carriers can be reused.

    - The parameters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)RedCap UE, and the requirements for both will be the same.

    - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

  - Define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands.

    - FFS: How to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands.

  - FFS: RAN4 to further discuss how to define the cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx and 2 Rx (e)RedCap UE after issue1-1-1.

2. RRC_CONNECTED mode mobility

2.1 Handover

Issue 2-1-2: RACH-based HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For NR RACH-based HO requirements,

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

      - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

- Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

Issue 2-1-3: RACH-less HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For NR RACH-less HO requirements,

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the legacy FR1-NTN RACH-less based HO requirements for normal UEs.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-less based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

      - The relaxations of Tsearch is the same as the relaxations defined for RACH-based HO For 1Rx (e)RedCap UEs with FR1-NTN bands.

Issue 2-1-4: Time/location-based CHO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

- For time/location-based CHO without L3 measurement,

  - RAN4 to reuse the legacy FR1-NTN Time/location-based CHO without L3 measurement defined requirements for normal UEs for 2Rx (e)RedCap UE with FR1-NTN.

  - Reuse the same relaxation on Tsearch of RACH-based HO requirements for 1Rx (e)RedCap UE with FR1-NTN compared to 2Rx (e)RedCap UEs.

- For time/location-based CHO with L3 measurement,

  - RAN4 to reuse legacy FR1-NTN Time/location-based CHO with L3 measurement requirements defined for normal UEs with the following adaptation:

    - For Tmeasure, i.e., Tidentify_intra/inter_with_index or Tidentify_intra/inter _without_index defined in Section 9 for 1Rx/2Rx (e)RedCap UE with FR1-NTN can be reused.

Issue 2-1-5: Satellite switching with re-sync

Online agreement

< Agreement >:

  - For Satellite switching with re-sync requirements,

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy Satellite switching with re-sync requirements for normal UEs (including both hard satellite switch with re-sync and soft satellite switch with re-sync).

    - For hard Satellite switching with re-sync requirements, for 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

      - Define the relaxation on Tsearch by extending one more [SSB or SMTC] periodicity on the top of Tfirst_SSB:

- Tsearch = Tfirst_SSB + [TSSB or TRS] ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

    - FFS: whether to define soft Satellite switching with re-sync requirements for 1Rx (e)RedCap UEs with FR1-NTN bands.

Issue 2-1-6: RRC Re-establishment requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs in high level.

    - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

      - Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by [1] more samples for FR1 known and unknown intra/inter-frequency cell. (In TN system, [] are still in the spec)

Issue 2-1-7: RRC Connection Release with redirection requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs in high level.

    - Depend on the supported UE capabilities, the existing NR NTN requirements may need be revised.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

      - Relax the Tidentify-NR by 1 more samples for FR1.

Issue 2-1-8: RA requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the legacy random access requirements defined for normal UEs with FR1-NTN.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds for random access requirements

    - +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

    - -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

    - Sub-topic 2-2 Timing for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-2-1: Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - The existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands.

    - UE transmit timing

    - UE timer accuracy

    - Timing Advance

  - The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

    - Sub-topic 2-3 Signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-3-1: RLM/BFD/CBD requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs in high level.

    - Depend on the supported UE capabilities, the existing NR NTN requirements may need be revised.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows:

      - Aggregation level (CCE) of 16.

      - Bandwidth (PRBs) of 48.

    - For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

    - The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

      - For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Issue 2-3-2: Other requirements in signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Online agreement

< Agreement >:

  - The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

    - Active BWP switch delay,

    - active TCI state switch delay

    - UE specific CBW change requirements

    - Pathloss reference signal switching delay

  - The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

    - Sub-topic 2-4 Measurement Procedure for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-4-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands,

    - Proposal 1 (CATT, Samsung, CMCC, HW): RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

      - Depend on the supported UE capabilities (e.g., multiple SMTC),, the existing NR NTN requirements may need be revised.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - Proposal 1 (CATT, Samsung, CMCC, ZTE, HW): The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

      - Proposal 1a (QC): Extended delays for PSS/SSS detection.

      - Proposal 1b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples.

    - Proposal 2 (CATT, [Samsung?], CMCC, HW): The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

      - Proposal 2a (QC): Extended delays for SSB index identification (PBCH decoding).

      - Proposal 2b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples.

    - Proposal 3 (CATT, CMCC, HW): The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

    - Proposal 4 (CATT, CMCC, ZTE, HW): The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

    - Proposal 5 (CATT, CMCC, HW): The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Issue 2-4-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

  - Proposal 1 (vivo, [QC?], HW):

    - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused.

    - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred to, and the same relaxation for accuracy should be reused.

    - Sub-topic 2-5 L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Issue 2-5-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

  - For 2Rx (e)RedCap UEs with FR1-NTN bands,

    - Proposal 1 (CATT, Samsung, CMCC, ZTE): RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands,

    - Proposal 1 (CATT, Xiaomi, Samsung, [QC?], CMCC, ZTE, [OPPO?], HW): RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

      - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

3 Topic #3: The impact of HD-FDD

    - Sub-topic 3-1 General HD-FDD related impact

Issue 3-1-1: General HD-FDD related impact

<Way Forward>:

  - Proposal 1 (CATT, Xiaomi, Samsung, QC, CMCC, ZTE, Ericsson, HW):

    - The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

    - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

    - Proposal 1a (CATT, HW, MTK):

      - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

    - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects:

      - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions.

    - Proposal 1c (CMCC):

      - Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

      - Wait for further RAN1 progress which may impact Scheduling availability requirement.

    - Sub-topic 3-2 The specific impact of HD-FDD on NR Handover

Issue 3-2-1: The impact on NR Handover due to HD-FDD

<Way Forward>:

  - Proposal 1 (CMCC):

    - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

    - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

- One SSB is available during T∆

- One SSB is available during TIU

  - Proposal 2 (Ericsson):

    - RAN4 to reuse NR SAN FR1 – NR SAN FR1 Handover and NR Satellite switching with re-synchronization requirements for Red-cap NTN and additionally add the following applicability rule for HD-FDD case,

      - For RedCap NTN UE with HD-FDD, the handover requirements and Satellite switching with re-synchronization requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

  - Proposal 3 (HW):

    - For HD-FDD (e)RedCap UE with FR1-NTN bands, the CHO and satellite switching with re-sync requirements are met provided that

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

  - Proposal 4 (Nokia):

    - Only support hard satellite switching for RedCap UEs

4 Topic #4: Others

    - Sub-topic 4-1 Specification structure

Issue 4-1-1: Specification structure

<Way Forward>:

- Proposal 1 (CATT, Ericsson, HW):

  - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands.

  - Utilize the reference method for the requirements that can be reused.

- Proposal 2 (Nokia):

  - Adopt RedCap subclauses under NTN sections (under Suffix C).

    - Sub-topic 4-2 UE capabilities for (e)RedCap UE with FR1-NTN bands

Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1 (Nokia): RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

  - Proposal 1a (CMCC): Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

  - Proposal 1b (Ericsson): RAN4 to define a new capability declaring (e) RedCap for NTN.