3GPP TSG-RAN WG4 Meeting # 113													R4-2420114

Orlando, USA, 18/11/2024 to 22/11/2024

Agenda item:			7.25.3

Source:	Moderator (Nokia)

Title:	Ad hoc minutes for [113][229] NR_XR_Ph3

Document for:	Information

# Ad hoc minutes

Issue 1-1: Deployment scenarios

- Background information:

  - In RAN4 #112 bis the following agreement regarding deployment scenarios was reached R4-2416864:

< Agreement>: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation <Way forward>: - Further discussion needed for - FR2-2 - NR-DC, EN-DC, NE-DC

- Proposals related to frequency range

  - Option 1 (Huawei, Mediatek): Deprioritize FR2-2 scenario for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

  - Option 2 (Nokia, Ericsson): FR2-2 is not considered in this WI.

- Proposals related to dual connectivity

  - Option 1 (Ericsson): EN-DC and NE-DC are not considered in this WI, based on WID.

  - Option 2 (Interdigital, Mediatek): Deprioritize discussions for NR-DC, EN-DC and NE-DC.

  - Option 3 (Nokia): RAN4 to not consider dual connectivity deployment scenarios for XR enhancements.

  - Option 4 (ZTE): NR-DC, EN-DC and NE-DC are applicable for measurement skipping.

  - Option 5 (vivo): Send an LS to RAN2 and RAN1 to ask if DC scenarios are supported for gap cancellation from their perspective.

- Recommended WF

  - FR2-2

    - Option 1 – Deprioritize FR2-2 discussion

    - Option 2 – FR2-2 is NOT considered for measurement gap skipping

  - NR-NR Dual connectivity

    - Option 1 - Is NOT considered for measurement gap skipping

    - Option 2 - Is considered for measurement gap skipping

  - EN-DC and NE-DC

    - Option 1 - Is NOT considered for measurement gap skipping

    - Option 2 - Is considered for measurement gap skipping

Notes from Tuesday 19th November

- Companies views on EN-DC and NE-DC

  - Moderator: EN-DC and NE-DC for DCI-based solution

    - Option 1 (Interdigital, ZTE, Apple, Ericsson, Nokia) - Is NOT supported for measurement gap skipping by the WID

    - Option 2 (Qualcom)- Is considered for measurement gap skipping

    - Option 3 (vivo)– Ask RAN1/2 if supported

  - Interdigital – DC or ENDC involves 2 schedulers. DCI skipping could collide in 2 schedulers. We should at least deprioritize for now. Since scheduler should be priority.

  - Vivo – Those DC cases are complicated. We need clarity whether RAN1/RAN2 is supporting it.

  - Apple – DC can only be supported if RRC solution is supported. For smie static add this proposal

    - Option 4 (Apple) If only DCI based solution is supported, we should not support DC. If RRC based soluition is supported, DC can be supported

  - ZTE – For all DC scenarios we cannot see additional workload. If follow dynamic scheduling indication, it is hard to work.

  - Qualcomm – Don’t need to exclude anything at this point. If same solution applies for all scenarios we don’t need to artificially exclude the scenarios. We should first see if there is additional impact from RAN4 perspective.

  - Nokia – From WID it is not included EN-DC nor NE-DC. It has NR-DC. We see additional impact in RAN4 with those DC scenarios, including types of measurements. We have not considered DC in Rel 1718 MG work items. So DC should be deprioritized altogether. RAN4 should start with assumption without DC considered.

  - Ericsson: EN DC and NE DC are clearly not supported by the WID.

  - Interdicital -

  - Vivo –

  - Tentative agreement: EN-DC and NE-DC for DCI-based solution

    - Is NOT supported for measurement gap skipping by the WID

Notes from Wednesday 20th November

- Companies views on EN-DC and NE-DC

  - Moderator: confirm if other companies not present on Tuesday are ok with tentative agreement.

- Companies views on NR-DC

  - Moderator: Try to quickly take comments regarding the NR-NR Dual connectivity

    - Option 1 (Apple: assuming EN-DC is out of scope) - Is NOT considered for measurement gap skipping

    - Option 2 - Is considered for measurement gap skipping

    - Option 3 (vivo, Ericsson) send LS to RAN1/RAN3 asking clarification

    - Option 4 (Qualcomm) Keep FFS.

  - Vivo: in general it is the same as EN-DC issue. I understand RAN3 supports it form the WID. But if information exchange between nodes is supported for NR-DC has to be considered

  - Ericsson: Since it is mentioned in the WID, other groups are considered XR rel 18 to be supported in NR-DC. So it is difficult to exclude from RAN4 scope. So confirmation fomr RAN1/RAN3

  - Apple: Different read in this extension. The RAN3 objective is about the Rel 18 XR features. We would like to handle in the same was as EN-DC and NE-DC.

  - Qualcomm: Considering WID extension for NR-DC it would be prematute to decide on this topic. We can keep open until there is more progress in RAN2/RAN3. Main goal is requirements first.

Agreements

  - FR2-2

    - FFS

  - NR-DC

    - FFS

  - EN-DC and NE-DC for DCI-based solution

    - Is NOT supported for measurement gap skipping by the WID

Issue 1-2: Types of measurements to be considered

- Proposals regarding which types of measurements are considered for XR enhancements

  - Proposal 1 (Apple, Interdigital, Huawei, Nokia, Mediatek, Ericsson):

    - Intra-frequency

  - Proposal 2 (Apple, Interdigital, Huawei, Nokia, Mediatek, Ericsson):

    - Inter-frequency

  - Proposal 3 (Apple, Interdigital, Huawei, Nokia, Mediatek, Ericsson):

    - LTE inter-RAT (NR-E-UTRAN FDD/TDD)

  - Proposal 4 (Interdigital, Huawei, Nokia, Mediatek, ZTE):

    - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

  - Proposal 5 (vivo):

    - UTRAN FDD measurements are supported for gap skipping

  - Proposal 6 (Ericsson):

    - If EN-DC scenario is supported in RAN4 XR requirements, then for UE in EN-DC, the XR-related requirements apply when the UE is not configured with the following inter-RAT measurements (can only be configured by E-UTRAN PCell):

      - E-UTRAN – UTRAN FDD/TDD,

      - E-UTRAN – GSM, GSM, cdma2000 1 x RTT and HRPD,

      - FFS: E-UTRAN – NR.

- Recommended WF

  - NR and LTE measurements

    - Tentative agreement

      - The following measurement types are considered for XR measurement skipping

        - Intra-frequency

        - Inter-frequency

        - LTE-inter- RAT (NR-EUTRAN FDD/TDD)

    - UTRAN and measurements for SRVCC

      - Option 1: XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

      - Option 2:  UTRAN FDD measurements are supported for gap skipping

Notes from Wednesday 20th November

  - Companies views on NR and LTE measurements

    - Moderator: proposal for tentative agreement

      - The following measurement types are considered for XR measurement skipping

        - Intra-frequency

        - Inter-frequency

        - LTE-inter- RAT (NR-EUTRAN FDD/TDD)

  - Companies views on UTRAN and measurements for SRVCC

    - Moderator: discus options for UTRAN and measurements for SRVCC

      - Option 1 (ZTE, Nokia): XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

      - Option 2:  UTRAN FDD measurements are supported for gap skipping

    - Qualcomm: This feature is for NR and currently we don’t have requirements for UTRA measurement in spec.

    - Interdigital: If Ue is configured to measure UTRAN where ther eis no XR service, why should we cancel those measurements?

    - Ericsson: We need to support also UTRA FDD specific measurements. We can discuss if this is an important scnerio if UE is performing those measurement during the XR operation. 2G is coupled to EN-DC

    - ZTE: We cant see obvious scenario for XR device to work under 2G 3g. To accelerate discussion, focus on most important case

    - Nokia: Same as ZTE. If look at SA scenario we have only interRAT for LTE. NO reason to support UTRAN or 2G. Otherwise we need to study exactly the aspects of how this service is conveyed to 3G. Good approach to reuse SA and connected more

    - Qualcomm: the network doesn’t know which gap is used for which type of measurement. So if UE is confired to measure 2G/3G

    - MTK: Option 1 means if the UE is configured with 3Gwe don’t have to follow the requirements. If the DCI is cancelling 3G we most likely will not skip the gap. Better not to configure from the beginning.

    - Nokia: Minimum requirements today are based on inter-RAT measurements for LTE, but not other technologies. Method would be not specify requirements if UE is configured for 2G/3G

    - Vivo: this is just for voice service. If this is urgent it should be prioritized. This is unclear why XR devices need voice services. Don’t see anything different compared ot other measurements. But if companies think this is a problem we can prioritize the UTRA FDD, meaning UE don’t need to skip this gap.

    - Interdigital: The sceneario is, there is XR service and voice call, so the UE is doing ahandover for voice continuity.

    - Qualcom: Try WF.

    - ZTE: This service is delay sensitive, so we ndont need to take too much effort for this scenario.

    - Moderator: Tentative agreement

      - Option 1 () The XR RAN4 requirements for gap skipping are not applicable when the UE is configured for measurements for SRVCC

      - Option 2 () - The UE is not required to fulfil the requirements of gap skipping indicated by DCI if the UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

      - Option 3 () - The scenario where the UE is required to measure SRVCC or 2G/3G for EN-DC/NE-DC and XR measurement skipping is not supported. The XR RAN4 requirements for gap skipping are not applicable when the UE is configured for measurements for UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

      - Option 4 () – the XR gap skipping mechanism is not applicable when the UE is configured for measurements for SRVCC

      - Option 5 (vivo, Ericsson, Interdigital, ) –

      - Tentative agreement: The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

Agreements

- The following measurement types are considered for XR measurement skipping

  - Intra-frequency

  - Inter-frequency

  - LTE-inter- RAT (NR-EUTRAN FDD/TDD)

Tentative agreement

- The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

Issue 4-2: Principles for defining time offset

- Proposals

  - Proposal 1 (Ericsson): RAN4 will define the minimum time offset between the end of the received DCI and the MG indicated by the DCI.

  - Proposal 2 (Ericsson): The minimum time offset can be different for FR1 and FR2.

  - Proposal 3 (Ericsson): The minimum time offset between the end of the PDCCH symbol carrying DCI and the first subframe of the target measurement gap includes:

    - DCI processing time,

    - measurement gap timing advance (MGTA) configuration,

    - FFS any other delays.

  - Proposal 4 (Ericsson): FFS: The minimum time offset accounts for UL timing adjustment for UL XR data within the target MG.

  - Proposal 5 (Ericsson): FFS: The minimum time offset accounts for a guard period for DL-to-UL switching for UL XR data within the target MG.

- Recommended WF

  - Discussion needed

Notes from Wednesday 20th November

- Companies view

  - Apple: As long as we know the time offset, the network can use this info regardless of MGTA being configured or not. UE and network know when the gap start.This is to provide guidance for the network.

  - Qualcomm: the exact is the point fomr DCI to when the gap starts. We don’t want separate formula taking into account different factors. Single value as in preMG should be enough

  - Nokia: Agree that delay is taking into account where the actual MG occasion starts. This is clear understanding between UE and netorkw.

  - Vivo: The definition of timing offset is clear enough. From DCI to the start of gap occasion to be skipped. This is already considering the MGTA.

  - Interdigital: the gap skiping indication will be in the DCI. The DCI is scheduling data

  - QC: the main concern is on the scheduler. Don’t need to go into discussion like in RAN1. This is more about the implementation, and re recalculation of all the gap occasions. All software scripts have to be rebuild.

  - Moderator: no agreement.

Issue 4-1: Time offset requirement for measurement gap skipping

- Proposals

  - Option 1 (Apple): 7 ms

  - Option 2 (Qualcomm, vivo): 5 ms

  - Option 3 (ZTE): Smaller than 5 ms

  - Option 4 (Nokia): 1ms

  - Option 5 (Interdigital): assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

  - Option 6 (Mediatek): If the XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, then the time offset requirements for MG skipping can be 5ms.

  - Option 7 (CMCC): N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window  (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214).

  - Option 8 (Huawei):

    - 5 ms if both NR frequency layers and LTE inter-RAT is to be measurement within the MG.

    - If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms.

- Recommended WF

  - Candidate options

    - Option 1: 5 ms

      - Option 1a: with some condition

      - Option 1b – without condition.

    - Option 2: less than 5 ms

      - Option 2a: 1 ms

      - Option 2b: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window  (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214).

      - Option 2c: assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

Notes from Tuesday 19h November

  - Ericssson: Is MGTA is included. Is it something that needs to be added

  - Apple – This is regardless of MGTA. This is only related to the start point of the gap occasions

  - Vivo – we can look at the definition of the time offset. We are talking about the exact gap here.

  - Nokia – we think we should investigate shorter offsets, from our simulations this is very benneficial. The case of N2 could also be considered.

  - Ericsson – we need to be clear if these numbers are equally applicable if the DCI is scheduling UL or DL. For UL there are more issues to consider like extra switching time.

  - ZTE – What the UE has to do during this period? Re-schedule between measurement and data. Replan sharing multiple Mos with this gap. Recompute the CSSF factor. But as long as UE can do the 1st step the UE can receive the data. So we believe processing can be less than 5 ms

Notes from Wednesday 20th November

  - Qualcomm: can remove conditions

  - Moderator: new candidate options:

    - Option 1: 5 ms

    - Option 2: less than 5 ms

      - Option 2a: 1 ms

      - Option 2b: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window  (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214).

      - Option 2c: assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

      - Option 2d: If only NR frequency layer is to be measurement within the MG, shorter time offset could be considered. E.g. 4 ms.

    - Option 3: UE capability

      - 5 ms

      - 1 ms?

      - N2?

  - Huawei: add option 2d

  - CMCC: reason for Option 2b, positions PRS processing window is introduced. RAN1 introduced capability where data has higher priority than PRS. So to support this case the UE has to decide what to do in this window. This is very similar to what we are discussing here.

  - Qualcomm: The procedure for dropping PRS and gaps are very different. One is for positioning, and MG is much more broadly used measurement that has broader repercussion. Lot more procedures from UE side. 2nd part. We use 5 ms for preMG because of the time the UE has to reprogram measurements. Huawei proposed option 2d, but I think we are already considering interRAT measurements. Does this measn

  - MTK: PRS is more similar to intra freq without gaps?

  - ZTE: Here the situation is not the same as Pre MG. In preMG during activation after the switch the UE has to perform measurement during the gap. But here the UE just need to cancel the gap. No need to recalculate CSSF and replan gap sharing between different MO. This is less than pre mG

  - Ericsson: If the gaps is skipped, the UE will have to understand that the legacy requirements do not apply anymore.

  - Vivo: Here we are only considering if the NW indicates to skip, we need time. If the DCI indicating the gap is used, than more time is used, in this case time for preparing measurement is needed

  - Nokia: We have 1 ms. We seen good performance benefits if the value is as low as possible. We can see that ther eis contrainst from UE side. But here it is only cancelling. So we should investigate smaller values. If we have smaller values we can do different UE capability for time offset.

    - Option 3: UE capability

      - 5 ms

      - 1 ms?

      - N2?

  - Qualcomm: Companies need to understand that UEs will not design different measurement engine only for XR. Are other UE vendors comfortable with those values

  - Mediatek: We discussed a lot on pre MG. Need to discuss values closer to preMG. We can have basic agreement of 5 ms, and other companies want other values needs to be discussed.

  - Apple: Even for preMG the delay is not 5 ms, because of the BWP siwthcing delay of 1, 2, 3 ms. For activation it is longer. 5 ms is already shorter than pre-MG.

  - OPPO: We are not in favour of UE capability. We need to consider wort case. 5 ms is compromise frm our side. For the preparation time of 1 ms is not feasible.

  - Ericsson: Is siwthicng time for UL also included in the time offset?

  - Qualcomm: we don’t need to consider UL switching time from the UE. It is including UL switthcing time.

  - Nokia: If we discuss such figures, we have 2 type sof BWP requirements. We defined also for other values having in mind that some UEs might support it. We may not need to go down to 1 ms, but we need better values.

  - Apple: why are we comparing the gap with BWP swithcing. The reason we mentioned pre-MG is because we have BWP activation as part of the preMG activation.

  - Vivo: do we attempt to reply the LS? We can use 5 ms ans FFS other options.

  - Qualcomm: anything less than 2 ms might be too hard. We can talk about other values

  - Moderator: UE capability for measurement skipping indication

      - Baseline feature 5 ms

      - Optional capability

        - Option 1: 1 ms

        - Option 2: 2 ms

        - Option 3 (Mediatek): 3 ms

  - Mediatek: Option 3 is something we can look in the future

  - Ericsson: We can reply to RAN1 that we are defining a time offset in the range of 2 ms to 5 ms.

  - Interdigital: we need to take into consideration what Ericsson said.

  - Nokia: To be clear, this is the measurement skipping offset, not measurement skipping cancellation.

  - Vivo: how this can help RAN1 in their work. IIf ther is different UEs the network have to handle different UEs. There is no benefit for the network of different capability. We can discuss if 5 ms can be reduced in certain conditions.

  - ZTE: This value is bennefiction, because data scheduling is per UE. Even if the UE has different capability the network can adapt the scheduler per UE.

  - Huawei: If we cannot agree on the values we should not send the options to RAN1. We share similar views as Mediatek. I am not sure if 1 ms and 2 ms will ever be possible. For network scheduler it needs to consider this UE capability.

  - Nokia: We agree with ZTE that it is benneficial to get different values. We agree that this can be challenging value, but we have to look into the future what could be possible. For now we can indicate that RAN4 has a baseline value of 5 ms and investigating the other values.

  - Qualcomm: we are ok to send the LS if 1 and 2ms are not considered.

  - CMCC: For the reply LS, we can inform RAN1 that 5 ms is baseline and RAN4 is discussing shorter value.

  - Moderator: UE capability for measurement skipping indication

      - Baseline feature 5 ms

      - Optional capability

        - Option 1: 1 ms

        - Option 2: 2 ms

        - Option 3 (Mediatek, QC, Apple, Huawei): 3 ms

  - Meditek: we can have a baseline agreement, and reply to RAN1 without mentioning the value.

  - Moderator: Tentative agreement

    - Baseline feature 5 ms

    - Optional capability

      - Option 1: 1 ms

      - Option 2: 2 ms

      - Option 3 (Mediatek, QC, Apple, Huawei): 3 ms

    - Send LS to RAN1 that Alt 1-1 is feasible without mentioning the Toffset.

  - QC, need to remove 1 ms from options.

  - Vivo: need to discuss with semi static

  - MTK: Tentative agreement

    - Baseline feature 5 ms

    - Optional capability 3 ms

  - NTT DOCOMO: Is this only for MG skipping, or also without gap skipping?

  - QC:  only to MG skipping. We haven’t agreed on without gaps

  - Moderator: Tentative agreement

    - Baseline feature 5 ms

    - Optional capability 3 ms

  - Ericsson: need to be clear about time offset definition. Including time switching for UL.

  - Apple: As long as the MG occasion is cancled, the network can schedule the UE as legacy.

  - Moderator: Tentative agreement

    - Baseline feature 5 ms

    - Optional capability 3 ms

    - Capture in the LS to RAN1

Agreement

- Tentative agreement

  - Baseline feature 5 ms

  - Optional capability 3 ms

  - Capture in the LS to RAN1

Issue 3-1: How to capture impact of measurement skipping

- Previous WF

<Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information.

- Proposals related to measurement accuracy requirements

  - Option 1 (Nokia, Interdigital, Ericsson, CMCC): Reuse existing measurement accuracy requirements.

  - Option 2 (Mediatek): RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

- Proposals related to measurement delay requirements:

  - Option 1: Proposals without distinction of XR and legacy requirements

    - Option 1a (Nokia): As minimum requirements, cell detection, index reading, and measurement period is extended.

    - Option 1b (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

    - Option 1c (Qualcomm): extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network.

    - Option 1d (Ericsson): RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

  - Option 2: Proposals with distinction of conditions where measurement impact is not needed

    - Option 2a (Interdigital):

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

    - Option 2b (CMCC):

      - Apply legacy measurement delay and accuracy requirements for the following condition:

        - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI

      - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

- Recommended WF

  - For measurement accuracy, please check if Option 1 is agreeable

  - For measurement delay discussion is needed.

Notes from Wednesday 20th November

- Companies view

Issue 5-1: General on UAI

- Proposals related to measurement occasions

  - Option 1: Define UAI related to measurement occasions

    - Option 1a (CMCC, Huawei, Qualcomm): number of occasions which can be skipped

    - Option 1b (CMCC): number of measurement occasions that are needed for measurement

    - Option 1c (Apple): UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 1d (Huawei, vivo): patterns of gaps positions of MG that can be skipped

    - Option 1e (vivo): study how UE assistance information/UE capability is reported when DRX is configured.

    - Option 1f (Qualcomm): Maximum number of consecutive MGs that can be skipped

    - Option 1g (Qualcomm): MG type that can be skipped

  - Option 2:

    - Option 2a (Nokia): Do not define UAI related to measurement occasions.

    - Option 2b (Mediatek): RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution

    - Option 2c (Ericsson): RAN4 will not specify any XR-related requirement, based on XR UAI.

      - Draft LS response in R4-2419707.

  - Option 3: The information about gaps not used for measurement are reported as UE capability.

  - Option 4 (Ericsson): The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions.

- Proposals related to channel conditions

  - Option 1 (Nokia): For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

- Recommended WF

  - Please discuss which option can be agreed

  - For conditions related to measurement occasions, consider if UAI is needed/feasible, and if it may be defined as UE capability as in Option 3.

Notes from Wednesday 20th November

- Companies view

Topic 6 Additional signalling aspects

Issue 6-1: Semi-static schemes

- Proposals

  - Option 1 (Interdigital, Nokia): Follow RAN1 working assumption and consider DCI only.

  - Option 2 (Apple, Huawei, Mediatek, Qualcomm, CMCC): also consider RRC/semi-static approach.

    - Option 2a (Apple): In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

    - Option 2b (Huawei): RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion.

    - Option 2c (Mediatek): RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

    - Option 2d (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

    - Option 2e (CMCC):  In addition to DCI solution, it is proposed to introduce semi-static schemes.

  - Option 3 (vivo):

    - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported).

    - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

- Recommended WF

  - Please discuss the proposals, considering the existing agreements from RAN1.

Notes from Wednesday 20th November

- Companies view

Issue 6-2: Skipping indication on frequency domain

- Proposals

  - Option 1 (ZTE): For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

  - Proposal 2 (Nokia):

    - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs.

    - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated.

  - Proposal 3 (vivo): It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

- Recommended WF

  - This issue has not been discussed yet in RAN4. Companies views is needed.

Notes from Wednesday 20th November

- Companies view

Issue 7-1: Response on RAN1 working assumption and time offset

- LS from RAN1:

  - R1-2407561	LS out	LS on gaps/restrictions that are caused by RRM measurements	RAN1, Nokia, RAN1#118

# 1 Overall description RAN1 discussed solutions to enable transmission/reception in gaps/restrictions that are caused by RRM measurements. The following working assumption and agreement were taken: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).. # 2 Actions To RAN4 ACTION: RAN1 respectfully asks RAN4 to check the working assumption and inform RAN1 if there are concerns that RAN1 should address.

- Proposals

  - Option 1: Options confirming feasibility of Alt 1-1

    - Option 1a (Interdigital, Nokia): RAN4 confirms that working assumption in Al. 1-1 is feasible

      - The work for the time offset delay X determination is ongoing and further updates will be sent to RAN1 in the future.

    - Option 1b (ZTE): Reply RAN1 LS with confirmation of the scheduling DCI indication used for skipping indication.

    - Option 1c (Ericsson): RAN4 can confirm the RAN1 working assumption, with the understanding that the UE will expect XR data on the same carrier frequency, on which the corresponding DCI is received.

  - Option 2: proposals confirming RAN1 working assumption and requesting additional schemes to be considered

    - Option 2a (CMCC): confirm RAN1 working assumption about DCI indication. In addition to DCI solution, it is proposed to introduce semi-static schemes.

    - Option 2b (Mediatek): RAN4 to send reply to RAN1 with the following information: (i) XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC, (ii) time offset for gap skipping is equal to 5ms, and (iii) RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

- Recommended WF

  - Please discuss the options.

Notes from Wednesday 20th November

- Companies view

Issue 2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Option 1 (Huawei, Mediatek): RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

  - Option 2 (Interdigital, vivo, Nokia, Ericsson): As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

- Recommended WF

  - The selection of the measurement gap types is a topic that has been taking long discussion. Please consider if a combination of proposals 1 and 2 can be considered as a baseline.

Notes from Wednesday 20th November

- Companies view

Issue 1-3: L3 Measurements without gaps with scheduling restrictions

- Proposals

  - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported

    - Option 1a (CMCC, Apple): Support skipping of measurements without gaps

    - Option 1b (NTT DOCOMO): Support skipping of measurements without gaps for 100MHz CBW band

  - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported

    - Option 2a (Interdigital, vivo, ZTE, Nokia, Qualcomm, Ericsson): Measurement skipping does not apply for measurements without gaps

    - Option 2b (Huawei, Mediatek): Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

- Recommended WF

  - Please consider the options. Question for discussion include, do the L3 measurement without gaps of proponents of Option 1 include which of the following cases

    - Rel 15 measurements without gaps

    - Rel 16 intra-frequency needForGaps

    - Rel 16 inter-frequency measurements for UE supporting interFrequencyMeas-Nogap-r16

    - Rel 17 UE reporting nr-NeedForGapNCSG-reporting-r17

    - Rel 18 UE reporting nr-NeedForInterruptionReport-r18

Notes from Wednesday 20th November

- Companies view

Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Proposals

  - Option 1: Support skipping of L1 measurements and procedures.

    - Option 1a (CMCC): Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

    - Option 1b (Apple): it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

    - Option 1c (NTT DOCOMO): At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

  - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures.

    - Option 2a (Interdigital, Huawe?, vivo, ZTE, Nokia, Qualcomm, Ericsson): Measurement skipping does not apply for L1 measurements and procedures

    - Option 2b (Mediatek): RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

- Recommended WF

  - Discussion needed

Notes from Wednesday 20th November

- Companies view

# Annex A WID information based on RP-241771

- 3 Justification This WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. This WI aims to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination, see also TR 22.847, TR 23.700 60. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. Related Potential Impacts has been proposed as follows: a) Enhanced RAN Awareness, by signaling from Core Network and/or indication by UE, b) Enhancements User Plane, e.g. Scheduling, LCP, Resource allocation, Discard. c) Support for multiple DRX configurations, without the potentially problematic restrictions of Pre-Rel-19 2nd DRX. This WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7. This WI aims to facilitate efficient and effective support of scheduling to enable high system capacity, e.g. by relaxing time constraints for resource allocation as far as possible, while meeting delay requirements/avoiding too late PDUs. Related Potential Impacts has been proposed as follows: a) by UE LCP prioritization, rate enforcement / starvation avoidance b) UE BSR DSR enhancements for better gNB awareness c) Awareness of application buffering working point d) Other UE MAC enhancements based on awareness of delay and/or deadline. RLC-AM is useful to limit data loss, however RLC-AM feedback or retransmission triggering mechanisms are not well adapted for short packet delay budgets applicable to XR traffic. Also, for RLC AM, considerable amounts of data may be in-flight, i.e. in the window, and there is no current way to avoid retransmitting this data, even if the data is old. In Release 18, the support of dual connectivity was not addressed in RAN3 and this WI aims at specifying the required enhancements. - 4 Objective - 4.1 Objective of SI or Core part WI or Testing part WI The Rel-19 XR Phase 3 objectives are as follows: - Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information-based approach as an alternative. RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE. NOTE: Review in RAN#106 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q1/25 onwards. - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion. - Specify Core requirements related to the above objectives as necessary [RAN4].