3GPP TSG-RAN WG4 Meeting # 113										 R4-2420234

Orlando, US, Nov. 18 – Nov. 22, 2024

Title: 	WF on Further NR mobility enhancements

Agenda Item:	5.23.3

Source: 	MediaTek Inc.

Document for:	Approval

# 0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

# 1 Topic #1: LTM - Core part

## Sub-topic 1-1 Early Sync

Issue 1-1-1: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay and PDCCH-order RACH delay

Agreement at coffee break

<Agreement>:

- RAN4 to remove “FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG” in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

  - Differentiate the cases based on whether the SSB is to be measured within or outside gap.

Issue 1-1-2: Whether to consider early TCI state activation for multiple cells at the same time

Follow RAN4#112’s guidance:

RAN4#112 Issue 1-2-1: Whether to consider early TCI state activation for multiple cells at the same time < Agreement>: - Interested companies can have further offline discussion. If no further progress, the following will be agreed in the Nov meeting. - No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

<Agreement>:

- No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

## 1.2 Sub-topic 1-2 L1-RSRP Measurement

Issue 1-2-1: Spec organization

< Agreement > Conclude the issue in RAN4#114:

  - Option 1 (vivo):

    - Although same UE behaviour on L1-RSRP measurement for current serving cell is assumed from RRM requirements perspective, no matter it is configured by either CSI-ResourceConfig under csi-MeasConfig or LTM-CSI-ResourceConfig-r18 under ltm-Config-r18, it is still proposed to capture LTM L1 measurement on serving cell in clause 9.14 of TS 38.133, so as to better differentiate UE support of either rxTimingDiff-r18 or multiCellL1-measRTD-greaterThan-CP-r18.

    - To better capture UE capabilities related to the max number of layers/cells/resource UE can measure under LTM configurations, LTM related serving cell L1 measurement shall be captured under 9.14.

    - Capture all intra-frequency LTM L1 measurement requirements, including serving cell L1 measurement requirements if counted as LTM L1 measurement, in 9.14. The impact to serving cell L1 measurement, which is configured by CSI-ResourceConfig, due to Rx beam sharing with neighbour cell LTM L1 measurement is captured by PL1_sharing in 9.5.4.1.

## 1.3 Sub-topic 1-3 Cell switch delay requirements for Pcell/PSCell

#### 1.3.1 T/F fine tracking: Tfirst-RS and Tmargin

Issue 1-3-1-1: T/F tracking when TRS as QCL source in cell switch delay

Agreement at coffee break

<Agreement>:

  - it is RAN4 common understanding that UE is allowed to use TRS for fine time tracking, if it comes earlier than SSB. Current core requirements do not need to be updated.

Issue 1-3-1-2: Conditions of Tfirst-RS =0 in cell switch delay

< Agreement>: Conclude the issue in RAN4#114

  - Option 1:

    - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

      - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

        - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

      - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 160ms.

  - Option 2:

    - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

      - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

      - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms with conditions

        - Supporters are encouraged to propose the conditions for discussion next meeting

      - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

#### 1.3.2 Conditions of Fast RRC decoding

Issue 1-3-2-1: Which cell(s) TLTM-RRC-processing = 0 apply to when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability?

<Agreement>: Conclude the issue in RAN4#114

  - Option 1 (HW)

    - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

      - The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion, and the wording can be updated.

  - Option 2 (Nokia, ZTE, E///)

      - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

  - Option 3 (Apple, HW)

      - The issue is closed

#### 1.3.3 Cell Switch without L1 measurement report in FR2?

Issue 1-3-3-1: Whether are cell switch delay requirements applicable to FR2 without L1 measurement report?

< Agreement >

    - Not to extend cell switch delay requirements to FR2 without L1 report and remove the following in known TCI state definition for cell switch.

      - [-	The target TCI state in the cell switch command is activated not more than TBD ms before the reception of the cell switch command and SNR of the SSB associated to TCI state ≥ -3dB; or]

      - [-	The target TCI state in cell switch command is activated before receiving the cell switch command and the SSB associated to target TCI state is available at least once every TBD ms after the TCI state activation command is received and SNR of the SSB associated to TCI state ≥ -3dB; or]

#### 1.3.4 Known cell definition

Issue 1-3-4-1: Whether and how to update the known condition for candidate cell SSB, candidate cell TCI and LTM target cell?

< Agreement> this issue is closed.

#### 1.3.5 PL-RS

Issue 1-3-5-1: Clarification on agreed “Conditions of no extra time for PL-RS measurement in cell switch delay” in RAN4#112

RAN4#112 Issue 1-4-2-1: Conditions of no extra time for PL-RS measurement in cell switch delay <Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. - Further discuss the wording in the CR next meeting.

<Way Forward>

  - RAN4 to further clarify the intent of the agreement regarding PL-RS measurement maintenance.

## 1.4 Sub-topic 1-4 UE feature

Issue 1-4-1: Capability for fast RRC decoding

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-6 | Fast processing of LTM candidate cell RRC configuration | 1. Indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and Scell(s) in LTMCandidateConfig(s) for MCG and SCG, that UE can store the configurations. 2. Indicates the maximum number of LTMCandidateConfigs that UE can support fast processing . | 45-3a or 45-4a in RAN1 feature list |

Agreement at coffee break

<Agreement>:

  - Send LS to RAN2 to inform them that 45-3a/45-4a (MAC-CE activated joint LTM TCI states) are not the prerequisite FG of 39-6 “Fast processing of LTM candidate cell RRC configuration”.

Issue 1-4-3: Clarification on LTM L1-RSRP related capabilities

< Agreement > For RAN4 39-3-6:

    - ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

## 1.5 Sub-topic 1-5 RAN2 LS on the fast RRC processing for LTM

Issue 1-5-1: Answer to Q1

Q1: As far as the network configures a number of total serving cell(s) + SpCell/SCell(s) in LTM candidate configurations which is up to maxNumberStoredConfigCells and where up to maxNumberConfigs LTM candidate configurations are included within the configured total cells, is it the correct understanding that UE will be capable of performing early ASN.1 decoding on all the configured LTM candidate configurations?

Online agreement

< Agreement >:

  - Reply “Yes” to Q1 in RAN2 LS on the fast RRC processing for LTM.

Issue 1-5-2: Answer to Q2

Q2: Does the UE need to know the number of SpCells and SCells within the configured LTM candidate configurations before doing the early ASN.1 decoding of an LTM candidate configuration?

Online agreement

< Agreement >:

  - Reply “No” to Q2 in RAN2 LS on the fast RRC processing for LTM.

  - Further discuss what additional information to include in the reply LS if needed.

Issue 1-5-3: Answer to Q3

Q3: Is it possible that the number of serving cell(s) + SpCell/SCell(s) in LTM candidate configurations exceeds the UE reported maxNumberStoredConfigCells?

Online agreement

< Agreement >:

  - Yes.

  - Capture the following in the LS:

    - RAN4 also agreed that both early TCI state activation command and PDCCH-order can be used to trigger early RRC decoding.

    - Further discuss wording in the LS regarding the condition under which UE can successfully perform early ASN.1 decoding.

## 1.6 Sub-topic 1-6 RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM

Issue 1-6-1: RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM (R2-2409360/ R4-2417508)

Online agreement

< Agreement >:

  - no need to reply the LS, and discuss whether and how to reflect the interFreqL1-OnlyInBC-r18 capability in 38.133.

# 2 Topic #2: Improvement on SCell/SCG setup delay - Core part

Issue 2-1-1: Mis-alignment between RAN2 and RAN4 spec when measReselectionCarrierListNR is not configured

Discuss in the CR directly if the components have reached initial consensus.

# 3. Topic #3: Performance Part

## 3.1 Sub-topic 3-1 LTM

Issue 3-1-1: Test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2)

    - < Wayforward > In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

Note: Ericsson needs further check.

Issue 3-1-2: Test cases for SSB based L1-RSRP measurement in FR1 – intra-f & inter-f without gap (A.6.6.26.1 & A.6.6.28.1.3)

Discuss in the CR directly.

Issue 3-1-3: TCI state configurations

< Way Forward > FFS the following proposal:

  - Proposal 1 (vivo): In RRM test case design for LTM, RAN4 avoid TCI configuration with ‘QCL-D’ or ‘UL TCI’ to UE in FR1, which means only DL or Joint TCI with QCL-A/C configuration, and pathloss RS configuration if necessary, shall be provided to UE in FR1, even if UE supports ltm-MAC-CE-SeparateTCI-r18.

## 3.2 Sub-topic 3-2 Improvement on SCell/SCG setup delay

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay?

< Agreement >Conclude the issue in RAN4#114

  - Option 1 (ZTE, Huawei):

    - Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement.

  - Option 2 (CATT):

    - RAN4 need to discuss if the understanding of ‘If a UE has already passed the test for R16 EMR and R18 EMR tests, no matter how the validity time of the measurement results is defined, the UE shall be able to complete valid reporting because its reporting function has been verified to be good’ is right.

    - If the above understanding is assumed, then the current R18 test cases design is enough.

  - Option 3 (Nokia):

    - Add Test case for Idle mode fast CA/DC eEMR measurement for FR1 with valid reporting.