3GPP TSG-RAN WG4 Meeting #113	R4-2420331

Orlando, United States 18th – 22nd November 2024

Agenda Item:	7.2.6

Source:	Qualcomm Incorporated

Title:	WF on 15GHz UE parameters

Document for:	Approval

# 1	Introduction

This WF captures the agreement for IMT UE matters for [113][119] FS_NR_IMT - UE RF parameters

# 2	Agreements

Power dynamic range

- 56 dB

Spectral mask

- Emission mask for 200 MHz BW is based on the CA mask as defined in TS 38.101-1, § 6.5A.2.2.

ACLR

- 24 dB with the following note

  - User equipment (UE) ACLR value is derived considering the UE needs to meet the Occupied bandwidth (OBW) requirements, and is higher than the ACLR needed for co-existence.

Maximum output power

- 23 dBm

Noise figure

- 13 dB

Spurious emissions

- 200 MHz BW requirements the same as TS 38.101-1 § 7.6A Tables 7.6A.2.1-1 and 7.6A.3-1 Bandwidth Class C for blocking levels and § 7.7 Table 7.7A-1 Bandwidth Class C for spurious response.

ACS

- 24 dB