3GPP TSG-RAN WG4 Meeting # 113						R4-2420364

Orlando, US,18th–22nd November 2024

Source:		Ericsson, Huawei

Title:			TP to TR 38.769:  A-IoT general overview

Agenda item:		7.21.1

Document for:		Approval

Introduction

In this paper, TP for system parameter for TR 38.769 is provided.

TP text proposal

- 6.7.1	System parameters

- 6.7.1.1	Operating band

According to the SID (RP-240826), Ambient IoT uses FR1 licensed spectrum in FDD. NR band n8 can be used as an example band.

Table 6.7.1.1-1 summarized the A-IoT system spectrum usage from co-existence scenario. It can be observed that The spectrum usage for A-IoT operation can be flexible, and it depends on several factors, such as whether the reader is A-IoT BS  or intermediate UE, and whether CW transmission is deployed inside or outside topology. For the case which different spectrum is used for D2R and R2D, the FDD band definition can be kept. As CW transmission may interfere the R2D reception, the R2D and D2R should be TDM and therefore, for FDD band definition, it should be half duplex mode.

Table 6.7.1.1-1: Spectrum usage for Ambient IoT

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW2D transmission |
| --- | --- | --- | --- |
| A-IoT BS | DL spectrum | DL spectrum | DL spectrum |
|  | DL spectrum | UL spectrum | UL spectrum |
|  | UL spectrum | DL spectrum | DL spectrum |
|  | UL spectrum | UL spectrum | UL spectrum |
| Intermediate UE as reader | UL spectrum | UL spectrum | UL spectrum |
|  | UL spectrum | DL spectrum | DL spectrum |

- 6.7.1.2	Channel bandwidth

The R2D waveform is agreed to be OFDM-based waveform, therefore the channel bandwidth definition can follow the channel bandwidth definition in NR. The R2D channel bandwidth can be defined as one or multiple PRBs with guardband if necessary. The value for this can be determined during the WI phase.

- 6.7.1.3	Channel arrangement

The channel spacing may be specified for D2R FDM operation for device 1/2a.

For Ambient IoT BS and intermediate UE, channel spacing and channel raster may be needed for deployment purpose, for example, to co-deploy with existing NR network.