3GPP RAN WG4 Meeting #113		R4-2420374

Orlando, USA, 18th – 22nd November 2024	(revision of R4-2417740)

Agenda item:	7.2.4.1

Source:	Apple

Title:	Co-existence simulation results for the 14800-15350MHz frequency range

WI/SI:	FS_NR_IMT_4400_7125_14800MHz

Release:	Rel-19

Document for:	Discussion

1	Introduction

The WRC-23 conference identified three frequency ranges as potential IMT candidates and tasked ITU-R WP5D to conduct the corresponding sharing and compatibility studies. In turn, the ITU-R WP5D sent an LS to several bodies, including 3GPP, asking to provide technical parameters for the candidate frequencies [1]. In response to that LS from WP5D and based on the technical input from RAN WG4 [2], the 3GPP RAN#103 meeting approved a new SI [3], purpose of which will be to study and provide the corresponding IMT technical parameters.

According to the WRC-23 resolution and the corresponding LS from ITU-R WP5D, there are three frequency ranges, for which RAN WG4 is tasked to provide the IMT-2030 parameters: 4400-4800MHz, 7125-8400MHz and 14800-15350MHz. During the previous RAN WG4#112bis meeting several agreements were made for the 15GHz UE IMT parameters, but there are remaining open issues, such ACLR and ACS, values for which should be decided based on the co-existence simulations. In this discussion paper we present our co-existence simulation results for the urban macro scenario for the 14800-15300MHz range.

2	Co-existence simulations for 15GHz

The general parameters for the co-existence simulations are the same as agreed during the previous RAN4#112bis meeting. For the sake of further clarity, Annex A also provides more detailed information on which simulation parameters are used including the general deployment parameters and the BS antenna parameters. For the UE antenna a simple omnidirectional antenna is assumed.  It should be noted that our simulations use slightly different models compared to the agreed 3GPP assumptions, short summary of which is presented below:

-	Building penetration loss. While the 3GPP model proposes 50% low/high loss ratio, our simulations use the ITU model from Recommendation ITU-R P.2109. Both models provide comparably the same penetration loss values, but the ITU model is slightly biased towards "lower" losses. Annex B presents the graphical comparison of both models.

Next sub-sections present the DL and UL co-existence results:

-	Section 2.1: DL co-existence results for the coordinated urban macro deployment

-	Section 2.2: UL co-existence results for the coordinated urban macro deployment

## 2.1	DL co-existence results

For the DL co-existence results the ACLR BS is fixed to be 45dB, but the UE ACS varies from 15 to 33dB as summarised below in Table 2.1-1. The summary of DL throughput loss is presented in Table 2.1-2 with separate statistics for the average and 5%-tile losses. For the previous studies on 10-10.5GHz it was concluded that UE ACS of 31dB would be sufficient. Since this study considers even higher frequencies with more advanced AAS systems, the required UE ACS can be 27-30dB to ensure quite low throughput losses.

Table 2.1-1: Summary of the UE ACS and ACIR values

| ACS UE | ACLR BS | ACIR |
| --- | --- | --- |
| 15 | 45 | 15 |
| 20 | 45 | 20 |
| 30 | 45 | 30 |
| 33 | 45 | 32 |

Table 2.1-2: DL throughput loss

| ACS UE | ACIR | Average throughput loss (%) | 5%-tile throughput loss (%) |
| --- | --- | --- | --- |
| 15 | 15 | 0.47 | 0.623 |
| 20 | 20 | 0.232 | 0.247 |
| 30 | 30 | 0.49 | 0.054 |
| 33 | 32 | 0.026 | 0.006 |

Observation 1:	Based on the DL co-existence simulation results the UE ACS can be around 27-30dB

## 2.2	UL co-existence results

For the UL co-existence results the BS ACS is fixed to 45dB, but the UE ACLR varies from 5 to 30dB as summarised below in Table 2.1-2. The summary of UL throughput loss is presented in Table 2.1-2 with separate statistics for the average and 5%-tile losses. While studying the 10-10.5GHz range, it was concluded that 24dB UE ACLR is sufficient for a UE with an omnidirectional antenna to co-exist with another IMT system. So, for the higher frequency range even a lower ACLR can be adopted as confirmed by the co-existence studies.

Table 2.2-1: Summary of the UE ACLR and ACIR values

| ACS BS | ACLR UE | ACIR |
| --- | --- | --- |
| 45 | 5 | 5 |
| 45 | 10 | 10 |
| 45 | 15 | 15 |
| 45 | 20 | 20 |
| 45 | 22 | 22 |
| 45 | 23 | 23 |
| 45 | 24 | 24 |
| 45 | 30 | 30 |

Table 2.2-2: UL throughput loss

| ACS UE | ACIR | Average throughput loss (%) | 5%-tile throughput loss (%) |
| --- | --- | --- | --- |
| 5 | 5 | 1.2 | 1.5 |
| 10 | 10 | 0.6 | 0.59 |
| 15 | 15 | 0.3 | 0.22 |
| 20 | 20 | 0.13 | 0.14 |
| 22 | 22 | 0.099 | 0.131 |
| 23 | 23 | 0.079 | 0.1 |
| 24 | 24 | 0.06 | 0.1 |
| 30 | 30 | 0.022 | 0.01 |

Observation 2:	Based on the UL co-existence simulation results the UE ACLR can be around 22-24dB.

# 3	Conclusions

In this discussion paper we have presented our DL and UL co-existence simulation results for the 14.8-15.35GHz frequency range. Based on our analysis we suggest considering the ACLR and ACS values that are more relaxed comparing to the previous studies for the 10-10.5GHz range.

Observation 1:	Based on the DL co-existence simulation results the UE ACS can be around 27-30dB

Observation 2:	Based on the UL co-existence simulation results the UE ACLR can be around 22-24dB.

# 4	References

RP-240024, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz", ITU-R WP5D

R4-2102840, "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)", RAN WG4.

RP-240765, "Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz"

3GPP TR 38.921, "Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz", v17.1.0

Annex A: Simulation parameters

Table A-1: General simulation parameters

| Parameter | Units | Value |
| --- | --- | --- |
| Frequency | MHz | 15000 |
| Network layout |  | Urban macro |
| Network layout | BS | 19x3 |
| Inter-site distance | m | 450 |
| UE distribution (horizontal) |  | uniform |
| Number UE served by BS | - | 1 |
| System bandwidth | MHz | 100 |
| BS transmit power | dBm | 46 |
| BS height | m | 20 |
| BS antenna | - | 4 subarray (16x24) |
| BS antenna mechanical tilt | deg | -6 |
| BS receiver noise figure | dB | 8 |
| Minimum coupling loss (UL) | dB | 55 |
| UE transmit power | dBm | 23 |
| UE height | m | 3GPP model |
| UE antenna gain | dBi | 0 |
| UE indoor ratio | Indoor outdoor | 20% 80% |
| UE receiver noise figure | dB | 11 |
| Minimum coupling loss (DL) | dB | 80 |
| Handover margin | dB | 3 |
| Penetration loss | - | Recommendation ITU-R P.2109 |
| Propagation model (within systems) | - | UMa |
| Propagation model (between aggressor to victim | - | UMa |
| Shadowing correlation | - | 1.0 0.5 |

Table A-2: BS AAS configuration

| Parameter | Units | Value |
| --- | --- | --- |
| Polarization (note 1) | - | Dual polarization |
| Number of elements H | - | 16 |
| Number of elements V | - | 24 |
| Horizontal spacing |  | 0.5 |
| Number of sub-array elements in vertical line | - | 4 |
| Pre-sub-array downtilt | deg | 3 |
| Minimum vertical coverage | deg | 90 |
| Maximum vertical coverage | deg | 100 |
| Minimum horizontal coverage | deg | -60 |
| Maximum horizontal coverage | deg | 60 |
| Antenna element peak gain | dBi | 6.4 |
| Azimuth 3dB beamwidth | deg | 90 |
| Elevation 3dB beamwidth | deg | 65 |
| Front to back ratio | dB | 30 |
| Side lobe suppression | dB | 30 |

Annex B: 3GPP and ITU building penetration losses

Figure B-1: Histogram of the building penetration loss of 3GPP and ITU-R P.2109 models