3GPP TSG-RAN WG4 Meeting #113	R4-2420400

Orlando, USA, 18 ‒ 22 November, 2024

Title: 	WF on UE RF requirements for MIMO enhancement

Agenda Item:	6.19.4

Source: 	Samsung

Document for:	Approval

<Topic 1> 3Tx

<Sub-topic 1-1> Power class

<Agreement>: PC for 3Tx requirement

- 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19

<Way forward>: FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type.

<Sub-topic 1-3> MPR

<Agreement>: PC3 for 3Tx

- Use the same MPR as MPR for PC3 single carrier.

<Agreement>: PC2 for 3Tx

- Use the same MPR as MPR for PC2 single carrier.

  - FFS on whether PC2 MPR with 1Tx or 2Tx will be applied

<Way forward>: FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx. Subject to  Tx Diversity support

<Sub-topic 1-4> MIMO layer for mode 0

<Agreement>: Supported layer for mode 0

- For the performance verification of 3Tx, use 1-layer only for mode 0.

<Topic 2> Asymmetric TRPs

<Sub-topic 2-1> UE RF impact

<Way forward>: FFS on study of UE RF requirement for different DL/UL TCI states in the WI or TEI

- R4-2417632 and R1-2410870 are for information.