3GPP TSG-RAN WG4 Meeting # 113	R4-2420403

Orlando, US, 18th – 22nd November, 2024

Title: 	WF on NR NTN HPUE

Agenda Item:	7.8.5

Source: 	Samsung

Document for:	Approval

Topic #1: Co-existence study

Sub-topic 1-1 How to treat different options for results discussions

Issue 1-1: Basic principles for co-ex conclusion

Agreement:

For co-ex conclusion discussion, to follow similar principles in deriving ACIR as documented in TR 38.863 in Rel-17.

It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case. Moreover, the following considerations are adopted to deal with major disputes for the selected cases’ results in each scenario: - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. - If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.

Issue 1-2: TN deployment scenario for co-ex conclusion

Agreement:

For co-ex study result conclusion discussion, in principle, to use Urban results instead of Rural results which aligns with previous agreement and worst case.

Issue 1-3: TN BS antenna type for co-ex conclusion

Agreement:

For co-ex study result conclusion discussion, in principle, to use AAS results instead of non-AAS results as worst case.

Issue 1-4: Wrap-around outside TN cells, and Issue 1-5: Isolation distance between NTN UE and TN cluster edge

Agreement:

Companies can provide the simulation results with or without wrap-around, and TR will capture the results with or without wrap-around separately.

- With wrap-around: Enable wrap-around assuming 1.5km isolation.

- Without wrap-around: Disable wrap-around assuming X isolation.

  - FFS whether X should be 0km or 0/1.5km.

Sub-topic 1-2 ACIR/ACLR discussion from co-ex study

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study

Agreement:

Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective:

For NR-NTN PC2:

- Less than 22 dB with 1.5km isolation distance assumed (baseline assumption)

- [26.5 dB] with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For NR-NTN PC1.5:

- Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption)

- [30.2] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For NR-NTN PC1:

- Less than 23.7 dB with 1.5km isolation distance assumed  (baseline assumption)

- [28.3/32.7] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For IoT-NTN PC2:

- Less than 21.1 dB with 1.5km isolation distance assumed;

- Less than [20.3] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

For IoT-NTN PC1:

- Less than 21.9 dB with 1.5km isolation distance assumed;

- Less than [24.2] dB with 0km isolation distance assumed.

  - Interested companies are encouraged to provide results on this case

Topic #2: HPUE Tx requirements

Issue 2-1: MOP

Agreement:

Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE

Issue 2-2: MPR/A-MPR

Agreement:

MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation.

Issue 2-3: SEM

Way forward:

- Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point.

- Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes.

Issue 2-4: ACLR

Agreement in #112bis meeting: Issue 2-5: ACLR Agreement: - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.

Way forward:

- Further discuss the ACLR requirements for NR-NTN HPUE

  - Take into account the co-ex study outcomes in this #113 meeting with previous agreements.

Topic #3: HPUE Rx requirements

Issue 3-1-2: RSD Requirements for PC2 on n254

Agreement:

For PC2 on n254: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx

Issue 3-1-3: RSD requirements for PC2 on n255

Way forward:

For PC2 on n255: Further discuss the follow assumptions

- Option 1: Duplex isolation as 50~52; PA interference as -107 dBm/Hz (P2/R4-2419073)

- Option 2: Duplex isolation as 51; PA interference as -117 dBm/Hz (O1/R4-2419540)

Issue 3-1-4: RSD requirements for PC2 on n256

Way forward:

For PC2 on n256: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx as starting point

- To be confirmed by further discuss the following assumptions:

  - Option 1: Duplex isolation as 50~52; PA interference as -117.8 dBm/Hz

  - Option 2: Duplex isolation as 51; PA interference as -124 dBm/Hz

Issue 3-4: Others general proposals w.r.t RSD

Way forward:

- Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

Open issues

Issue 2-5: NR-NTN PC2 UE

Agreement in #112bis meeting: Issue 2-2: UE architecture/type considerations for NR-NTN PC2 UE Agreement: - Follow previous agreement Issue 3-1-5: Consideration of HPUE Architecture - Agreement: - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements. - ....

- Proposals

  - Option 1: In Rel-19 RAN WG4 can focus on 26dBm 1TX (PC2) for handheld devices. (P1a/R4-2417747)

  - Option 2: For handheld UE, 1Tx and 2Tx are covered with existing TN 26dBm and 23dBm PAs respectively. (P1/R4-2419521)

Issue 2-6: NR-NTN PC1.5 UE

Agreement in #112bis meeting: Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not

- Proposals

| PC1.5 UE | 1Tx | 2Tx |
| --- | --- | --- |
| Handheld | To specify: Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015, P1/R4-2419521 FFS: | To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532 Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015 FFS: |
| Non-handheld | To specify: Not to specify: P1/R4-2419668, P3/R4-2419072 FFS: P5/R4-2418200, P1/R4-2419521 | To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532, P2/R4-2419521 Not to specify: P1/R4-2419668, P3/R4-2419072, P1b/R4-2417747 FFS: |

Issue 2-7: NR-NTN PC1 UE

Agreement in #112bis meeting: Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.

- Proposals

| PC1 | 1Tx | 2Tx | 3Tx | 4Tx |
| --- | --- | --- | --- | --- |
| Handheld | To specify: Not to specify: P1/R4-2419521 FFS: | To specify: Not to specify: P5/R4-2418200, P1/R4-2419521 FFS: | To specify: P1/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200 | To specify: Not to specify: FFS: |
| Non-handheld Only non-handheld: P2/R4-2419015, P1/R4-2419072 | To specify: P1a/R4-2417747, P2/R4-2419521 Not to specify: FFS: | To specify: Not to specify: P5/R4-2418200 FFS: | To specify: P2/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200 | To specify: P4/R4-2419072 Not to specify: FFS: |

Issue 3-1-1: General considerations

- Proposals

  - Option 1: For power class 2, there is no differentiation of handheld and non-handheld UE. (P2b/R4-2418201)

  - Option 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE. (P2/R4-2419717)

Issue 3-2: RSD requirements for PC1.5

- Proposals

  - Option 1: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE (P1/R4-2417815)

  - Option 2: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only (P3/R4-2418201)

  - Option 3: no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016)

  - Option 4: [2.2] dB RSD for PC1.5 with 2Tx architecture for n256.

Issue 3-3: RSD requirements for PC1

- Proposals

  - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (P4a/R4-2418201)

    - Option 1-1: FFS for PC1 with 2 or 3 Tx cases (P4b/R4-2418201)

  - Option 2 no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016)