3GPP TSG-RAN WG4 Meeting #113	R4-2420492

# Orlando, USA, 18th October – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2552 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.101-1 correcting PC2 and PC3 IMD MSD´s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_TDD_NR_CADC_SUL_R18-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correcting PC2 and PC3 MSD |  |  |  |  |  |  |  |  |
| Summary of change: |  | MSD value for CA_n7-n20 moved from PC2 to PC3 table MSD value for UL n78(2A) for DL CA_n20-n78 moved from PC2 to PC3 table MSD value defined for CA_n8-n77 PC2. Now also added for PC3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correction is not made |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| CA_n1-n46 | n1 | 1950 | 5 | 25 | 2140 | 5 | FDD | IMD5 |
|  | n46 | 5660 | 20 | 100 | 5660 | N/A | TDD | N/A |
| CA_n1-n77 | n1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2130 | 17 | FDD | IMD516 |
|  | n7712 | 3310 | 10 | 1 (RBSTART=25) | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 1.7 | FDD | IMD717 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3675 | 10 | 1 (RBSTART=44) | 3675 |  |  |  |
| CA_n1-n102 | n1 | 1922.5 | 5 | 25 | 2112.5 | 13 | FDD | IMD3 |
|  | n102 | 5957.5 | 20 | 100 | 5957.5 | N/A | TDD | N/A |
| CA_n1-n105 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| CA_n3-n18 | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
|  | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |
| CA_n3-n26 | n3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD24 |
| CA_n3-n20 | n3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | FDD | IMD4 |
| CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | N/A | FDD | IMD914 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 15.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A | FDD | IMD57 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | [2.2] | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBSTART=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=3) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=0) | 3780 |  |  |  |
| CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| CA_n5-n12 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A18 | FDD | N/A |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 |  |  |  |
|  | n12 | N/A | 5 | N/A | 743.5 | 20.8 | FDD | IMD11 |
| CA_n5-n13 | n5 | 828 | 5 | 25 | 873 | 25 | FDD | IMD3 |
|  | n13 | 783 | 5 | 25 | 752 | N/A | FDD | N/A |
| CA_n5-n14 | n5 | 836 | 5 | 25 | 881 | 25 | FDD | IMD34 |
|  | n14 | 791 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 25 | FDD | IMD3 |
|  | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A18 | FDD | N/A |
|  |  | 838.9 | 10 | 10 (RBSTART=28) | 883.9 |  |  |  |
|  | n14 | N/A | 5 | N/A | 765.5 | 26.2 | FDD | IMD9 |
| CA_n5-n29 | n5 | 829 | 10 | 10 (RBSTART=0) | 874 | N/A18 | FDD | N/A |
|  |  | 838.9 | 10 | 10 (RBSTART=36) | 883.9 |  |  |  |
|  | n29 | N/A | 5 | N/A | 725.5 | 16.0 | FDD | IMD13 |
| CA_n5_n41 | n5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n5-n77 | n5 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD413 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 5.5 | FDD | IMD513 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 |  |  |  |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD417 |
|  | n7812 | 3340 | 10 | 1 (RBSTART=25) | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=25) | 3780 |  |  |  |
| CA_n7-n20 | n7 | 2512 | 5 | 50 | 2632 | N/A | FDD | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | FDD | IMD3 |
| CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD34 |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | FDD | N/A |
| CA_n7-n40 | n7 | 2510 | 5 | 25 | 2630 | 23 | FDD | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
| CA_n7-n46 | n7 | 2550 | 10 | 50 | 2670 | 26.8 | FDD | IMD24 |
|  | n46 | 5220 | 20 | 100 | 5220 | N/A | TDD | N/A |
| CA_n7-n66 | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 7.1 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 15 | FDD | IMD5 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 |  |  |  |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 15 | FDD | IMD5 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=25) | 3700 |  |  |  |
| CA_n8-n20 | n8 | 892.5 | 5 | 25 | 937.5 | 25 | FDD | IMD3 |
|  | n20 | 849.5 | 5 | 25 | 808.5 | 25 | FDD | IMD3 |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| CA_n12-n66 | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n66 | 1765 | 5 | 25 | 2115 | 5.0 | FDD | IMD4 |
| CA_n12-n77 | n12 | 702 | 5 | 20 | 732 | 5.5 | FDD | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n12-n78 | n12 | 710 | 5 | 25 | 740 | 5.5 | FDD | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
| CA_n13-n77 | n13 | 782 | 5 | 20 | 751 | 5.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
| CA_n14-n77 | n14 | 793 | 5 | 20 | 763 | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 15.6 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 8.4 | FDD | IMD48 |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 4.5 | FDD | IMD58 |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |
| CA_n18-n78 | n18 | 827.5 | 5 | 25 | 872.5 | 8.3 | FDD | IMD49 |
|  | n78 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
| CA_n20-n78 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
|  | n20 | N/A | 5 | N/A | 800 | 8.6 | FDD | IMD415 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=25) | 3750 |  |  |  |
| CA_n24-n77 | n24 | N/A | N/A | N/A | N/A | N/A | FDD | IMD410 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBSTART=221) | 2640 |  |  |  |
| CA_n25-n48 | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 |  |  |  |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1980 | 2.7 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 (RBSTART=7) | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 (RBSTART=0) | 3760 |  |  |  |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 | N/A | FDD | N/A |
| CA_n26-n77 | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD413 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD513 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
|  | n78 | N/A | 10 | N/A | 3336 | 11.1 | FDD | IMD4 |
|  | n26 | 824 | 5 | 12 (RBSTART=0) | 869 | N/A | TDD | N/A |
|  |  | 839 | 5 | 12 (RBSTART=0) | 884 |  |  |  |
| CA_n28-n50 | n28 | 730 | 10 | 50 | 775 | 15.3 | FDD | IMD2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
|  | n28 | 740 | 10 | 50 | 785 | 6.0 | FDD | IMD44 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
| CA_n28-n74 | n28 | 705.5 | 5 | 25 | 760.5 | 24.6 | FDD | IMD2 |
|  | n74 | 1466 | 5 | 25 | 1514 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 11.3 | FDD | IMD44 |
|  | n74 | 1431 | 5 | 25 | 1479 | N/A | FDD | N/A |
|  | n28 | 709 | 5 | 25 | 764 | N/A | FDD | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | FDD | IMD4 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | FDD | N/A |
|  | n74 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | FDD | IMD5 |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 8.5 | FDD | IMD415 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 705.5 | [8.6] | FDD | IMD4 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=17) | 3455 | N/A | TDD | N/A |
|  |  | 3805 | 10 | 1 (RBSTART=0) | 3805 |  |  |  |
| CA_n28-n78 | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
|  | n30 | N/A | 5 | N/A | 2352.5 | 3.2 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=17) | 3455 | N/A | TDD | N/A |
|  |  | 3825 | 10 | 1 (RBSTART=0) | 3825 |  |  |  |
| CA_n41-n66 | n41 | 2545 | 90 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBSTART=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11 | FDD | IMD4 |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 17 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| CA_n46-n77 | n46 | N/A | 20 | N/A | 5155 | N/A | TDD | IMD5 |
|  | n7712 | 3385 | 10 | 1 (RBSTART= 25) | 3385 | N/A | TDD | N/A |
|  |  | 3975 | 10 | 1 (RBSTART= 25) | 3975 |  |  |  |
|  | n46 | N/A | 20 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 20 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| CA_n46-n78 | n46 | N/A | 20 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 20 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBSTART=8) | 3855 |  |  |  |
|  | n66 | 1730 | 5 | 25 | 2130 | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBSTART=0) | 3875 |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | 25 | 2150 | 1.7 | FDD | IMD7 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=7) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=0) | 3750 |  |  |  |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |
| CA_n67-n78 | n67 | N/A | 5 | N/A | 748 | 8.6 | SDL | IMD415 |
|  | n7812 | 3376 | 10 | 1 (RBSTART=25) | 3376 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=25) | 3750 |  |  |  |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n70-n78 | n70 | 1705 | 5 | 25 | 2005 | 31 | FDD | IMD2 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n70 | 1705 | 5 | 25 | 2005 | 5.0 | FDD | IMD5 |
|  | n78 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD513 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| CA_n77-n85 | n77 | 3590 | 10 | 25 | 3590 | N/A | TDD | N/A |
|  | n85 | 712 | 5 | 25 | 742 | 5.5 | FDD | IMD5 |
| CA_n78-n102 | n7812 | 3320 | 10 | 1 (RBSTART=25) | 3320 | N/A | TDD | N/A |
|  |  | 3680 | 10 | 1 (RBSTART=25) | 3680 |  |  |  |
|  | n102 | 6280 | 20 | 100 | 6280 | N/A | TDD | IMD44 |
| CA_n78-n105 | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
|  | n105 | 682.5 | 5 | 25 | 631.5 | 5.5 | FDD | IMD5 |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Void. NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE8: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 – 3800 MHz, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: Void. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz NOTE 15: This band is subject to IMD6 also which MSD is not specified NOTE 16: This band is subject to IMD7 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n78 frequency range restriction of 3400 – 3800 MHz or 3300 – 3600 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 18: This component carrier is affected by IMD due to CA_n5B for which the MSD is not specified. |  |  |  |  |  |  |  |  |

Table 7.3A.5-1a: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 | 5 | 25 | 2140 | 35.8 | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2130 | 31 | FDD | IMD515 |
|  | n7712 | 3310 | 10 | 1 (RBSTART=25) | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3675 | 10 | 1 (RBSTART=44) | 3675 |  |  |  |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 20.0 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 19.10 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 18.4 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
|  | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 23.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |
|  | n3 | N/A | 5 | N/A | 1877.5 | N/A17 | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
| CA_n3-n774 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=3) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=0) | 3780 |  |  |  |
| CA_n5-n77 | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD44,13 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 | N/A | TDD | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n5-n784 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 (RBSTART=25) | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=25) | 3780 |  |  |  |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | [15.8] | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 29.9 | FDD | IMD515 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 | N/A | TDD | N/A |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 29.9 | FDD | IMD515 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=25) | 3700 | N/A | TDD | N/A |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | FDD | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 21.5 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| CA_n12-n77 | n12 | 702 | 5 | 20 | 732 | 11.7 | FDD | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n13-n77 | n13 | N/A | 5 | N/A | 750 | 8.6 | FDD | IMD415 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 (RBSTART=25) | 3885 |  |  |  |
|  | n13 | 781 | 5 | 25 | 750 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 (RBSTART=25) | 3885 | N/A | TDD | N/A |
|  | n13 | 782 | 5 | 20 | 751 | 20.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
| CA_n14-n77 | n14 | 795.5 | 5 | 15 | 765.5 | 11.7 | FDD | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | TDD | N/A |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 24.6 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 17.5 | FDD | IMD416 |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 10.5 | FDD | IMD516 |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |
|  | n18 | 827.5 | 5 | 25 | 872.5 | 18.4 | FDD | IMD48 |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | TDD | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 11.7 | FDD | IMD58 |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |
|  | n25 | 1860 | 5 | 25 | 1940 | 15.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 32.1 | FDD | IMD24 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 19.1 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1980 | 13.6 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 (RBSTART=7) | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 (RBSTART=0) | 3760 |  |  |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 23.8 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n28-n78 | n28 | N/A | 5 | N/A | 780 | 18.5 | FDD | IMD414 |
|  | n7812 | 3310 | 10 | 1 (RBSTART=7) | 3310 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=0) | 3700 |  |  |  |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| CA_n41-n66 | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 16.3 | FDD | IMD4 |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | [2.7] | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 32 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 34.33 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 31 | FDD | IMD515 |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 | N/A | TDD | N/A |
| CA_n66-n78 | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | 25 | 2150 | 13.6 | FDD | IMD7 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=7) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=0) | 3750 |  |  |  |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 37 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 18.4 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n71-n77 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD513 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 18.5 | FDD | IMD414 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| CA_n77-n85 | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 702 | 5 | 20 | 732 | 11.7 | FDD | IMD5 |
| NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Void. NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE8: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: Void. NOTE 10: Void. NOTE 11: Void. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: This band is subject to IMD6 also which MSD is not specified. NOTE 15: This band is subject to IMD7 also which MSD is not specified. NOTE 16: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. NOTE 17: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |

---End of changes---