# 3GPP TSG-RAN WG4 Meeting #114	R4-2500010

Athens, Greece, 17th – 21st Feb, 2025

3GPP TSG RAN WG1 Meeting #119	R1-2410917

Orlando, FL, USA 18th – 22nd November 2024

Title:	Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell

Response to:	R1-2409350 (R4-2416913)

Release:	Rel-19

Work Item:	Netw_Energy_NR_enh

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact person:	Ajit Nimbalker, Ajit (dot) Nimbalker (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 would like to thank RAN4 for the LS on SSB relation in On-demand SSB and SSB adaptation on Scell and would like to provide the following replies.

Obj 1

Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?):

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?):

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst.

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB.

Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?):

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?):

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Obj 3

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

# 2	Actions

To RAN4:

ACTION: 	RAN1 respectfully asks RAN4 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #120	17th – 21st February 2025		Athens, GR

TSG-RAN WG1 Meeting #120bis	7th – 11th April 2025			TBD China