3GPP TSG-RAN WG4 Meeting #114												R4-2500041

Athens, Greece, 17 – 21 February, 2025

Agenda item:			7.14.2.2

Source:	Huawei, HiSilicon

Title:	on NTN OTA test

Document for:	Approval

# Introduction

The WF [1] in the last meeting did not agree on the issue of CP (Circular Polarisation) as shown below. This contribution suggests ways to measure CP TRP and TRS using LP (Linear Polarisation).

Issue 3-2-4: How to resolve UE antenna circular polarization issue

Proposals

  - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do.

    - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system.

  - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments.

- WF

  - FFS

# Discussion

Both reference [2] and [3] demonstrate that CP performances such as total power and axial ratio can be derived by making 3 or 4 LP measurements.

The method in [2] uses two pairs of orthogonal measurements with a comment that one of the measurements can be derived from the other three, viz. three measurements would be sufficient. However, it claims that four measurements improve accuracy. Fig.1 in [2] shows a generic diagram of elliptic polarisation. The derived phase difference between two orthogonal electric fields, major axis inclined angle and axial ratio are given in equations (10), (11) and (12) in [2], respectively.

[3] adopts essentially the same approach as [2], but with 3 LP measurements in any directions. Namely orthogonality among LP measurements is not required. Fig.5 in [3] provides the layout of an elliptic polarisation, together with the angles of 3 LP measurements. In [3], the derived left and right handed circular polarisations are given equations (44), (45), axial ratio is in equation (45) and major axis inclined angle is in equation (60).

Neither [2] nor [3] can determine the handedness of CP due to the sign uncertainty in determining the phase difference between orthogonal electric fields in CP.

The derivations in [2] and [3] are for CP transmission, e.g. in measuring total radiated power (TRP). The same procedure can equally be applied to total radiated sensitivity (TRS) measurement. To reduce the number measurements in TRS from three to two, the phase difference derived from TRP measurement can be re-used for TRS due to Tx and Rx reciprocity. This may introduce an error in FDD cases because Tx and Rx are not on exactly the same frequency. However, such error could be insignificant as the frequencies between uplink and downlink in FDD bands are relatively close to each other. In addition, two out of three LP measurements used in [3] can be made orthogonal to align with E and E in order to minimise the impact on the existing TRP and TRS test procedures.

Proposal 1 : to adopt the method in reference [3] to derive circular polarisation TRP and TRS and axial ratio using 3 linear polarisation (LP) measurements with two out of three LP measurements orthogonal to each other to align with E and E in order to minimise the impact on the existing TRP and TRS test procedures if devices self-declare use of circular polarisation.

The third LP measurement in addition to E and E in Proposal 1 may necessitate mechanism to rotate measurement antenna. It may be difficult to retro-fit such mechanisms if not already available in existing systems. In those cases, a linear polariser can be placed in front of the chamber measurement antenna in a direction other than E and E.

A wire grid polariser is a good candidate and allows polarisation in a specific direction to go through. If measurement probe is a horn antenna, the horn antenna needs to take measurement in two orthogonal directions (e.g. E and E) in order to record the signal power polarised in the direction of the polariser (viz. perpendicular to the wire grid) to serve as the third LP measurement. This is because the polariser should not be aligned with either E or E and measurement in two orthogonal directions would fully capture the power after the polariser. If the probe is a linear antenna (e.g. dipole), the loss due to polarization misalignment between linear probe antenna and polariser needs to be taken into account.

The ohmic loss from the polariser is typically small and can be ignored. No additional calibration is needed for the TRP and TRS measurement system with a polariser in place because the polariser is considered as part of incoming wave. The weight of the polariser with mounting mechanism is hopefully acceptable. For example, the following steps could be used in the measurement.

Step 1: make normal measurement with TRP and TRS without the polariser.

Step 2: place the polariser in front of the probe(s), then make one more measurement. This would serve as the third LP measurement.

Proposal 2: a linear polariser can be placed in front of the chamber measurement antenna in a direction other than E and E to serve as the third measurement required in Proposal 1 if existing systems have difficulty in rotating chamber measurement antenna and if devices self-declare use of circular polarisation.

If CP axial ratio is not of any interests, viz, only TRP and TRS are required, the TRP and TRS under circular polarisation can be derived from two orthogonal LP measurements as shown below assuming devices employ circular polarisation. This means no change to the existing TRP and TRS measurement configuration.

Figure 1  Elliptic polarisation layout

Figure 1 shows the layout an elliptic polarisation with its major and minor axes coinciding with X- and Y-axis for easy of derivation. Pa and Pb are two points on the ellipse with an angle of 90 degrees between them. Namely the vector (0,Pa) and (0,Pb) are orthogonal to each other.

A typical ellipse can be described by the equation 1 with c and d representing the lengths of major and minor axes. If the ellipse represents elliptic polarisation, then the associated total power would be equal to c2+d2.

x2c2+y2d2=1                                               equ.1

Any point Pa on an ellipse can be represented by { c*cos() , d*sin() }. A point Pb at right angle relative to Pa can be represented as { c*cos(±) , d*sin(±) }, where the sign ± denotes two cases, viz Pb is 90 degree ahead of Pa counter clockwise if it is plus, or 90 degrees behind if it is minus. In the context of circular polarisation, the sign ± would denoted CP handedness. If LP measurements are made along the directions of Pa and Pb, e.g. E and E in TRP and TRS methodology, the associated power would be expressed in equation 2.

[c*cos()]2 + [d*sin()]2 + [c*cos(±)]2 + [d*sin(±)]2                        equ.2

Equation 2 can be re-written as equation 3 by using trigonometry identities.

[c*cos()]2 + [d*sin()]2 + [c*sin()]2 + [d*cos()]2 = c2 + d2                      equ.3

Equation 3 implies that the total power of an elliptic polarisation can be measured by LP measurement in any two orthogonal directions. In other words, there is no need to derive the phase difference between the major and minor axes and inclined angle of the elliptical polarisation to arrive at total radiated power.

In the context of TRP and TRS measurement methodology, the two orthogonal directions for LP measurements can be chosen as E and E and existing procedures can still be used to derive CP performance using LP measurements. The shortcoming of this approach is that CP axial ratio cannot be calculated.

Proposal 3: TRP and TRS of circular polarisation can be derived from two linear polarisation measurement in orthogonal directions assuming devices employ circular polarisation if devices self-declare use of circular polarisation. Hence existing TRP and TRS test procedures can be used for circular polarisation without the ability to assess axial ratio and CP handedness.

Currently most, if not all, devices employs linear polarisation. In those cases, the measured TRP and TRS using existing test procedures under linear polarisation needs to be “mapped” to a circularly polarised receiver or transmitter. In the worst case, such mapping would imply a 3dB loss. However, satellite antennas using circular polarisations might not be perfect in the sense that the phase difference between two orthogonal components may not be exactly 90 degrees and their amplitudes could differ too. There are two ways to map such loss.

- Apply a fixed loss. This means existing test procedures reflect performance under circular polarizations.

- Multiple the E and E at every grid point by a circular polarisation vector with agreed imperfections.

Regardless how the loss is mapped, existing test procedures using linear polarisations could reflect circular polarisation performance. However, this does not encourage devices to use circular polarisation to improve real network performances.

Observation 1: existing test procedures using linear polarisations could reflect circular polarisation performance. However, this does not encourage devices to use circular polarisation to improve real NTN network performances.

# Conclusion

This contribution makes the following proposals.

Proposal 1 : to adopt the method in reference [3] to derive circular polarisation TRP and TRS and axial ratio using 3 linear polarisation (LP) measurements with two out of three LP measurements orthogonal to each other to align with E and E in order to minimise the impact on the existing TRP and TRS test procedures if devices self-declare use of circular polarisation.

Proposal 2: a linear polariser can be placed in front of the chamber measurement antenna in a direction other than E and E to serve as the third measurement required in Proposal 1 if existing systems have difficulty in rotating chamber measurement antenna and if devices self-declare use of circular polarisation.

Proposal 3: TRP and TRS of circular polarisation can be derived from two linear polarisation measurement in orthogonal directions assuming devices employ circular polarisation if devices self-declare use of circular polarisation. Hence existing TRP and TRS test procedures can be used for circular polarisation without the ability to assess axial ratio and CP handedness.

Observation 1: existing test procedures using linear polarisations could reflect circular polarisation performance. However, this does not encourage devices to use circular polarisation to improve real NTN network performances.

# Reference

- R4-2419808 WF for [113][325] TRP_TRS_Ph3

- Improved Measurement Method of Circularly-Polarized Antennas Based on Linear-Component Amplitudes, D. Wang, et al, Open Journal of Antennas and Propagation, Vol.5, pp.36-45, March 2017, DOI: https://doi.org/10.4236/ojapr.2017.51004

- Determining Antenna Polarization Using Amplitude-Only Measurements With Linear-Polarized Antenna in Three Positions, R. Zentner, et al, IEEE Access, 2024, Vol.12, pp.136542-136564, DOI: 10.1109/ACCESS.2024.3463477