3GPP TSG-RAN WG4 Meeting #114												R4-2500043

Athens, Greece, 17 – 21 February, 2025

Agenda item:			7.14.2.3

Source:	Huawei, HiSilicon

Title:	on MIMO OTA tests

Document for:	Approval

# Introduction

This contribution discusses the issues listed below in WF [1] from the last meeting.

Issue 1-3: Dynamic MIMO OTA test and UE performance metric

Agreement:

- For discussing performance metric perspective, further discuss whether the measurement results of UMi and UMa should be combined.

- Down-selection of a single channel model for performance requirements is not precluded.

Issue 2-1-1: Target bands for Rel-19 MIMO OTA requirements

Agreement:

- Single band is needed to replace n79. Which band is FFS.

- Companies are encouraged to share views on how to down-select within n7, n38, and n20. Consider the consistency with Rel-19 TRP TRS requirements work.

Issue 2-1-3: Device type for MIMO OTA measurement campaign

Agreement:

- Rel-19 MIMO OTA TRMS measurement campaign is targeted on smart phone without power class restriction.

# Discussion

Regarding issue 1-3, UMa and UMi channels represent totally different propagation conditions. It is in the interests of the network operators to include a variety of conditions. Besides, tests under UMa and UMi channels can be carried out separately and combining the data from two channels is via data processing after the measurement. Therefore, it would be useful to include both UMa and UMi channels.

Proposal 1: include both UMa and UMi channels via post measurement data processing.

Issue 2-1-1 and Issue 2-1-3 are related in the sense of reference sensitivity reduction allowed from PC3 to PC2 in FDD bands as given in Table 7.3.2-1c for non TxD and Table Table 7.3.2-1d for TxD in 38.101-1 [2]. The latest WID [3] indicates band n3, n7, n8, n20, n77 for MIMO OTA. However, both band n3 and n8 have allowed reference sensitivity degradations going from PC3 to PC2. In other words, if Rel-19 MIMO OTA TRMS measurement campaign is targeted on smart phones without power class restriction, PC2 and PC3 MIMO OTA performance for band n3 and n8 could be different. Because different conducted reference sensitivity values would, by inference, lead to different MIMO OTA performance values for PC3 and PC2, especially under noise limited scenarios.

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n14 | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 1.2 | 1.2 | 1.3 | 1.2 | 1.2 | 5.8 | 6.0 | 6.5 |  |  |
| n3 | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n8 | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n14 | 1.1 | 1.3 |  |  |  |  |  |  |  |  |
| n25 | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

The above reference sensitivity reduction from PC3 to PC2 in FDD bands is at maximum power transmission, viz. 23dBm for PC3 and 26dBm for PC2. In this case, the agreement in issue 2-1-3 may not produce any adverse effect in MIMO OTA measurement campaign due to the fact that 15dBm uplink transmit power is used in MIMO OTA tests. However, there could be a MIMO performance impact in deployed networks, e.g. near cell edge or in a deep fade, where maximum UE transmit power is required. In other words, the current MIMO OTA test settings do not fully reflect the scenarios in deployed networks.

Observation 1: the agreement under issue 2-1-3 in WF of RAN4 #113 may not produce any adverse effect in MIMO OTA measurement campaign. However, PC2 devices in FDD bands may have a degraded MIMO performance at or near maximum transmit power.

In fact, the uplink transmit power of 15dBm in MIMO OTA tests was selected based on tests from [4]. However, no explanation for the MIMO performance degradation was given when uplink transmit power is above 15dBm as shown in Figure 2 from [4]. Assuming the cause is power leakage from Tx to Rx, it would be interesting to replicate the tests shown in Figure 2 from [4] using a NR TDD band.

Observation 2: it would be interesting to replicate the tests shown in Figure 2 from R4-1704080 using a NR TDD band to ascertain the cause MIMO performance degradation when uplink transmit power is above 15dBm in FDD bands.

Regarding issue 2-1-1, the WID [3] lists band n3, n7, n8, n20, n77 as priority. To target 4 bands, it would be representative to select band n3, n7, n77, (n8 or n20). Namely choose one band between n8 or n20 because both of them are regarded as low bands.

Proposal 2: down select the following four bands: n3, n7, n77, (n8 or n20) and choose either n8 or n20 to represent low band.

# Conclusion

This contribution makes the following proposals and observations.

Proposal 1: include both UMa and UMi channels via post measurement data processing.

Observation 1: the agreement under issue 2-1-3 in WF of RAN4 #113 may not produce any adverse effect in MIMO OTA measurement campaign. However, PC2 devices in FDD bands may have a degraded MIMO performance at or near maximum transmit power.

Observation 2: it would be interesting to replicate the tests shown in Figure 2 from R4-1704080 using a NR TDD band to ascertain the cause MIMO performance degradation when uplink transmit power is above 15dBm in FDD bands.

Proposal 2: down select the following four bands: n3, n7, n77, (n8 or n20) and choose either n8 or n20 to represent low band.

# References

- R4-2419910 WF for [113][326] MIMO_OTA_Ph3

- TS 38.101-1 v18.5

- RP-243277 Revised WID: WI on FR1 TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3

- R4-1704080 Uplink power control for MIMO OTA