3GPP TSG-RAN WG4 Meeting #114												R4-2500044

Athens, Greece, 17 – 21 February, 2025

Agenda item:			7.14.3.1

Source:	Huawei, HiSilicon

Title:	on TRP and TRS tests

Document for:	Approval

# Introduction

This contribution discusses the issues listed below in WF [1] from the last meeting.

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis)

Agreements:

  - RAN4 further discuss how to specify requirements for Size 2 (narrow) UE.

  - Further align the understanding on target of preliminary measurement campaign.

    - Option 1: for defining the offset value, on top of size 1 requirements

    - Option 2: for collecting data to generate CDF and define requirements based on CDF ( this is traditional measurement campaign, the difference is only less number of devices required)

Issue 4-1-4: 2Tx related MU impacts

WF

  - FFS

Issue 4-2-7: How to specify 1Tx requirements in Rel-19

Agreements:

  - Further discuss whether a proper methodology (e.g., threshold) is needed to identify and exclude outlier results of TRP TRS data pool.

# Discussion

To minimise effort on measurement campaign for smartphone size 2 using PDA hand, it is convenient to use TRP and TRS requirements for smartphone size 1. If this is agreeable, TRP and TRS requirements for smartphone size 2 in other bands can also adopt this approach in future. This would constitute a sizable saving in test time.

Proposal 1: apply the size 1 TRP and TRS requirements to size 2. Namely, offset=0 in option 1 under issue 4-2-1 in WF R4-2419808.

Regarding issue 4-1-4, the text from B.2.7.1.1 in [2] is reproduced for reference.

The uncertainty limits of this effect are calculated by means of the following formula (uj22 of [12]):

Phase centre limits%=± estimated offset from the axis of rotationrange length·100

Normally antennas are located on the rim of a smartphone for FR1 frequencies. Typically the main transmit antenna in 1Tx case is placed on either the top or the bottom edge of a smartphone. The fact that current term MU for 1Tx described in Annex B.2.7.1.1 of [2] is zero means that top, bottom as well as side edges of a smartphone is considered as the centre of rotation. In case of 2Tx with one antenna on the top edge and another at bottom edge, the equivalent centre for 2Tx is in the middle or gravitational centre of a smartphone. Meaning that the 2Tx case could be closer to, certainly not further away from, the centre of rotation than 1Tx case. By interference, the MU term under B.2.7.1.1 for offset of DUT phase centre from axis(es) of rotation should remain zero for 2Tx cases.

Proposal 2: the MU term under B.2.7.1.1 in TS 38.870 for offset of DUT phase centre from axis(es) of rotation should remain zero for 2Tx cases.

Regarding issue 4-2-7, the previous decision making using CDF from measurement campaign should be used. It is difficult to define a criterion for outliers.

Proposal 3: use CDF to determine TRP and TRS thresholds without identifying outliers.

# Conclusion

This contribution makes the following proposals.

Proposal 1: apply the size 1 TRP and TRS requirements to size 2. Namely, offset=0 in option 1 under issue 4-2-1 in WF R4-2419808.

Proposal 2: the MU term under B.2.7.1.1 in TS 38.870 for offset of DUT phase centre from axis(es) of rotation should remain zero for 2Tx cases.

Proposal 3: use CDF to determine TRP and TRS thresholds without identifying outliers.

# References

- R4-2419808 WF for [113][325] TRP_TRS_Ph3

- TS 38.870 v18.3 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)