3GPP TSG-RAN4 Meeting #114	R4-2500085

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.19.3

Source:	VIAVI Solutions Ltd

Title:	Views on outstanding Testability and Interoperability Issues for Beam Management

Document for:	Discussion

# Introduction

3GPP defined a normative work including beam management testability and interoperability aspects and RAN4 needs to specify core requirements for this use case. The latest updated work item description is given in RP-242399 [1].

In this document we discuss three remaining decisions identified in R4-2418151 [2] on testability and interoperability issues for beam management:

- Beam prediction KPIs (Sub-topic 3.4)

- Measurement error impact evaluation (Sub-topic 3.6)

- Channel model for beam prediction (Sub-topic 3.11)

# Discussion

## 2.1 Beam Prediction KPIs

Different metrics/KPIs have been discussed and captured in the TR. For beam management requirements/tests, the following test metrics are identified and could be considered:

- Option 1: RSRP accuracy

- Option 2: Beam prediction accuracy

  - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

  - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

  - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

- Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

  - Related measurement accuracy can be considered to determine x

- Option 4: combinations of above options

The current proposals as outlined in RAN4#113 are as follows:

- Option 1: Use Option 1

- Option 2: Use Option 2

- Option 3: Use combined Option 2 and 3:

- Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB”

    - FFS: K = 2 and x = 3 as baseline

- Option 4: Add new metric:

    - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived.

- Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference).

    - Consider defining multiple tests under different conditions but with the same outcome(same predicted beam)

- Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction

    - RSRP absolute accuracy applies to all predicted beams

In general, it is important to predict a set of strong beams from the candidate beams and hence beam prediction accuracy is an effective metric to measure the successful beam predictions. All three sub-options can be used depending on the size of the prediction output. To allow the network to perform the best beamforming, apart from beam indices, the RSRP values for predicted beams should be accurate as well. This could potentially provide elimination of less dominant beams for a scenario where the UE is configured to provide K beam predictions, but the total number of dominant beams (beams with large RSRP values) are less than K. As another potential scenario, the network can apply a beamformer using a combination of predicted beams to transmit the data after beam prediction stage. In this case, accurate RSRP estimates are required to adjust the weights of the predicted beams in the combination. As a result, both beam prediction accuracy and RSRP accuracy are quite important to measure beam management performance.

Proposal 1: Adopt Option 6 where both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction.

## 2.2 Measurement Error Impact Evaluation

RSRP Error modeling in simulations

Initial simulation assumptions for assessing the impact that the measurement error has on prediction accuracy/ performance were agreed in the previous RAN4 meeting. The RSRP measurement error modeling should be further discussed/refined.

Issue 3-6:	Measurement error impact evaluation

- Proposals:

  - Option 1: model RF error as same within an RF module but varies across modules, FFS the distribution

  - Option 2: Gaussian distribution with +/- 2.5dB for BB errors and total of +/-6.5 for BB+RF

  - Option 3: model RF error as

    - An offset value in the original dataset;

    - Truncated Uniform distribution;

    - Truncated Gaussian distribution.

- Recommended WF

  - Model error as RF error + BB error

    - RF error as a fixed bias for each UE module, FFS which distribution

    - BB error with Gaussian distribution with 0 mean and sigma=0.6dB

Should the BB error distribution be changed based on SNR?

RF errors result from the hardware impairments of the RF modules in the transceiver chains and can be modelled randomly without any SNR impact. On the other hand, BB errors are affected from signal processing operations where RSRP measurement is applied. In general, the known pilot symbols (such as PSS, SSS, DMRS) are used to measure RSRP values at the UE side where receiver thermal noise and intra/inter-cell interference affect the measurement performance. In that sense, BB errors can be modelled again randomly but with relation to SNR (or SINR).

Proposal 2: Adopt Option 2 where error range (or standard deviation) for the BB part is a function of SNR (or SINR). For RF part, a constant error range (or standard deviation) can be used.

## Channel Model for Beam Prediction

- Proposals

  - Option 1: multi-AoA TDL

  - Option 2: simplified CDL

  - Option 3: further study both multi-AoA TDL and simplified CDL

  - Option 4: CDL without simplification

  - Option 5: others

- Recommended WF

  - To be discussed whether the channel model can be downscoped to Option 1 or Option 2. If Option 3 is preferred, it should be clarified what needs to be analyzed further.

Aspects to be analysed:

- How to simplify the CDL channels. Is it necessary to stick to currently used CDL models (CDL-A,.., CDL-D) or can the channels be further simplified to just a few clusters

- How many spatial beams can be generated with multi-AoA TDL(can the TE emulate beams reaching the UE between the AoDs) and what is the relationship between angular distance between the probes and emulated beams/codebook.

- Others?

CDL channel models are useful for beamforming applications as they allow to model different multipath channel components with sufficient angular resolution. RAN 1 study uses CDL channel models for link-level simulations [3]. Although existing CDL channels are accurate for beam prediction framework, there is no strong need for using exact CDL models. Beamforming operation cancels many multipath components in the angular region not covered by the beamwidth of the transmission and hence it might be possible to simplify the CDL models by limiting the total number of multipath components in the CDL model. It is also possible to put extra constraints on angle and delay values for multipath components to be modelled considering the beamforming weights to be applied.

Proposal 3: Adopt Option 2 where simplified CDL channel models are used. Put a limit for total number of clusters and constraints for angle/delay values for different clusters depending on the beamforming operation.

# Conclusion

We make the following proposals in this document:

Proposal 1: Adopt Option 6 where both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction.

Proposal 2: Adopt Option 2 where error range (or standard deviation) for the BB part is a function of SNR (or SINR). For RF part, a constant error range (or standard deviation) can be used.

Proposal 3: Adopt Option 2 where simplified CDL channel models are used. Put a limit for total number of clusters and constraints for angle/delay values for different clusters depending on the beamforming operation.

# References

- RP-242244, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#106, Madrid, Spain, December 9-12, 2024.

- R4-2418151, “Topic summary for [113][129] NR_AIML_air”, Qualcomm, RAN4#113, Orlando, FL, USA, November 18-22, 2024.

- 3GPP TR 38.843, Table 6.3.1-2