3GPP TSG-RAN WG4 Meeting #114                                   R4-2500118

Athens, Greece, February 17 – February 21, 2025

Source: 	Murata Manufacturing Co Ltd.

Title: 	Discussion on UE RF requirement impact for DL fragmented carriers

Agenda item:	7. 6. 3

Document for:	Discussion

Introduction

In the last RAN4#113 meeting, the topic on DL fragmented carriers was discussed and the WF [1] was approved. It was agreed that companies are encouraged to bring analysis with their own assumptions following the table format in [2]. In this contribution we discuss the reference sensitivity degradation derivation for PCC and SCC further and provide our view.

Discussion

DL fragmented carriers study item was introduced recently. The purpose of this SI is to reduce the number of Rx signal paths from two to one supporting non-contiguous DL CA and to utilize the remaining Rx signal path for another CC to increase the data throughput. This is because some operators have non-contiguous or fragmented carriers within their own licensed band.

The reference UE RF architecture (one Rx RF chain) has been agreed within the companies but to go further forward, the UE RF requirement impacts should be carefully reviewed. One of the important RF requirements is the reference sensitivity so we only focus on how much reference sensitivity would be impacted for DL fragmented carriers in this contribution. In other words, the in-gap interferer requirements such as ACS, IBB, NBB or other operator’s DL CC from same BS location are not within the scope of this paper.

From LTE non-contiguous DL CA discussion, the ΔRIBNC concept has been introduced for relaxation of DL SCC due to UL PCC Rx band noise. The requirement is highly dependent on the BW of PCC and SCC, Wgap and UL PCC RB allocation. It should be noted that this requirement is derived based on the RF front end block diagram which divides the path for each CCs after external LNA or before down conversion mixer of RFIC receiver as shown in Figure 1 in [3]. This kind of block diagram can be referred as partially shared architecture since the duplexer and external LNA support both the PCC and SCC DL path.

It is well analysed in [3] that below 1) ~ 3) factors are not affecting the sensitivity performance of one Rx RF chain compared to the shared RF architecture. However, we can expect the increased noise figure from AGC adjustment and decreased ADC SNR due to Tx leakage 4) would result in the additional composite noise and sensitivity degradation of PCC and SCC [2], [3]. The main cause of this is the lack of attenuation of low pass filter (LPF) embracing both PCC and SCC down conversion signals inside RFIC since the cutoff frequency of the LPF is much higher than that of LPF which only covers one CC each.

- Rx band noise due to PCC Tx nonlinearity (RFIC impairment, PA noise and duplexer isolation)

- DL image falling into another DL CC (image rejection ratio of down conversion mixer)

- IMD2 noise due to Tx leakage (duplexer isolation and IIP2 of receiver)

- Increased noise figure from AGC adjustment and decreased ADC SNR due to Tx leakage (LPF attenuation)

Table 1 shows the proposed sensitivity degradation calculation for partially shared architecture and one Rx RF chain architecture. The main difference compared to the table in [2] is that the proposed calculation format is focused on the composite noise level at the PRX and DRX antenna port separately. Then the sensitivity degradation is derived from MRC with diversity gain.

To calculate the ΔRIBNC for one Rx RF chain, the existing ΔRIBNC for partially shared architecture needs to be analysed first. In other words, the additional interferer noise level, i.e. UL PCC Rx band noise should be calculated reversely based on the specified ΔRIBNC in requirements (e.g. 4.7dB for Band 3) and assumed interferer noise imbalance between PRX path and DRX path (-88dBm at PRX and -97dBm at DRX with 9dB interferer noise imbalance). After that, this interferer noise is equally applied to one Rx RF chain calculation. It should be noted that there would be no interferer noise at PCC for both RF architectures for both PRX path and DRX path.

The detailed analysis and difference from table in [2] will be discussed in the following sections.

Table 1 Link budget table for ΔRIBNC calculation

Consideration on LNA and mixer down-conversion gain

From the analysis in [2] and [3], the Tx leakage level going into LPF of one Rx RF chain architecture is the most impacting factor for the reference sensitivity degradation. Therefore, the LNA and mixer down-conversion gain should be considered. It is RFIC implementation dependent, so different companies might have different numbers.

Observation 1: The LNA and mixer down-conversion gain should be considered, but it is RFIC implementation dependent.

Consideration on different Tx leakage level at PRX and DRX

Considering the RFFE architecture for FDD (PRX path through duplexer and DRX path through antenna coupling), it is expected that Tx leakage level going into DRX path is lower than PRX path due to antenna isolation between PTX antenna and DRX antenna. In other words, the two additional noise increases, described in the following sections, should be considered separately for the PRX path and the DRX path. This applies to both PCC path and SCC path.

Increased noise figure from AGC adjustment at DRX

In the table in [2], the NF is increased from AGC adjustment since Tx leakage is less attenuated through low pass filter after down conversion mixer compared to the partially shared RF architecture. As previously mentioned, the Tx leakage level going into DRX path should be lower than PRX path, which means the NF increase from AGC adjustment in DRX might be less than PRX as in Table 1 (e.g. 10dB vs 8dB). It is worth noting that the Tx leakage level difference between PRX path and DRX path can be up to 18dB in this example (-26.4dBm vs -44.4dBm), which means the noise figure increase at DRX path might be less than 8dB depending on the Rx gain table operation algorithm.

Figure 1 shows the calculated ΔRIBNC depending on the NF increase from AGC adjustment at DRX path after MRC. Both PCC and SCC ΔRIBNC show around 7dB difference from 0dB NF increase and 10dB NF increase at DRX path. Therefore, the NF increase from AGC adjustment at DRX path should be considered separately for accurate calculation.

Figure 1. Calculated ΔRIBNC versus NF increase from AGC adjustment at DRX path

Observation 2: The NF increase from AGC adjustment at the DRX path should be considered separately, but it is RFIC Rx gain table dependent.

    - Noise at ADC

Similarly, the ADC noise would be different for PRX path and DRX path for both PCC and SCC since the Tx leakage level going into ADC would be different. That means the ADC noise for DRX path would be less as in Table 1 (-82.4dBm vs -100.4dBm, 18dB difference) for both PCC and SCC with same ADC ENOB.

Observation 3: The ADC noise should be considered for the PRX path and the DRX path separately, but it is ADC design dependent.

Sensitivity degradation using MRC

It should be noted that the reference sensitivity is calculated after correlated MRC for PRX path and DRX path. The DRX path, having a lower interferer noise level, reduces the MSD level by leveraging diversity gain.

As discussed before, the increased noise figure from AGC adjustment and ADC noise for DRX (PCC, SCC) should be lower than PRX (PCC, SCC). Also, the RX band noise from PCC TX is less dominant for DRX in SCC due to antenna isolation (-88dBm vs -97dBm). In conclusion, the resulting sensitivity degradation would be different or less when considering MRC of PRX and DRX noise compared to the case where MRC is not considered. Therefore, we propose to use MRC of PRX and DRX noise for the sensitivity degradation link budget as in Table 1.

Observation 4: Sensitivity degradation from MRC would be less than the case where MRC is not considered.

Based on the above discussion, we would like to propose below.

Proposal 1: RAN4 to use the revised Table 1 to calculate ΔRIBNC as a starting point with consideration of observation 1~4.

Conclusion

In this contribution, we discussed the UE RF requirement impact for DL fragmented carriers especially for reference sensitivity and provided our view as follows:

Observation 1: The LNA and mixer down-conversion gain should be considered, but it is RFIC implementation dependent.

Observation 2: The NF increase from AGC adjustment at the DRX path should be considered separately, but it is RFIC Rx gain table dependent.

Observation 3: The ADC noise should be considered for the PRX path and the DRX path separately, but it is ADC design dependent.

Observation 4: Sensitivity degradation from MRC would be less than the case where MRC is not considered.

Proposal 1: RAN4 to use the revised Table 1 to calculate ΔRIBNC as a starting point with consideration of observation 1~4.

References

[1]	R4-2420337, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc, 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[2]	R4-2418718, “On RF requirements of fragmented carriers”, Huawei, HiSilicon, 3GPP TSG RAN WG4 Meeting #113, Orlando, US, November 18th – 22nd, 2024

[3]	R4-2415360, “Discussion on UE RF requirement impact for DL fragmented carriers”, Murata Manufacturing Co Ltd., 3GPP TSG RAN WG4 Meeting #112-bis, Hefei, China, October 14th – 18th, 2024