3GPP TSG RAN WG4 #114															     R4-2500216

Athens, GR, February 17th – 21st 2025

Agenda Item:		7.28.2

Source:				InterDigital France R&D, SAS

Title:		NR_XR_Ph3 RRM core requirements analysis

Document for:		Discussion and decision

# Background

During RAN1 Meeting #117, an LS to RAN4 [1] related to the UE assistance information on skipping RRM measurement occasions for NR XR has been approved. Further, in RAN1 Meeting #118 an agreement was made on the NW scheduling indication for a skipping occasion [2].

In the last RAN4 meeting #113, the discussions on this topic progressed. As a result, an LS  reply in [4] has been approved while a WF has been agreed in the meeting in [3].

# Discussion

In this contribution, we share our analysis on gap skipping measurements occasions for NR XR case and the NW scheduling mechanism. The LS reply from RAN4 to RAN1 mentions:

RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability. It is also RAN4 understanding that the time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network according to TS 38.331. Furthermore, from RAN4 RRM requirements perspective, Alt 1-1 is not supported for EN-DC and NE-DC scenarios. Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication-based solutions to skip measurement gap occasions.

In RAN4 meeting #113, the agreed WF in [3] treats a variety of topics that we will address in the following sections. The inclusion of the semi-static indication-based solutions from RAN4 perspective in the response LS to RAN1 will be addressed as well in the next section.

# NR deployment scenarios and MEASUREMENT types

In the last meeting, the following agreements and were made:

## Issue 1-1: Deployment scenarios < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC

One of the FFS points was about the NR dual connectivity support for this feature. In our view, dual connectivity scenarios are more complex due to the involvement of two scheduling entities, that may be uncorrelated when considering the UE measurements gap skipping through DCI signaling. The scheduling in DC deployments may lead to colliding DCI gap skipping orders, and thus more difficult to address.

Also, if there is an application of some flavor of semi-static indication mixed with dynamic solutions this may be for the NR-DC case.

Observation 1: NR-DC deployments involve different schedulers and thus supporting gap skipping may lead to combined solutions involving semi-static and or UE assistance with dynamic DCI gap skipping orders.

Proposal 1:  If NR-DC gap skipping is to be supported, semi-static indication/UE assistance combined with DCI dynamic indications solutions are required.

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported - Option 1a: Support skipping of measurements without gaps - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported - Option 2a: Measurement skipping does not apply for measurements without gaps - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction.

In our opinion, the WI targets the L3 scheduled measurements gaps, and thus we propose to continue discussions considering Option 2 from the above list.

Proposal 2: Support Option 2a: Measurement skipping does not apply for measurements without gaps.

## Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures. - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures. - Option 2a: Measurement skipping does not apply for L1 measurements and procedures - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

In our view, the measurement gaps are not supposed to affect the intra-frequency operations and procedures. Thus, we are proposing supporting Option 2 for further work.

Proposal 3: Support Option 2a: Measurement skipping does not apply for L1 measurements and procedures.

## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM - Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping. - Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

LTM is supposed to resolve the latency problem in mobility and reduce the HO failure. Thus, related measurements, triggering conditions and delay shall not be affected by this feature. We also supported to treat in RAN4 L3 measurements, a RAN1 decision.

Proposal 4: Support Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM.

.

# Measurement with gaps

The WF from last meeting describe a few options on what gaps type to consider moving forward.

Issue 2-1 and Issue 2-2 are to be discussed again:

## Issue 2-1: General approach for defining requirements for measurements with gaps <Agreement>: - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

The following issue reiterates the assumed definitions to be used in discussions and ask for further input.

## Issue 2-2: Types of measurement gaps <Way forward>: - In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG.

In our view, Type-1 should be the priority, as they are supported by all UEs, while Type-2 and/or concurrent gaps should be a second priority. This is simply a direct alignment with the previous agreement on Issue 2-1.

Proposal 5: Support Type 1 as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

# XR skipping requirements impact

In this section, we assume that L3 measurements are considered. That means average is used over the measurement samples. If the accuracy is to be maintained, then the number of samples should be preserved, meaning a delay time will be assumed.

## Issue 3-1: How to capture impact of measurement skipping <Way forward>: Proposals related to measurement delay requirements: - Option 1: Proposals without distinction of XR and legacy requirements - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended. - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. - Option 2: Proposals with distinction of conditions where measurement impact is not needed - Option 2a: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. - Option 2b: - Apply legacy measurement delay and accuracy requirements for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged

At this stage, we support Option 2a which offer more flexibility.

Proposal 6: Support Option 2a:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Options based on scaling factor - Option 1a: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 1b: Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. - Option 1c: introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor. - Option 1d: pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor). - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Option 2a: dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

In our view, if the delay calculation is required, the simplest way is to follow Option 2 which offers a direct mode of calculation/evaluation.

Proposal 7: Support Option 2a: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

# Need/feasibility of UE assistance information

The WF mention the following options:

## Issue 5-1: General on UAI <Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Proposals related to measurement occasions - Option 1: Define UAI related to measurement occasions - Option 1a: number of occasions which can be skipped - Option 1b: number of measurement occasions that are needed for measurement - Option 1c: UE indicates NW a status whether it can tolerate measurement cancellation. - Option 1d: patterns of gaps positions of MG that can be skipped - Option 1e: study how UE assistance information/UE capability is reported when DRX is configured. - Option 1f: Maximum number of consecutive MGs that can be skipped - Option 2: - Option 2a: Do not define UAI related to measurement occasions. - Option 2b: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution - Option 2c: RAN4 will not specify any XR-related requirement, based on XR UAI. - Option 3: The information about gaps not used for measurement are reported as UE capability. - Option 4: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions. - Proposals related to channel conditions - Option 1: For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

If the UAI is to be supported, then it shall contain a bitmap, or a skipping rule based on the RRC configured gaps. The UE gets configured with a L3 gaps by network. Then UE, based on the configuration received replies to the network with a bitmap or a sort of gaps skipping rule.

If the UAI is not supported, then the network will make its own decisions when to skip a gap based on pure buffer status and channel conditions.

We don’t have a strong opinion. However, for UAI case we prefer to leave the gap skipping rules for UE implementation.

Proposal 8: If UAI mechanism is agreed, the rules for UAI gap skipping shall be left for UE implementation.

# additional skipping schemes

In the last meeting, in addition to the dynamic schemes, semi-static schemes were brought back into discussion even though RAN1 already discussed and excluded them one by one.

Here are the options retained in the WF:

## Issue 6-1: Semi-static schemes <Way forward>: - Option 1: Follow RAN1 working assumption and consider DCI only. - Option 2: also consider RRC/semi-static approach. - Option 2a: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. - Option 2b: RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion. - Option 2c: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. - Option 2d: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2e: In addition to DCI solution, it is proposed to introduce semi-static schemes. - Option 3: - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported). - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

In our opinion, RAN4 should focus on the dynamic scheme that was agreed in RAN1. Thus, we are strongly supporting Option 1 from the above list, assuming NR-DC is not part of Rel-19 objectives for XR gap skipping.

Proposal 9: Support Option 1: Follow RAN1 working assumption and consider DCI only.

Another topic is related to the skipping indication on frequency domain, which can be correlated with the gap configuration. The following proposals were retained in the WF:

<Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3: It needs to be clarified that when per-FR gaps are configured, the indication of the particular gap to be skipped by DCI or MAC-CE/RRC is for which gap pattern(s).

From the feature coverage perspective, we believe that Proposal 1 has the potential to resolve the issue. The second proposal may be covered by Proposal 1 if per-FR gap configuration capability is there since there will be a direct applicability to all the CCS falling in the same FR gap. For Proposal 3, we believe that would be applicable to CA or NR-DC case (if supported). But for simplicity, it seems that Proposal 1 would cover most of the cases.

Proposal 10: Support Proposal 1 from the above listed options:

Proposal 1: For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

# Conclusions

This contribution discussed our analysis related to measurements occasions skipping for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: NR-DC deployments involve different schedulers and thus supporting gap skipping may lead to combined solutions involving semi-static and or UE assistance with dynamic DCI gap skipping orders.

Proposal 1:  If NR-DC gap skipping is to be supported, semi-static indication/UE assistance combined with DCI dynamic indications solutions are required.

Proposal 2: Support Option 2a: Measurement skipping does not apply for measurements without gaps.

Proposal 3: Support Option 2a: Measurement skipping does not apply for L1 measurements and procedures.

Proposal 4: Support Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM.

Proposal 5: Support Type 1 as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

Proposal 6: Support Option 2a:

      - For the cases where measurement impact is needed

        - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

          - FFS how to extend.

      - For the cases where measurement impact is not needed

        - FFS the feasible conditions and necessary UE assistance information.

Proposal 7: Support Option 2a: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

Proposal 8: If UAI mechanism is agreed, the rules for UAI gap skipping shall be left for UE implementation.

Proposal 9: Support Option 1: Follow RAN1 working assumption and consider DCI only.

Proposal 10: Support Proposal 1 from the above listed options:

Proposal 1: For the gap/restriction skipping impact in frequency domain,

    - For the gap skipping, depend on whether the UE capable of per-FR gap

      - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

      - Otherwise, no need to distinguish different FR.

    - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

# References

- R1-2405736, “LS on UE assistance information”, RAN1 Meeting #117, Fukuoka, Japan, May 2024

- R1-2407512, “LS on gaps/restrictions that are caused by RRM measurements”, RAN1 Meeting #118, Maastricht, NL, August 2024

- R4-2420103 “WF on RRM requirements for XR_Ph3, RAN4 Meeting #113, Orlando, US, November 2024

- R4-2420198, LS response on gap/restrictions that are caused by RRM measurements, Qualcomm, November 2024, Orlando, US