# 3GPP TSG-RAN WG4 Meeting #114	R4-2500270

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.2

Source: 	OPPO

Title: 	CSI reporting requirement framework for CSI prediction

Document for:	Approval

- Discussion

For CSI prediction, following agreements have been made in RAN4#113 meeting,

Channel model for CSI prediction tests

Agreement:

- For channel model for CSI prediction tests, TDL channel models will be used.

  - FFS on whether CDL channel model is needed for generalization test.

Reference throughput

Agreement:

- reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook

Testing conditions

Agreement:

- use static conditions as baseline.

  - FFS on whether to have non-static condition

According to our understanding, throughput should be utilized to evaluate the model inference performance and existed RAN4 test examples for “reporting of PMI” can serve as a reference, e.g. as captured in 38.101-4, “The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively.”

In terms of the relationship to legacy requirements, as a baseline test, what we define is the minimum performance requirement, which is not to verify how much performance gain an AI/ML function/model can bring. The assessment of performance gain should be considered when RAN1 introduces and evaluates CSI features, rather than when RAN4 defines test cases. Furthermore, under current test conditions, e.g., by TDL channel, even if we define a higher performance testing requirement, it still has little significance for the practical application of AI/ML solutions. Therefore, we can just reuse the legacy PMI requirement (γ) as a baseline test, and let other options/proposals with higher performance requirements be further studied.

The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

γAI/ML=tue_AI/MLtrnd

In the definition of γAI/ML,  [e.g., for 32TX PMI requirements, FFS other TX values], tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

- Note1: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance, γAI/ML could be calculated for each predicted CSI and then do the average within the CSI prediction window, or tue_AI/ML and trnd could be calculated within the CSI prediction window first and then get the averaged γAI/ML value.

- Note2: 32TX PMI could be the baseline, FFS other values.

- Note3: TDL channel could be the baseline, FFS other channel conditions.

- Note4: whether tests should be conducted at a given SNR or a SNR range should be further considered.

Proposal 1: The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

γAI/ML=tue_AI/MLtrnd

In the definition of γAI/ML,  [e.g., for 32TX PMI requirements, FFS other TX values], tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 3: 32TX PMI could be the baseline, FFS other values.

Proposal 4: TDL channel could be the baseline, FFS other channel conditions.

Proposal 5: Whether tests should be conducted at a given SNR or a SNR range should be further considered.

Figure 1 Reference block diagram for 1-sided model

Figure 1 provides the reference block diagram for 1-sided model as agreed previously. For CSI prediction, following steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

Proposal 6: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

In addition, it should be noted that if the test is limited only to the legacy approach (e.g., TDL channel), it is challenging to assert that the performance test outcomes can reflect the effectiveness of the tested model. This is because for a data-driven AI/ML algorithm, the TDL channel model is too simple to mimic. Even if a model achieves good test results on the TDL channel, it is still hard to say that it can work on UMa and CDL channels, let alone in complex field channels. Therefore, besides the TDL channel based tests(TDL could be a baseline test), CSI-related tests may be conducted under CDL channel [or other more practical channel conditions] to check a generalized performance.

Regarding the static/non-static scenarios/conditions for testing, first to consider testing cases in static scenarios and configurations is a good choice. After having some feasible testing cases for static configurations, RAN4 can further consider and evaluate whether to introduce non-static testing scenarios and configurations.

Proposal 7: Besides the TDL channel based tests(TDL could be a baseline test), CSI-related tests may be conducted under CDL channel [or other more practical channel conditions] to check a generalized performance.

Proposal 8: After having testing cases for static configurations, RAN4 can further consider and evaluate whether to introduce non-static testing scenarios and configurations.

- Conclusions

In this contribution, we have discussed the testability and interoperability issue on AI/ML based CSI enhancement and got the following proposals:

Proposal 1: The requirements for AI/ML based CSI prediction(or AI/ML based PMI prediction) could be specified in terms of the ratio:

γAI/ML=tue_AI/MLtrnd

In the definition of γAI/ML,  [e.g., for 32TX PMI requirements, FFS other TX values], tue is 90 % of the maximum throughput obtained at [a given SNR, or a SNR range ] using the precoders configured according to the UE reports, and trnd is the throughput measured at [a given SNR, or a SNR range ] with random precoding.

Proposal 2: γAI/ML could be obtained within a CSI prediction window. If a CSI prediction window contains more than one CSI prediction instance,

Option1: γAI/ML is calculated for each predicted CSI and then do the average within the CSI prediction window

Option2: tue_AI/ML and trnd are calculated within the CSI prediction window first and then get the averaged γAI/ML value.

Proposal 3: 32TX PMI could be the baseline, FFS other values.

Proposal 4: TDL channel could be the baseline, FFS other channel conditions.

Proposal 5: whether tests should be conducted at a given SNR or a SNR range should be further considered.

Proposal 6: For CSI prediction, following test steps could be considered:

- Step1: TE sends CSI-RS to DUT, within a CSI observation window (signal generating and transmitting)

- Step2: DUT conducts the CSI measurement within the CSI observation window (CSI-RS receiving)

- Step3: DUT conducts the CSI prediction within a CSI prediction window (Inference)

- Step4: DUT reports the predicted CSI (CSI reporting)

- Step5: TE calculates the γAI/ML for performance [and calculates the measurement/inference latency for LCM]

Proposal 7: Besides the TDL channel based tests(TDL could be a baseline test), CSI-related tests may be conducted under CDL channel [or other more practical channel conditions] to check a relatively generalized performance.

Proposal 8: After having testing cases for static configurations, RAN4 can further consider and evaluate whether to introduce non-static testing scenarios and configurations.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface