3GPP TSG-RAN4 Meeting #114	R4-2500486

# Athens, Greece,  16th – 21st February, 2025

Title:	On 200 MHz BW and 2 layer MIMO capabilities

Source:	Qualcomm Incorporated

Agenda item:	5.21.1

Release:	Rel-18

Document for:	Approval

# 1.	Introduction

RAN4 received and LS from RAN2 asking if 200 MHz channel BW should be mandatory. LS also asks if UL MIMO should be mandatory. This paper discussed those questions and proposes a response.

# 2. 	Discussion

RAN2 was motivated to ask this since they are discussing which NTN FR2 features should be mandatory and should they copy same mandatory features as TN FR2 has.

200 MHz channel BW deployments happened only very late and that created a problem in enabling this mandatory feature in UEs since there was no network to confirm the functionality with. It was more favorable to deploy 2x100 MHz instead. For NTN, this same problem is like to happen and therefore, 200 MHz channel BW should be made optional. Infect, also 50 MHz channel BW should be made optional too.

Proposal 1: For FR2-NTN, define only 100 MHz as mandatory channel BW.

UL MIMO is hardly relevant for satellite services since channel is almost always AWGN. Despite terminals are dual polarized, we propose nog to mandate UL MIMO support.

Proposal 2: Make UL MIMO support optional for FR2-NTN.

Finally, we propose to respond to the LS [1] with the text in the appendix.

Proposal 3: Agree text for the LS in Appendix.

# Conclusion

We discussed and proposed the incoming LS [1] and proposed  text for the reply in Annex

Proposal 1: For FR2-NTN, define only 100 MHz as mandatory channel BW.

Proposal 2: Make UL MIMO support optional for FR2-NTN.

Proposal 3: Agree text for the LS in Appendix.

# Reference

[1]	R4-2500014, “LS on UE Capabilities for FR2-NTN”, RAN2

# Appendix: Draft LS

3GPP TSG-RAN4 Meeting #114	R4-25xxx

Athens, Greece,  16th – 21st  February, 2025

Title:	LS on LS on UE Capabilities for FR2-NTN

Response to:	LS R4-2500014 (R2-2411195) on LS on UE Capabilities for FR2-NTN  from RAN2

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Ville Vintola

vvintola ( a ) qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

Ran4 respectfully thanks RAN2 for the questions in LS .

Ran4 has discussed the issue of 200 MHz channel BW and made the agreement that only 100 MHz channel BW should be mandatory for FR2-NTN UEs.

In addition, RAN4 also agreed that UL MIMO is not mandatory feature for the FR2-NTN terminals.

# 2	Actions

To: RAN2

ACTION: 	RAN4 asks RAN2 to take the above information in to account in their work

# 3	Dates of next WG RAN4 meetings

3GPP RAN 4  RAN4#114-bis	2025-04-07 - 2025-04-11 China, CN

3GPP RAN 4  RAN4#115 	2025-05-19 - 2025-05-23	Malta, MT