3GPP TSG-RAN4 Meeting #114	R4-2500487

# Athens, Greece, 17th – 21st February, 2025

Title:	n1 NS_05 and NS_05U PC3 A-MPR proposal

Source:	Qualcomm Incorporated

Agenda item:	4.2.1

Release:	Rel-19

Document for:	Approval

# 1.	Background

n1 NS_05 requirements are relaxed since the PHS service is closed. However, the DECT service in the same range 1884.5MHz - 1915.7MHz must still be protected in Japan. The updated Japanese regulations were captured, the PHS protection requirement was modified to protect DECT, and an A-MPR region definition table was agreed for PC3 NS_05 as a starting point for A-MPR re-evaluations [1]. This paper fixes a minor typo in the agreed NS_05 table, proposes new A_MPR regions for NS_05U requirement, and proposes the re-evaluated PC3 A-MPR values for NS_05 and NS_05U requirements for 5MHz, 10MHz, 15MHz and 20MHz channel bandwidths.

# 2. 	Discussion

## 2.1	Measurement and simulation assumptions

We analyze the A-MPR regions and values for n1 NS_05 and NS_05U with the modified PHS requirements, through simulations and measurements.

The simulations and measurements were done for channel bandwidths 5MHz, 10MHz, 15MHz and 20MHz, wherein the channels were analyzed at different Fc values based on analysis in [1], and A-MPR values were proposed accounting for moderate implementation margin to accommodate possible variations in production.

We also use the following impairment levels in our measurements and simulations:

- Carrier feedthrough = -28dBc

- Image = -28dBc

- CIM3 = -60dBc

## 2.2 	Evaluation results and proposals

### 2.2.1	Proposed PC3 NS_05 and NS_05U A-MPR region definitions

Shown below in the tabular figure 1 is the previously agreed NS_05 A-MPR region definitions from [1] in blue, with a minor fix proposed and highlighted in red in the blue area

Proposal 1: Change three > signs to ≥ in agreed AMPR region definition

Below tabular figure 1 also introduces regions Region A4 and Region A5 in right side in grey color to account for NS_05U A-MPR which were not accommodated in the previously agreed regions table.

Figure 1. Proposed PC3 NS_05 and NS_05U A-MPR region definitions

Proposal 2: Add new regions as in Figure 1 to the NS_05U AMPR definition

The NS_05 regions that include combination of Region A1 and Region A2 are represented graphically in the plots in Figure 2 below for various applicable channel bandwidths.

Figure 2. Graphical representation of proposed PC3 NS_05 A-MPR region definitions

The NS_05U regions that include a combination of Region A1, Region A2, Region A4 or Region A5 (based on channel bandwidth) are represented graphically in the plots in Figure 3 below for various applicable channel bandwidths and specific center frequency ranges.

Figure 3. Graphical representation of proposed PC3 NS_05U A-MPR region definitions

### 2.2.2	Proposed PC3 NS_05 and NS_05U A-MPR values

Table 1 below shows the proposed PC3 A-MPR values for NS_05 regions noted in Figure 1.

| NS_05 |  | 5M | 10M @<1930.7 |  | 15M @1927.5 |  | 15M @1932.5 | 20M @<1947.5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A1 | A2 | A1 | A2 | A2 | A1 | A2 |
| DFT-s-OFDM | BPSK | 0.0 | 2.5 | 2.0 | 2.5 | 2.5 | 2.5 | 3.5 | 3.5 |
|  | QPSK |  | 4.0 | 3.0 | 4.0 | 3.5 | 3.0 | 4.0 | 4.0 |
|  | 16QAM |  | 4.0 | 3.0 | 4.5 | 4.0 | 4.0 | 5.5 | 5.0 |
|  | 64QAM |  | 4.5 | 3.5 | 4.5 | 4.0 | 4.0 | 5.5 | 5.0 |
|  | 256QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  | 5.0 | 4.0 | 6.0 | 5.5 | 5.0 | 6.0 | 5.5 |
|  | 16QAM |  | 5.0 | 4.0 | 6.0 | 5.5 | 5.0 | 6.0 | 5.5 |
|  | 64QAM |  | 5.0 | 4.0 | 6.0 | 5.5 | 5.0 | 6.0 | 5.5 |
|  | 256QAM |  |  |  |  |  |  |  |  |

Table 1. Proposed PC3 NS_05 A-MPR values

Table 2 below shows the proposed PC3 A-MPR values for NS_05U regions noted in Figure 1.

| NS_05U |  | 10M @<1930.7 |  |  | 15M @1927.5 |  |  | 15M @1932.5 |  |  | 20M @<1947.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A4 | A2 | A5 | A4 | A2 | A5 | A4 | A2 | A5 | A1 | A2 | A5 |
| DFT-s-OFDM | BPSK | 2.5 | 2.0 | 2.0 | 2.5 | 2.5 | 2.0 | 2.0 | 2.5 | 2.5 | 3.5 | 3.5 | 2.0 |
|  | QPSK | 4.0 | 3.0 | 2.5 | 4.0 | 3.5 | 2.5 | 2.5 | 3.0 | 2.5 | 4.0 | 4.0 | 2.5 |
|  | 16QAM | 4.0 | 3.0 | 3.0 | 4.5 | 4.0 | 3.0 | 3.0 | 4.0 | 3.0 | 5.5 | 5.0 | 3.0 |
|  | 64QAM | 4.5 | 3.5 | 3.0 | 4.5 | 4.0 | 3.0 | 3.0 | 4.0 | 3.0 | 5.5 | 5.0 | 3.0 |
|  | 256QAM |  |  |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | 5.0 | 4.0 | 4.5 | 6.0 | 5.5 | 4.5 | 4.5 | 5.0 | 4.5 | 6.0 | 5.5 | 4.5 |
|  | 16QAM | 5.0 | 4.0 | 4.5 | 6.0 | 5.5 | 4.5 | 4.5 | 5.0 | 4.5 | 6.0 | 5.5 | 4.5 |
|  | 64QAM | 5.0 | 4.0 | 4.5 | 6.0 | 5.5 | 4.5 | 4.5 | 5.0 | 4.5 | 6.0 | 5.5 | 4.5 |
|  | 256QAM |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: When NS_05U is signaled for 5 MHz channel bandwidth, A-MPR is defined in Table 6.2.3.1-2. NOTE 2: When NS_05U is signaled for 10, 15 and 20 MHz channel bandwidths which are placed such that value of Fc is other than the specified ranges in A-MPR region definitions table (xx, TBD), then the A-MPR value is as defined in Table 6.2.3.1-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2. Proposed PC3 NS_05U A-MPR values

Proposal 3: Define new AMPR values for changed NS_05 and NS_05U requirements according to table 1 and table 2 above

# Conclusion

This contribution proposes A-MPR regions and values for PC3 n1 NS_05 and NS_05U modified requirements based on measurements and simulations. Companies are encouraged to provide their A-MPR values for each proposed region and the compare their results to the proposed A-MPR values. Our proposals are as follows:

Proposal 1: Change three > signs to ≥ in agreed AMPR region definition

Proposal 2: Add new regions as in Figure 1 to the NS_05U AMPR definition

Proposal 3: Define new AMPR values for changed NS_05 and NS_05U requirements according to table 1 and table 2 above

# Reference

[1] R4-2420481, “WF on PHS protection update”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[2]	R4-2500488, “(NR_newRAT-Core) n1 NS_05 and NS_05U PC3 A-MPR re-evaluations”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece, 17th – 21st February, 2025