3GPP TSG-RAN4 Meeting #114	R4-2500491

# Athens, Greece,  16th – 21st February, 2025

Title:	Initial views on RF impact for NTN IoT TDD

Source:	Qualcomm Incorporated

Agenda item:	7.31.4

Release:	Rel-19

SI: 	IoT_NTN_TDD-Core

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

WID [1] was approved with new TDD mode for NTN IoT and a new band definition for 1616-1626.5 MHz. WF [2] was agreed in RAN4#113 with some good progress in RF domain requirements. In this paper we provide our view of some key RF requirements to be addressed for this feature.

# 2. 	Discussion

## 2.1	Duplex switching time

Duplex switching time is not specified for LTE based system as it is specified for NR based in TS 38.211 clause 4.3.2. Instead it is implicitly defined e.g. in transient period timing diagrams. For NB IoT TDD this was discussed in Rel-16 and after liaison exchange, RAN4 agreed [3] that the duplex switching time for NB-IoT TDD is 40 µsec.

For this feature, since the frame design is agreed by RAN1 to be N=9, U=D=8, and index of the UL and DL slot is always the same [4] , agreement copied to Annex. This gives long time between TX and RX even with the 1200 km elevation where propagation time is 4 msec since there are three empty slots in between. This yields > 15 milliseconds for UE to switch from TX to RX. This is visualized in Figure 1. In practice the time is even longer since there are gaps surrounding the simplex timeslot.

Figure 1. Timings from UE perspective and Duplex switching time

Observation: NB IoT TDD NRX-TX is more than enough for NTN NB IoT frame structure and does not need further discussions.

## 2.2	AMPR

As written in WF [2], the AMPR work can benefit from B254 AMPR but the work for B254 should be completed. The frequencies are not exactly aligned,

[249] is 1616-1626.5 MHz and B254 is 1610-1626.5 MHz and protected regions are just below 1610 MHz and above 1628.5 MHz. The AMPR work that is dependent of the emissions falling to higher side of the bands may be re-used but the emissions on the lower side of the [249] band need to be carefully analysed.

Proposal 1: Focus on AMPR work for the lower side emissions for band [249].

In addition to the additional spurious emissions, there are additional Spectral Emission mask requirements. These can be copied from B254 work since they are Fc agnostic.

## 2.3	RMC definition for NTN IoT

New RMC’s are needed for REFSENS and output power related requirements. These should be aligned with the recent agreement in RAN1 for the frame structure and re-used from FDD NTN NB IoT for appropriate parts.

## 2.4 	Freq error

Frequency error and Doppler handling for it may need some additional work. Especially the part with the repetition segmentation seems unnecessary:

Clause 6.4B.1of TS36.102:

…

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

…

Since UL transmissions are not constant, due to TDD system nature, it makes sense to have UE update the pre-compensation between every active UL slot. There is plenty of time, as shown in Figure 1.

Proposal 2: Update frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot.

It seems unrealistic to mandate UE to maintain phase continuity between two UL slots since they are very far from each other in time. RAN4 does not specify phase continuity currently but if UE is not allowed to update the pre-compensation between the UL slots, it implies there is a requirement for some continuity between slots.

# Conclusion

We observed and proposed.

Observation: NB IoT TDD NRX-TX is more than enough for NTN NB IoT frame structure and does not need further discussions.

Proposal 1: Focus on AMPR work for the lower side emissions for band [249].

Proposal 2: Update frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot.

# Reference

[1]	RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2419795, “Way Forward for [113][317] IoT_NTN_TDD”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, 18th – 22nd November, 2024

[3]	R4-1711536, “Reply LS to R1-1715304 LS on minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs”, RAN4 (Ericsson), 3GPP TSG RAN WG2 Meeting #84bis, Dubrovnik, Croatia, 9th – 13th October 2017

[4] R1-2500003, Report of RAN1#119 meeting

# Annex: Agreement in RAN1