3GPP TSG-RAN WG4 Meeting #114	R4-2500498

# Athens, Greece, 17th - 21th, Feb 2025

Title:	Discussion on MPR enhancement for FR2 CA and draft LS

Source:	Samsung

Agenda item:	7.1.1.2.3

Document for:	Discussion

1.	Introduction

In RAN4#112bis meeting, the configuration based MPR enhancement for FR2 CA was agreed [1] and draft CR was endorsed [2]. In RAN4#113 meeting, activation based MPR enhancement was discussed and some progress was made, but there are still remaining issues [3] and the feature CR was postponed to this meeting.

In this contribution, we further discuss the open issues.

2. 	Discussion

2.1 Emission requirements

The emission requirements are still an open issue for the enhanced MPR.

Issue 3-1-1: Conditions of CC activation based enhancement Agreement in main session and WF: - For the Rel-19 activation based enhancement, FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration where all the activated UL CCs forms a contiguous block. - FFS emission requirements are activated UL CCs BW based or Cumulative aggregated channel bandwidth (CABW) based

Based on TS 38.101-2 specification, the existing emission requirements for FR2 intra-band contiguous CA are all based on UL aggregated channel bandwidth, regardless of DL aggregated channel bandwidth, i.e., CABW. It can be observed that the emission requirements are depending on configured bandwidth. Even enhanced MPR was introduced for activation status, we see no reason to change the applicable scheme for emission requirements. That is, the existing emission requirements still work without changes.

Proposal 1:	No change is needed to existing emission requirements for enhanced MPR feature.

2.2 Optional UE capabilities

It was agreed to introduce distinct enhancement capabilities for configuration-based enhancements and activation-based enhancement, further discussion is needed if the capabilities should be mutually exclusive.

Issue 3-1-2: Optional UE capability Agreement in main session and WF: - For Rel-19 FR2 UE MPR, define distinct enhancement capabilities for configuration-based enhancements and activation-based enhancement. - FFS if the capabilities should be mutually exclusive

Literally, if we discuss configuration-based enhancement and activation-based enhancement together, it looks natural to consider the configuration-based enhancement as precondition of activation-based enhancement. However, this is kind of misleading due to the term we used. The configuration-based enhancement is actually uplink dependent MPR enhancement reflecting some UE with decoupled implementations between downlink and uplink, while activation-based enhancement is activation-status dependent MPR enhancement reflecting some UE with fast tuning capability. The legacy MPR is downlink dependent and not adaptable to the activation enhancement. A summary is provided in Table 1 to show a full picture of MPR dependency in Rel-19.

Table 1. MPR dependency in Rel-19

|  | fr2-MPR-ImprovementUplinkDependent-r19 | fr2-MPR-ImprovementActivationDependent-r19 | MPR dependency |
| --- | --- | --- | --- |
| Legacy UE | No | No | Downlink configured CCs （CABW） |
| New UE in R19 | Yes | No | Uplink configured CCs |
| New UE in R19 | No | Yes | Downlink activated CCs (within CABW) |
| New UE in R19 | Yes | Yes | Uplink activated CCs |

Proposal 2:	Introduce new UE capabilities which are not mutually exclusive to reflect the status indicated in table 1.

Previously the discussion is focused on ‘configuration-based’ MPR enhancement, which looks misleading when activation based MPR enhancement is also introduced. It is suggested to call ‘uplink dependent’ MPR enhancement instead of ‘configuration-based’ MPR enhancement.

Proposal 3:	To avoid misleading, the two levels of MPR enhancements (configuration based & activation based) should be renamed as uplink dependent MPR and activation dependent MPR.

Given the new capabilities are complicated and are not so straightforward, it is better to send RAN2 LS to trigger RAN2 discussion earlier.

Proposal 4:	Send RAN2 LS to trigger the work early.

2.3 MPR applicability when bandwidth basis = 400Mhz or 800MHz

It was observed in last meeting that the applicable MPR may not be correct when bandwidth basis = 400MHz or 900MHz. The boundary is not aligned with CA BW class.

Sub-topic 3-2: Others - Proposals in R4-2418337 (Samsung) - Proposal 1: Highlight 800MHz upper limit for the enhanced MPR feature when applying MPR values in the tables. - Proposal 2: RAN4 to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz. - WF - FFS proposal 2 with the identified discrepancy of CA BW class and MPR table - FFS if 800MHz is explicitly referenced as an upper limit

Looking at the derivation when R15 MPR table was introduced, the MPR value within a certain bandwidth range is actually determined by the upper bound, one of example is shown in Figure 1 from [4]

Figure 1. MPR delta due to large BW (quoted from [4])

So the upper bound of bandwidth in MPR table “≤” should be used rather than “<”, e.g. ≤400MHz, ≤800MHz, which is also aligned the boundary in the CA BW class range definition in Table 5.3A.4-1 of TS 38.101-2. A companion CR is prepared in [5].

Proposal 5:	Correct the applicable MPR when bandwidth basis=400 MHz or 800MHz as indicated in companion CR R4-25xxxxx.

It is common understanding that 800MHz is the upper limit of uplink aggregated bandwidth so far for FR2 UL contiguous CA. Given it is already reflected in Table 5.5A.1-1 of TS 38.101-2, so it may not necessary to duplicate such restriction in both places.

Proposal 6:	800MHz upper limit of uplink aggregated bandwidth can be addressed in Table 5.5A.1-1 of TS 38.101-2

3. 	Conclusion

Proposal 1:	No change is needed to existing emission requirements for enhanced MPR feature.

Proposal 2:	Introduce new UE capabilities which are not mutually exclusive to reflect the status indicated in table 1.

Proposal 3:	To avoid misleading, the two levels of MPR enhancements (configuration based & activation based) should be renamed as uplink dependent MPR and activation dependent MPR.

Proposal 4:	Send RAN2 LS to trigger the work early.

Proposal 5:	Correct the applicable MPR when bandwidth basis=400 MHz or 800MHz as indicated in companion CR R4-2500499.

Proposal 6:	800MHz upper limit of uplink aggregated bandwidth can be addressed in Table 5.5A.1-1 of TS 38.101-2

A Draft LS is in Annex

4. 	References

- R4-2417204     “WF on power domain enhancements”, Huawei, HiSilicon

- R4-2417110     “draft CR to TS 38.101-2 for FR2 CC configuration based MPR improvement”, NTT DOCOMO, INC., vivo

- R4-2420527     “WF on power domain enhancements”, Huawei, HiSilicon

- R4-1811487    “Update FR2 CA MPR”, Qualcomm

- R4-25xxxxx    companion CR

Annex: Draft LS

Title:	Draft LS on introduction of new UE capabilities for FR2 MPR improvement

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_RF_Ph4-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

The MPR of legacy UE is based on CABW i.e., downlink dependent and configuration dependent. In Rel-19 NR_ENDC_RF_Ph4 work item, MPR was enhanced for some UE implementations. RAN4 agreed to introduce two new UE capabilities for MPR improvement in Rel-19 for FR2 intra-band contiguous CA to address different UE implementations. One is for uplink dependent MPR, the other is activation dependent MPR. The two new UE capabilities are optional and applicable since Rel-19. The applicable MPR for different UEs can be summarized in table below

|  | fr2-MPR-ImprovementUplinkDependent-r19 | fr2-MPR-ImprovementActivationDependent-r19 | MPR dependency |
| --- | --- | --- | --- |
| Legacy UE | No | No | Downlink configured CCs （CABW） |
| New UE in R19 | Yes | No | Uplink configured CCs |
| New UE in R19 | No | Yes | Downlink activated CCs (within CABW) |
| New UE in R19 | Yes | Yes | Uplink activated CCs |

RAN4 respectfully requests RAN2 to introduce the UE capabilities for FR2 intra-band contiguous CA MPR improvement.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully requests RAN2 to introduce the UE capabilities for FR2 intra-band contiguous CA MPR improvement by taking above into account.

3. Date of Next TSG WG RAN4 Meeting:

TSG-RAN4 Meeting#114-bis  	 		07 – 11 April 2025			Wuhan, China