3GPP TSG-RAN WG4 Meeting #114	R4-2500504

# Athens, Greece, 17th - 21th, Feb 2025

Title:	Discussion on testability for LP-WUR

Source:	Samsung

Agenda item:	7.26.2.4

Document for:	Discussion

1.	Introduction

Testability is one of the scopes for this work item but no much progress yet, it is important to converge on high level framework and then move forward onto details.

In this contribution, we share our views on LP-WUR testability.

2. 	Discussion

There kinds of RRC states are involved for LP-WUS feature, i.e., RRC_Idle, RRC_inactive and RRC_Connected. For requirement and test, a high level agreement is achieved in last meeting [1]:

Issue 4-1-1: alignment of common understanding on LP-WUS requirements and corresponding testing Agreements: - The same LP-WUS RF requirements for all three RRC states are assumed. But only one RRC state needs to be tested.

Only one RRC state needs to be tested. Then it is next question which RRC state is to be selected or prioritized. For FR1, supporting LP-WUS in RRC_Idle states showed most obvious power saving benefits. Though UE may support not all 3 RRC states, the large possibility is the FR1 LP-WUS UE would either support RRC_Idle or support RRC_Idle & RRC Connected states. Therefore it can be expected that test method based on RRC_Idle could cover most available UEs. It is reasonable to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

Proposal 1:	RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

High level framework has been discussed for several meeting cycles, and the latest agreements are as following [1]:

Issue 4-1-2: General framework of interface between the MR and the LR to verify RF performance Agreements: - Group discuss and conclude the general framework for LP-WUS performance verification, consider the following as starting point: - Option 1: Counter the detection rate without waking up the MR would be enough to verify LP-WUS RF performance. - FFS if the MR should only be woken up when data transfer the system simulator is needed. - Other options are not precluded.

There have been sufficient discussions on this issue and it can be concluded that waking up MR for each detected LP-WUS is not practical for conformance test. It is more practical to just counting the detected LP-WUS signal and not to wake up the MR before the test counting is over. It is proposed to agree on this high level framework and then move forward.

Proposal 2:	RAN4 to counter the detection rate without waking up the MR to verify LP-WUS RF performance.

Following to the high level framework in proposal 2, a test mode is needed to enable the testing without waking up MR when LR receives LP-WUS signal.

There is also proposal to leave the discussion to RAN5. In our understanding, without conclusion on test mode in RAN4, the testability objectives can not be completed for this work item since this is the first step of testability. In addition, there is risk of further delay to enable this feature if RAN4 leave too much testability to RAN5, the debates would occur in RAN5 again later. There is similar case in Rel-18 beam correspondence for initial access which also testability objective, now RAN5 progress is still slow due to limited RAN4 solid outcome on testability.

As a grand new feature it is better for RAN4 to fully study and complete the testability aspects rather than to leave to RAN5.

Proposal 3:	RAN4 to fully study and complete the testability aspects rather than to leave to RAN5.

3. 	Conclusion

Proposal 1:	RAN4 to prioritize RRC_Idle when developing test method for FR1 UE LP-WUR RF testing.

Proposal 2:	RAN4 to counter the detection rate without waking up the MR to verify LP-WUS RF performance.

Proposal 3:	RAN4 to fully study and complete the testability aspects rather than to leave to RAN5.

4. 	References

- R4-2420528    “WF on UE RF requirements for LP-WUS”, vivo