3GPP TSG-RAN WG4 Meeting #114	R4-2500506

# Athens, Greece, 17th - 21th, Feb 2025

Title:	On performance metric for FR1 dynamic MIMO

Source:	Samsung

Agenda item:	7.14.2.3

Document for:	Discussion

1.	Introduction

In the latest TR 38.762 v0.1.0 [1], the Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e., UMi and Uma, while it is further mentioned that whether and how to combine the CTMT metrics for different each device orientations, positions/rotations, routes/channel models is FFS.

Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e., TMTUMi=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMi and TMTUMa=TP1(t1),..., TP1(tN), ...,TPm(t1),..., TPm(tN),..., TPM(t1),..., TPM(tN)UMa Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF CTMTUMi,10%=CDF10%TMTUMi CTMTUMi,50%=CDF50%TMTUMi CTMTUMi,90%=CDF90%(TMTUMi) CTMTUMa,10%=CDF10%TMTUMa CTMTUMa,50%=CDF50%TMTUMa CTMTUMa,90%=CDF90%(TMTUMa) Whether and how to combine the CTMT metrics for different each device orientations, positions/rotations, routes/channel models is FFS.

In recent RAN4 meetings, whether combining both Umi and Uma channel models into a single data pool is also under discussion.

In this contribution, we share our views on above open items.

2. 	Discussion

2.1 CTMT performance metric

Regarding “Whether and how to combine the CTMT metrics for different each device orientations, positions/rotations, routes/channel models”, this FFS item is partially related with CTIA requirement [2] which requires separate CTMT recording and reporting for each device orientation and rotation. So in CTIA there will be 3 percentile point * 4 device orientations * 4 positions/rotations = 48 CTMT values recorded and reported for a single channel model, and it is not determined how to apply a pass/fail criteria for these 48 CTMT values. In our understanding, a requirement for different each orientation and rotation will lead to a performance metric determined by worst-case performance scenario and may not reflect a UE’s average performance.

Observation 1:	a requirement for different each orientation and rotation will lead to a performance metric determined by worst-case performance scenario and may not reflect a UE’s average performance.

Now 3 percentile points are introduced, i.e., the 10%, 50%, and 90%-ile CDF. It is possible that a UE showing best performance at 90%-ile may not show best performance at 50% and 10%-ile, and vice versa. In usual OTA metric, average performance is adopted as performance metric when handling low/middle/high channels, or consider both average and minimum performance. To some extent, the 10%-tile CDF reflects the minimum value and the 50%-tile CDF reflects the average value. The 90%-tile reflects the best cases which is usually not considered in previous OTA requirements.

Observation 2:	To some extent, the 10%-tile CDF reflects the minimum value and the 50%-tile CDF reflects the average value. The 90%-tile reflects the best cases which is usually not considered in previous OTA requirements.

Based on above observations, it is suggested not to specify the CTMT requirements for different each orientation and rotation, and further discuss if 90%-tile CDF is suitable for specifying minimum requirements.

Proposal 1:	RAN4 not to specify the CTMT requirements for different each orientation and rotation, and further discuss if 90%-tile CDF is suitable for specifying minimum requirements.

2.2 Uma and Umi

The agreements about combining Uma and Umi channel models are collected as below [3][4]:

Agreements: (RAN4#112bis) - Consider dynamic channel MIMO OTA tests to include both UMa and UMi channels with tests on UMa and UMi channels carried out separately and their measurement results combined into a single data pool for processing. Agreement: (RAN4#113) - For discussing performance metric perspective, further discuss whether the measurement results of UMi and UMa should be combined. - Down-selection of a single channel model for performance requirements is not precluded.

If the final CTMT requirements are to be specified for different each orientation and rotation, it seems not so meaningful to combine Umi and Uma. It is necessary to determine “Whether and how to combine the CTMT metrics for different each device orientations, positions/rotations” firstly and then decide if combining Umi and Uma into a single data pool.

Proposal 2:	RAN4 to make decision on “Whether and how to combine the CTMT metrics for different each device orientations, positions/rotations” firstly and then decide if combing Umi and Uma into a single data pool.

3. 	Conclusion

Observation 1:	a requirement for different each orientation and rotation will lead to a performance metric determined by worst-case performance scenario and may not reflect a UE’s average performance.

Observation 2:	To some extent, the 10%-tile CDF reflects the minimum value and the 50%-tile CDF reflects the average value. The 90%-tile reflects the best cases which is usually not considered in previous OTA requirements.

Proposal 1:	RAN4 not to specify the CTMT requirements for different each orientation and rotation, and further discuss if 90%-tile CDF is suitable for specifying minimum requirements.

Proposal 2:	RAN4 to make decision on “Whether and how to combine the CTMT metrics for different each device orientations, positions/rotations” firstly and then decide if combing Umi and Uma into a single data pool.

4. 	References

- R4-2416433    3GPP TR 38.762 v0.1.0 (2024-10)

- CTIA 01.42

- R4-2416579    “WF for [112bis][324] MIMO_OTA_Ph3”, CAICT

- R4-2419910    “WF for [113][326] MIMO_OTA_Ph3”, CAICT, vivo