3GPP TSG-RAN WG4 Meeting #114	R4-2500511

Athens, Greece, 17th – 21th February, 2025

Agenda Item:	7.19.1

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we discuss following issues from last meeting:

- Issue: Post-deployment handling

- Issue: Delay requirements for AI to non-AI transition

2	Discussion

2.1 Issue: Post-deployment handling

In last meeting, a comprehensive summary on post deployment issue has been proposed by moderator [1]. A list of issues were captured in the summary. In this paper, we present our general view on post-deployment handling first, and then give initial thinkings on the issues.

List of issues proposed in moderator summary: - Whether to keep option 1 and/or option 2 - When to use option 1 or option 2 - Where validation/testing for option 1 is carried out - How to design option 1 - How to design option 2 - Proposals for new option 3 - Interaction with other WGs

In RAN4 #112bis, two aspects that would impact performance are identified:

- Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance

- Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

Regarding to the 1st aspect, it can be further divided into,

- update or fine-tuning: conducting conformance tests/validation for all potential models or updates would induce significant testing workload, of which the feasibility and necessity are both questionable. Conformance test or validation for major update is needed before loading to on-site devices to ensure the performance meeting requirements. Criteria for distinction of major and minor update might include the scope of changes introduced, the impact on hardware capability, the extent of engineering required, and the potential risks involved. Specifying clear rules is preferred to help in planning and managing tests more effectively.

The performance of minor update or fine-tuning could be guaranteed by device vendors via sufficient offline tests before loading to on-site devices. Certain efforts are required to boost the reliability of offline tests. Establishing a dataset that can cover as many scenarios/conditions that are likely to be encountered in real deployment as possible would be a way out. The source and the composition of the dataset could be, for example, collected from previous conformance tests, or through public datasets in the industry, or collected internally by device vendors.

We understand, either conformance test for major update or offline tests for minor update, is still within the effort of pre-deployment. Performance monitoring and LCM is indispensable to check whether the functionality/model works normally in the field after being deployed. The effectiveness of monitoring and LCM is required for timely reaction to varied conditions in the field to avoid severe performance degradation, which needs to be guaranteed via particular requirements and tests. Additionally, RAN4 #111 agreed that UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode. So, fallback to non-AI mode can be taken as baseline mechanism for post-deployment handling.

- Addition/removal of model: if addition/removal of models indicates the enable/disable of applicable functionality, this relates to the discussion of “LCM for UE-sided model” in RAN2. This apparently shall be addressed within the scope of performance monitoring and LCM.

Regarding to the 2nd aspect, data drift/mismatch between field conditions and training data is theoretically inevitable because the field condition is time-varying and statistical features cannot be exhausted in the tests. Even though model validations are carried on before deployment, certain procedures are still needed to check how the functionalities/models actually operate in the field after being deployed, which we think can only be handled via performance monitoring and LCM. In our view, critical efforts deployment would be to introduce field data in training stage to minimize the impact of data drift/mismatch.

Observation 1: For update or fine-tuning of models, conformance tests could be conducted for major update, and offline tests could be conducted for minor update or fine-tuning.

Observation 2: Even with conformance tests or offline tests, Performance monitoring and LCM is indispensable to check whether the functionality/model works normally in the field after being deployed.

Observation 3: If addition/removal of models indicates the enable/disable of applicable functionality, this relates to the discussion of “LCM for UE-sided model” in RAN2, which could be addressed via performance monitoring and LCM.

Observation 4: For the aspect of data drift/mismatch, validation before deployment cannot fully address the potential performance degradation. Performance monitoring and LCM is needed to check how the functionalities/models actually operate in the field after being deployed. Introducing field data into training stage is critical to minimize the impact brought by the data drift/mismatch.

Accordingly, some initial thinkings are given to the issues brought up in last meeting:

- Whether to keep option 1 and/or option 2

- When to use option 1 or option 2

- Where validation/testing for option 1 is carried out

Issue 1~3 could be discussed in a bundle. We suggest to prioritize option 2 (performance monitoring and LCM procedures). RAN4 should also aim to specify requirements and tests for option 2 to guarantee the effectiveness.  For option 1, in our understanding, conducting validations in lab functions nearly the same as conformance tests, which we think needs to done for major update or new version release. If conducting validation in field means validating the model with field data, it could be realized by device vendor in offline manner, no need to be a formal procedure. If it means verifying the model through real operation in the field, the procedure is same as performance monitoring and LCM.

- How to design option 1

Identify the necessity of option 1 first before further discussion on details.

- How to design option 2

Detailed designs on performance monitoring and LCM shall be specified in other WGs. RAN4 is to specify requirements and tests for monitoring and LCM to guarantee the effectiveness.

- Proposals for new option 3

Pending the discussion.

- Interaction with other WGs

We are open for further discussion.

Proposal 1: Prioritize option 2 (performance monitoring and LCM) for post-deployment handling. RAN4 to specify requirements and tests on monitoring and LCM to guarantee the effectiveness.

Proposal 2: Suggest to discuss feasibility and potential solutions of building dataset with field data for both offline performance tests and mitigating the impact of data drift/mismatch.

2.2 Issue: Delay requirement for AI to non-AI transition

Relevant agreements in RAN2 are excerpted as below.

Agreement (RAN2 #125bis): •For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). •“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19 Agreement (RAN2 #126): •For BM use case, As a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive Agreement (RAN2 #127bis): •UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) Agreement (RAN2 #128): •When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Based on RAN2 progress, the case of “UE-autonomous without reporting to NW” is precluded. Remaining possible cases are:

- NW-decision and NW-initiated: delay requirement of this case shall be considered as the duration from UE receiving fallback command to UE being ready to starting transmission/reception with non-AI configuration.

- NW-decision and UE-initiated: the difference from 1st case is, the LCM request is initiated by UE and NW responses to the request based on own inner decision. The LCM request may be in form of monitoring output at the UE. Thus, delay requirement of this case shall be considered as the duration from UE request to UE being ready to starting transmission/reception with non-AI configuration.

- UE-decision, reporting and NW-initiated: monitoring result is reported by UE for this case. It has not decided whether the reporting is event-triggered or not. If so, an additional component of delay uncertainty from when the UE receives event configuration until a condition exists at the measurement reference point which will trigger the reporting, shall be incorporated in the delay requirement. If not, it would be analogous to 2nd case.

For beam management, RAN1 agreed to support Type-1 performance monitoring and FFS Type-2. According to definition in TR 38.843, Type-1 includes aforementioned case 1 and 3.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

For position case 1, RAN1 agreed to support option A and FFS option B. Option A may include aforementioned case 2 and 3 since the content of monitoring outcome has not been decided. Position case 3a/3b are both NW-side model which is out the scope of the discussion. Position case 2a/2b have been treated as 2nd priority without sufficient progress on monitoring discussion. Overall, all 3 possible cases currently are still on table for further discussion.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: • Option A. The target UE side performs monitoring metric calculation. o The target UE may signal the monitoring outcome to the LMF. o FFS: content of monitoring outcome • FFS: Option B

Observation 5: For beam management, RAN1 agreed to support Type-1 performance monitoring, which includes case 1 (NW-decision and NW-initiated) and 3 (UE-decision, reporting and NW-initiated).

Observation 6: For positioning case 1, RAN1 agreed to support option A of performance monitoring, which includes case 2 (NW-decision and UE-initiated) and 3.

Proposal 3: Suggest to consider delay requirement of AI fallback to non-AI, including:

- NW-decision and NW-initiated

- NW-decision and UE-initiated

- UE-decision, reporting and NW-initiated

Conclusion

In this contribution, the issue of post-deployment and delay requirement of AI to non-AI transition are discussed with following observations and proposals:

Observation 1: For update or fine-tuning of models, conformance tests could be conducted for major update, and offline tests could be conducted for minor update or fine-tuning.

Observation 2: Even with conformance tests or offline tests, Performance monitoring and LCM is indispensable to check whether the functionality/model works normally in the field after being deployed.

Observation 3: If addition/removal of models indicates the enable/disable of applicable functionality, this relates to the discussion of “LCM for UE-sided model” in RAN2, which could be addressed via performance monitoring and LCM.

Observation 4: For the aspect of data drift/mismatch, validation before deployment cannot fully address the potential performance degradation. Performance monitoring and LCM is needed to check how the functionalities/models actually operate in the field after being deployed. Introducing field data into training stage is critical to minimize the impact brought by the data drift/mismatch.

Proposal 1: Prioritize option 2 (performance monitoring and LCM) for post-deployment handling. RAN4 to specify requirements and tests on monitoring and LCM to guarantee the effectiveness.

Proposal 2: Suggest to discuss feasibility and potential solutions of building dataset with field data for both offline performance tests and mitigating the impact of data drift/mismatch.

Observation 5: For beam management, RAN1 agreed to support Type-1 performance monitoring, which includes case 1 (NW-decision and NW-initiated) and 3 (UE-decision, reporting and NW-initiated).

Observation 6: For positioning case 1, RAN1 agreed to support option A of performance monitoring, which includes case 2 (NW-decision and UE-initiated) and 3.

Proposal 3: Suggest to consider delay requirement of AI fallback to non-AI, including:

- NW-decision and NW-initiated

- NW-decision and UE-initiated

- UE-decision, reporting and NW-initiated

Reference

[1] R4-2418152, Topic summary for [113][130] NR_AIML_air_part2, Moderator (Ericsson)